Tactical Approaches

Tactical Approach Guide Notes & Disclaimers:

All of these approaches assume an execution altitude of 500’ AGL.

If executing "turning" approaches from higher AGL altitudes, then plan to extend downwind segments to give a long enough final to intercept the desired glidepath. For Straight-Ins, plan to increase slowdown distance by

1 NM for every 500’ increase of altitude.

Variations are authorized!  Refer to Vol 25 for more ideas or get with an Instructor…   These examples are provided as a solid starting point; feel free to tweak the approaches.  Approach timing math is listed in this guide to help you calculate your own variations (higher entry airspeeds, longer finals, etc).

Wind analysis is EVERYTHING!  Don’t rely on these no-wind calculations exclusively, winds can and will affect these numbers significantly…

Calculating in KTAS (vs KIAS) keeps the playing field level in terms of density altitude, temp, etc…   Doing the math ahead of time in terms of TAS will save you from unwanted surprises downstream!  (go-arounds, etc)

What is the "right" length to use for a final approach?  Often 0.5 NM is referenced, but that distance leaves little-to-no room to make inevitable corrections once rolled out on final.  Using a 1.0 NM final is more conservative and comfortable, but leaves one exposed for a longer time in a vulnerable position.  Most experienced aviators will agree that planning for a 0.75 NM final is the best compromise of threat exposure and wiggle-room availability.  Try several different distances and determine what is most comfortable for you.  Keep in mind crew experience as well as threat level!

How aggressive should the approach be?  After all, why limit bank angle to 45˚ when one could use 60˚?  In line with the considerations of how long final should be, bank angle in maneuver execution is a function of both limiting threat exposure balanced with crew capability…  Keep in mind how bank angle can affect MWS coverage – for more info on that, reference Vol 25 or see an Instructor.  Smaller bank angles slow the approach pacing down, but also yield a larger overall groundtrack through maneuver execution.  Both of those factors increase threat exposure.  On the other hand, 60˚AOB turns can be pretty sporty at 500’ AGL and require very precise entry parameter calculations – much like a short final, using max bank angles leaves little room for corrections as the maneuver progresses.  Most experienced aviators will agree that 45˚AOB is generally the best "happy medium".

*** This product is both a compilation of many separate peoples’ work along with my own 2¢.  Thanks to those other folks and their great work! ***

1.0

2.0

4.0

3.0

5.0

Flaps 100%

Gear Down

Max Effort Threshold

Flaps 50%

250 KTAS

Slowdown

Favorable Winds

210 KTAS

Slowdown

Tactical Straight-In

- Suggested Techniques

(Information below is based on a 5º glidepath final)

  - Establish desired entry altitude and airspeed

  - Determine the required slowdown distance to
slowdown based on airspeed and altitude

    - Use 0.25 NM per 10 KTAS to determine 
slowdown distance based on airspeed

      - 250T to 110T = 140 knots

      - 140 ÷ 10 = 14

      - 14 x 0.25 NM = 3.5 NM required for slowdown

plus 1.0 NM final equals 4.5 NM total

    - Determine the distance required from the appr end to 
intercept an acceptable glidepath

      - A 3º glidepath = 300 ft/NM

      - 500 ft ÷ 300 ft/NM = 1.67 NM

      - 500 ft per NM = 5º glide path

    - A slowdown from 250 KTAS to intercept a 5º 
glidepath requires 4.5 NM (see Level Slowdown charts)

  - Execute a level slowdown and configure on speed...

    - Flaps 50% by 2.5 NM

    - Gear down by 2.0 NM

    - Flaps 100% by 1.5 NM

    - Threshold speed by 1.0 NM

| Intercept glidepath... | Avg Speed | Dist (NM) | Time (Sec) |
| --- | --- | --- | --- |
| A-B | 195 | 0.9 | 16" |
| B-C | 155 | 0.3 | 6" |
| C-D | 140 | 0.9 | 24" |
| D-E | 120 | 0.75 | 23" |
| Total from Break: 1 min + 10" |  |  |  |

½ - 1 NM  Final

- Considerations

  - Deceleration factor based on heavy-weight aircraft

  - Increase slowdown distance 1 NM per 500 feet
of increased altitude

  - Increase SD .15 NM per 10 knots of tailwind

  - If planning with KIAS is preferred,

use SMOE x .25 NM as deceleration factor per 10 KIAS

3.    Approach Timing Considerations

Average Speed during slowdown (250T -> 110T) = 180T (3 NM/min)

- 3.5 NM at 180T ≈ 1.2 minutes

- 1.0 NM at 110T ≈ 0.6 minutes

Total for 4.5 NM Slowdown through Landing ≈ 1.8 minutes

-->  Turn radius at 195 kts ~ 3500'

-->  Turn radius at 140 kts ~ 1800'

#### Tactical Beam

- Suggested Techniques

(Calculations below are based on 220 KTAS initial entry airspeed)

  - Enter at 220 KTAS perpendicular to the LZ

  - Cross the runway at least 1500 to 2000 feet from the approach end

  - Delay break up to 1 - 2 seconds depending on the winds!

  - Break left/right using 45º AOB and reduce power to flight idle

  - Extend flaps to 50% on speed

  - Expect to roll out with approximately 3 – 4000 ft spacing at about 160 KIAS

  - Fly a drift-killed heading to parallel final and extend gear as speed permits

  - Roll off the perch approx 5-6 seconds after rolling out of break turn onto downwind

  - Expect to roll off the perch with the gear in transit and not more the 150 KIAS

  - Use 45º AOB in the final turn and maintain 140 KIAS (or approach speed)

  - Rolling out on final, extend flaps to 100% and slow to threshold speed

  - Expect about a ½ - 1 NM final depending on winds.  Expect to descend in the final turn (once you consider yourself established on your desired glidepath profile)

- Considerations

A break turn using 30º AOB requires a 30º AOB turn to final (downwind lateral offset from LZ centerline should be approx 1 NM)

# Final Turn

45º AOB

140 KIAS or approach speed

Descend in turn to meet glidepath

  - Plan CAREFULLY for undershooting / overshooting winds…

# Final

Flaps 100%

Max Effort Threshold

###### Perch Point

5 – 6 sec after break turn rollout

150 KIAS

Flaps 50%

Gear in transit

3 – 4000 ft

C

D

B

###### 220 KIAS

E

###### Break Point

1 – 2" past LZ (winds!)

Use 45º AOB

Power to flight idle

Flaps 50% on speed

A

Tactical Teardrop

- Suggested Techniques

(Information below is based on the more conservative 1.5 DTG Teardrop Break and

220 KTAS initial entry airspeed)

  - Enter at 220 KTAS

  - Initiate a break at 1.5 NM DTG from the approach end of the landing zone

  - Break left/right using 30˚ AOB and reduce power to Flight Idle

  - Roll out on a 30˚ offset from ingress heading (which is opposite from final course)

  - Extend flaps to 50% on speed

  - Abeam the approach end of the landing runway, turn to parallel final...

  - Expect to turn to parallel with approximately 5,000 feet / 0.8 NM lateral spacing

  - Fly a drift killed heading to parallel final and extend gear as speed permits

  - Roll off the perch 15 - 17 seconds after passing abeam the approach end

  - Expect to roll off the perch with the gear in transit and not more than 150 KIAS

  - Use 30 - 45˚ AOB in the final turn (5000 ft offset spacing requires ~35˚ AOB)

  - Maintain 140 KIAS / approach speed until established on final

  - Rolling out on final, extend flaps to 100% and slow to threshold speed

  - Plan for a 0.75 NM final; expect descent in the final turn to meet desired glidepath

- Consideration:  Plan CAREFULLY for undershooting / overshooting winds…

###### Perch Point

15-17" after abeam

150 KIAS

Flaps 50%

Gear in transit

# Turn to Parallel

Abeam approach end

Gear down speed permitting

Expect 5000ft / 0.8NM spacing

|  | Avg Speed | Dist (NM) | Time (Sec) |
| --- | --- | --- | --- |
| A-B | 195 | 1.7 | 30" |
| B-C | 155 | 0.75 | 17" |
| C-D | 140 | 1.3 | 34" |
| D-E | 120 | 0.75 | 23" |
| Total from Break: 1 min + 44" |  |  |  |

C

B

# Final Turn

30º - 45º AOB

140 KIAS or appr  speed

~ 5000 ft

E

D

30º

A

# 0.75 NM Final

Flaps 100%

Max Effort Threshold

~1.5 NM

###### Break Point

1.5 NM from app end

Use 30˚ AOB

Establish 30˚ hdg offset

Power to flight idle

Flaps 50% on speed

***  Initiating break turn at 1.5 DTG will give ~5000’ lateral offset from LZ centerline and requires ~35˚ AOB turn-to-final

***  Initiating break turn at 1.0 DTG will give ~3600’ lateral offset from LZ centerline and requires ~45˚ AOB turn-to-final

Teardrop Variants   --  " The CLUB "

- Suggested Techniques

(Timing calculations below are based on 220 KTAS initial entry airspeed)

  - Enter maneuver on extended runway centerline & opposite direction of final…

  - Break left/right using 45˚ AOB once crossing the approach end of the LZ, cut power to Flight Idle

  - Hold 45˚AOB turn until offset 60˚ from initial ingress heading…

  - Upon reaching 60˚ heading offset, turn opposite direction using 45˚AOB

  - Configure as speed throughout maneuver permits, and complete checklists…

  - Expect to roll off the perch with the gear in transit and not more than 150 KIAS

  - Maintain 140 KIAS / approach speed until established on final

  - Rolling out on final, extend flaps to 100% and slow to threshold speed

  - Plan for a ~0.75 NM final; expect descent in the final turn to meet desired glidepath

Consideration:  Plan CAREFULLY for undershooting / overshooting winds…

# Final Turn

45º AOB

140 KIAS or   approach speed

###### Perch Point

~150 KIAS

Flaps 50%

Gear in transit

~3600 ft

E

D

# Switching Directions….

Make it a smooth, controlled                                       roll to opposite direction!

Turn should take 4-6 seconds…

60º

C

B

###### Break Point

At Approach End…

Use 45˚ AOB

Establish 60˚ hdg offset

Power to flight idle

Configure on speed

A

|  | Avg Speed | Dist (NM) | Time (Sec) |
| --- | --- | --- | --- |
| A-B | 200 | 0.6 | 11" |
| B-C | 180 | 0.3 | 6" |
| C-D | 160 | 0.4 | 5" |
| D-E | 140 | 0.9 | 24" |
| E-A | 120 | 0.75 | 23" |

Total from Break:  1 min + 9”

# Initial Entry….

Other Teardrop Variants

45º

Variations of teardrop:

A normal downwind entry will set you up for the same base turn…

A turning or straight ahead slowdown may be used…

220 KTAS

SLOWDOWN

2 NM

1 NM

BREAK TURN

220 KIAS

SLOWDOWN

APPROX 1.0 NM

FROM THRESHOLD

PERCH

150 KIAS,

FLAPS 50,

GEAR IN TRANSIT

0.75–1.0

NM FINAL

~ 4000 FT

BASE TURN

45˚ AOB

140 KIAS or APPR SPEED

Note: Distance shown is to the perch point.  Offset ~4000 feet from LZ, abeam 0.75-1.0 NM final.  Perch at 140 KIAS

~ 4000 FT

Note: Distance shown is to the threshold.

Assuming ~150 kts, timing for 15-18” past abeam approach end will give ~ 0.75 mile final

How do you get DTG and determine lateral offset?

Use SCNS!  Put the approach end coords in the RENDEVOUZ page for DTG and use your SCNS final course   crosstrack to determine lateral offset

Tactical Steep Considerations

- PLAN at least 4000' AGL

  - Consider all surrounding terrain, not just runway elevation

- Based on experience…

  - Plan to lose ~1000' of altitude per 90 degrees of turn

-  At 130K GW, ~1250' per 90 (30º AOB) or ~ 750' (45º AOB)

- Plan 25-30 seconds per 90º turn (45º AOB)

- 2250 - 2500 fpm descent rate (level or turning)

- Timing - compare:

-  4 turns at 25 sec each = 1 min 40 sec

-  4000 feet at 2250-2500 fpm = 1 min 40 sec

- COMMON ERRORS

  - Ground references from 4000' / 8000' / 12000', etc AGL?

  - Not retarding throttles to flight idle during descent

  - Airspeed/bank angle control in descent

  - Failure to account for winds

  - Failure to adjust final turn for altitude or position

- NOT RECOMMENDED!

  - Slipping aircraft to lose altitude/airspeed

  - Excessive bank angle, rudder turns

  - Flying less than approach speed

- OTHER CONSIDERATIONS

  - Standard turn rate/radius charts are based on level turns and do not apply to tactical steep approaches

  - 45˚ AOB descending turn compares to 30˚ AOB level turn

  - Use other planning tools based on experience

  - Greatest descent rates are for light-weight aircraft

  - Energy dissipation problem -- heavy aircraft can’t lower the nose as far without overspeeding flaps, thus require a lower descent rate

  - Descent rates are about the same for level or turning

  - Airspeed is the limiting factor (145 KIAS max)

________________________________________________________________________________________________________________________________________________________

SAFELY MANEUVERING AT MAX EFFORT THRESHOLD SPEED ?!? …  Yes!

- Worst case - max GW 130,000 lbs assault landing:

50% flap speeds are 143, 133, 115 KIAS

Max Effort speeds are 128, 118, 108 KIAS

50% flap, 45˚ AOB stall speed is 118 KIAS

100% flap, 45˚ AOB stall speed is 108 KIAS

- Threat reactions at Max effort threshold speed...

Same as 50% flap, 45˚ AOB stall speed (power off)

10 kts above on 100%, 45˚ AOB stall (power off)

- So, at 100% flaps, using 30º AOB and power ON to execute threat reactions gives a three-way pad on stall speed

- Bottom line - You can safely maneuver at threshold speed!

Tactical Steep Execution

- Suggested Techniques

  - Enter initial at an altitude approximately 4,000 feet above field elevation (as a min)

  - Slow to 140 KIAS and configure with flaps 50% and gear down at approx 4 NM

  - Initiate break at 1.0 – 0.5 NM from the approach end (depending on winds)

  - Simultaneously reduce power to flight idle and track flaps to 100%

  - Let the nose to drop to maintain 140 KIAS and break left/right using 30˚ AOB (45˚ AOB is max).  As a general rule, look for 8-10˚ nose-low and ~2500 VVI…

  - Ensure power is set at zero torque (out of NTS) and use pitch to maintain 140 KIAS

  - Plan to make a continuous turn to final and cross check altitude through every 90˚

  - Expect to lose about 1000 feet per 90˚ of turn

  - Roll out on final and slow to approach speed

  - Expect ~ 0.5 – 0.75 NM final and to roll out on a steep glideslope (≥5˚)

- Consideration:

  - Plan CAREFULLY for undershooting / overshooting winds, and consider the difference between winds at altitude and winds on the surface….

  - To help remain unpredictable, consider Figure-8s etc when executing from higher altitudes

BEGIN

SLOWDOWN

~4NM

PERCH

~2000 AGL

0.75 NM FINAL

~ 6000 FT

BREAK

140 KIAS

FLAPS 100%

FLIGHT IDLE

30˚ AOB

ENTRY

FROM 4000

or

8000, 12000, etc.

above LZ

TACTICAL STEEP

OPPOSITE

DIRECTION

7000 AGL

FAVORABLE

WINDS

FAVORABLE

WINDS

“Minimum Spacing” LZ Operations

- The MATH

  - Stick with feet and seconds for units

  - Knots x 1.69 = feet per second

  - When slowing (straight ahead or turning):

(Starting speed + ending speed) divided by 2 = average speed

Level deceleration ~ 0.25 NM per 10 KTAS lost

- 90 degree turn:

Distance (feet) = 1.57 multiplied by radius in feet (1.57 = ½ Pi)

Time (sec) = distance (ft) divided by speed (feet per sec)

or... 90˚ divided by turn rate (degrees per sec)

- LANDING / GROUND OPERATIONS

  - Landing Roll-out:

Touchdown ~100 KIAS to rollout 20 KIAS = 65 KIAS average

(Assuming pilot doesn’t stomp on the brakes)

65 x 1.69 ≈ 100 feet per second

3500 ft gives you 35 seconds, and 5000 ft gives you 50 seconds

- 90˚ turn ≈ 10 seconds

- 180˚ turn ≈ 25 seconds

- Taxi Ops:

Down the LZ, 25 knots ≈ 40 fps

Down the taxiway, 15 knots ≈ 25 fps

- Landing to clear at THE END:

35 sec for 3500 ft LZ

10 sec for 90˚ turn

+ 10  sec to clear the LZ

=  55 sec (round up to use 1 minute spacing for planning!)

- Add 15 seconds for a 5000 ft LZ…

(Longer than that, and you can probably do a formation landing)

- Landing with back taxi to clear at:

MIDFIELD             APPROACH END

3500 ft LZ:                         35 sec                          35 sec

180˚ turn:                           25 sec                          25 sec

Back taxi at 25 knots:        40 sec (1500 ft)           100 sec (3500 ft)

90˚ turn:                             10 sec                          10 sec

Clear the LZ        :           + 10  sec                      + 10 sec

= 120 sec (2 min)        = 180 sec (3 min)

  - Add 15 seconds for a 5000 ft LZ…

(Longer than that, and you can probably do a formation landing)

TURN RADIUS from TRUE AIRSPEED

20

40

30

25

35

45

50

55

60

* For checklist versions of both this diagram and more, reference local 36 AS drive M:\Tactics\Gouge and Study Materials\Hydro's Gouge\Wally's Gouge...  "Checklist Rate and Turn Guide"