1.  During the aircrew briefing, the aircraft commander must advise the loadmaster when the mission profile requires flight below 800 feet AGL with the door open.

a.  True

b.  False

a

55-130, Vol 2, 3.9.2, Warning

2.  For equipment airdrop, the aircraft must be level at drop altitude and on drop airspeed by:

a.  The “ONE MINUTE ADVISORY”

b.  Green light time

c.  The “FIVE SECOND ” call

d.  There is no specified time

b

55-130, Vol 2, 3.14.4

3.  If an ascending slowdown is required, climb at:

a.  Enroute airspeed

b.  130 KIAS

c.  140 KIAS

d.  The appropriate speed for the airdrop being accomplished

c

55-130, Vol 2, 3.14.5

4.  No Drop decisions:

a.  Are made by any crewmember after the “ONE MINUTE ADVISORY”

b.  Are made by any crewmember after the “FIVE SECOND ” call

c.  Must be acknowledged by the copilot and loadmaster

d.  a & c

e.  b & c

d

55-130, Vol 2, 3.18.1

5.  A “NO DROP” will be called if the slowdown checklist is not complete by the “ONE MINUTE ADVISORY”.

a.  True

b.  False

b

55-130, Vol 2, 3.18.1, Note

6.  If surface winds are unknown, advise the jumpmaster and Army airborne mission commander when drop altitude winds exceed:

a.  17 knots

b.  25 knots

c.  30 knots

d.  150 knots

c

55-130, Vol 2, 3.18.2

7.  Which of the following is not true concerning Race Track Procedures?

a.  Race Tracks are not recommended for combat operations

b.  Accomplish all checklists during the Race Track

c.  Checklists may be compressed, except for the “ONE MINUTE ADVISORY”

d.  All of the above are true

d

55-130, Vol 2, 3.20

8.  For a Verbally Initiated Release System (VIRS) airdrop,:

a.  “STANDBY” indicates approximately 5 seconds prior to the release point

b.  “STANDBY” indicates approximately 10 seconds prior to the release point

c.  “STANDBY” indicates approximately 1 minute prior to the release point

d.  “STANDBY” indicates you may drop on the Navigator’s CARP when ready

a

55-130, Vol 2, 3.24.2.3

9.  During jumpmaster-directed airdrops,:

a.  The jumpmaster begins spotting procedures one minute out

b.  The jumpmaster begins spotting procedures two minutes out

c.  The copilot turns on the green light at the “FIVE SECOND” call

d.  The lead aircraft relays jumpmaster “NO DROP” decisions to the formation

b

55-130, Vol 2, 3.25.4

10.  High altitude drop procedures:

a.  Are used when a small arms threat precludes conventional low altitude operations

b.  Are used when the drop altitude is above 5,000 feet AGL

c.  Are limited to 18,999 feet MSL if all of the personnel on the aircraft have not accomplished the 	   	     appropriate physiological training

d.  None of the above

a

55-130, Vol 2, 3.31

11.  Pre-breathing is required when un-pressurized flight above _____ will be flown.

a.  18,999 feet MSL

b.  18,000 feet AGL

c.  FL 250

d.  18,000 feet MSL

d

55-130, Vol 2, 3.33.2

12.  Quick don masks and emergency equipment are approved for pre-breathing procedures.

a.  True

b.  False

b

55-130, Vol 2, 3.33.5, Note

13.  Ramp and Door paratroop drops are limited to ____ paratroopers per pass.

a.  20

b.  10

c.  40, if both anchor cables are used

d.  There is no restriction to the number of paratroopers

a

55-130, Vol 2, 3.41.3

14.  The jumpmaster must be informed when the point of impact falls within ____ of any DZ boundary.

a.  200 yds

b.  100 yds

c.  150 yds

d.  150 feet

c

55-130, Vol 2, 3.42.3

15.  Door Bundles may be as heavy as ____ , excluding the weight of the parachutes.

a.  5,000 lbs

b.  100 lbs

c.  500 lbs

d.  1,000 lbs

c

55-130, Vol 2, 3.44.1

16.  The max airdrop load to be extracted over the ramp is ____ for C-130 E/H airplanes 62-1784 and up.

a.  25,000 lbs

b.  At the discretion of the user, not to exceed CG limits

c.  42,000 lbs

d.  10,000 lbs

c

55-130, Vol 2, 3.45

17.  CDS airdrops at less than 104,000 lbs gross weight are not recommended.

a.  True

b.  False

a

55-130, Vol 2, 3.46.2

18.  Which of the following are true concerning Combat Rubber Raiding Craft (CRRC) Airdrops?

a.  CRRC’s are released using the standard CDS checklist, to include emergency procedures

b.  2 CRRC’s and 18 paratroopers or 1 CRRC and 19 paratroopers may be dropped on one pass

c.  CRRC’s may be dropped as a door bundle

d.  a & b

e.  b & c

d

55-130, Vol 2, 3.48.1

19.  For a given aircraft gross weight, the flap setting ____ as the number of bundles increases past 4 and 10.

a.  Increases

b.  Decreases

c.  Stays the same

d.  Can increase or decrease, depending on the drop altitude

a

55-130, Vol 2, Fig 3.4

20.  When dropping a SATB-C, the loadmaster will use the equipment checklist and rig it in the bomb rack.

a.  True

b.  False

a

55-130, Vol 2, 3.52

21.  Minimum emergency bail-out altitude for free-fall parachutists is:

a.  1,000 feet AGL

b.  400 feet AGL

c.  2,000 feet AGL

d.  2,000 feet MSL

c

55-130, Vol 2, 3.54.2

22.  With multiple 28-foot extraction chutes deployed outside the aircraft, do not slow below:

a.  Drop airspeed

b.  Max effort takeoff speed

c.  Max effort threshold speed

d.  Max effort takeoff speed or max effort threshold speed, whichever is greater

b

55-130, Vol 2, 3.57.6

23.  When notified of a CDS malfunction,:

a.  Extend additional flaps to lower the nose to maintain a slight nose down attitude

b.  Set 50% flaps

c.  Set 70% flaps

d.  Set 100% flaps

a

55-130, Vol 2, 3.58.1

24.  If a CDS malfunction was due to a static-line retriever failure, the mission may continue using the opposite static line retriever and manually activating the retriever rewind switch at FS 245.

a.  True

b.  False

a

55-130, Vol 2, 3.58.1.5, Note

25.  During combat personnel airdrops,:

a.  Paratroopers should stop exiting the aircraft after “RED LIGHT”

b.  Paratroopers may continue to exit the aircraft after “RED LIGHT”

c.  Compute CARP and “RED LIGHT” without regard for safety (buffer) zones

d.  a & c

e.  b & c

e

55-130, Vol 2, 3.64.3

ANSWER SHEET

1.  a 	55-130, Vol 2, 3.9.2 Warning

2.  b	55-130, Vol 2, 3.14.4

3.  c	55-130, Vol 2, 3.14.5

4.  d	55-130, Vol 2, 3.18.1

5.  b	55-130, Vol 2, 3.18.1, Note

6.  c	55-130, Vol 2, 3.18.2

7.  d	55-130, Vol 2, 3.20

8.  a	55-130, Vol 2, 3.24.2.3

9.  b	55-130, Vol 2, 3.25.4

10.  a	55-130, Vol 2, 3.31

11.  d	55-130, Vol 2, 3.33.2

12.  b	55-130, Vol 2, 3.33.5, Note

13.  a	55-130, Vol 2, 3.41.3

14.  c	55-130, Vol 2, 3.42.3

15.  c	55-130, Vol 2, 3.44.1

16.  c	55-130, Vol 2, 3.45

17.  a	55-130, Vol 2, 3.46.2

18.  d	55-130, Vol 2, 3.48.1

19.  a	55-130, Vol 2, Fig 3.4

20.  a	55-130, Vol 2, 3.52

21.  c	55-130, Vol 2, 3.54.2

22.  b	55-130, Vol 2, 3.57.6

23.  a	55-130, Vol 2, 3.58.1

24.  a	55-130, Vol 2, 3.58.1.5, Note

25.  e	55-130, Vol 2, 3.64.3