## Towed Parachutist:

## Pilot - Lower Gear and 100% flaps (paratroop door drops only) - Maintain min of drop Alt and airspeed avoid flying over or upwind of water or built up areas - Shallow coordinated turns away from the parachutist - CP - Red Light “ON”

## LM - Inform Pilot and have the jumpmaster stop the remaining jumpers - Jumpmaster or safety will determine entanglement and make a recommendation via LM - Priority: paratroop door – retrieve; ramp and door - cut if conscious (AC final authority)

## CDS Malfunction:

## Pilot - Extend additional flaps as required (Tech – to the horn… 70%) - Lower nose to slight nose low (don’t over control) - Maintain drop altitude and airspeed, avoid flying over or upwind of built up areas and water - CP – Red Light “ON”

## LM - De-arm CDS switch - Notify pilot of malfunction - Alert parachutists to stay forward/clear of the load (if req’d) - Raise aft anchor supports (if required) - Clearance to close ramp and door (if jammed in ramp area stop closing action when released from up-lock) - Secure load – completion of drop checklist - When dropping double stick using CVR, LM will delay notifying the pilot until both sticks have exited the aircraft

## Heavy Equipment Malfunctions:

## Pilot (all situations) - Maintain drop altitude and airspeed - Avoid flying over or upwind of water or built up areas - CP - Red light “ON”

LM (extraction chute fails/falls or loose platform) - Notify pilot of malfunction - Alert parachutists to stay forward/clear of the load (if req’d) - Emergency aft restraint (fwd to aft) - Raise aft anchor supports (if required) - Clearance to close ramp and door - Re-lock and secure platform – completion of malf/drop checklist

LM (single parachute outside) - Notify pilot of malfunction (after pulling right hand emergency handle) - Alert parachutists to stay forward/clear of the load (if req’d) - Emergency aft restraint (fwd to aft) - Cut aft of the attachment point - Raise aft anchor supports (if required) - Clearance to close ramp and door - Re-lock and secure platform – completion of malf/drop checklist

LM (multiple 28 ft fail/fall) - Notify pilot of malfunction - Alert parachutists to stay forward/clear of the load (if req’d) - Raise aft anchor supports (if required) - Clearance to close ramp and door - Re-lock and secure platform – completion of malf/drop checklist

LM (multiple 28’ outside aircraft) - Pull right hand emergency handle - Left hand Simul handle (10-20 seconds steady pull for stuck locks) - Notify pilot and brace for impact NOTE: Pilot - max power, no slower than VMETO speed, look for a place to land