# AIRPLANE GENERAL

## WING – 222’ 9”

## STAB – 68’ 9”

LENGTH – 247’ 10”

TAIL HEIGHT – 64’ 9”

## MAIN WHEEL BASE WIDTH – 36’

BASIC A/C WEIGHT – 375,000

## GROUND SPOILER’S – INBOARD (4 PER SIDE)

## FLIGHT SPOILER’S – OUTBOARD (5 PER SIDE)

PAYLOAD – 290,000

SEATS

FLT STATION – 6

## RELIEF CREW – 7

## COURIER – 8

TROOP – 75

NOISE HAZARD

T/O PWR – 1000’

IDLE – 500’

ENGINE NOISE – 34’ IN FRONT

WAKE / EXHAUST = 50 MPH

800’ AT T/O PWR

## 315’ AT TAXI -

## 52’ AT IDLE

## RADOME

## FUELING – 46’

## PEOPLE/ELECT/EXPL- 34’

### APU TEMP / NOISE HAZARD – 20’

## PORTABLE O2 BOTTLES – 16

O2 BOTTLE RECHARGER’S – 10

5/FLT DECK

3/ CARGO

2/ TROOP

# ENGINES AND APU’S

## TF-39 ENGINES WITH THRUST REVERSERS

FLAT RATED CONSTANT THRUST

43,000 LBS THRUST AT SEA LEVEL AT 22 DEG C

8 TO 1 BYPASS RATIO

CORE CONTRIBUTES 15% THRUST

ENGINE SECTIONS

FAN

COMPRESSOR

COMBUSTORS

TURBINE

ACCESSORY DRIVE

FAN

N1 - 1 ½ STAGE

COMPRESSER N2 (CORE ENGINE)

N2 – 16 STAGE AXIAL FLOW

VARIABLE VANE SYSTEM:

-- INLET GUIDE VANES

--1ST SIX STAGES OF STATOR VANES

--TWO HYD ACTUATORS OPERATED BY FUEL PRESSURE

--PREVENTS COMPRESSOR STALLS

--ENHANCES ACCELERATION

--PREVENTS OVERHEATING

--DRIVES ENGINE ACCESSORY SECTION

--SOURCE OF BLEED AIR

COMBUSTOR

2 IGNITERS

TURBINE

--2 STAGE HIGH SPEED

--DRIVES COMPRESSOR (N2)

--AFT OF HIGH SPEED - 6 STAGE LOW SPEED DRIVES FAN (N1)

--FAN AND COMPRESSOR INDEPENDENT

ACCESSERY SECTION

--FORWARD END OF COMPRESSOR

--ALL ACCESSORIES EXCEPT N1 TACH GENERATOR

# ENGINES AND APU’S

PAGE 2

ACCESSORIES

--2 HYD PUMPS                                    --FUEL PUMP

--CSD                                                   --FUEL CONTROL

--STARTER                                               --AIR / OIL SEPARATOR

--LUBE / SCAVENGE PUMPS                   	--N2 TACH GENERATOR

FUEL PUMP

2 STAGES

BOOST / HIGH PRESS IMPELLER

FUEL  BOOST HEATER  FILTER  HIGH PRESS BY PASS

VALVE

IF FILTER IS CLOGGED FUEL WILL BYPASS FILTER

FUEL CONTROL UNIT

METER FUEL

LIMITS FUEL FLOW VARIATIONS

SCHEDULES OPERATION OF VARIABLE VANES

PRESSURIZING AND DRAIN VALVE FUEL MANIFOLD

FUEL MANIFOLD

30 NOZZLES IN CLUSTERS OF THREE

START SYSTEM

PROVIDES AND CONTROLS APU AIR FOR START

AIR FROM EXTERNAL SOURCE, APU, ANOTHER RUNNING ENGINE

RECEIVES AIR THROUGH STARTER CONTROL VALVE

STARTER BUTTON DEPRESSED OPENS STARTER CONTROL VALVE

CUTOUT SWITCH DISENGAGES AT 46 +/- 3 RPM

APU SHUTS DOWN DURING OR IMMEDIATELY AFTER START

DO NOT ATTEMPT A RESTART

HAVE MAINTENANCE CHECK ACCESSORY DRIVE SHAFT

ATTEMPTED RESTART COULD RESULT IN FIRE WITH A

FAILED DRIVE SHAFT

CAUSE FOR FAILED DRIVE SHAFT SHALL BE DETERMINED

IF BUS CONTROL CIRCUITRY AT FAULT, MUST BE FIXED

PRIOR TO FLIGHT

IGNITION SYSTEM

EXCITER AND 2 SPARK IGNITERS

2 SUB SYSTEMS

HIGH ENERGY TO INITIATE IGNITIO                                                 		       LOW ENERGY PROVIDES CONTINUOUS IGNITION

# ENGINES AND APU’S

PAGE 3

START SEQUENCE

11% - FUEL AND START IGNITION TO RUN

25% - LIGHTOFF

35% - LOW OIL PRESSURE LIGHT OFF

46% +/-3 – BUTTON OUT / STARTER DISENGAGE

55% - FUEL FLOW PEAK 2200PPH DROPS TO 1200PPH

65% - ENGINE STABLE, N2 = 61%-67%

OIL SYSTEM

LUBRICATES ENGINE BEARINGS AND ACCESSORY DRIVE SECTION

OIL TANK

TOTAL CAPACITY – 12.9 GALS

SERVICED UP TO – 9.1 GALS

OPERATION LEVEL – 6.1 GALS

LUBE AND SCAVENGE PUMPS

ENGINE DRIVEN

SUPPLIES OIL TO BEARINGS AND ACCESSORY GEAR BOX

SUPPLY PRESSURE VARIES WITH N2 SPEED AND OIL TEMP

AIR OIL SEPARATOR

COLLECTS OIL MIST FROM BEARING SUMPS

SEPARATES AIR AND VENTS IT THRU VENTS LOCATED ON EACH

PYLON

INDICATIONS OF OIL SYSTEM FAILURE

LOSS OF QUANTITY (LEAK) MUST BE FOUND VISUALLY

INCREASE IN OIL PRESSURE (10 PSI)

LOW OIL PRESSURE < 10 PSI (DROP OF 5 PSI)

FILTER DIFFERENTIAL PRESSURE WARNING LIGHT

HIGH OIL TEMPERATURE

OIL FLOW

TANKOIL PUMPENGINE2 SCAVENGE PUMPSSCAVENGE

OIL FILTERFUEL OIL COOLERTANK

# ENGINES AND APU’S

PAGE 4

FIRE / OVERHEAT DETECTION AND PROTECTION

4 BOTTLES, 2 IN EACH INBOARD ENGINE PYLON

CONTINUOUS LOOP – GIVES VISUAL INDICATION OF FIRE/OVHT

LOCATED IN ENGINE NACELLE AND LOWER PYLON

OPTICAL AT BOTTOM OF ENGINE – ENG FIRE INDICATION ONLY

IF SHORT CIRCUIT OCCURS ENGINE FIRE DET LIGHT ON FE

ANNUNCIATOR PANEL AND ON ENGINE AND PYLON DETECTOR

PANEL ILLUMINATE

ENGINE FIRE INDICATED BY STEADY LIGHT IN FIRE HANDLE AND

STEADY ENGINE OVERHEAT LIGHT ON CENTER INST PANEL

ONE BOTH BOTTLES CAN BE DISCHARGED INTO A SINGLE

NACELLE BY SELECTING EITHER NORMAL OR ALTERNATE

BOTTLE OUT LIGHTS INDICATE WHICH BOTTLES HAVE FIRED

7 BUS OPERATION (NO DETECTION)

FIRE HANDLE POWERED

FIRE EXTINGUISHER SYSTEM FULLY FUNCTIONAL

NO BOTTLE OUT LIGHTS

3 BUS OPERATION (NO DETECTION)

FIRE HANDLES NOT POWERED

FIRE EXTINGUISHER SYSTEM FULLY FUNCTIONAL

ENGINE FIRE HANDLE

CLOSES FUEL CONTROL SHUTOFF VALVE & TURNS IGNITION OFF

DE-ENERGIZES THE GENERATOR

CLOSES CABLE OPERATED FUEL SHUTOFF VALVE ON FORWARD

WING BEAM

CLOSES HYDRAULIC SHUTOFF VALVE AND DEPRESSURIZES THE

HYDRAULIC PUMPS

CLOSES VALVE IN THRUST REVERSER HYDRAULIC PRESSURE

LINE ON #2 AND #3 ENGINES AND DEACTIVATES THE

EMERGENCY RETRACT VALVE

CLOSES BLEED AIR SHUTOFF VALVE

EXPOSES FIRE EXTINGUISHER BUTTON

DE-ENERGIZES STARTER CONTROL VALVE TO OPEN POSITION

THRUST REVERSER

TO EXTEND NEED TOUCHDOWN OR SPIN UP SIGNAL

AIRBORNE CAN ONLY EXTEND INBOARD TR’S

REVERSE THRUST = 1/3 OF FORWARD THRUST

ALTERNATE HYD PRESS FOR #2 TR IS #1 HYD SYSTEM

ALTERNATE HYD PRESS FOR #3 TR IS #4 HYD SYSTEM

# ENGINES AND APU’S

PAGE 5

THRUST REVERSER COMPONENTS

TRANSLATING COWL – MOVES FORWARD AND AFT POWERED BY

FOUR HYDRAULIC ACTUATORS

FIXED CASCADES – WHEN THE TRANSLATING COWL’S EXTENDED

THEY ARE EXPOSED DIRECTING FAN EXHAUST FORWARD

BLOCKER DOORS – DIRECT EXHAUST FORWARD THROUGH THE

FIXED CASCADES

BLOCKER LINKS - WHEN THE COWL EXTENDS THEY POSITION

THE BLOCKER DOORS AGAINST THE FAN COWL

HYDRAULIC SHUTOFF VALVES – ALLOWS PRESSURE TO ENTER

THE SYSTEM WHEN THE THROTTLES ARE MOVED TO OR

FROM THE REVERSE POSITION

EMERGENCY RETRACT SWITCHES – USED TO SELECT AN

ALTERNATE HYDRAULIC SOURCE, MUST BE HELD IN THE

RETRACT POSITION

THROTTLES – MUST BE LIFTED OVER STOP TO ENTER REVERSE

THROTTLE INTERLOCK

- INFLIGHT PREVENT OUTBOARDS FROM BEING PLACED

## IN REVERSE, ALSO LIMITS INBOARDS TO REVERSE IDLE

###### ACTIVATED THROUGH TOUCHDOWN & SPINUP SIGNALS

###### CAN BE OVERRIDDEN BY 35 LBS PRESSURE

REVERSE THRUST LIMITER

- LIMITS THE AMOUNT OF REVERSE THRUST APPLIED

- BASED ON RWY TEMP AND PRESSURE ALTITUDE

## THRUST REVERSER PRESSURE LIGHT – ON WHEN SYSTEM IS

PRESSURIZED

THRUST REVERSER NOT LOCKED LIGHT

- ON WHEN ONE OF 4 COWL LOCKS IN UNLOCKED POSITION

## BROKEN GROUND WIRES / STUDS AND DISCONNECTED

## CANNON PLUGS WILL CAUSE NOT LOCKED LIGHT

## THRUST REVERSER EXTENDED LIGHTS – ON WHEN LIMIT

SWITCH NOT CLOSED

# ENGINES AND APU’S

PAGE 6

THRUST REVERSER RESTRICTIONS INFLIGHT

- DO NOT EXTEND OUTBOARD THRUST REVERSERS

- DON’T MOVE INBOARD TR’S TO REVERSE IDLE TIL IDLE RPM

- NEVER EXCEED REVERSE IDLE

- OUTBOARD ENGINES MUST BE IN FLIGHT IDLE

- DO NOT ADVANCE INBOARD THROTTLES PAST FLIGHT IDLE

UNTIL TR’S ARE FULLY RETRACTED

- DO NOT EXTEND TR’S WITH FLAPS DEPLOYED

- ON GROUND LIMITED TO 60 TO 150 KTS

- FAN BLADE INSPECTION REQUIRED IF EXTENDED BELOW 60 KT

APU’S

SOURCE OF BLEED AIR AND ELECTRICS ON GROUND

ONLY ONE APU AT A TIME CAN SUPPLY ELECTRICAL POWER

BOTH APU'S CAN BE USED FOR BLEED AIR AT THE SAME TIME

PROVIDES AIR CONDITIONING

PROVIDES HYD PRESSURE THROUGH THE ATM

HYDRAULIC STARTING THROUGH ACCUMULATOR PRESSURE

FIRE DETECTION LOOPS

2 NITROGEN BOTTLES FOR FIRE RESPONSE

APU ACCUMULATORS CHARGED BY HYDRAULIC SYS #1 AND #4

CAN ALSO CHARGE ACCUMULATORS –EXTERNAL / HAND PUMP

APU FIRE DETECTION / PROTECTION

COVER’S BOTH ATM’S AND APU’S

GROUND FIRE ACTIVATES AUDIBLE HORN

TWO FIRE BOTTLES LOCATED IN RIGHT AFT WHEEL WELL

INFLIGHT APU OPERATIONS

PROHIBITED EXCEPT FOR EMERGENCIES

ATTEMPTED STARTS ABOVE 25,000 MAY BE UNSUCCESSFUL

APU INLET DOOR WILL NOT CLOSE ABOVE 170 KTS

# HYDRAULICS

## 4 SEPARATE HYDRAULIC SYSTEMS

- 2850 PSI TO 3150 PSI

- SERVICE CENTERS IN CARGO PROVIDE ACCESS TO SYSTEMS

- SYSTEMS #2 AND #3 PRIMARY FOR FLIGHT CONTROLS

- SYSTEMS #1 AND #4 PRIMARY FOR:

- FORWARD AND AFT CARGO DOORS

- CREW ENTRANCE DOOR

- APU START SYSTEM

- AIR REFUELING SYSTEM

- LANDING GEAR SYSTEM

- ALSO TO UTILITY AND FLIGHT CONTROLS

- EACH ENGINE HAS 2 HYDRAULIC PUMPS

- 3 PTU’S PERMIT TRANSFER OF HYD PRESS BETWEEN SYSTEMS

- HYD PRESS CAN BE SUPPLIED BY 2 ATM’S ONE FOR EACH APU

- RAT IS CONNECTED TO HYD SYSTEM #2

- LIMITED CAPABILITY FOR LIMITED FLIGHT CONTROLS

MAIN COMPONENTS

- PUMPS

- RESERVOIRS LOCATED AT EACH SERVICE CENTER

- #1 AND #4 10 GALS

- #2 AND 3# 6 GALS

- SUCTION BOOST PUMPS

- AT BOTTOM OF RESERVOIR

- OVERCOME PRESSURE DROP IN TRANSMISSION LINES

- INSURES POSITIVE INLET PRESS AT ATM’S AND ENGINE

PUMPS

- PUMP IS HYDRAULIC MOTOR DRIVEN

- ELECTRIC SUCTION BOOST PUMP

- LOCATED AT #1 SYS AND #4 SYS SERVICE CENTERS

- PROVIDES SAME FUNCTION AS SUCTION BOOST PUMP

- PYLON SHUTOFF VALVES

- ISOLATE ENGINE DRIVEN PUMPS

# HYDRAULICS

## PAGE 2

ENGINE DRIVEN PUMPS

- 2 VARIABLE VOLUME, PRESSURE COMPENSATED ON EACH ENG

- CAPABLE OF 60 GALS PER MINUTE

- ON ENGINE START TOP PUMP IS PRESSURIZED

- FLUID IS COOLED BY HYDRAULIC COOLERS IN #2 AND #3 MAIN

#### FUEL TANKS

- IN CASE OF FAILURE A CHECK VALVE PREVENTS FUEL FROM ENTERING THE HYDRAULIC SYSTEMS

- ONE PUMP ON 2 NON ADJACENT ENGINE’S MAY BE INOPERATIVE PROVIDED ALL PTU’S ARE OPERATIVE, FILTERS HAVE BEEN CHECKED AND ALL PUMPS WILL DEPRESS

- PRESS LOW LIGHT WILL COME ON WHEN PRESSURE DROPS BELOW 1350 (+/- 150) PSI

PTU’S

- 3 PTU’S  #1-2,  #2-3,  #3-4

- #1-2, #3-4 PTU’S ON FOR TAKEOFF

- TRANSFER POWER FROM LIGHTLY LOADED SYSTEM TO A

#### HEAVILY LOADED SYSTEM

- SYSTEM TO BE PRESSURIZED MUST CONTAIN HYD FLUID

- 2 HYDRAULIC MOTORS CONNECTED BY SHAFT

- #1-2 PTU LOCATED IN THE LEFT LANDING GEAR POD

- #3-4 PTU LOCATED IN THE RIGHT LANDING GEAR POD

- #2-3 PTU LOCATED ON AFT SPAR OF CENTER WING BEAM

- PTU MANUAL OVERRIDE BUTTON AT EACH SERVICE CENTER MANUALLY PRESSURIZES A HYDRAULIC SYSTEM WITH THE FAILURE  OF ELECTRICS

## ATM’S

- WITH A HYD PUMP LOCATED IN EACH APU COMPARTMENT

- CONNECTED TO #1 AND #4 HYDRAULIC SYSTEMS

- SUPPLY HYDRAULIC POWER DURING GROUND OPERATIONS

- CAN BE USED IN FLIGHT IF NECESSARY

- CAPACITY OF 40 GAL PER MINUTE

- RUN BY BLEED AIR SUPPLIED BY ENGINES, APU’S OR EXTERNAL

- PRIOR TO START THE ELECTRIC SUCTION BOOST PUMP MUST BE ON AND THE FLOOR HEAT OFF

- FIRE WILL BE INDICATED BY A STEADY LIGHT IN THE APU FIRE HANDLE AND A FLASHING APU FIRE LIGHT ON THE PILOTS ANNUNCIATOR PANEL

# HYDRAULICS

## PAGE 3

## RAT

- HYDRAULIC POWER LIMITED TO FLIGHT CONTROLS

- CONNECTED TO #2 HYDRAULIC SYSTEM

- #2 ENGINE HAS 4 SOURCES OF HYDRAULIC PRESSURE

- LOCATED IN FORWARD PORTION OF LEFT WHEEL WELL

- WILL DEPLOY AUTOMATICALLY

- CSD UNDERSPEED BOTH INBOARD AND ONE OUTBOARD ENG

- MAIN LANDING GEAR TOUCHDOWN SWITCH PREVENT

#### AUTOMATIC DEPLOYMENT ON GROUND

- OVERRIDE BUTTONS LOCATED AT THE #2 SERVICE CENTER

ENABLE MANUAL DEPLOYMENT OR RETRACTION

HYDRAULIC SERVICING

- LOCATED AT #3 SERVICE CENTER

- CAN SERVICE ALL 4 SYSTEMS FROM ONE POINT

- HYDRAULIC SYSTEM BEING SERVICED SHOULD BE DEPRESSED (FASTER)

HYDRAULIC SYSTEM FAILURES

- FOR ANY MALFUNCTION HAVE SCANNER MAKE VISUAL     INSPECTION FOR FIRE OR LEAKS

- TIME PERMITTING RUN MADAR TROUBLESHOOTING

- LOSS OF SYSTEM PRESSURE, NO FLUID LOSS, USE PTU TO REPRESSURIZE AND TURN ON ATM’S FOR SYS #1 AND #4

- MADAR MONITORS ENGINE DRIVEN PUMPS, ATM’S, AND PTU’S FOR EXCESSIVE CASE DRAIN

- ATM PRESSURE LIGHT WITH ATM SWITCH OFF

## ATM RUNS BACKWARDS

IMMEDIATELY DEPRESS SYSTEM TO PREVENT FIRE

- #1 AND #4 HYDRAULIC BOOST PUMP FAILURE

TURN ON ASSOCIATED ELECTRIC SUCTION BOOST PUMP

- #2 AND #3 HYDRAULIC BOOST PUMP FAILURE

TREAT AS TOTAL LOSS OF SYSTEM QUANTITY

- HYDRAULIC SYSTEM OVERPRESSURE > 3400 PSI

DO NOT PULL FIRE HANDLE

DO NOT DEPRESS PUMPS

ENVIRONMENTAL

BLEED AIR SUPPLIES AIR FOR:

AIR CONDITIONING

CARGO FLOOR HEAT

CABIN PRESSURE

ENGINE START

LPPS

ENGINE ANTI-ICE

ATM

PRIMARY SOURCE OF BLEED AIR IS 8TH STAGE OF COMPRESSOR

LOW POWER SETTINGS OR HIGH DEMAND 16TH STAGE IS SOURCE

DUCT SYSTEM

NACELLE AND PYLON MANIFOLDS

LEFT AND RIGHT WING DUCT MANIFOLDS

CENTER DUCT MANIFOLD

BLEED AIR SUPPLIED BY ENGINES THRU WING DUCT MANIFOLDS

EACH ENGINE HAS A PYLON SHUTOFF VALVE

EACH WING HAS A WING ISOLATION VALVE

TO ISOLATE WING DUCT MANIFOLD

CLOSE PYLON SHUTOFF VALVES FOR EACH ENGINE

CLOSE WING SHUTOFF VALVE

TO ISOLATE CENTER DUCT MANIFOLD

CLOSE BOTH WING ISOLATION VALVES

CLOSE FORWARD AND AFT CARGO FLOOR HEAT VALVES

CLOSE BOTH APU ISOLATION VALVES

CLOSE APU BLEED LOAD AND FLOW CONTROL VALVES

BLEED DUCT OVERHEAT DETECTION

3 SEPARATE LOOPS

AIR CONDITIONING

USES ENGINE, APU, OR EXTERNAL  BLEED AIR

2 SYSTEMS , OPERATE TOGETHER OR CAN OPERATE ALONE

BOTH USED TO PRESSURIZE THE AIRCRAFT

LEFT SYSTEM SERVICES FLIGHT DECK AND TROOP

RIGHT SYSTEM SERVICES THE CARGO COMPARTMENT

CAN MAINTAIN 8000’ CABIN AT 40,000 FT

OUTFLOW VALVE CONTROLLED LEAKAGE

# ENVIRONMENTAL

PAGE 2

AIR CONDITIONING CON’T

FOR PRESSURIZATION – ONE PACK AND FLOOR HEAT

CAN DEPRESSURIZE THE AIRCRAFT IN 60 SEC

OVER PRESS SAFETY VALVES MAINTAIN 8.7 PRESS DIFFERENTIAL

ICE DETECTION

DETECTOR PROBE LOCATED ON NOSE

ENGINE AND NACELLE ANTI-ICE ON INLET DUCTING AND INLET

GUIDE VANES AND ENGINE PRESSURE RATIO PROBE

WINDSHIELD HEAT

3 FRONT WINDOWS

VINYL BETWEEN GLASS HEATED TO PROTECT FROM ICE / FOG

AND BIRD STRIKES

PITOT HEAT

INDEPENDENT DE-ICE FOR ANGLE OF ATTACK

FLOOR HEAT

CENTER BLEED AIR DUCT

2 FLOOR HEAT TEMPERATURE CONTROL VALVES

3 RECIRCULATING FANS

BLEED AIR SUPPLIED THROUGH ISOLATION VALVES AND PASSES

THROUGH THE ATM PRIORITY VALVE THEN TO THE FLOOR

HEAT TEMPERATURE CONTROL VALVES

PRESSURIZATION SYSTEM

CAN BE CONTROLLED AUTOMATICALLY OR MANUALLY

AFI 11-2C-5 VOL 3 – AIRPLANE MUST BE PRESSURIZED FOR ALL

FLIGHTS

TO PRESSURIZE BOTH AC SYSTEMS OR ONE SYSTEM AND FLOOR

HEAT IS NECESSARY

CABIN ALTITUDE IS MAINTAINED BY CONTROLLING THE

OUTFLOW VALVE ASSEMBLY

CONTROLLED MANUALLY DURING 7 BUS AND 3 BUS OPERATION

CABIN LOW LIGHTS ON, WHEN CABIN ALTITUDE EXCEEDS

10,000 FT

# ENVIRONMENTAL

## PAGE 3

AUTOMATIC VISUAL AND OXYGEN WARNING SYSTEMS, 12,000 TO

14,500 FT CABIN ALTITUDE

-    OXYGEN LIGHT ON THE CONTINUOUS FLOW PANEL

ILLUMINATES

-    WHITE DOME LIGHT ON BRIGHT IN THE CREW RELIEF,

COURIER, TROOP, AND CARGO COMPARTMENTS

-    ALL NO SMOKING LIGHTS ON

-    WARNING HORNS SOUND AND WILL CONTINUE TO SOUND

UNTIL SILENCED OR BELOW 11,500 FT

# ELECTRICAL

## AC POWER

## 4 ENGINE DRIVEN GENERATORS CONNECTED BY THE CSD

17 ELECTRICAL BUSES

STEPDOWN TRANSFORMERS CONVERT 115V AC TO 26V AC

LEFT AND RIGHT APU’S PROVIDE POWER ON GROUND

EMERGENCY GENERATOR

DC POWER

FURNISHED BY 2 TRANSFORMER RECTIFIERS

8 DC BUSES

2 NICKEL CADMIUM BATTERIES

EMERGENCY GENERATOR

GENERATORS THRU THE CSD PROVIDE 400 CYCLE 115V AC

CSD CONVERTS ENGINE SPEED TO A CONSTANT 8000 RPM OUTPUT TO

MAINTAIN 400 CYCLES / SEC

CAN BE DISCONNECTED TO TAKE GENERATOR OFF LINE

CAN ONLY BE RESET ON THE GROUND

GENERATORS

RATED AT  80 KILOVOLT AMPS, SUPPLIES 115/200V AC 400HZ

GENGCUGLCBTCTIE BUS

GENERATOR CONTROL UNIT (GCU)

PROTECTS THE GENERATOR FROM SYSTEM FAULTS AND SHORTS

CONTROLS THE GLC AND THE BTC

GENERATOR LINE CONTACTOR

CONNECTS EACH GENERATOR TO A MAIN AC BUS

OPERATED MANUALLY BY THE GENERATOR CONTROL SWITCH

BUS TIE CONTACTOR

CONNECTS THE MAIN BUSES TO THE TIE BUS

ALLOWS PARALLEL OPERATION OF THE 4 GENERATORS

GENERATOR FAILURE

POWER SUPPLIED TO FAILED GENERATORS BUSES THRU TIE BUS

GROUND OPERATIONS

POWER COMES THRU THE TIE BUS  FROM A POWER CART OR APU

# ELECTRICAL

PAGE 2

AIRCRAFT BUSES

FWD MAIN AC BUS 1, 2, 3, 4

AFT MAIN AC BUS 1, 2, 3, 4

MONITOR AC BUS 1,2

PWR FROM #2 AND #3 GENERATORS

AVIONICS AC BUS 1, 2

PWR THRU PTR’S

NAV AC BUS 1, 2

PWR FROM AVIONICS AC BUS 1

NAV 28V AC BUS 1, 2

PWR FROM NAV AC BUS 1, 2 RESPECTIVELY

ISO AC BUS

PWR FROM #1 PTR & ISO BUS POWER RELAY

ISO 26V AC BUS

PWR FROM AVIONICS IOSLATED AC BUS

EMERG AC BUS

PWR THRU #1 PTR & EMERG BUS POWER RELAY

POWER TRANSFER RELAY (PTR)

ENSURE MULTIPLE SOURCES OF 115V AC AVAILABLE

2 PTR’S

#1 AND #4 MAIN BUSES PRIMARY SOURCE OF POWER

#2 AND #3 MAIN BUSES BACKUP SOURCE OF POWER

AUTOMATICALLY SWITCH IN CASE OF POWER FAILURE

APU

HAS SAME GENERATOR AS THE ENGINE’S

ONLY ONE CONNECTED FOR ELECTRICS AT A TIME

HAS GCU

DC POWER

2 TRANSFORMER RECTIFIERS, CONVERT 115V AC TO 28V DC

RECEIVE POWER THRU #1 AND #2 PTR’S

DC BUSES

PROTECTED BY

CIRCUIT BREAKERS

CURRENT LIMITER

PERMITS SINGLE TR TO POWER ENTIRE DC SYSTEM

REVERSE CURRENT RELAYS

ALLOWS ONE WAY FLOW OF CURRENT

# ELECTRICAL

PAGE 3

TRANSFORMER RECTIFIERS

LOCATED AT FWD RIGHT SIDE OF CARGO COMPARTMENT

CONVERT 115AC TO 28V DC

RECEIVE POWER THROUGH THE #1 AND #2 PTR’S

ONE CAN CARRY THE ENTIRE LOAD

BATTERIES

NORMAL AND STANDBY

24V DC 5 AMP / HR

DC BUSES

AVIONICS DC BUS 1, 2

AVIONICS ISO DC BUS

ISO DC BUS

EMERG DC BUS          CAN BE POWERED BY BATTERY OR

BATTERY BUS             EMERGENCY AC/DC POWER SYSTEM

IN CASE OF TR FAILURE

EMERGENCY AC/DC GENERATOR (EMERGENCY POWER SUPPLY)

GENERATOR DRIVEN BY #2 ENGINE HYDRAULIC PRESSURE

115V AC / 28V DC

POWERS

PILOTS FLIGHT INSTRUMENTS

MINIMUM WARNING AND CONTROL

NAVIGATION

COMMUNICATION

BUSES POWERED BY THE EMERGENCY GENERATOR

EMERG AC AND DC BUSES

ISO AC AND DC BUSES

AVIONICS ISO AC AND DC BUSES

BATTERY BUS

ELECTRICAL SYSTEM MALFUNCTIONS

GENERATOR FAIL LIGHT INDICATES A MECHANICAL FAILURE

GENERATOR OUT LIGHT INDICATES EITHER A CSD OR  A

GENERATING SYSTEM COMPONENT MALFUNCTION

(DIFFERENTIAL FAULT, NEUTRAL CURRENT FAULT)

FOUR MAIN GENERATORS OUT

POSSIBLY CAUSED BY PARALLING CIRCUIT MALFUNCTION

OR LIGHTING STRIKE

MAINTAIN LEVEL FLIGHT DUE TO ENGINES GRAVITY FEED

TO DISTINGUISH BETWEEN FOUR GENERATORS OUT AND FOUR

ENGINE FLAMEOUT CHECK N2 RPM

# ELECTRICAL

PAGE 4

APU INFLIGHT OPERATION

USE ONLY IN CASE OF EMERGENCY

ABOVE 20,000 FT FULL OUTPUT MAY NOT BE AVAILABLE

OPERATION MORE THAN 3 MIN WILL CAUSE SKIN DAMAGE AFT

LOSS OF DC POWER INDICATIONS

SLAT INDICATOR SHOWS INTRANSIT

FLAP INDICATOR 9 O’CLOCK (A), 11 O’CLOCK (B)

GEAR INDICATIONS SHOW INTRANSIT

ALL WARNING FLAGS ON CO-PILOTS HIS AND ADI

CO-PILOT LOSES INTERCOM

# LANDING GEAR SYSTEM

## COMPONENTS

- 4 MAIN AND 1 NOSE GEAR

- MAINS 6 WHEEL DUAL TANDEM BOGIES 2 WHEELS FWD, 4 AFT

- NOSE 4 WHEEL STEERABLE

- EACH MAIN CAN MOVE INDEPENDENTLY

- CAN TAXI OVER 8 IN DROP

- 2 DOORS FOR EACH MAIN OPERATED BY MECH LINKAGE

- #1 HYDRAULICS PROVIDE PRESSURE FOR AFT MAINS

- #4 HYDRAULICS PROVIDE PRESSURE FOR FWD MAINS

- #1 HYDRAULICS PROVIDE PRESSURE FOR NOSE

- NORMAL RETRACTION / EXTENSION TIME = 25 SEC

- GEAR EMERGENCY EXTENSION TIME IS APPROX. 3 MIN

MAIN LANDING GEAR MOVEMENT

- HYDRAULIC PRESSURE DISENGAGES POWER BACK COLLAR

- BOGIES ROTATE 90 DEGREES TWO FRONT WHEELINBOARD

- HYDRAULIC PRESS HOLDS GEAR POSITION DURING RETRACT

- DOWNLOCK ACTUATOR UNLOCKS SIDEBRACE

- HYDRAULIC MOTER RETRACTS GEAR

- DOORLOCKS AND OVERCENTER LINK HOLD GEAR RETRACTED

- EXTENSION IS REVERSE

LANDING GEAR HANDLE WARNING LIGHT

- ON WHEN HANDLE IS UP AND THE NOSE OR ANY MAIN LANDING

GEAR DOOR LOCKS ARE NOT LOCKED

- ON WHEN HANDLE IS DOWN AND THE NLG NOT DOWN AND

LOCKED OR ANY MLG POSITION COLLAR IS NOT ENGAGED

BOGIE PITCH LIGHT

- ON WHEN PITCH IS GREATER THAN 15 DEGREES NOSEDOWN

OR 11 DEGREES NOSEUP AND SIDE BRACE IS LOCKED

- STRUCTURAL DAMAGE CAN OCCUR IF LIGHT IS ON AND THE

GEAR HANDLE IS PLACED DOWN (DUE TO ERRONEOUS SIDE

BRACE LOCKED SIGNAL)

- RETRACTION ATTEMPTED WITH A LIGHT ON CAN CAUSE THE

BOGIE TO MISS THE TRACK CAUSING STRUCTURAL DAMAGE

# LANDING GEAR SYSTEM

## PAGE 2

## EMERGENCY SWITCH ON LIGHT

LIGHT ON WITH NLG UP AND LOCKED AND EMERGENCY EXTEND

IN NORMAL POSITION, INDICATES FAILURE OF NLG DOOR OPEN

SWITCH CARD, NLG WILL ATTEMPT TO EXTEND THROUGH DOORS

ON WHEN ANY MLG EMERGENCY EXTEND SWITCH EXTEND AND

MLG IS DOWN AND LOCKED / COLLAR LOCK ENGAGED

LANDING GEAR WARNING HORN

SOUNDS WHEN FLAPS > 80%, NLG NOT DOWN AND LOCKED OR ANY

MLG COLLAR LOCK NOT ENGAGED

SOUNDS WHEN AIRSPEED < 200 KTS AND;

ADS SAFE

THROTTLE BELOW A MINIMUM POWER SETTING

NLG NOT DOWN AND LOCKED OR ANY MLG COLLAR NOT LOCKED

## NOSE LANDING GEAR

- 4 DOORS MOVE OUTWARD WHEN OPENING

- #4 HYD AND ELECT MOTORS USED FOR EMERG RETRACTION

NOSE LANDING GEAR MOVEMENT

- HYDRAULIC PRESSURE TO UNLOCK LOCKS AND OPEN DOORS

- HYDRAULIC PRESSURE TO NOSE GEAR LOCK AND HYD MOTOR

- FIBER OPTIC SCOPE AFT OF CREW ENTRANCE DOOR

NOSE WHEEL STEERING

- USE STEERING WHEEL OR RUDDER PEDAL’S

- HYDRAULIC SYSTEM #1

- 60 DEGREES DEFLECTION FOR NOSE STEERING

- 5 DEGREES [ +2, -1] DEFLECTION FOR RUDDER PEDAL STEERING

-NOT AVAILABLE  WITH #1 HYDRAULICS FAILURE

- #4 HYDRAULICS USED FOR EMERG NOSE WHEEL STEERING

- ANY MAIN GEAR LOCKED AT ANGLE > THAN FOUR DEGREES LIMIT NOSE WHEEL STEERING TO 30 DEGREES DEFLECTION

- WITH AIRCRAFT STOPPED, ALL MLG 0 DEGREES, NOSE GEAR MAY BE TURNED 45 DEGREES PRIOR TO TAXI

- WITH CG AFT OF 32% RAPID MOVEMENT CAN CAUSE LOSS OF EFFECTIVENESS, ESPECIALLY ON WET OR ICY SURFACES, RE-CENTER AND RE-ENGAGE TO RECOVER

# LANDING GEAR SYSTEM

PAGE 3

AFT MAIN LANDING GEAR CASTERING

- UP TO 20 DEGREES FROM CENTER

- REDUCES TURNING RADIUS

- MINIMIZES TIRE SCUFFING

- TO CASTERREMOVE HYDRAULIC PRESSURE BLOCK

- USES HYDRAULIC PRESSURE TO RETURN TO CENTER

- ASYMMETRY BETWEEN AFT BOGIES REACHES 8 DEGREES, BOGIE NEAREST CENTER STOPS CENTERING

- WHEN REACHING 5 DEGREES DIFFERENCE BOTH BOGIES CONTINUE TO CENTER

- MAIN LANDING GEAR FREE LIGHTS MUST BE ON TO ENSURE BOGIES ARE

- AVOID BRAKING TO A STOP IN A TURN

- IF STOPPED IN A TURN NLG LIMITED TO 30 DEGREES DEFLECTION FROM STOPPED POSITION

- 180 DEGREE TURN REQUIRES 150 FT OF PAVED SURFACE WITH GEAR CATERED AND 228 FT WITH GEAR FIXED

- WHEN TAXIING ABOVE 20 KTS ANTI-SKID SHOULD BE ON

- ANY MLG MORE THAN 4 DEGREES FROM CENTER DON’T TAXI

- ALTERNATE CASTER

- PLACE BOTH BOGIE POSITIONING SWITCHES OPPOSITE THE DIRECTION OF TURN

- WHEN COMING OUT OF A TURN,  BOTH SWITCHES MUST BE HELD IN OPPOSITE DIRECTION TO CENTER MLG

KNEELING

- FOR EASE OF CARGO LOADING

- CAN KNEEL INDIVIDUALLY

- CAN BE KNEELED INFLIGHT

- JACK SCREWS RAISE AND LOWER

BRAKES

- NORMAL MAIN GEAR BRAKES USE HYDRAULIC SYSTEM #4

- ALTERNATE MAIN GEAR BRAKES USE HYDRAULIC SYSTEM #1

- EMERGENCY BRAKES USE ACCUMULATOR PRESSURE

- FULLY CHARGED ACCUMULATOR3 BRAKE APPLICATIONS

- EMERGENCY BRAKE ACCUMULATOR CHARGED BY #4 HYD SYS

- EMERGENCY BRAKES SELECTED, EMERGENCY HYDRAULIC PRESSURE LIGHT ON WHEN PRESSURE IS GREATER THAN 1300 LBS

# LANDING GEAR SYSTEM

## PAGE 4

## INFLIGHT BRAKE CHECK

- PERFORM ANY TIME A MLG CB IS OPENED OR THE MLG IS EXTENDED WITH THE EMERGENCY EXTEND SWITCHES

- DO NOT PERFORM THE CHECK IF EMERGENCY BRAKES ARE SELECTED

ANTI-SKID

- MAX BRAKING EFFICIENCY

- CAN USE WITH EITHER NORMAL OR ALTERNATE BRAKES

- EMERGENCY BRAKES – NO ANTI-SKID AVAILABLE

ANTI-SKID WARNING LIGHT

- 1 OR MORE PAIR OF BRAKES FREE WHEEL FOR MORE THAN 3 SEC CAUSES THE DET FAIL LIGHT TO ILLUMINATE

- LIGHT ILLUMINATES

- ANTI-SKID SWITCH OFF

- BRAKE SWITCH EMERGENCY

- 2 ELECTRIC POWER SOURCES  THAT SUPPLY THE ANTI-SKID

CONTROL BOX LOST

- ANTI-SKID TEST ARM AND #3 PUSHBUTTON ON

- ANTI-SKID SWITCH ON AND OVER VOLTAGE EXISTS

COLD WEATHER OPERATIONS

- DO NOT TAKEOFF WITH MORE THAN ½ INCH OF SLUSH OR WATER ON THE RWY OR MORE THAN 3 INCHES OF DRY SNOW

- IF SLUSH OR WATER ON THE RWY WITH TAKEOFF PERMITTED, DELAY THE GEAR RETRACTION FOR 2 MINUTES

- AT TEMPERATURES BELOW –25 DEGREES FOR 6 HOURS, MLG RETRACTION TIME = 40 SEC AND NLG RETRACTION TIME = UP TO 2 MINUTES

- COLD SOAKS FOR 16 HOURS BELOW –25 DEGREES REQUIRE COMPLETE INSPECTION FOR HYDRAULIC LEAKS UPON PRESSURIZATION

- AT TAXI SPEEDS > 5 KTS SKIDDING CAN BE EXPECTED

- NOSE WHEEL INPUTS > 20 DEGREES

- POWER BACK OF AFT MAIN GEAR

# FLIGHT CONTROLS

## ROLL CONTROL

- AILERONS AND SPOILERS

- MOVEMENTS ARE TRANSFERRED BY CABLES, QUADRANTS,

#### PUSH PULL RODS AND LINKAGES

- INTERCONNECT PINS BETWEEN CONTROL WHEELS

- CAN SHEAR PINS IN CASE OF EMERG PERMITTING USE OF ONE

## AILERON

- LEFT AILERON POWERED BY HYDRAULIC SYSTEM #1, #2

- RIGHT AILERON POWERED BY HYDRAULIC SYSTEM #2, #4

- MAX AILERON TRAVEL25 DEGREES UP AND 15 DEGREES DN

- AILERON SYSTEM OFF LIGHTS 1450 (+/- 200) PSI

FLIGHT SPOILERS

- NINE SPOILER PANELS ON EACH WING

- FLIGHT SPOILERS5 OUTBOARD

- GROUND SPOILERS4 INBOARD

- SPOILER CAUTION LIGHTS ON1450 (+/- 200) PSI

AILERON INTERCONNECT SHEAR PROCEDURE

- DISENGAGE AUTOPILOT

- REDUCE AIRSPEED BELOW 250 KTS

- TURN BOTH CONTROL WHEELS TOWARD THE CENTERLINE

- EASY TOO OVER CONTROL AFTER SHEAR IF RIGHT WING IS OPERABLE SYSTEM DUE TO PACKS AND FEEL SPRING BEING ON PILOTS SIDE

SPOILER RATIO SHIFTER

- LIMITS SPOILER EXTENSION WHEN FLAPS ARE LESS THAN 80%

- A SPOILER RATIO SHIFTER LIGHT DURING APP/LANDING WITH FLAPS > 80%, PLACE BOTH RATIO SHIFTER SWITCHES TO FLAPS DOWN, LIGHT SHOULD GO OUT. IF LIGHTS REMAIN ON PLACE BOTH RATIO SHIFTER SWITCHES IN THE UP POSITION

- A SPOILER RATIO SHIFTER LIGHT WITH FLAPS < 80%, PLACE BOTH RATIO SHIFTER SWITCHES TO THE UP POSITION, THE LIGHTS SHOULD GO OUT. IF NOT PLACE BOTH SWITCHES TO THE FLAPS DOWN POSITION.

AILERON TRIM

- MAX AILERON DEFLECTION USING TRIM10 DEGREES

- NOT AVAILABLE IN 7 BUS OR 3 BUS OPERATIONS

# FLIGHT CONTROLS

## PAGE 2

PACKS (PILOT ASSIST CABLE SERVO)

- ASSISTS IN OVERCOMING CONTROL WHEEL BREAKOUT FORCE

- INITIAL RESISTANCE TO MOVEMENT IS REDUCED BY 40%

- NOT AVAILABLE IN 7 BUS OR 3 BUS OPERATION

ALDCS

- REDUCES WING GUST LOADS

- FLAPPING MOVEMENTS

- USES AILERONS TO DAMPEN MOVEMENT AUTOMATICALLY

- DISENGAGES WHEN PITCH OR LATERAL AUG DISENGAGES

PITCH CONTROL

- INBOARD AND OUTBOARD ELEVATORS

- TRIM PROVIDED BY FULLY MOVABLE HORIZONTAL STAB

- INPUTS TRANSFERRED BY CABLES,QUADRANTS, PUSH PULL RODS AND LINKAGES

- ALSO HAS INTERCONNECT ROD SHEAR PINS

- INBOARD ELEVATORS MOVED BY TWO ACTUATORS

- ONE ACTUATOR POWERED BY HYD SYS #2

- ONE ACTUATOR POWERED BY HYD SYS #3

- OUTBAORD ELEVATORS MOVED BY 3 ACTUATORS

- ONE ACTUATOR POWERED BY HYD SYS #1

- TWO ACTUATORS POWERED BY HYD SYS #3

- MAX ELEVATOR TRAVEL 25 DEGREES UP AND 15 DEGREES DOWN

- HYDRAULIC SYSTEM OFF LIGHTS 1450 (+/- 200) PSI

ELEVATOR INTERCONNECT SHEAR PROCEDURE

- DISENGAGE AUTOPILOT

- REDUCE AIRSPEED BELOW 250 KTS

- SHUTOFF #3 HYDRAULIC POWER TO BOTH OUTBOARD ELEVATORS

- INBOARD ELEVATORS POWERED ONLY BY ONE SYSTEM

- DO SHEAR OUT BY HOLDING RIGHT CONTROL COLUMN FIXED AND PULL ON LEFT COLUMN

- IF IT DOESN’T WORK REVERSE PROCEDURE

- REACTIVATE #3 OUTBOARD ELEVATOR HYDRAULICS

# FLIGHT CONTROLS

## PAGE 3

PITCH TRIM

- HORIZONTAL STAB MOVED BY TWO HYD JACKSCREW MOTORS

- #2 HYD SYS POWERS NUT DRIVE FOR NORMAL & MANUAL TRIM

- #1 HYD SYS POWERS SCREW DRIVE FOR ALTERNATE TRIM

- MANUAL PITCH TRIM AVAILABLE IN 7 BUS AND 3 BUS OPS

- AUTOPILOT USES ALTERNATE PITCH TRIM

- AUTO TRIM FAULT WITH AUTOPILOT ENGAGED INDICATES POSSIBLE ALTERNATE PITCH TRIM FAILURE

- PITCH TRIM DISCONNECT BUTTON ON CONTROL WHEEL WILL DISCONNECT BOTH NORMAL AND ALTERNATE PITCH TRIM

PACKS

- OVERCOME CONTROL BREAKOUT FORCE

- REDUCES FORCE BY 35%

- NOT AVAILABLE IN 7 BUS OR 3 BUS OPERATION

ELEVATOR ARTIFICIAL FEEL

- BOB WEIGHTS – OPPOSING FORCE

- CENTERING SPRINGS

- VARIABLE FEEL UNIT APPLIES OPPOSING FORCE TO AIR IMPACT

- USES HYDRAULIC SYSTEMS #1 AND#4

DIRECTIONAL CONTROL

- UPPER AND LOWER RUDDERS

- NORMALLY NOT USED DURING FLIGHT DUE TO FLIGHT AUGMENTATION SYSTEM

- USES CABLES, QUADRANTS, PUSH PULL RODS AND LINKAGES

- NO SHEAR PINS

- UPPER RUDDER POWERED BY HYD SYS #1 AND #3

- LOWER RUDDER POWERED BY HYD SYS #2 AND #3

- SYSTEM OFF LIGHTS ON BELOW 1450 (+/- 200) PSI

RUDDER LIMITER

-     PREVENT DAMAGE TO AIRFRAME BY LARGE INPUTS AT HIGH

## SPEEDS

- BELOW 153 KTS35 DEGREES DEFLECTION

- 153 – 237 KTS12 DEGREES DEFLECTION

- 238 KTS AND ABOVE4 DEGREES DEFLECTION

RUDDER FEEL AND TRIM PROVIDED BY FEEL AND CENTERING SPRINGS

# FLIGHT CONTROLS

## PAGE 4

EMERGENCY RUDDER CONTROL PROVIDED BY YAW AUG SYSTEM

- LIMITED TO 20 DEGREES DEFLECTION FROM CURRENT TRIMMED POSITION

AFCS (AUTOPILOT)

- PITCH, ROLL, AND ALTITUDE CONTROL

- HAS AUTO THROTTLES AVAILABLE

- REQUIRE PITCH, YAW AND LATERAL AUGMENTATION FOR OPERATION

FLIGHT AUGMENTATION

- PROVIDES STABILITY IN PITCH, YAW, AND ROLL

- PROVIDES YAW AND ROLL DAMPING

- PROVIDES TURN COORDINATION

- MAX FLIGHT CONTROL TRAVEL AUTHORITY

- ELEVATOR10 DEGREES

- RUDDER20 DEGREES

- AILERON15 DEGREES

- PACKS USED TO HELP MOVE AILERONS AND ELEVATORS

- AVAILABLE DURING 7 BUS BUT NOT 3 BUS OPERATIONS

- FIRST STEP IN ALL AUGMENTATION FAILURES IS TO ATTEMPT TO RESET THE SYSTEM

DUTCH ROLL RECOVERY

- USE SMALL AILERON INPUTS

- DO NOT USE RUDDER

- REDUCE AIRSPEED AND ALTITUDE

GO AROUND ATTITUDE SUBSYSTEM

- 2 CHANNEL COMPUTER WITH SELF TEST SYSTEM

- PROVIDES PITCH COMMANDS FOR BEST ANGLE OF CLIMB

- ROTATION

- TAKEOFF

- CLIMB OUT

- GO AROUND

# FLIGHT CONTROLS

## PAGE 5

FLIGHT CONTROL FAILURES

- FAILURE OF OR TURNING OFF BOTH SYSTEMS ON A SINGLE

## AILERON WILL CAUSE THE AILERON TO FLOAT PRODUCING A

MILD ROLL

- FAILURE OR TURNING OFF BOTH SYSTEMS POWERING THE

## RUDDER WILL CAUSE IT TO FLOAT TO NEUTRAL

-   PLACE THE RUDDER LIMITER TO MIN Q TO OBTAIN

## MAXIMUM (35 DEGREES) DEFLECTION

-   BOTH UPPER AND LOWER RUDDER INOPERATIVE RESULTS

IN A REDUCED AUTHORITY IN THE LANDING

CONFIGURATION

UNCOMMANDED FLIGHT CONTROL INPUTS

- DISENGAGE THE FLIGHT AUGMENTATION SYSTEM

- DETERMINE THE MALFUNCTIONING SYSTEM

-   ENGAGE SYSTEMS ONE AT A TIME TO DETERMINE

THE SOURCE OF THE PROBLEM

- DEACTIVATE THE MALFUNCTIONING SYSTEM

HIGH CONTROL FORCES INFLIGHT IN THE PITCH AXIS

- DISENGAGE THE AUTOPILOT

- AFCS MASTER POWER OFF

- TURN OFF THE PITCH PACKS

- TURN OFF ELEVATOR FEEL

- ISOLATE THE MALFUNCTIONING SYSTEM

# WING FLAPS AND SLATS

## COMPONENTS

- FLAPS RIDE ON RAILS ON TRAILING EDGE OF WING

- SLATS ON LEADING EDGE

- CONNECTED TO GEARBOX POWERED BY TWO HYD MOTORS

- HYDRAULIC MOTORS POWERED BY HYD SYS #1 AND #4

- GEARBOX DRIVES TORQUE TUBES IN EACH WING

- TORQUE TUBES DRIVE JACK SCREWS TO MOVE FLAPS & SLATS

- FLAPS AND SLATS EXTEND OR RETRACT IN 29 SEC

## SLAT DECOUPLER

- CONNECTS FLAP AND SLAT TORQUE TUBES

- IF SLATS BIND IT DISCONNECTS SLATS ALLOWING NORMAL OPERATION OF FLAPS

- IF SLATS DECOUPLE INFLIGHT DO NOT RECOUPLE

SLAT CLUTCH AND BRAKE ASSEMBLY

- HOLDS SLATS IN PLACE ONCE POSITIONED

- USES HYDRAULIC SYSTEM #1 AND #4

ASYMMETRY DETECTOR

- PREVENTS ONE WINGS TORQUE TUBES FROM TURNING FASTER

## THAN OTHER WING

- IF ASYMMETRICAL CONDITION DETECTED BRAKES LOCK FLAPS AND SLATS IN POSITION

SLAT DRIVE DISCONNECT SWITCH

- OVERHEAD CONSOLE

- USED TO DISCONNECT SLATS FROM FLAP DRIVE SYSTEM

ASYMMETRICAL FLAP OR SLAT INDICATION

- CHECK ASYMMETRY AND BRAKE ON LIGHTS

- BOTH LIGHTS ON INDICATE ASYMMETRY SYSTEM HAS CLOSED THE SOLENOID-OPERATED SHUTOFF VALVE AND ENGAGED THE TORQUE BRAKES

- ASYMMETRY LIGHT IS ALWAYS ACCOMPANIED BY A BRAKE LIGHT

- BRAKE LIGHT MAY COME ON ALONE

# WING FLAPS AND SLATS

PAGE 2

FLAP AND SLAT MALFUNCTION LIGHTS

- LE SLAT OR TE FLAP ASYMMETRY LIGHT ON

## -  THE SYSTEM HAS DETECTED AN ASYMMETRICAL

## CONDITION

- LE SLAT OR TE FLAP LIGHT ON

-  THE ASSOCIATED BRAKE IS ENGAGED

- SLAT FLAP SYNC LIGHT ON

-  FLAPS HAVE REACHED 40% AND SLATS ARE NOT FULLY

## EXTENDED

-  FLAPS ARE UP & THE SLATS ARE NOT FULLY RETRACTED

WITH THE ASYMMETRY AND BRAKE LIGHTS ON AND FLAPS AND SLATS NOT FULLY EXTENDED OR RETRACTED RESET MAY BE POSSIBLE, SEE SECTION III PROCEDURES

GROUND SPOILERS

- ALL 9 DEPLOYED ON GROUND

- REDUCE WINGS LIFT AND INCREASE BRAKE EFFICIENCY

- DEPLOYED ANYTIME MLG ARE ON THE GROUND

STALLIMETER SYSTEM

- 2 INDEPENDENT SYSTEMS

- STALLIMETER COMPUTER HAS 2 CHANNELS: PILOT / CO-PILOT

- 2 INDICATIONS

- WARNING HORN IF ANGLE OF ATTACK EXCEEDS

## PROGRAMMED VALUE

- CONTROL STICK SHAKER IF ANGLE OF ATTACK EXCEEDS

SET VALUE

GPWS (GROUND PROXIMITY WARNING SIGNAL)

- WARNS OF POTENTIALLY DANGEROUS FLIGHT PATH

- THE FOLLOWING ACTIVATE

- EXCESSIVE DESCENT RATE BELOW 2500 FT

- EXCESSIVE CLOSURE RATE WITH THE GROUND

## - NEGATIVE CLIMB

- IMPROPER GEAR AND FLAP CONFIGURATION

- EXCESSIVE GLIDE SLOPE DEVIATION

- DECISION HEIGHT

- NINE AURAL WARNINGS

- 2 LIGHTS PULL-UP AND BELOW GLIDE SLOPE –1.3 DOT

- CORRECTIVE ACTION FOR NUISANCE WARNINGS IS TO PULL THE GPWS CIRCUIT BREAKER

- INHIBIT IS USED TO DISABLE FLAP WARNINGS

# FUEL SUPPLY SYSTEM

##### FUEL SYSTEM COMPONENTS

- 12 WING TANKS

- TOTAL FUEL 49,626 GALS / 332,500 LBS

- TANKS INTERNALLY SEALED

- 4 MAIN TANKS

- DIRECT FUEL SUPPLY TO ENGINES THRU 2 BOOST PUMPS

- 4 AUXILIARY TANKS

- 4 EXTENDED RANGE TANKS

- FUEL NORMALLY TRANSFERRED -  AUX/EXTMAINENGINE

- FUEL CAN BE DIRECTLY SUPPLIED FROM ANY TANK TO ENGINE

- FUEL CAN BE TRANSFERRED FROM ANY ONE TANK TO ANY OTHER TANK

- #2 AND #3 MAIN TANKS (INBOARDS) SUPPLY FUEL TO APU’S

BOOST PUMPS

- IN TANKS AND PROVIDE FUEL PRESSURE TO THE ENGINE OR TRANSFER FUEL

- TANKS ARE VENTED THOUGH A COMMON VENT SYSTEM

- ALSO USED TO JETTISON FUEL

- OUTBOARD PUMP IS PRIMARY

PUMP SUMP BOX

- CONTAINS THE 2 BOOST PUMPS

- PROVIDES A POSITIVE SUPPLY OF FUEL TO BOOST PUMPS

- KEPT FULL BY A ONE WAY FLAPPER VALVE

FUEL SCAVENGE EJECTORS

- SCAVENGE RESIDUAL FUEL AND DIRECT IT TO THE SUMP BOX

FUEL MANIFOLD

- SUPPLY FUEL TO ENGINE

- TRANSFER FUEL FROM TANK TO TANK

- CROSS FEED FROM TANK TO ENGINE

- REFUEL AND DEFUEL

- JETTISON FUEL

- AERIAL REFUEL

# FUEL SUPPLY SYSTEM

## PAGE 2

FUEL VALVES

- FILL VALVES – EXT TANKS AND AUX TANKS MAIN TANKS

- ISOLATION VALVES – EXT TANKS AND AUX TANKS ENGINE IN CASE OF MAIN TANK BOOST PUMP FAILURE

- SEPARATION VALVES – USED TO JETTISON OR TRANSFER FUEL

- CROSSFEED VALVES – CROSSFEED FROM TANK TO ENGINE

- FUEL SHUTOFF VALVES – ISOLATE MAIN FUEL TANK FROM ENGINE WHEN FIRE HANDLE PULLED

SUCTION FEED CAN ONLY BE ACCOMPLISHED FROM THE MAIN TANKS

FUEL TANK VENTING

- PRIMARY AND SECONDARY CLIMB AND DIVE  VALVES

- MAINTAIN PROPER TANK PRESSURE

BOOST PUMPS

- DELIVER FUEL UNDER PRESSURE TO THE ENGINES

- JETTISON OR TRANSFER FUEL

- 2 ELECTRIC SUCTION BOOST PUMPS - #1 SYS AND #4 SYS

FUEL JETTISON MASTS ARE ON TRAILING EDGE OF WING

-     WITH ALL BOOST PUMPS OPERATING9000 LBS/MIN

FUEL TANK PRESSURIZATION

- USES NITROGEN GAS

- PROVIDES A SLIGHT POSITIVE PRESSURE

- PRESSURED THROUGH THE VENT SYSTEM

- NITROGEN STORED IN TWO DEWERS

- HEAT EXCHANGERS CONVERT LIQUID NITROGEN TO GAS

FUEL SCRUBBING

- REDUCES OXYGEN DISSOLVED IN FUEL

- PREVENTS RELEASE OF FLAMMABLE GASES

- MIXES NITROGEN WITH FUEL

REFUELING

- SINGLE POINT

- 2400 GAL / MIN AT 50 PSI + /- 5 (4 RECEPTACLES)

# FUEL SUPPLY SYSTEM

## PAGE 3

ENGINE DRIVEN FUEL PUMP

- BOOST IMPELLER ELEMENT DIRECTS FUEL UNDER BOOST

#### PRESSURE THE FUEL HEATERHIGH PRES ELEMENT

- HIGH PRESSURE ELEMENT DIRECTS FUELTHE  FUEL CONTROL

FUEL HEATER PREVENTS ICING IN THE FUEL SYSTEM

FUEL FILTER

- BETWEEN THE FUEL HEATER AND THE HIGH PRESSURE ELEMENT

- BYPASS VALVE ALLOW FUEL TO BYPASS FILTER AT A SET PRESSURE (CLOGGED FILTER)

FUEL CONTROL UNIT

- METERS FUEL AND PROVIDES ENGINE STABILITY

- LIMITS FUEL FLOW VARIATIONS

- ENSURES FAST STALL FREE ACCELERATION & DECELERATION

- SCHEDULES OPERATION OF THE VARIABLE COMPRESSOR VANES AND INLET GUIDE VANES

- TOTAL CONTROLPROVIDES OPTIMUM PERFORMANCE

FUEL OIL COOLER

- BASICALLY A HEAT EXCHANGER

- FUEL PASSING THROUGH COOLS OIL WHILE FUEL IS ALSO HEATED

FUEL DIFFERENTIAL PRESSURE LIGHT

- FUEL FILTER BECOMES CLOGGED AND BYPASS VALVE OPENS

- CAN BE A RESULT OF ICING OR CONTAMINATION

- PLACING THE FUEL HEATER SWITCH ON FOR 1 MIN SHOULD CAUSE THE LIGHT TO GO OUT IF ICING WAS THE CAUSE

PUMP OUT LIGHT

- FIRST STAGE FAILURE IF ENGINE CONTINUES TO RUN

- FUEL HEATER AND FILTER OPERATION LOST WITH FIRST STAGE FAILURE

SUMP LOW WARNING LIGHTS

- ON WHEN FUEL QUANTITY IN MAIN OUTBOARD TANK DROPS

## BELOW 1600 LBS

- SLOW TO 280 KTS OR .825M TO PREVENT ADVERSE WING LOADING

# FUEL SUPPLY SYSTEM

PAGE 4

FUEL QUANTITY INDICATOR FAILURE

- DO NOT MOVE OR DISCONNECT ANY INDICATOR IN FLIGHT

- ENROUTE MAY CONTINUE MAY CONTINUE MISSION WITH NO MORE THAN ONE FUEL INDICATOR PER WING INOPERATIVE AND THE SYMMETRICALLY OPPOSITE INDICATOR OPERATIVE

- DO NOT RESET FUEL SYSTEM ASSOCIATED CB’S

# DEFENSIVE SYSTEMS

## MWS

- CONTROLLED BY THE PILOT

- VISUAL AND AUDIO ALARM

- AUTOMATIC EJECTION OF FLARES

CMDS

- CONTROLLED BY FLIGHT ENGINEER

- MANUAL EJECTION OF FLARES

CONTROL INDICATOR MOUNTED ON PILOTS GLARESHEILD

4 OPTICAL SENSORS

- PROVIDE 360 DEGREE DETECTION

- 2 IN NOSE CONE

- 2 IN TAIL CONE

PISTOL GRIP FLARE DISPENSERS

- 1 IN FLIGHT ENGINEER STATION

- 2 AT CARGO DOORS 7L AND 7R

SQUIB POWER RELAYS PREVENT FIRING ON THE GROUND

# SECONDARY FLIGHT DISPLAY SYSTEM

## PROVIDES ATTITUDE, ALTITUDE AND AIRSPEED INDICATIONS

COMPONENTS

- STANDBY ATTITUDE INDICATOR

- AIR DATA UNIT

REQUIRES PITOT AND STATIC PRESS, AND 28V DC TO OPERATE

- POWER FROM 28V DC EMERGENCY BUS

- BACKUP POWER BY SAI BACKUP CB VIA SAI BACKUP RELAY

BUILT IN TEST

- PBIT ON POWER UP

- CBIT CONTINUOUS UNTIL POWER REMOVED

BRIGHTNESS CONTROLLED BY AMBIENT SENSORS AUTOMATICALLY

OR BY PILOTS INST PANEL LIGHTING

SAI

- 360 DEGREES ROLL, BANK MARKED IN 10, 20, 30, 45, & 60 DEGREES

- 50 DEGREES OF PITCH, MARKED IN 5 DEGREE INTERVALS

ALTITUDE PRESENTATION

- RIGHT SIDE DIGITAL READOUT

- TAPE DISPLAY IN 1000 FT INTERVALS

- METRIC ALTITUDE DISPLAY IN LOWER LEFT CORNER

BAROMETRIC PRESSURE

- LOWER RIGHT CORNER IN INCHES OF MERCURY

- UPPER RIGHT CORNER IN MILLIBARS OF MERCURY

ATT BUTTON

- ALIGN DISPLAY ATTITUDE

- DEPRESS THEN RELEASE

- NOT TO BE RESET UNLESS 7 DEGREE OR > ERROR WHEN COMPARED TO PRIMARY ADI OR HORIZON

# SECONDARY FLIGHT DISPLAY SYSTEM

## PAGE 2

AIRSPEED INDICATION

- LEFT CENTER OF DISPLAY

- TAPE DISPLAY 80 KTS RANGE / 10 KT INCREMENTS

- MACH INDICATION IN UPPER LEFT CORNER

- RED WARNING STRIP ON AIRSPEED TAPE WHEN APPROACHING MAX SPEED

ON START 3 MIN INITIALIZATION PERIOD

- REQUIRES BANK < 7 DEG, PITCH < 7 DEG, YAW< 2.5 DEG

# TRAFFIC ALERT & COLLISION AVOIDANCE SYSTEM

( TCAS )

SELF CONTAINED AVOIDANCE SYSTEM THROUGH USE OF MODE S TRANSPONDER

TCAS FUNCTIONS

- DETERMINES IF THREAT EXISTS

- TRACK AND DISPLAY THREATS, PROXIMITY TRAFFIC, AND NON THREAT TRAFFIC

- CALCULATE VERTICAL EVASIVE MANEUVERS

- PROVIDE AUDIO ADVISORIES

- COORDINATE EVASIVE MANEUVERS BETWEEN TWO OR MORE TCAS AIRCRAFT

TCAS HORIZONTAL SURVEILLANCE  AREA

- 80 NM FORWARD

- 15 NM EITHER SIDE

- 12 NM BEHIND

VERTICAL SURVEILLANCE AREA + /- 9000 FT

TCAS WILL TRACK 50 INTRUDERS, BUT WILL ONLY DISPLAY TEN HIGHEST THREATS

TRAFFIC CLASSIFICATIONS

- NON-THREAT, PROXIMITY, TRAFFIC ADVISORY, & RESOLUTION

TCAS WILL ISSUE VISUAL ANS AUDIO CLIMB AND DESCENT COMMANDS

DISPLAY WILL SHOW OTHER AIRCRAFTS

- RANGE AND BEARING

- RELATIVE ALTITUDE

- VERTICAL TREND IF > 500 FT / MIN

- ONLY WORKS FOR TRANSPONDER EQUIPPED AIRCRAFT

- FOR NON ALTITUDE REPORTING TRAFFIC TCAS PROVIDES ADVISORIES ONLY

VSI / TRA DISPLAY

- MAXIMUM TWELVE AIRCRAFT

- TRAFFIC INFORMATION TIME AND ALTITUDE

- VERTICAL SPEED FOR RA’S

CONTROL TCAS THROUGH FMS / CDU IFF PAGE

# TRAFFIC ALERT & COLLISION AVOIDANCE SYSTEM

## PAGE 2

## TWO ANTENNA’S TOP AND BOTTOM OF AIRCRAFT

MODE S PROCESSOR LOCATED IN AVIONICS BAY 3

TCAS MODE S PROCESSOR

- SURVEY AIRSPACE

- DETECT RANGE, BEARING, AND CLOSURE RATE

- COMPUTE VERTICAL EVASIVE MANEUVERS

- ISSUE AND DISPLAY RA’S

NON THREAT TRAFFIC

- SYMBOL – OPEN CYAN DIAMOND

- GREATER THAN 1200 FT ABOVE OR BELOW

- GREATER THAN 6 MILES

PROXIMITY TRAFFIC

- SYMBOL – CLOSED CYAN DIAMOND

- WITHIN + / - 1200 FT

- CLOSER THAN SIX MILES

TRAFFIC ADVISORY

- SYMBOL – SOLID YELLOW CIRCLE

- TIME TO CPA 20 TO 48 SEC

RESOLUTION ADVISORY

- SYMBOL – SOLID RED SQUARE

- TIME TO CPA 15 TO 35 SEC

- TCAS CAN RESOLVE UP TO THREE SIMULTANEOUS RA’S

- INTRUDER’S ALTITUDE IS PROVIDED BELOW SYMBOL IF REPORTING

- TREND ARROW TO RIGHT OF SYMBOL OF CLIMB OR DESCENT IS GREATER THAN 500 FT / MIN

VERTICAL DISPLAY

- OTHER THAN NORMAL SELECTED DISPLAYED IN LOWER LEFT

## CORNER

- NORMAL – 2700 FT ABOVE AND BELOW

- ABOVE – 2700 FT BELOW AND 9000 FT ABOVE

- BELOW – 2700 FT ABOVE TO 9000 FT BELOW

# TRAFFIC ALERT & COLLISION AVOIDANCE SYSTEM

PAGE 3

FLAGS

- TCAS OFF – STANDBY MODE

- TA ONLY – RA’S INHIBITED ( IE FORMATION)

- TCAS FAIL – UNUSABLE

- VSI FAIL – LOSE VERTICAL SPEED INPUT, NO RA’S

- RA FAIL – SITUATIONAL AWARENESS ONLY

- TD – NO TRAFFIC DISPLAY

TCAS ADVISORY RESTRICTIONS

- 1450 FT AGL – INCREASE DESCENT RA’S INHIBITED

- 1100 FT AGL – DESCENT RA’S INHIBITED ON TAKEOFF

- 900 FT AGL – DESCENT RA’S INHIBITED

- 500 FT AGL – ALL RA’S INHIBITED

# FMS / GPS

## EQUIPMENT REPLACED

- INS CDU’S, FSAS / INS CDU

- IFF

- UHF AND VHF / NAV COMM PANELS

- TACAN CONTROL PANELS

- INS STATUS PANELS (PILOT AND CO-PILOT INST PANELS)

EQUIPMENT ADDED

- CDU’S

- FMS POWER CONTROL PANEL

- COMM / NAV CONTROL PANEL

- DATA LOADER AND CARTRIDGE

- BSIU’S (BUS SUBSYSTEM INTERFACE UNITS)

- GPS RECEIVERS

FMS POWER CONTROL PANEL

- TURN ON COMPONENTS

- TURN ON LEFT TO RIGHT AND REVERSE FOR OFF

FMS AUXILIARY CONTROL PANEL

- FSAS ENGAGE PUSHBUTTON

- - ENGAGE FSAS FLIGHT MODES

- FMS HEADING PUSHBUTTON

- - ALLOWS SELECTION OF TRUE OR MAGNETIC HEADING

## FOR CDU’S AND FLIGHT INSTRUMENTS

- FMS BUS SPLIT SWITCH

- - LAST RESORT SWITCH IF DATA BUS 1553 LOC KS UP

- WILL SPLIT BUS INTO TWO COMPONENTS

- RESTORES PARTIAL CAPABILITIES

- ALLOWS  FOR TROUBLE SHOOTING

- #1 & #3 CDU ON ONE BUS, AND # 2 CDU ON THE OTHER

COMM / NAV VOLUME CONTROL PANEL

- PROVIDES CENTRAL CONTROL FOR COMM / NAV VOLUME

FMS/INS STATUS PANEL

- UPCOMING WAYPOINT ALERTS

- CDU MESSAGE ALERTS

- RAIM ALERTS

- INS HEADING AND ATTITUDE WARNINGS

# FMS / GPS

## PAGE 2

CDU’S

- PRIMARY INTERFACE WITH FMS SYSTEM

- ALPHA NUMERIC AND FUNCTION KEYS

- ARROW (SCROLL KEYS) AND LINE SELECT (LS) KEYS

- DISPLAY

- 8 LINES, 22 CHARACTERS, 19 FOR THE SCRATCHPAD

- LINES 1, 3, 5, 7 LS KEYS

- LINE 2-TITLE, LINE 4-INFORMATION

- LINE 6-ANNUNCIATOR(STATUS), LINE 8-SCRATCHPAD

- ANY SINGLE CDU CAN RUN SYSTEM

- FIRST CDU POWERED UP IS BUS CONTROLLER

- BUS CONTROLLER

- DOES ALL COMPUTATIONS AND PAGE DISPLAYS

- COMMUNICATE WITH ALL EXTERNAL EQUIPMENT

- REMOTE CDU’S

- PROCESS KEY STROKES

- DISPLAY BUS CONTROLLER BUILT PAGES

- BACKUP TO BUS CONTROLLER CDU

BUS SUBSYSTEM INTERFACE UNITS

- INTERFACE WITH CDU’S AND NON DATA BUS (1553) RADIOS AND AIRCRAFT SYSTEMS

- PROVIDE NUMBER CRUNCHING FOR INU / GPS INTEGRATED NAVIGATION SOLUTION

DATA LOADER AND CARTRIDGE

- STORE AND LOAD WORLDWIDE NAVIGATION DATA (GPS ALMANAC)

- MAGNETIC VARIATION TABLES AND REFERENCE DATABASE

- ALTERNATE FLIGHT PLAN DATABASE (UP TO 40)

- 60,000 WAYPOINTS FROM RAPID DATA BASE

- 10,000 (SIDS,STARS,GPS APPROACHES AND AIRPORTS)

- 40,000 RUNWAYS FOR VISUAL APPROACHES

- CAN CONTAIN 40 FLIGHT PLANS AND GPS ALMANAC

- 8 VOR/LOC PRESETS, 28 VHF AND 28 UHF PRESETS

# FMS / GPS

PAGE 3

GPS RECEIVER

- ONE LOCATED IN EACH CDU

- 2 GPS’S

- CAN TRACK 5 SATELLITES AT A TIME

- 4 SATELLITES REQUIRED FOR 3 DIMENSIONAL POSITION

- 5TH SATELLITE CHECKS INTEGRITY OF OTHER FOUR

- RECEIVER WILL PROVIDE A RAIM LIGHT IF IT SUSPECTS A NAVIGATION SOLUTION IS INACCURATE DUE TO BAD SATELLITE DATA

GPS ANTENNA

- 2 160 DEGREE OMNI DIRECTIONAL ON TOP OF AIRCRAFT

ANTENNA ELECTRONICS

- SIGNAL PRE-AMPLIFICATION

- NULL GENERATION (BLOCK SECTIONS TO LIMIT INTERFERENCE)

KYK-13 DECODES HIGH ACCURACY P CODE SIGNAL

CDU KEYS

- FUNCTIONCALL UP SPECIFIC TOP LEVEL PAGES

- LINE SELECTACCESS TO LOWER LEVEL PAGES, TOGGLE BETWEEN MODES, ENTER DATA IN A FIELD, AND COPY DATA TO THE SCRATCHPAD

- SCROLL ARROWSCROLL THROUGH PAGE DISPLAYS

- ALPHANUMERICINPUT DATA

UDID (USER DEFINED IDENTIFIERS)

- ASSIGNED TO WAYPOINTS

- ENTER A ( / ) FOLLOWED BY UP TO 9 CHARACTERS

- UDID WILL DISPLAY RATHER THAN ORIGINAL WAYPOINT

I-NAV SOLUTIONS

- GPS, INU1, INU1/GPS, INU2, INU2/GPS, INU3, AND INU3/GPS

DIFFERENCE BETWEEN INS AND INS/GPS SOLUTIONS FOLLOWING LOSS GPS

-     INU OUTPUTS ARE DRIFT CORRECTED USING DRIFT ERRORS COMPUTED WHILE GPS DATA WAS AVAILABLE

# EMERGENCY PROCEDURES

## BASIC RULES

- MAINTAIN AIRCRAFT CONTROL

- ANALYZE THE SITUATION AND TAKE PROPER ACTION

- LAND AS SOON AS CONDITIONS PERMIT

AFTER COMPLETING EMERGENCY PROCEDURES

- DECLARE THE EMERGENCY WITH ATC / COMMAND POST

- INCLUDE THE FOLLOWING:

- DESCRIPTION AND EXTENT OF EMERGENCY

- ASSISTANCE REQUIRED

- PERSONS ON BOARD

- FUEL ON BOARD AND LANDING FUEL

- INTENTIONS

COORDINATE TRIM ACTUATION WITH THE CO-PILOT

ON LANDING ONLY EXCEED REV IDLE WITH SYMMETRICAL ENGINES

LANDING DISTANCE IS NORMALLY COMPUTED WITH OUT USING REVERSE AS A FACTOR

IF HYDRAULIC SYSTEM IS POWERED BY THE PTU, MAKE SURE THE ATM IS ON FOR GEAR AND FLAP OPERATION

PLAN FOR LOSS OF 2ND ENGINE AND COMPUTE 100% FLAP VAPP AND 2 ENGINE VMCA

AT LOWER GROSS WEIGHTS THE 2 ENGINE VMCA CAN BE GREATER THAN 100% FLAP VAPP

EMERGENCIES THAT REQUIRE IMMEDIATE ENGINE SHUTDOWN

- ENGINE FIRE

- ENGINE FLAMEOUT FOR UNKNOWN REASONS

- EXCESSIVE FUEL FLOW

- PYLON FIRE ( USE PYLON FIRE CHECKLIST)

- UNCOMMANDED THRUST REVERSER INDICATIONS ( TR NOT LOCKED OR EXTENDED LIGHT) UNDER 250 KTS

- VISIBLE FLUIDS COMING FROM THE ENGINE OR PYLON THAT CAN’T BE STOPPED

- ENGINE DISINTEGRATION

# EMERGENCY PROCEDURES

PAGE 2

IF ENGINE FLAMES OUT OR IS SHUT DOWN

- RUN CHECKLIST

- NRT ON GOOD ENGINES

- ASK FOR MAX 3 ENGINE ALTITUDE FROM ENGINEER

- DECLARE EMERGENCY

- REQUEST DESCENT CLEARANCE IF REQUIRED

- REQUEST MAX RANGE FROM ENGINEER

WHEN SELECTING MIN Q WITH RUDDER LIMITER SWITCH

- YOU WILL HAVE FULL RUDDER TRAVEL – 35 DEGREES

- YOU WILL HAVE FULL RUDDER TRIM – 12 DEGREES

- BEFORE MOVING SWITCH CHECK THAT NO MORE THAN 4 DEGREES OF RUDDER TRIM ABOVE 238 KTS IS HAS BEEN INPUT

REQUIRED SHUTDOWNS USING PRECAUTIONARY ENGINE SHUTDOWN

- EXCESSIVE ENGINE VIBRATION ( IMMEDIATE SHUTDOWN)

- ENGINE OVERHEAT

- TIT OVER TEMPERATURE

- HIGH, LOW, OR NO OIL PRESSURE

- HIGH OIL TEMPERATURE

- N1 OR N2 OVERSPEED

- NACELLE BLEED DUCT FAILURE

- FAILURE OF BLEED DUCT VALVES TO CLOSE WITH DUCT OVERHEAT

- FAILURE OF CSD TO DISCONNECT OR RECONNECTS INFLIGHT

- START VALVE OPEN LIGHT REMAINS ON AFTER AIRSTART

- HYDRAULIC PRESSURE OVERPRESSURE

- UNCOMMANDED THRUST REVERSER EXTENSION OR FAILURE TO RETRACT ABOVE 250 KTS

CSD FAILURE INDICATIONS, CHECK N2 NOT BETWEEN 75% AND 80%

# EMERGENCY PROCEDURES

## PAGE 3

LANDING GEAR EXTENSION USING EMERGENCY EXTEND SWITCHES

- LANDING GEAR MAY NOT ROTATE ABOVE 185 KTS

- USE 40% FLAPS AND 100% FLAP APPROACH SPEED + 30 KTS

- MLG SHOULD INDICATE IN-TRANSIT WITHIN 5 SEC

- NLG SHOULD INDICATE IN-TRANSIT WITHIN 10 SEC

- BOTH SHOULD TAKE APPROXIMATELY 3 MIN TO EXTEND

- WHEN ACCUMULATOR PRESSURE IS USED FOR BOGIE ROTATION MLG WILL NOT ROTATE ABOVE 170 KTS

- NEVER DO A INFLIGHT BRAKE CHECK WHEN USING EMERGENCY BRAKES WITH LOSE OF NORMAL AND ALTERNATE BRAKING

- IF MLG EMERGENCY EXTEND SWITCH IS LEFT EXTENDED NORMAL AND ALTERNATE BRAKES, AND CASTORING ARE NOT AVAILABLE

- IF NLG EMERGENCY EXTEND SWITCH IS LEFT EXTENDED NORMAL NOS WHEEL STEERING WILL NOT BE AVAILABLE

PULLING THE ENGINE FIRE HANDLE

- CLOSES THE ENGINE FUEL SHUTOFF VALVE

- TURNS OFF THE IGNITION

- DE-ENERGIZES THE GENERATOR

- MECHANICALLY SHUTS OFF FUEL TO THE NACELLE BY CLOSING A SHUTOFF VALVE

- CLOSES BLEED AIR SHUTOFF VALVE IN THE PYLON

- CLOSE HYDRAULIC SHUTOFF VALVE AND DEPRESSURIZES THE HYDRAULIC PUMPS

- CLOSES VALVE IN TR HYDRAULIC PRESSURE LINE AND DE-ENERGIZES ALTERNATE RETRACT VALVE

- DE-ENERGIZES THE STARTER CONTROL VALVE CLOSED

- EXPOSES THE FIRE EXTINGUISHER DISCHARGE BUTTON

# EMERGENCY GENERATOR

## PARALLEL GENERATOR OPERATION - BUS TIES CLOSED

ISOLATED OPERATION – BUS TIES OPEN

WHEN EMERGENCY GENERATOR IS ON THE EMERGENCY BUS POWER RELAY IS ALWAYS DE-ENERGIZED

EMERGENCY GENERATOR OPERATION

- MANUALLY BY MOVING THE INST POWER SWITCH TO EMERG

- AUTOMATICALLY WHEN ALL FOUR GENERATORS ARE OFF LINE WHEN OPERATING IN PARALLEL MODE

- AUTOMATICALLY WHEN #1 AND #2 GENERATOR ARE OFF LINE WHEN OPERATING IN ISOLATED MODE

WHEN EMERGENCY GENERATOR IS OPERATING THE NAV AC BUS #1 IS ALWAYS DE-ENERGIZED

NAV AC BUS #1 OFFLINE YOU LOSE

- TACAN'S

- BDHI’S

- ADF’S

- AUTOPILOT

WHEN EMERGENCY GENERATOR IS SOLE SOURCE OF ELECTRICS (EXCLUDING BATTERY) = 7 BUS OPERATION

- ISOLATED AC AND DC BUSES

- AVIONICS ISOLATED AC AND DC BUSES

- EMERGENCY AC AND DC BUSES

- BATTERY BUS

# ELECTRICAL FIRE CHECKLIST

## PURPOSE IS TO DEPOWER BUSES TO ISOLATE FIRE

BEST OPTION= ISOLATE FIRE WITHOUT RUNNING CHECKLIST

BOLDFACE=”OXYGEN MASKS – ON”( LIMITED VALUE IF FUMES ARE TOXIC)

ATTEMPT TO GET TO VFR CONDITIONS

LAND THE AIRCRAFT AS SOON AS POSSIBLE

MAJOR PHASES

- PRIOR TO “GENERATORS – OFF”

- SELECT UHF #1 AS PRIMARY RADIO

- SELECT INS #3, PILOT SELECTED WILL BE POWERED BUT

## - OTHERS WILL BE ON BATTERY POWER

- LOSE HF RADIO’S

- DECLARE EMERGENCY

- GRAVITY FEED ALTITUDES

- LOSE CO-PILOT, NAV, AND JUMP INTERPHONE

- LIGHTS= PILOTS INST FLOOD, MAG COMPASS, CHART

- 7 BUS OPERATION= SEE HANDOUT

- PRIOR TO “ISOLATED BUS – OFF”

- RADIO TRANSMISSION, YOU WILL HAVE NO RADIO’S

- SELECT INS #3 OR #1, PILOT SELECTED WILL BE POWERED

## BUT OTHER TWO WILL BE ON BATTERY POWER

- 3 BUS OPERATION= SEE HANDOUT

TO NAVIGATE IN THREE BUS OPERATION

- INS #1 MUST BE SELECTED

- SELECT STR PAGE

- USE XTK ERROR AND DA / TKE(TRACK ANGLE ERROR) TO MAINTAIN COURSE

- LOSE ALL OTHER COMPASS INFORMATION

# 7 BUS OPERATION

PILOT WILL HAVE:

PILOTS ADI, HSI, AND RATE OF TURN

INTERPHONE FOR PILOT, FE, AFT AND TROOP

VHF #1

UHF #1

VHF NAV #1 / ILS #1

IFF

MANUAL PITCH TRIM ONLY

SELECTED INS #1 OR #3, OTHERS ON BATTERY

VSFI’S

ENGINE FIRE PROTECTION (NO DETECTION)

LEFT HAND AC AND FLOOR HEAT

ANTI-SKID BRAKES

MANUAL GEAR EXTENSION (BUTTON DOWN)

MASTER CAUTION WARNING SYSTEM

PILOT INST FLOOD, MAG COMPASS, & CHART LIGHTS

N2 INDICATIONS

RAT AUTO AND MANUAL CONTROL

APU FIRE DETECTION / PROTECTION

FLAPS AND SLATS (SLATS WILL DECOUPLE)

# 3 BUS OPERATION

## PILOT WILL HAVE

PILOTS ADI (OFF FLAGS SHOW), AND RATE OF TURN

INTERPHONE FOR PILOT, FE, AFT AND TROOP

IFF

MANUAL PITCH TRIM ONLY

SELECTED INS #1 OR #3

VSFI’S

ENGINE AND APU FIRE PROTECTION

LEFT HAND AC AND FLOOR HEAT

MANUAL GEAR EXTENSION (BUTTON DOWN)

PILOT INST FLOOD, MAG COMPASS,& CHART LIGHTS

RAT AUTO AND MANUAL CONTROL

FLAPS AND SLATS (SLATS WILL DECOUPLE)

# LANDING GEAR OPERATION

## MLG EXTENSION

- GEAR LEVER DOWN

- HYDRAULIC PRESSURE TO BOGIE ROTATION ACTUATOR TO HOLD BOGIE’S IN 90 DEGREE POSITION

- HYDRAULIC PRESSURE TO OUTBOARD DOORLOCK ACTUATORS, GEAR DOWNLOCK CYLINDER, AND GEAR DRIVE MOTORS

- GEAR DOWN HYDRAULIC PRESSURE TO DOWNLOCK ACTUATOR TO LOCK THE SIDE-BRACE

- DOWNLOCK SWITCH ACTUATED HYDRAULIC PRESSURE TO BOGIE ROTATION ACTUATOR

- FORWARD ROLLINGCOLLAR LOCK SEQUENCE VALVE MECHANICALLY OPENEDHYDRAULIC PRESSURE SUPPLIED TO INTERCOLLAR LOCK ACTUATOR

NLG EXTENSION

- GEAR LEVER DOWN

- HYDRAULIC PRESSURE UNLOCK DOOR UPLOCKS AND OPEN DOORS

- DOORS OPENPROXIMITY SENSORS ENERGIZE SOLENOID OPERATED VALVESHYDRAULIC PRESSURE TO NLG UPLOCKS AND HYDRAULIC DRIVE MOTOR

- DRAG BRACE UNLOCKED BY GEAR LOCK ACTUATOR

- FULLY EXTENDED GEAR LOCK ACTUATOR LOCKS DRAG BRACE

DO NOT ATTEMPT TO RETRACT THE LANDING GEAR WITH ANY OTHER

INDICATION THAN GREEN WHEELS

ALL MLG REMAIN GREEN WHEELS ON RETRACTION, WAIT 2 MIN FOR

WHEEL SPIN DOWN

IF EMERGENCY EXTENSION IS USED ABOVE 185 KTS MLG MAY FAIL TO

ROTATE

MLG WILL NOT ROTATE ABOVE 170 KTS USING ACCUMULATOR

PRESSURE FOR EXTENSION

# FIRE SUPPRESSION SYSTEM

## FIRE PREVENTION, DETECTION, AND SUPPRESSION

MAIN COMPONENTS

- FUEL TANK INERTING AND PRESSURIZATION SUBSYSTEM

- NITROGEN FIRE SUPPRESSION SUBSYSTEM

- PANEL LOCATED ON FE’S OVERHEAD PANEL, AND A PANEL LOCATED IN THE NOSE WHEEL WELL TO FIGHT SOME FIRES

FUEL TANK PRESSURIZATION SYSTEM

- USES LIQUID NITROGEN STORED IN TWO 750 LB DEWERS

- LIQUID NITROGEN IS CONVERTED TO A GAS BY HEAT EXCHANGERS IN OUTBOARD SECTION OF EACH MAIN FUEL TANK

- 11-2C-5, VOL 3 REQUIRES ALL FUNCTIONS OF FSS OPERATIONAL FOR FLIGHT

FIRE DETECTION AND PROTECTION

- FIRE DETECTION IS PROVIDED IN THE NITROGEN PROTECTED

## AREAS BY OPTICAL DETECTORS

- DISCHARGE OF NITROGEN LASTS FOR 45 SEC, ACCOMPANIED

#### BY A DISCHARGE LIGHT

- PUSHING ANY BUTTON TO THE RIGHT OR BELOW ONE ALREADY ACTIVATED ON THE PANEL CANCELS PREVIOUS ZONE SELECTIONS

- AUTOMATIC DISCHARGE IN ZONES 1 – 4

-   RIGHT AND LEFT INBOARD AND OUTBOARD WINGS

-   ARMED WHEN ANY OF THE 4 FUEL AND START IGNITION

SWITCHES ARE IN THE RUN OR AIR START POSITION

# WEATHER RADAR

## USED FOR WX AVOIDANCE

RANGE = 300 MILES

TRUE PICTURE OF CELLS WITHIN 100 NM

HAS CONTROL SWITCH FOR PILOT OR CO-PILOT CONTROL

WX RADAR TEST

- INS ON AND IN ALIGNMENT STATE 8 OR LESS

- WX RADAR CONTROL SWITCH TO PILOT OR CO-PILOT

- FUNCTION SELECTOR TO TEST

- TILT TO 0 DEGREES

- ANTENNA STAB ON

- GAIN TO AUTO

- RANGE SET TO 150 MILES

- TARGET CLEAR / INT TO MID RANGE

- SELECT NORMAL

- GREEN, YELLOW, RED, YELLOW, GREEN

- UNIFORM NOISE BAND80 TO 100 NM

- TEST IN UPPER LEFT CORNER

- 150 / 30 IN UPPER LEFT CORNER

- 3 COLOR LEGEND BAR IN UPPER LEFT CORNER RED, YELLOW, GREEN

- 5 AZIMUTH CURSERS AND 5 RANGE MARKERS IN BLUE

IN STANDBY RECEIVER AND TRANSMITTER ARE IN WARM-UP MODE

RADAR TAKES 3 MIN TO WARM UP WHEN TURNED FROM OFF TO STBY

OPERATION RESTRICTIONS

- 34 FT FROM GROUND PERSONNEL AND EXPLOSIVES

- 46 FT FROM REFUELING OPERATIONS

TURN OFF PILOT AND CO-PILOT RANGE MARKERS PRIOR TO FUNCTION SWITCH TO OFF

# AFCS

SLEW SWITCHES FOR AIRSPEED, ALTITUDE AND MACH WILL WORK WITH THE AFCS MASTER POWER SWITCH OFF

SYSTEMS REQUIRED FOR OPERATION

- PITCH AUGMENTATION

- YAW AND LATERAL AUGMENTATION

- DON’T ENGAGE PITCH AUTOPILOT UNTIL 10 SEC AFTER MASTER POWER SWITCH ON

- TO ENGAGE LATERAL MODE OF AUTOPILOT

- TURN KNOB IN THE DETENT

- NO MORE THAN 2 LBS ROLL FORCE ON CONTROL WHEEL

TURN KNOB OVERRIDES ALL LATERAL MODES, 35 DEGREES BANK MAX

CWS – ROLL WINGS LEVEL WHEN RELEASED, BANK < 7 DEGREES A MODEL AND < 3 DEGREES B MODEL

HORIZONTAL NAV MODESOURCE IS FROM PILOTS NSP CHOICE

HDG MODE

- HAS PRIORITY OVER ALL OTHER HORIZONTAL MODES

- HEADING CONTROL DETERMINED BY POSITION OF HDG BUG

- WHEN HEADING MODE IS DESELECTED AUTOPILOT FLYS COURSE SET IN THE PILOTS HSI

HORIZONTAL NAVIGATION MODE IS DISENGAGED WHEN TURN KNOB IS USED

IAS/MACH PITCH

- AUTOPILOT CONTROLS ATTITUDE TO MAINTAIN A SELECTED ALTITUDE

- AUTOPILOT PITCH ENGAGED

- PITCH WHEEL IS DISENGAGED

- SLEW SWITCH CONTROLS ALTITUDE

- MOVE SLOWLY WHEN CHANGING ALTITUDE

ALTITUDE CAPTURE

- SET DESIRED ALTITUDE IN VSFI WITH SLEW SWITCH

- ENGAGE ALTITUDE CAPTURE AT LEAD POINT

- AUTOMATICALLY DISENGAGES PITCH MODE

- INITIATES LEVEL OFF

- DISENGAGES ALTITUDE CAPTURE

- ENGAGES ALTITUDE HOLD

# AFCS

## PAGE 2

## AUTOPILOT USE IN THUNDERSTORMS

- USE AUTOPILOT BASIC MODES

- DON’T USE ALT / MACH HOLD MODES

- DON’T HELP AUTOPILOT

- MINIMUM AUTOPILOT PITCH CONTROL CHANGES

- WATCH FOR DISCONNECT

- SET POWER

- DON’T CHASE ALTIMETER, LET ALTITUDE VARY

DO NOT SELECT APP / LDG MODE IN FSAS UNTIL VERTICAL NAV MODE IS VERIFIED DISENGAGED

COUPLED ILS SEQUENCE

- SELECT ILS MODE

- DISENGAGE HDG MODE

- CHECK RADAR ALTIMETERS

- 30 DEGREE INTERCEPT ARMS AUTOPILOT FOR AUTOPILOT CAPTURE OF LOCALIZER

- CAPTURE LOCALIZER

- LOCALIZER CAPTURE PRIOR TO GS INTERCEPT

- REDUCE AIRSPEED TO 170 KTS OR APP + 20 ON LOCALIZER INTERCEPT COURSE

- ENGAGE VERTICAL NAVIGATION MODE

- ENGAGE ATS MODE