# SHORT FIELD LANDING BRIEFING GUIDE

Runway Environment

Weather, pressure altitude, temperature

Approach

Navaids/SCNS/radar altimeter

Hazardous terrain/obstructions

Landing Runway:

Elevation, length, width, type surface, markings

TOLD:

Runway Requirements – Ground roll + 500’, but not less than 3000’ (peacetime)

Approach/threshold/touchdown speeds (Gusts?)

Brake cooling time

Aircraft Configuration:

Weight </= 130,000 lbs

Fuel:

Externals – empty		<21,900 lbs total fuel in Main Tanks

Outboard - Strut pressure		Foam		Fuel (max)

215		yes/no		6200/6600

250		yes/no		5800/ 6200

285		yes/no		5000/5420

Inboards – 500 to 1000 lbs less than outboards

Bleeds – open/closed?

Flaps – 100% on final

Copilot Duties

Monitor airspeed (threshold speed +/- 3 knots), aimpoint (short or long), glidepath, and alignment

Touchdown – advise if short or long (short: keep it on the ground – long: go around)

Flaps on go-around – airspeed

Control of yoke on landing

Emergency brakes (if called for, I’ll be off the brakes)

Flight Engineer

Monitor engine instruments

Report malfunctions by engine number

Runway Emergencies:

Directional control malfunction

Loss of normal brakes/anti-skid

Flight Deck/ Cargo Compartment:

Top Bunk Clear/Loose items secured

Cargo/passengers secure and clear of rolling stock

# SHORT FIELD TAKEOFF BRIEFING GUIDE

Runway Environment

Departure conditions

Hazardous conditions

Threats

TOLD – Requirements/Speeds:

Brake cooling time

Bleed air valve

Charted MFLMETO (corrected for VMCA?)

Acceleration check time?  (VT/O > VRef):

Acceleration time reached – Nav states “Time”

At VRef – CP states “Go” (guard yoke)

At VT/O – CP states “Rotate”

Refusal Speed

Takeoff Speed:

Higher of Vmeto or Vmca (one engine inop in ground effect), unless an actual obstacle exist (in which case Vmca is disregarded)

Obstacle Clearance Speed:

Max effort obstacle clearance speed or Vmca + 10 knots (if using Vmca for T/O)

If obstacles are a factor, use Vmeto and max effort obstacle clearance speed without VMCA corrections.  The pilot will ultimately make the decision to use Vmeto versus Vmca based on a consideration of all applicable data, including: weather, runway length, TOLD, Vmeto, Vref, Vmca, Vmcg, applicable airfield survey and a review of hazards, obstructions, and terrain both laterally and along the climb out flight path

Emergency Procedures:

Abort – Before VRef – Between VRef and VT/O – Near VT/O

What to abort for?

What’s off the end of the runway?

Call “Reject”?  Or just state the problem?

Loss of normal/anti-skid brakes

Fuel dumping/cargo jettison

Departure Procedures

Departure Instructions

Nav aids/SCNS/Radar Altimeter

Secure flightdeck/cargo compartment

Reminders:

Do not lift off at “Go” unless VRef = VT/O

Raise gear immediately when airborne

Minimum flap retraction speed = obstacle clearance  + 10 kts

CP – backup yoke/brakes/throttles

Briefing required when accomplishing a max effort takeoff:

VMCG > VRef or CFL > runway available

Runway less than 80 ft wide, authorized or directed by MAJCOM