ASSAULT TAKE-OFF BRIEFING

1.  REQUIRED:

Vmca > Vr    //  CFL > Rwy available   //   Rwy < 80’ wide

2.  RESTRICTIONS

Minimum runway	-- MFLMETO x 60’ / 3000’ (peace)

3.  RUNWAY ENVIRONMENT

Runway surface -- braking action (TOLD assumes dry, paved)

Obstacles

Takeoff continued (N) - - Height/Distance / MSA / 3 eng performance ?

Takeoff aborted - - Area past departure end

Weather

Ceiling/Vis

Crosswind - in “recommended” area (corrected for RCR)

4.  TOLD

Required torque

Ground run (corrected for Vmca)

Refusal

Speeds			no obstacle		obstacle

T.O.			Vmca,ige			V meto

Climb			Vmca + 10			Vmeto + 10

Min flap		Vmca + 20			Vmeto + 20

Normal takeoff speed

Acceleration check time

5.  CREW DUTIES

P	-- call “hack” @ brake release

-- nose wheel

E	-- monitor engine inst / power

-- bleeds - closed

CP	-- yoke forward / wings level

-- “reject time” if speed fails (-3 knots)

-- “go” / “refusal”

-- ready for ESP / emer. brakes / ready to TO

N	-- “timing” at brake release

-- “time”

-- monitor departure / obstacles

6.  EMERGENCY PROCEDURES

Directional control probs. only

-- Prior to refusal - “reject”

-- After refusal - advisory

All other emergencies will be handled airborn enroute to big runway

7.  DEPARTURE

ASSAULT LANDING BRIEFING

1.  REQUIRED:

Rwy < ‘normal’ lndg dist  //   80’ wide

2.  RESTRICTIONS

Rwy > gnd roll + 500’  //  3000’ (peace)

Gross wgt < 130,000 # (waiver)

Fuel:	215 struts --  <6200# outboard  // <23500# total

285 struts --  <5000# outboard  // <18700# total

Externals - empty

540 fpm (max)

wind - “recommended” area (corrected for RCR)

3.  RUNWAY ENVIRONMENT

Weather

Ceiling/Vis / Wind

Obstacles / Terrain / MSA (N)

Approach

ARA (night, wx)

SCNS - LZ

LZ -- Lenght / Width / Surface / Markings

4.  TOLD

Weight / Fuel / CG good for takeoff ?

Ground roll

Speed - approach, touchdown

Brake cooling -- 3 consecutive landings (15 min. gear down cooling time)

5.  CREW DUTIES

E	-- close bleeds when “flaps - 100%”

-- monitor engines

-- for go-around - call max up to max power

CP	-- 3 slow /  5 hot

-- call aimpoint, centerline

-- landing - short (keep on gnd) / long - “go-around, long touchdown”

-- expect yoke right after touchdown -- keep back pressure

-- emergency brakes //  ESP

N	-- monitor approach on radar / VFR chart

-- keep SA if go-around

6.  REMINDERS

CLEAR TOP BUNK

WINDS AND WEIGHT