BI-1 (I3101): 6/5/05

- Instrument  Crosscheck

* IVSI w/g-meter better than CP VSI.  Still avoid chasing needle.

* 1)  Attitude: ADI, ball.  Also Alt, IVSI, A/S, Compass.  (T/S (bank) partial panel, Alt/VSI/A/S (pitch) partial panel.

2) Heading: HSI (wet compass partial panel,) any corrections on ADI, turn, roll out, correct heading.

3) A/S & Alt: Nose attitude + pwr.  Ceteris Parabus on either, other controls.  Off A/S or alt can be corrected by either/both.

1. Attitude control: [1] set power & attitude, study affects on attitude gyro [2] trim [3] confirm attitude by crosschecking instruments.

2a. Six basic flight instruments: attitude indicator, directional indicator (RMI, HSI), airspeed indicator, altimeter, VSI, and Turn & Slip Indicator.

2b. Pitot-static instruments: 2 separate sources for operation of pilot and copilot instruments (airspeed, altimeter, VSI).

2c. Turn & Slip Indicators: Pilot's is DC powered.  Copilot's is vacuum driven by engine bleed air pressure.

2d. The vertical gyro (attitude indicator) system is AC powered.

3. To obtain desired performance, must maintain correct attitude and power setting and trim!

4. To set parameters: [1] Power [2] Attitude [3] Trim  [4] Crosscheck (PATC)

5. Sequence for trimming aircraft: [1] Rudder [2] Elevator [3] Aileron

6. Primary scan instrument = attitude indicator.  Use rudder to keep ball centered.  Altimeter, IVSI, airspeed indicator & compass augment Att Ind.

7. Airspeed and Altitude are combination of attitude and power.

8. IVSI on pilot (left) side gives nearly instantaneous vertical speed info.  Copilot VSI lags.  Shows trends.  Don't chase needle!

9. Pilot's turn & Slip Indicator: Double Width (in the dog house) deflection is standard rate turn (SRT) (3 degrees per second).  Copilot's turn & Slip Indicator gives SRT with single needle width deflection.

10. BI Patterns: All turns standard rate (AOB and radius of turn varies with TAS).  For T-44, AOB is approx 16-18% TAS for a standard rate turn.  As airspeed decreases, AOB required for SRT decreases.  AOB is more critical than rate of climb!

11. ATTITUDE (STICK) = AIRSPEED!!! ALTITUDE = POWER!!!  (STick = AIRspeed, Power = ALtitude - STAIR PAL)

12. Adjust pitch slow and smooth to transition to climb/descent from normal cruise.

13. Once climb/descent established, cross check altimeter against clock to make power corrections to correct.  Adjust pitch with every power change.

14. SRT by smooth roll to AOB req to put turn needle at SRT position.  Make SRT with clock needle at 6 or 12 position.

[1] Slight pitch up to compensate for loss of lift.  Prolonged turns req power addition.

[2] Lead roll out by 1/3 AOB.

[3] As wings roll level, anticipate nose up tendency.

[4] Adjust AOB to max of 30 degrees to catch up in a turn.

15. All maneuvers at 150K, 1900RPM.

- Instrument Takeoff

* Used night/overwater.  Combined outside/inside.  Consider possibility of abort.  Align a/c w/ centerline (heading bug.) complete checklist.  Release brakes, initially use visual references.  Apply max.  Transition to instruments, (heading /attitude /airspeed)  Rate of transition is as we lose outside references.  Essential to establish scan.  Rotate to 7-10 deg NH, use attitude indicator as primary reference.  Check altimeter + IVSI for two positive climbs, gear up.  While gear retracting, adjust attitude for airspeed, until climb A/S is reached.

- Full Panel Approach to Stalls

* Same as FAM.  See question.

Begin on assigned altitude and hdg at 150K.  Calculate stall speeds prior to for clean and dirty stalls prior to performing approach to stalls.

1. Pre-Stall checks: [LACY PC]	[1] Loose Items - Stowed

[2] Altitude - 5000'AGL min (VMC to deck) or 8000'AGL min (5000' above clouds)

[3] Cabin Sign - Both

[4] Yaw Damp - OFF

[5] Props - 1900RPM

[6] Clearing Turn - 90 degrees of turn at 30AOB

2. Clean, Power Off (COF)

[1] As soon as clearing turn starts, set power at 400ft-lbs

[2] Roll out after 90 degrees

[3] Simultaneously: Power - IDLE & Pitch up 10 degrees.  Trim to 100K.  Maintain wings level and 10 up.

[4] 1st indication of stall - Recover!

3. Clean, Power On (CON)

[1] As soon as clearing turn starts, set power at 400ft-lbs

[2] Roll out after 90 degrees

[3] Pitch 10 degrees up and trim for 100K.  Maintain wings level and 10 up.

[4] 1st indication of stall - Recover!

4. Dirty, Power Off (DOF)

[1] As soon as clearing turn starts: [A] set power at 400ft-lbs [B] Flaps - APPR [C] Gear down landing checklist

[2] Roll out after 90 degrees

[3] Flaps - FULL DOWN

[4] Simultaneously: Power - IDLE & Pitch up 10 degrees.  Trim to 100K. Maintain wings level and 10 up.

[5] 1st indication of stall - Recover

5. Dirty, Power On (DON)

[1] As soon as clearing turn starts:  [A] set power at 400ft-lbs [B] Flaps - APPR [C] Gear down landing checklist

[2] Roll out after 90 degrees

[3] Flaps - FULL DOWN

[4] Pitch up 10 degrees and trim for 100K.  Maintain wings level and 10 up.

[5] 1st indication of stall - Recover

6. Approach Turn

[1] As soon as clearing turn starts: [A] set power at 400ft-lbs [B] Flaps - APPR [C] Gear down landing checklist

[2] After 90 degrees, continue turn with 30 AOB!

[3] Simultaneously: Power - 300ft-lbs & Pitch up 10 degrees.  Trim to 100K.  Maintain 30 AOB and 10 up.

[4] 1st indication of stall - Recover (Gear UP, then Flaps UP)

7. Recovery procedures

[1] MAX, RELAX, ROLL (add Right Rudder): Max allowable power, Relax back pressure on yoke, Roll wings level

(with ball centered)

[2] Once level or climbing (with 85K or more): Flaps - APPR, Gear - UP, Flaps - UP

[3] Maneuver complete when climbing on assigned hdg, > 110K.

*Stall indications: [1] Stall warning horn [2] Wing buffet [3] Uncontrollable loss of altitude [4] Inability to maintain wings level/ selected roll attitude [5] 25 units AOA

- Control/Performance Concept

* 1) Lead with a desired pitch/power setting, (attitude/power) 2)  Trim for that setting, 3)  Check performance instruments (airspeed/VSI/altitude).

* Attitude + Power = Performance.  1) Power, 2) Attitude, 3) Trim (Rudder, Elevator, Aileron) [Lead w/ Power]