BI-4 (I3104): 6/28/05

- Pitot-Static System

* Two internally heated pitot masts on the nose, 4 static pressure ports.  Ice electrically prevented in pitot system, protected by a/c structure in static system.

* Alternate static air port located behind pressure bulkhead.  Position error: shows higher and faster than you really are.

* Static air drain valves (3: pilot, co, alt. Pilot) located under alternate air select switch.

* Feeds altimeter, a/s, vsi on both sides.

- Flight Instrument Characteristics [Natops/Natops IFM]

* Airspeed Indicator

Pilot's is encoding and vibrates (makes it more accurate.)  DC fail kills encoder, vibrating.

* Altimeter

Within 75' of field elevation on the ground=acceptable for IFR.

* VSI

Trend Inst for Altitude... Takes 6 Seconds to equalize (Pilot's IVSI faster.)  Two types of error: lag and reversal.  Reversal happens during abrupt pitch changes.

Slight noseup when entering turn, nose down when out (Pilot's VSI due to accelerometer.)

* Turn + Slip

Pilot: 2 Needle Widths=SRT.  2/3 Needle Width=10'.  CP: Needle Width=SRT.  Also a trend instrument.  Gyro instrument.  Copilot Vacuum Powered off 2.0 in Hg by inline orifice.

Needle Calibration

[1] Turn to 10 deg prior to cardinal hdg

[2] Smoothly roll into SRT and note AOB

[3] Start clock rolling through cardinal hdg

[4] Note time to turn 90deg at SRT

[5] If 30 sec = good SRT

If < 30 sec = must put in LESS AOB to get SRT

If > 30 sec = must put in MORE AOB to get SRT

* Attitude Gyros

Gyroscope through synchros represents pitch + roll.  Vertical Reference is established by two gravity sensitive switches, through torque motors.  High or low erection rate of the gyros controlled by high or low voltage to torque motors.  Fast erect switch erects the gyro within 1.0' within 60 sec, 0.5' in 2 mins.

* Compass

Directional gyro primary reference, slaved (through transistorized slaving assembly) to flux detectors in each wingtip (saturable metal core reactor +-1 degree.)  Decouple when in polar regions (unreliable mag sensing.)

* RADALT

FM/CW (Frequency Modulation Constant Wave) type, Determines altitude by time of reflection back to reciever.

* 8-Day Clock

Mechanically Wound

* Standby Compass

Already Briefed.