C-5 Pilot Ops Limits

C-5 Pilot Evaluation Guide Attachment C										Jan 04

Limits

TF-39 ENGINE:

Static Thrust (below 22°C, at sea level):   43,000

ENGINE RPM LIMITS:

N1 Maximum		      106 %

N1 Idle			     17-29%

N2 Maximum		      102 %

N2 Idle			     61-67%

N1/N2 Vibration Range	     75-85%

N1 Icing Vibration Range	     67-87%

- Use Anti-Ice when temperature is below  8 °C with visible moisture

- On the ground with noted or suspected ice build-ups, momentarily advance engines to 90% N1 every  5  minutes

- Avoid prolonged flight in icing conditions below  60% N1.  If required, momentarily advance each engine, individually, to at least  90% N1 every  10   minutes above 15,000 ft or every  6 minutes below 15000 ft.

TIT LIMITS:

Thrust  TIT Range			Time

MOD

TRT			 940°C		5 Min

MRT			 930°C		30 Min

NRT			 915°C		Continuous

Idle Guide	      370- 525°C		Continuous

Starting		      800- 860°C		25 Sec

> 860°C		2 Sec

Transient	      940- 960°C		2 Min

960- 970°C		30 Sec

Max Reverse		 960°C		30 Sec

STARTING NUMBERS:							OXYGEN PRESSURE:

Duty Cycle								STATIC (NO FLOW) 275-450PSI

1 Minute on   30 Seconds off						Stabilized (Flow)         275-400PSI

1 Minute on   30 Seconds off      or       2 Minute on    5 Minutes off Cont

1 Minute on   30 Seconds off						CG LIMITS:

- Maximum stable fuel flow prior to light off:		               900 pph			Normal  19- 41%

- Indication of light off (TIT rise) after Fuel & start Ignition Run within: 30 seconds

- Start button should pop out by:			                46 ± 3% N2 RPM

- N1 RPM indication by:				               Idle N2 +  5 Min

- N2 RPM for airstart without start assist <10000 ft		               10% N2 RPM

OIL PRESSURE:

Minimum		10 PSI							OIL CONSUMPTION:

Idle Guide	         10-22 PSI							Home Station       1.1 Qt/Hr

80% N2 Guide	        20-30 PSI							Enroute Missions 2.2 Qt/Hr

ENGINE SUCTION FEED LIMITS:					HYDRAULIC PRESSURE

During Climb								Normal                      3000   ± 150 PSI

Outboard Engines:		 12,600 ft Maximum			Overpressure		 3400   PSI

Inboard Engines:			 20,000 ft Maximum			Min for NLG Extension	 2500   PSI

Min for 1 Brake App	 1000   PSI

During Cruise

Outboard Engines:		 32,000 ft Maximum

Inboard Engines:			 40,000 ft Maximum