Pitch and Bank								Angle of Attack

Max pitch on rotation	 13°   				Holding				 3°

Normal pitch on rotation	 8-10°				Approach (100% or 40% flap, no slats)	 5°

Max pitch after takeoff	 15°  				Approach (100% or 40% flap, slats)	 7.2°

Pitch attitude for windshear	 15°  - 18°   (below shaker onset)	Approach (No flap)			 7.8°

Max pitch for rapid descent	 15°  				Flaps up – stall			 15°

Max pitch below 200 feet	 10°  				Flaps down – stall			 18°

Max bank below 200 feet	15°

Max bank below 50 feet	 5°

Airspeeds

GENERAL									TURBULANCE

CLEAN RECOMMENDED		350__ KCAS/0.825M		LEAN – SEVERE TURBULANCE                 280_ KCAS/0.825M

RAPID DESCENT			350__ KCAS/0.825M		REC TURBULENCE PENETRATION         _240_ KCAS/0.74 M

DO NOT EXCEED 15° NOSE DOWN				ALDCS INOP – MODERATE / SEVERE     _240_ KCAS/0.825M

MAX TAXI UP TO EWP		__30_ KTS		ALDCS INOP – LIGHT             	           	 _300_ KCAS/0.825M

MAX TAXI AT EWP WEIGHTS		__15_ KTS		YAW AUG INOP		           	 _0.70 --_0.75M

LANDING LIGHT EXTENDED		_300_ KCAS/0.825M	 	  (DESCEND TO 31000_ FT OR LOWER)

DO NOT OPERATE >_25 MIN WHEN LESS THAN FULLY EXTENDED

WINDSHIELD WIPERS		_175_ KCAS

GOOD RAIN CLEARING UP TO	_150_ KCAS

ADEQUATE RAIN CLEARING UP TO	_175_ KCAS

WILL START TO FLOAT ABOVE	_150_ KCAS

THRUST REVERSERS							FLAPS

MAX WITH INVOARD T/R EXTENDED       _350_ KCAS/0.825M		40% FLAPS, SLAT EXT/RETRACTED        _215_ KCAS

MIN WITH INBOARD T/R EXTENDED         _215_ KCAS			62.5% FLAPS		            	  _195_ KCAS

T/R ON GROUND ABOVE REVERSE IDLE  _ 60_ KCAS - _150_ KCAS		62.5% FLAPS, NO SLATS	            	  _185_ KCAS

ABOVE REV IDLE BELOW _60 KCAS REQUIRES A FAN BLADE INSPECTION	LANDING FLAPS		            	  _180_ KCAS

LANDING FLAPS, NO SLATS	            	  _175_ KCAS

LANDING GEAR							SLATS EXTENDED, NO FLAPS	  _215_ KCAS

LANDING GEAR EXTENDED			_250_ KCAS/_0.60M

LANDING GEAR OPERATION			_250_ KCAS/_0.60M		OPERATING WEIGHTS (NON-EWP)

LANDING GEAR RECOMMENDED		_200_ KCAS			MAX RAMP	          _____772,000____ LBS

NLG MAY NOT RETRACT AT AIRSPEEDS > THAN	_225_ KCAS			MAX TAEKOFF	          _____769,000____ LBS

MLG MAY NOT ROTATE AT AIRSPEEDS >THAN	_185_ KCAS			MAX FUEL	          _____332,500____ LBS

NORMAL HYDRAULIC PRESSURE AVAILABLE					MAX ZERO FUEL	          _____665,000____ LBS

MLG WILL NOT ROTATE AT AIRSPEEDS > THAN	_170   KCAS			MAX INFLIGHT	          _____840,000____ LBS

APU ACCUMULATOR PRESSURE ONLY						MAX LANDING	          _____769,000____ LBS

NORM RAMP	          _____732,500____ LBS

TAKEOFF AND LANDING							NORM LANDING	          _____635,850____ LBS

ACCELERATION CHECK SPEED TOLERANCE	               -__3__ KCAS		NORM LANDING FUEL    _____159,250  ___ LBS

RUDDER EFFECTIVE FOR DIRECTOIN CONTROL	APPROX _60 _ KCAS

ELEVATOR EFFECTIVE FOR DIRECT CONTROL	APPROX _80 _ KCAS

Vapp INCREASE FOR ICE ON WING OR TAIL	              + _5 __ KCAS		AIR REFUELING

KNOWN OR SUSPECTED								A/R DOOR OPEN		_350_/0.825

MINIMUM TOUCHDOWN SPEED		       Vapp-_10 _ KCAS		NORMAL RENDEZVOUS	_300_/_0.72

RENDEZVOUS OUTSIDE 3NM_325_/_____

EMERGENCY AIRSPEEDS							1 NM			_300_/_0.72

MIN AIRSPEED ABOVE 10000 FT FOR AIRSTART       _210_ KCAS			KC-135 @ 1/2 NM		_270_/_0.65

WITHOUT STARTER ASSIST		                    				KC-10 @1/2NM		_290_/_0.70

RAT EXTENSION				_350_/0.825M			KC-135 CONTACT		_252_/_0.62

RAT RETRACTION				_180_/_0.45M			KC-10 CONTACT		_275_/_0.66

RAT OPERATION ABOVE 15000 FT		_190_				NOSE DOWN PITCH TRIM >2.7 DEG

RAT OPERATION BELOW 15000 FT		_175_			   	CG FORWARD OF 30%	_270_/_0.65

RAT OPERATION MIN FOR FLT INSTRUMENTS	_155_	   		   	CG AFT OF 30%		_300_/0.775

BELOW 175 KCAS, TURN ISO BUS “OFF” TO MAINTAIN EMERGENCY BUS ELECTRICAL POWER AND PRESSURE TO FLIGHT CONTROLS  AIRSTART NOT AVAILABLE WITH ISO BUS OFF

MAIN TANK LOW (DUE TO FUEL MISMANGMT)	_280_/0.825M

MAIN SUMP LOW				_280_/0.825M

THREE ENGINE ENROUTE CLIMB AIRSPEEDS

ABOVE 650,000 LBS			_250_/_0.60M

650,000 TO 550,000 LBS			_230_/_0.60M

550,000 TO 450,000 LBS			_210_/_0.60M

BELOW 450,000 LBS			_190_/_0.60M

TWO ENGINE CLIMB (ALL WEIGHTS)		_190_/_0.45M

DRIFT DOWN (2 OR 3 ENGINES AT NRT)		_250_/_0.60M