## ABNORMAL START

## (Ground EP)

- Condition lever -  Fuel Cutoff (note decrease in ITT) (LS)

- Ignition & Start Switch – Off (LS)

In the event of a hot start:

- Ignition & Start Switch – Starter only (for remainder of 40-sec starter operation) (LS)

## EMERGENCY ENGINE SHUTDOWN ON THE DECK

## (Ground EP)

- Condition levers – Fuel Cutoff (PF)

- Firewall valves – Closed (LS)

- Fire extinguishers – Discharged (For Fire) (PF)

- Gang Bar – As required (PF)

- Evacuate Aircraft – As required

BRAKE FAILURE

- Maintain directional control with rudder and asymmetrical power

- Try the other crewmembers brakes

- Stop the aircraft using propeller reversing to aid in deceleration

## BRAKE FIRE

## (Ground EP)

- Fire fighting assistance – Request (PNF)

- Stop aircraft (PF)

- Emergency Shutdown on Deck Checklist – Execute (PF)

# ABORTING TAKEOFF

(T/O EP)

- Announce intention to abort (PF)

- Power levers – Idle (PF)

- Reverse – As required (PF)

- Brakes – As required (PF)

If anticipating insufficient runway remaining:

- Condition levers – Fuel Cutoff prior to leaving prepared surface (PNF)

After the aircraft stops:

- Emergency shutdown on the deck checklist – Execute (PF)

RUNAWAY TORQUE PRIOR TO Vr

(T/O EP)

- Condition levers – Fuel Cutoff (PNF)

- Brakes – As required (PF)

## ENGINE FAILURE AFTER TAKEOFF

(In-Flight EP)

- Power – Maximum allowable (PF)

- Airspeed – Vlof or greater (PF)

- Gear – Up (LS)

- Propeller (inoperative engine) – Feather or Confirm Autofeather (PF)

- Airspeed – Vxse (until clearing obstacles, then Vyse) (PF)

- Flaps – Up (At Vyse) (PF)

- Emergency Engine Shutdown Checklist – Execute (PF)

## EMERGENCY ENGINE SHUTDOWN

(In-Flight EP)

- Condition Lever – Fuel Cutoff (PF)

- Propeller Lever – Feather (PF)

In case of confirmed/suspected fire or visible fluid leak:

- Firewall valve – Closed (LS)

- Fire extinguisher – Discharger (For Fire) (PF)

UNCONROLLABLE POWER

- Emergency Engine Shutdown Checklist – Execute (PF)

WINDMILLING AIRSTART

- Power Lever (failed engine) – Idle (PF)

- Prop Lever (failed engine) – Full Forward (PF)

- Condition Lever (failed engine) – Fuel Cutoff (PF)

- Firewall valve – Open (PF)

- Autoignition – Armed (ignition light on) (PF)

- Condition Lever (failed engine) – Low Idle (PF)

- Power – As Required

## SMOKE/FIRE OF UNKNOWN ORIGIN

(In-Flight EP)

- Oxygen masks – Don (100 percent) (PF,PNF)

- Mic Switches – Oxygen Mask (PF,PNF)

- Passenger oxygen – DEPLOY (as required) (RS)

## CABIN FIRE

(In-Flight EP)

1.  Oxygen masks – Don (100 percent) (PF,PNF)

- Mic Switches – Oxygen Mask (PF,PNF)

- Passenger oxygen – DEPLOY (as required) (RS)

ENGINE-DRIVEN FUEL PUMP FAILURE

- Emergency Engine Shutdown Checklist – Execute (PF)

## FUEL PRESSURE LIGHT ILLUMINATED

(In-Flight EP)

- Engine Nacelle – Check for fuel leak (PF,PNF)

If fuel is leaking:

- Emergency Engine Shutdown Checklist – Execute (PF)

PROPELLER FAILURE OR OVERSPEED

- Prop Lever – Adjust to normal RPM range (PF)

If rpm limits are within normal governing range, continue operation.  If normal governing range cannot be maintained, proceed as follows:

- Power Lever (affected engine) – Idle (PF)

- Prop Lever – Feather (PF)

- If propeller fails to feather, Propeller Fails to Feather Checklist – Execute (PF)

## EXPLOSIVE DECOMPRESSION

(In-Flight EP)

- Oxygen masks – Don (100 percent) (PF,PNF)

- Mic Switches – Oxygen Masks (PF,PNF)

- Passenger Oxygen – Deploy (as required) (PNF)

- Descend – As Required

## EMERGENCY DESCENT PROCEDURE

(In-Flight EP)

- Power Levers – Idle (PF)

- Prop Levers – Full Forward (PF)

- Flaps – As Required (PF)

- Landing Gear – As Required (LS)

- Airspeed – As Required (PF)

ELECTRIC ELEVATOR TRIM FAILURE

- Trim disconnect switch – Depress Fully (PF/PNF)

SPINS

- Power Levers – Idle (PF)

- Rudder – Full deflection in the opposite direction of turn needle (PF)

- Control Wheel – Rapidly forward and neutralize ailerons (PF)

- Rudder – Neutralize after rotation stops (PF)

- Control Wheel – Execute a smooth pullout (PF)

## SINGLE ENGINE WAVEOFF/MISSED APPROACH

(Landing EP)

- Power – Maximum (PF)

- Prop Lever – (operating engine) Full Forward (PF)

- Nose Attitude – Establish a positive rate of climb (PF)

- Gear – Up (LS)

- Airspeed – As Required (Vxse or Vyse) (PF)

- Flaps – Up (PF)

## WINDSHEAR ESCAPE MANEUVER

(Landing EP)

- Power – Maximum (PF)

- Autopilot – Disengage (PF)

- Pich Attitude/wings – Rotate towards 15 degrees/Gently Roll Wings Level (PF)

- Configuration – Do not change (PF)

When clear of the windshear emergency event:

- Waveoff Procedure (normal) – Execute (PF)

FLAP SYSTEM FAILURE

- Flaps – Return to previously selected position (PF)

- Flap Motor circuit breaker – Pull (LS)

## DITCH/FORCED LANDING

(Landing EP)

- Announce intention and time to ditch

## DITCH/FORCED LANDING (AIRCREW)

(Landing EP)

- Passengers – Briefed

- Don life vest – As Required

- Escape hatch – Remove and stow / jettison as directed

- Bags/cargo – Jettison As Required

- Brace position – Assume

- Evacuate aircraft – After violent motion stops

- Raft/survival equipment – As Required