ASSAULT TAKEOFFS

Auth when runway avail < CFL

Still need >3000’ (and gnd roll + 500’ for landing)

Norm rwy:80’w		Assault:60’w	Taxi:30’w

MFLMETO: Accel, lose eng, stop or go at VMETO

VRefusal: 2eng rev, 1 eng gnd idle, 1 windmilling

Gusts: Add to Rot, Toff, Appr, Thold, TDown (≤10kts)

Accel: If VRef<VTakeoff; highest ≤VRef-10, ±3 kts

VMETO: 1.2Vs pwr on	Norm: 1.1Vs pwr off

VMEObst: 1.3 Vs pwr on	Norm: 1.2 Vs pwr off

Takeoff		Climb Obst	Flaps up

VMCA		VMCA+10	VMCA+20 (tech)

VMETO		VMEObst		VMEObst+10

Brake

Rel

“Go”

Refusal

“Rotate”

METO

MCG

MCA

Norm

T\Off

Go

Stop

3 eng

≥3k’

Rwy

Avail

Good

Better

Best

ME Obst

MCA

+10

MCA

+20

ME Obst

+10

“Time”

Accel

“Gear”

“Flaps”

4 eng

ASSAULT LANDINGS

Need >3000’ and ground roll + 500’			Bunk

Compute 2 engines in reverse, 2 at ground idle		Bleeds

Brake @ 90 for 3 seconds max				Brakes

Consider 15 minutes brake cooling between assaults		Rwy\LZ

130k# max, 115k# considered heavyweight			Airspeeds

Fuel:	Mains: 23,500# total (max)			Check Fuel

Outboards: 6200#					Emergency

Inboards: Outboards -500#, if possible		NVG EPs

Externals: empty

ME VTHold: 1.28 Vs pwr off

Norm VTHold: 1.35 Vs pwr off

MISC

NVG: All in flight deck must have 		Fail<2nm:go missed appr

NVG Approaches: 300\1

Advisories: 20\10 min (Pers), 1min\5sec for CDS

Drop aspd\alt NLT “Green light”; NLT 1min(Pers)\2min (JM-directed)

SKE

36a/c, TWS:4nm, Ldr w\in10nm, 80nm min. btwn SKE formations

FCI:14 sec alwd btwn prep & exec	Priority: Alt, Hdg, Apsd

Depart: L:180kias,1500vvi	Wing: 190kias

Turns: 20°b (SD 10°)		Accel: 900°/600° TIT

Lead: Call ∆>10°h, >10kias	Lead ∆: 45° until 1 nm

Lead: Call passing 2000’levels	Wing: Report level

Wing: 2000’ITRK, 500’XTRK	EL: 12000’ITRK, 0 XTRK

Overruns: EL: XTRK+800’	Wing: XTRK+300’

Loss SKE: VMC-PPI inop: continue, all inop: VFR or RTB

IMC single - st\lvl: 500’, 30°/30 sec, resume base hdg

- in turn: roll out, 500’

Escape:140 ,1000vvi

PTurn: Entry w\in 20°&70°, PTalt+1k’, 150kias, desc OB\abeam

18 sec spacing turn IB, 140 @ FAF, 5-6k’ landing spacing

ALTITUDES

Min IFR: 1k’/2k’ above obst w\in 5 nm

MSA: 500’/400’+1cont above obst w\in 5 nm

ESA: 1k’/2k’ above obst w\in 22 nm

Day VMC: 300’ modified contour

NVG: 500’/400’+1cont above TERR w\in 3 nm

CHUM: w\in 22 nm, can cut to 10 nm of route

Altitude reference Day: L:terrain, CARA	W: Lead, CARA, terrain

Night: L: Altimeter	W: Lead, altimeter

FORMATION

Min WX: Appr mins, ≥200\1 (use dep alt if below appr mins)

Wing: 2000’ trail			EL: 12000’

Lead: Call ∆>15kias		CDS: 6000’ trail

WX w\out SKE: 1000vvi base alt, 30°/2 min [elems +500’ fwd]

w\SKE: 1000vvi base alt, 1000’XTRK, lead directs “SKE”

slow 15kts, back 4000’ITRK, move into SKE posit

O\head: 1500’,200kias, stack @1nm, brk 45°b, idle, 160/20, 150

Wing delay: brk 5 sec, perch 12-14 sec, land 20sec (15sec min.)

D\wind: 1000’, 200kias, brk 45°b, idle, 140, g\idle @4k’, rev @ 3k’

AIRDROP

XWind: Per\CDS(13);HE(17);SATB, PersWater(25);HVCDS(n\a)

Personnel: 64max, 20\static line, 130kts, 600xDZ,400’bailout, 2k’ freefall

EP-Door: gear\flaps,  away, retrieve; Ramp: cut if ready, retrieve if not

HE: 140kts, 42000# max, 600xDZ

CDS: 130\140kias, 16 bundles\stick, 6-8°deck, 400xDZ

DZ marking: JRACS OH, flanker 250m, trailing edge (Pers) 1000m

Notify, Aft restraint, Raise\Relock\Ramp, Completion of drop checklist Fwd throw: Cds(550’) He(450’) i() Pers(250’) Satb(150’)