#### Engine Fire Shutdown

* 1.

* 2.

### WARNING

The agent should be discharged when

________________________________

___________________________________

a. If condition persists, wait a minimum

of ________ after the first bottle is

discharged, and then  ____________

_______________________________

* 3.

## Note

If a propeller continues to rotate

above 3 percent NP, perform Propeller

Fails to Feather procedure.

4. CROSS FEED switch (manage fuel dis-

tribution) -- CLSD (CP)

#### Four-Engine Flameout

Propellers No. 1 and No. 4 autofeather any

time the FADEC detects autofeather criteria.

Propellers No. 2 and No. 3 either autofeather

or remain windmilling at 100 percent depend-

ing on logic within the MC. If the autofeather

criteria occur below 15,500 feet and three other

engines are running normally, an inboard

engine autofeathers after a 2-second delay to

give the engine time to recover. At or above

15,500 feet, or if another engine is shut down,

the inboard engine windmills at 100 percent. If

all four engines flameout, this logic ensures at

least one inboard propeller will be at 100 per-

cent rpm to supply electric and hydraulic

power.

Unless it occurs at low altitude and low airspeed,

a safe recovery from a four-engine

flameout can be accomplished. The procedure

involves selecting the inboard engines to

START before the outboards because at least

one of them will have 100 percent NP, result-

ing in a very quick start (within 30 seconds),

even at relatively low airspeed.

## Note

Airstart Envelope:

250 KIAS

25,000 feet for JP-5/8, JET A

22,500 feet for JP-4

* 1.

### WARNING

Adjust the attitude immediately

following a four-engine flameout to

maintain airspeed or accelerate.

The faster the airspeed the faster

the engines restart. Once the

engines are restarted, decreasing

the airspeed decreases the time to

unfeather the propellers.

NG must be below ____ percent when

START is selected or the FADEC

does not begin the start sequence

(schedule fuel and ignition).

* 2.

* 3.

* 4.

### WARNING

If the restart on engine No. 2 or No. 3

must be aborted, do not use the fire

handle.  Place the ENGINE START

switch to STOP. Pulling the fire han-

dle causes loss of hydraulic and generator

power as the propeller feathers.

## Note

After at least one engine is

restarted and NP is on-speed, if

any other engine fails to restart,

pull the respective fire handle and

place the respective ENGINE

START switch to STOP.

Disregard START VLV FAIL

ACAWS for the first engine

restarted since no bleed air is available

until one engine is on-speed.

At high airspeed, the outboard

engines may decrease to 28 percent

before the inboards. In this case,

select STOP, START, RELEASE on

each engine in the order they reach

28 percent.

* 5.

Auxiliary Power Unit Fire (Ground/In Flight)

* 1.

* 2.

### WARNING

The agent should be discharged after

the fire handle has been pulled when

an indication continues or if any

other indication or malfunction is

suspected that requires fire extinguisher

agent.

a. First bottle -- Discharge

Wait a minimum of ___ seconds after the first

bottle is discharged, if condition persists:

b. Remaining bottle Discharge

3. APU bleed air valve switch -- Closed

(P/CP/CM)

4. Ground Evacuation Procedure initi-

ate (if required) (P/CP/CM)

#### Ground Evacuation

* 1.

### WARNING

If a hot brake is suspected or main

wheelwell fire exists, set opposite

brake only.

* 2.

* 3.

* 4.

* 5.

* 6.

* 7.

* 8.

* 9.

### WARNING

If hot brakes are suspected or main

wheelwell fire exists, chock _____ gear

only.

#### Abort Procedures

Perform the following procedures as soon

as the decision  to abort is made:

* 1.

* 2.

### Caution

Do not move the power levers below

FLT IDLE above _____ KTAS during

aborted takeoff. Moving the power

levers below FLT IDLE above this

speed can lead to all engine-driven

generators tripping off-line due to an

over frequency condition caused by

excessively high propeller RPM transients.

Anti-skid protection will be

lost resulting in increased stopping

distance. If hard braking is applied

when this occurs, main tires could

blow causing possible loss of direc-

tional control and damage to the gear

doors and surrounding structure.

## Note

Pause for approximately 1 second

with the power levers at GND IDLE

to check for symmetric power, and for

the copilot to check for BETA indica-

tions below the HP gauges. The

Copilot should call ‘‘No BETA on (1, 2,

3, or 4)’’ if BETA is not indicated after

power reaches GND IDLE.

* 3.

##### Note

Any time after the power levers reach

FLT IDLE, the pilot should transfer

control of the control wheel to the

copilot and use the nosewheel steer-

ing as required. Forward pressure on

the control wheel increases the

nosewheel load and improves ground

tracking.

If power is symmetric:

* 4.

If power is asymmetric:

* 5.

* 6.

### WARNING

The copilot must check for BETA

indications. Selection of asymmetric

reverse will likely cause directional

control difficulties.

When the aircraft is under control:

7. Appropriate ACAWS crew actions --

Perform (P/CP)

###### WARNING

At the first indication of directional

control difficulties during four

engine reverse, immediately return

all power levers to GND IDLE until

the affected engine can be identified.

Use rudder and aileron,

nosewheel steering, differential

power, and differential braking, as

required.

If one BETA indication does not

illuminate after the power levers

are brought to ground idle, and no

other associated ACAWS occurs,

there is a remote possibility directional

control difficulties will occur

as airspeed slows. If necessary for

directional control, the engine fire

handle should be pulled on the

engine without the BETA

indication.

### Caution

Power levers should be moved from

full reverse to ground idle or the

brakes released just before the air-

craft comes to a full stop to prevent

the nose gear from coming off the

ground and causing possible tail contact.

If any loose material is on the

runway which can be lifted by the

propellers, return the power levers

toward GND IDLE by 50 KIAS, as

required, to prevent reduced visibility

and engine damage.

Engine Failure Below Refusal Speed

If an engine fails before refusal speed is

reached:

* 1.

Engine Failure After Refusal Speed

If engine failure or fire occurs after reaching

refusal speed, perform the following

procedures:

* 1.

* 2.

* 3.

### WARNING

Rotating at significantly less than ___ degrees

per second or rotating to a pitch atti-

tude significantly below the target

may delay the liftoff and result in not

clearing obstacles.

## Note

The target pitch attitude and the

takeoff trim setting were chosen so

that if the critical engine fails

between refusal speed and rotate

speed, the aircraft will be approximately

in longitudinal trim at obstacle

clearance speed passing 50 foot

height.

When safely airborne with a positive rate of

climb:

4. Landing gear -- Retract

### WARNING

Retracting gear and flaps simultane-

ously increases VMCA because of the

reduction in available hydraulic pres-

sure to the rudder booster assembly.

5. Initially establish 2 to 5 degree angle of

bank away from shut down engines and

then adjust angle of bank and rudder pedal

force for zero sideslip, and apply rudder

trim to reduce rudder pedal force.

### WARNING

Apply rudder trim, as required, after

takeoff to reduce rudder forces and to

increase margin above VMC speeds.

With rudder trim set, en route climb

speed, one engine inoperative, is

above VMCA2, even in low boost with

the flaps up.

6. After gear is up, lower the nose, as nec-

essary, to continue climbing and accel-

erating toward three-engine climb

speed.

7. Flaps -- Retract

### WARNING

Selecting the flap lever above 15 per-

cent increases VMCA by up to 38

knots due to the reduction in boost

pressure and the ATCS power restoration

schedule. Observe the minimum

flaps up safety speed of 135

knots, the worst case VMCA with

flaps up and low boost. Operating the

flaps and gear together increases the

minimum control speed due to reduction

in available hydraulic pressure.

If predicted climbout flight path is not

required:

a. Raise the flaps passing FUSS,

VOBS+25 but not less than 135 KIAS

If predicted climbout flight path is required:

b. Raise the flaps, as follows:

(1) Less than 100,000 pounds: raise

flaps passing FUSS, (135 KIAS)

(2) 100,000 to 140,000 pounds: begin

flap retraction at 135 KIAS and

end at FUSS, (VOBS+25)

(3) More than 140,000 pounds: raise

flaps in 10-percent increments for

each 5-knot increase in airspeed,

beginning at Minimum Flap

Retraction Speed (VOBS+5) and

ending at FUSS (VOBS+25)

After the flaps are retracted:

8. Power -- Reduce to maximum

continuous

9. Emergency procedures -- Perform as

required, when time permits

10. Climb at three-engine en route climb

speed

Fire/Smoke/Fumes Elimination

Any crewmember detecting fire, smoke, or fumes shall immediately alert the flight station. The pilot will direct crewmembers to fight the fire as required in the following checklist. If at any time during the execution of this checklist the fire, smoke, or fumes decrease in intensity, do not continue the checklist. Stop and ascertain

what systems are affected and if able, isolate

the component(s) and restore aircraft power.

### WARNING

If flammable fumes are present or

suspected, electrical equipment not

required to carry out the Fire/Smoke/

Fumes Elimination checklist should

not be turned on or off.

### Caution

If a fire is near an oxygen component

or there is a possibility that the oxy-

gen could increase the fire, consider

closing the oxygen manual shutoff

valve, providing portable oxygen bot-

tles are adequate for the situation.

## Note

If at any time during the execution

of this checklist the fire, smoke, or

fumes decrease in intensity, do not

continue the checklist. Stop and

ascertain what systems are

affected and if able, isolate the

component(s) and restore aircraft

power.

Depressurization at altitude

greatly assists in elimination of fire

because of the lack of oxygen at

altitude.

* 1.

* 2.

## Note

If crewmembers leave their crew

positions to assist in the emer-

gency, ensure that the oxygen reg-

ulator is switched OFF. When the

checklist is complete, repressurize

the aircraft as required and ensure

all oxygen regulators are OFF.

One of the pilots should remain on

oxygen until after landing.

3. AIR COND CARGO COMPT POWER

switch OFF (CP)

4. CROSS FLOW VALVE switches

MAN ON, CLOSE (CP)

5. UNDERFLOOR switch OFF (CP)

6. Pressurization mode select switch

NO PRESS (CP)

## Note

Depressurization at altitude greatly

assists in elimination of fire because

of lack of oxygen.

7. AUTO RATE selector MAX (CP)

8. Safety altitude Check (P)

9. Radio Call Transmit, as required

(CP)

10. Descent (as required) To a safe alti-

tude (P)

## Note

If an aircraft or cargo fire is known or

suspected, the loadmaster/observer

should don the portable oxygen and

attempt to locate and fight the fire.

11. Cause or source Isolate (ALL)

If the smoke is clearing and the source has

been isolated, then items 12 and 13 are

optional.

12. Flight deck escape hatch/paratroop

doors (below 150 knots and depressur-

ized) Cleared to Open. (Use ICS/PA/

Green Jump lights.) (CP/CM)

13. PRESSURIZATION panel AUX

VENT (CP)

## Note

The cargo lights may be reset when

no longer required by selecting

CARGO LIGHTS RESET on the

AMU LIGHTING page.

If electrical fire is suspected:

a. If visual conditions or better can be

maintained to a landing within 30

minutes and the smoke/fire cannot be

isolated; carry out items 14 and 15,

otherwise proceed to electrical bus

Load Shedding Procedures.

14. Fuel panel Tank to Engine (CP )

15. All generators Off (CP)

### WARNING

Fully charged batteries may only last

30 minutes.

## Note

Time and conditions permitting, refer

to Total AC Power Failure and Bat-

tery Power Only Considerations

paragraphs for additional guidance

and a list of equipment still powered.

The Fire/Smoke/Fumes Elimination check-

list is complete.

#### Rapid Decompression

Sudden and uncontrollable loss of cabin pressure is known as rapid decompression. This may result from losing a nonstructural member, such as a door or window, or from a rupture in the fuselage skin. If pressure loss is uncontrollable, notify crew and passengers to fasten seat belts and proceed as follows:

* 1.

* 2.

* 3.

Type of descent is dependent upon the

degree of structural damage. Use rapid

descent (gear and flaps up) without structural

damage, or rapid descent (gear and flaps

down), when structural damage necessitates.

If pressure loss occurred because of structural

failure, conduct a controllability check to establish the safe speed for completion of the flight.

### WARNING

The loadmaster/observer should

make an inspection of the fuselage

during descent (using a walk-around

oxygen bottle and wearing a

restraining harness or parachute) to

determine what caused the decom-

pression and the extent of any

damage.

* 4.