| EVENT: C2101 |  | DATE: |  | BRIEF TIME: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| INSTRUCTOR: |  |  |  | Aircraft: |  |  |  |
| BLOCK C21 | TITLE Cockpit Procedures |  | MEDIA CPT | Dual | Solo | Hrs 9.0 | H/X 1.5 |
|  |  |  |  | 6 | 0 |  |  |
| Syllabus Notes - Practice all checklists, applicable FTI briefings, radio calls, and basic aircraft control. Ensure student’s checklist proficiency is adequate to proceed to flight operations. - Multiple items are listed as discuss items. However, due to time constraints, it will not be possible to discuss all items prior to the simulator event. Therefore, a discuss item may be addressed during the sim and/or prior to the sim. - During all C21xx events, each normal checklist should be performed if it has been previously introduced or discussed. - C2101 and C2102, only normal checklists should be practiced. Due to the visual capability of the TC-12B simulator, all remaining C21XX events may include flight and visual maneuvers. For example, aborted takeoffs, SSE, basic air work, etc. Special Syllabus Requirements None Discuss Items - Seat/rudder adjustment, fuel management panel, pilot instrument panel, engine instruments and switches, center instrument panel, annunciator panel/analysis, copilot instrument panel, circuit breaker panels, control pedestal, overhead control panel, brake check, checklists: before start, engine start, after start, engine run-up, takeoff, after landing and secure. a. See NATOPS and FTI Chapter 1 and Appendix E Emergency Procedures None Review Material Operating Condition; Torque ft-lb; Max ITT; N1; Prop RPM; Oil Press; Oil Temp. Start; 1000 (5sec); -40 (min). Low Idle; 660; 52-55; 60 (min); -40 to 99. High Idle; 70-73; -40 to 99. Takeoff and Max Cont; 2230; 750; 101.5; 2000; 105-135; 10 to 99. Max Cruise/Climb; 2230; 725; 101.5; 2000; 105-135; 10 to 99. Max Reverse; 750; 88; 1900; 105-135; 10 to 99. Transient; 2750 (5sec); 850 (10sec); 102.6 (10sec); 2200 (5sec); 100 to 104 (5min). |