| EVENT: C2102 |  | DATE: |  | BRIEF TIME: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| INSTRUCTOR: |  |  |  | Aircraft: |  |  |  |
| BLOCK C21 | TITLE Cockpit Procedures |  | MEDIA CPT | Dual | Solo | Hrs 9.0 | H/X 1.5 |
|  |  |  |  | 6 | 0 |  |  |
| Syllabus Notes - Practice all checklists, applicable FTI briefings, radio calls, and basic aircraft control. Ensure student’s checklist proficiency is adequate to proceed to flight operations. - Multiple items are listed as discuss items. However, due to time constraints, it will not be possible to discuss all items prior to the simulator event. Therefore, a discuss item may be addressed during the sim and/or prior to the sim. - During all C21xx events, each normal checklist should be performed if it has been previously introduced or discussed. - C2101 and C2102, only normal checklists should be practiced. Due to the visual capability of the TC-12B simulator, all remaining C21XX events may include flight and visual maneuvers. For example, aborted takeoffs, SSE, basic air work, etc. Special Syllabus Requirements None Discuss Items - Takeoff procedures - While holding the brakes, advance the power levers to the 12 o’clock position, allow prop RPM to stabilize at 2000 RPM with torque indications matched. - Check engine and flight instruments and both nacelles for secure caps, panels, and no leaks. - Check autofeather armed lights on (N1>90%). The left seat shall report “Checked left” and the right seat shall report “Checked right” - Release the brakes, place heels on the deck, set at least 2100 ft-lbs of torque and check ITT within limits. Maintain directional control with rudders - The PNF may adjust the power levers as necessary to keep torque or ITT within limits. The PNF will call “Rotate” at VR. At VR, raise the nose smoothly to 7-10 degrees up. - Raise the gear when airborne with two positive indications of climb (increasing altitude and positive IVSI). Use the verbal/visual challenge using a thumbs-up and phrase “gear up”. If PNF agrees with the decision to raise the gear, PNF will also give a thumbs-up and say “gear up”. This is done to prevent inadvertent cycling of the landing gear. - Departure: After gear indicates up, check the flaps up and turn off the landing lights. Smoothly reduce power to 1900 ft-lbs and then the props to 1900 RPM. Adjust climb power, accelerate to the appropriate speed and comply with course rules (170 KTS, 500’) - Checklists: climb, cruise, descent, approach, landing - See FTI Chapter 1 and Appendix E - Climb checklist: wait until 1000’ AGL before calling for the checklist. If staying in the local area or below 5000’ AGL, only the abbreviated list items are required. Not required if staying in the touch-n-go patter or proceeding directly to Cabaniss or Corpus International via course rules. Maneuver; KIAS; N2; Torque; ITT. Maneuvering Cruise Fast Cruise Slow Cruise; 170 200 140; 1700 1700 1700; 1000 1200 750; 600 650 575. Cruise Climb SL-10K 10K-20K 20K-25K Above 25K; 155 135 125 115; 1900 1900 1900 1900; 2230 2230 2230 2230; 700 700 700 700. Vx Vy VXSE VYSE Emergency Descent; 100 121 115 121 245; 2000 2000 2000 2000 2000; 2230 2230 2230 2230 Idle; 700 700 700 700 Idle. - Landing procedures - Area departure: Complete the approach checklist in accordance with course rules. Common technique: ATIS, Brief (calculate fuel, give touch-n-go brief, and discuss your plan to get to the airfield), Checklist (call for the approach checklist). Terminate with Seagull, Kingsville approach, or Houston center and proceed with course rules. - Touch-and-go brief: Once safely on the runway, I will advance the power levers to 12 o’clock. You reset the flaps and trim and call “go” when the engines have spooled up, and call “Rotate” at VR with takeoff power set. - At “GO”, advance power levers to 2100 ft-lbs, do not remove your hand from the power levers until the aircraft is climbing and well clear of the ground. - PNF resets elevator trim to 3 degrees up after touchdown and checks props at 2000 RPM. - Use a wider downwind in a no-flap pattern and push the flap override button on the GPWS. - Pattern Entry procedures: - Break Entry: Enter the break (left except for 13L/31R) wings level at 170 KTS. Report the numbers or as directed. When cleared to break, roll into a level bank not to exceed 45 degrees while simultaneously reducing power to 800 ft-lbs. Select approach flaps and anticipate ballooning. Place props full forward, select gear down (182 KTS max) and complete the landing checklist. Adjust AOB to roll out on downwind with the runway on the fuel cap, descent to pattern altitude, placing the wingtip on the runway. - Downwind entry: Normally made at 1000’ AGL, preferably on a 45 degree angle. Intercept midfield approximately 1 mile abeam (WTD) and turn downwind. Transition to landing configuration and descent to pattern altitude. - Base entry: Arrive at the 90 degree position, configured, at 130 KTS, 500’ AGL, and landing checklist complete. - Interval: - TC-12, T-44, T-45: abeam the wing - TC-12, T-44, T-45 and a full stop: 30 degrees behind the wing - T-34: 45 degrees behind the wing 180 140 KTS 700-800/500-600 ft-lbs “Tower, Stingray XXX, left/right 180, 3 down and locked, full stop/touch-n-go” Downwind 1 WTD 800’ AGL 140 KTS 800-1000 ft-lbs Checklist by midfield Base 140/130 KTS 500’ AGL 20-30 AOB 700 IVSI Final 300’ AGL 132/120/105 KTS 30 AOB max 1000-1200’ threshold “3 down and locked review me complete” Upwind Gear down 140 KTS 1500-1600 ft-lbs Turn at 300’ AGL Crosswind 600’ AGL Flaps up/approach 140 KTS 30 AOB max Cross at Vref Landing speeds NF 110-120 AF 90-100 FF 80-90 No reverse <40 Emergency Procedures Abnormal Start If any of the following occur: ITT rate of increase likely to exceed 1000 degrees, no rise in ITT is evident within 10 seconds after selecting LOW IDLE with the condition lever, N1 does not accelerate normally to low idle, the CHIP DET light illuminates, proceed as follows: - Condition lever – FUEL CUTOFF (note ITT decrease) (LS)* - IGNITION and START switch – OFF (LS)* In the event of a hot start: - IGNITION and START switch – STARTER ONLY (for remainder of 40-sec) (LS)* CAUTION: Starter use is time limited to 40 seconds on, 60 seconds off, 40 seconds on, 60 seconds off, 40 seconds on, then 30 minutes off. For engine clearing, wait 60 seconds, then: - IGNITION and START switch – STARTER ONLY (min 15 seconds, max 40 seconds) Review Material Operating Condition; Torque ft-lb; Max ITT; N1; Prop RPM; Oil Press; Oil Temp. Start; 1000 (5sec); -40 (min). Low Idle; 660; 52-55; 60 (min); -40 to 99. High Idle; 70-73; -40 to 99. Takeoff and Max Cont; 2230; 750; 101.5; 2000; 105-135; 10 to 99. Max Cruise/Climb; 2230; 725; 101.5; 2000; 105-135; 10 to 99. Max Reverse; 750; 88; 1900; 105-135; 10 to 99. Transient; 2750 (5sec); 850 (10sec); 102.6 (10sec); 2200 (5sec); 100 to 104 (5min). |