T-44A Briefing Guides

EVENT: C4503

DISCUSS ITEMS: Aircraft operating limits, night visual approaches.

Aircraft operating limits – See VT-31 website (answers below).

Night visual approaches – Crosswind corrections are not as easy to anticipate due to a lack of visual cues. Fly a normal pattern. Concentrate on looking down the entire length of the runway to avoid angling. It is extremely important to hit pattern checkpoints to prevent having to make gross corrections. The most common night landing error is failing to flare sufficiently. Altitude cues are not readily apparent and the flare must be anticipated. Do not fixate on a spot in front of the nose. Sight toward the end of the runway and land with a visual picture of the centerline between your legs. Do not fixate inside the aircraft on rollout, especially during touch and go’s. Keep your scan outside and maintain centerline. The OBS may shine a flashlight on the trim panel (at IP’s discretion) to assist in resetting the trim during touch and go operations.

T-44A Operational Limits (April 2007)

| ## Airspeed KIAS | General | ### General |
| --- | --- | --- |
| VMO/VNE Max dive/level flight 227 | Pneumatic pressure norm operating range 12-20 psi | Aircraft Ceiling 31,000 ft |
| # VMCA Minimum controllable airspeed 86 | Max sink rate on landing 600 fpm | Right Squat Switch (GFISH) |
| VX Best angle of climb 102 | Max crosswind 20 knots | G – Landing Gear Circuit Inop, J-Hook engaged |
| VY Best rate of climb 108 | Max cabin pressure differential 4.7 psi | F – Flight hour meter inop on deck |
| VSSE Minimum safe 1 engine inop 91 | Gyro suction norm operating range 4.3-5.9 inHg | I – AOA Indexer Lights inop on deck |
| VXSE Best angle of climb single engine 102 | Min oxygen required for local/X-C flight 1000/1500 psi | S – Stall Warning inop on deck |
| VYSE Best rate of climb single engine 110 | Prop deicer ammeter normal operation 14-18 amps | H – R eng Lip Boot Heat Inop on deck |
| VFE Max flap extension/extended (full) 140 | Left squat Switch (HHHAP) | Memory Items/Bold Face |
| VFE Max flap extension/extended (appr) 174 | H – AOA heat 14V on deck vice 28V in-flight | Smoke/Fire of Unknown Origin |
| VLR Max landing gear retraction 145 | H – L eng lip boot heat inop on deck | 1) Crew – Alerted |
| VLE Max landing gear extension 155 | H – Gnd max (8 elements) enabled on deck | 2) Cabin Temperature Mode – OFF |
| VA Maneuvering speed 153 | A – Ambient air solenoid | 3) Vent Blower – AUTO |
| VMCG Min controllable speed on gnd 63 | P – Pressurization Controls: 1) Preset 2) Dump 3) Door | 4) Oxygen Masks/MIC switches (100%) – As Required |
| Max range glide 130 | Memory Items/Bold Face |  |
| Max endurance glide 102 | ##### Engine Failure After Takeoff | ### Primary Governor Failure/Malfunction |
|  | 1) Power – As Required | 1) Propeller RPM - Adjust to Normal RPM |
| ## Electrical | #### 2) Gear – UP | 2) Power Lever - IDLE |
| DC generator voltage 28.25  .8 VDC | 3) Airspeed – As Required (VXSE or VYSE) | 3) Propeller Lever - FEATHER |
| AC Inverter voltage 114  7 VAC | 4) Emergency Shutdown Checklist – Execute | 4) Alternate Feathering Checklist - As Required |
| AC Inverter frequency 400  6 Hz |  |  |
| Min battery voltage for APU charge 18 VDC | ##### Emergency Engine Shutdown On Deck | ### Emergency Descent |
| Min battery voltage for APU start 20 VDC | 1) Condition Levers – FUEL CUTOFF | 1) Power Levers – IDLE |
| Min battery voltage for BATT start 22 VDC | 2) Firewall Valves – CLOSED | 2) Propellers – FULL FORWARD |
| Starter Cycle (40sec, 60sec) X 2, then 40sec, 30min. | 3) Boost Pumps – OFF | 3) Flaps – As Required |
|  | 4) Fire Extinguisher – As Required | 4) Landing Gear - As Required |
| Interstage Turbine Temperature (ITT) | 5) Gang Bar – OFF | ### 5) Airspeed – As Required |
| Normal operating range 400–790 C | 6) Evacuate Aircraft | 6) Windshield Heat – As Required |
| Max allowable 790 C |  |  |
| Max continuous 790 C | ### Jammed Power Lever | Single Engine Waveoff |
| Max reverse 790 C | 1) Condition Lever (affected engine) – FUEL CUTOFF | 1) Power – Max Allowable (Est + ROC – Vxse min) |
| Max acceleration (up to 2 sec) 850 C | 2) Emergency Shutdown Checklist - Execute | 2) Flaps – APPROACH (unless already UP) |
| Max low idle 685 C |  | 3) Gear – UP |
| Max cruising 740 C | ##### Fuel Leaks | 4) Flaps – UP |
| Cruise climb 765 C | 1) Condition Lever – FUEL CUTOFF | 5) Props – 1900 RPM |
| Start Limit 925 (cutoff ) 1090 (2 sec) | 2) Emergency Shutdown Checklist - Execute |  |
|  |  | ### Brake Fire |
| ### Torque | Emergency Shutdown Checklist | 1) Firefighting Assistance - Request |
| Normal operating range 400–1315 ft-lb | 1) Power lever – IDLE | 2) Stop Aircraft |
| Max @ 1900 RPM 1520 ft-lb | 2) Prop lever – FEATHER | 3) Emergency Shutdown on Deck Checklist - Execute |
| Max @ 2200 RPM 1315 ft-lb | 3) Condition lever – FUEL CUTOFF |  |
| Max acceleration 2100 ft-lb | If Fire or Fuel Leak: | ### Abnormal Start |
|  | 4) Firewall valve – CLOSED | 1) Condition Lever - FUEL CUTOFF (note max ITT) |
| Turbine Tachometer (N1) | 5) Fire Extinguisher – As Required | 2) Starter - OFF |
| Normal Operating Range 50-101.5 % | 6) Bleed air – CLOSED |  |
| Low idle range 50-53 % |  | Aborted Takeoff |
| Hi idle range 64-67 % | ### Generator Failure | 1) Announce “Abort” |
| Max reverse 86 % | 1) Generator-OFF, Reset Momentarily, Then ON. | 2) Power levers – IDLE |
| Max acceleration 102.6 % | If generator will not reset: | 3) Reverse – As Required |
| Max continuous 101.5 % | 2) Generator – OFF | 4) Brakes – As Required |
|  | ### 3) Current Limiter (Battery Ammeter) – Checked | - - Immediately prior to departing the runway - - |
| Propeller Tachometer (N2) | ### Smoke and Fume Elimination | 5) Condition levers – FUEL CUTOFF |
| Normal operating range 1800-2200 rpm | 1) Oxygen Mask/Mic Switch (100%) - As Required | - - As soon as practicable - - |
| Max reverse 2100 rpm | 2) Pressurization - DUMP | 6) Firewall valves – CLOSED |
| Max during accel (2sec) 2420 rpm |  | 7) Boost pumps – OFF |
|  | ### Explosive Decompression | 8) Fire extinguisher(s) – As Required |
| ### Oil Temperature | 1) Oxygen Mask/Mic Switch (100%) - As Required | 9) Gang bar – OFF |
| Normal operating range 10-99 C | 2) Descend – As Required | 10) Evacuate aircraft |
| Max oil temp 99 C |  |  |
|  | ### Windmilling Airstart | Spin Recovery |
| ### Oil Pressure | 1) Power Lever (Failed Engine) - IDLE | 1) Power Levers - IDLE |
| Normal operating range 85-100 psi | 2) Propeller Lever (Failed Engine) – FULL FORWARD | 2) Rudder – Full deflection opposite direction of the turn needle |
| Max oil pressure 100 psi | 3) Condition Lever (Failed Engine) - FUEL CUTOFF | 3) Control Wheel – Rapidly forward |
| Min oil pressure 40 psi | 4) Firewall Valve - OPEN | 4) Rudder – Neutralize after rotation stops |
|  | 5) Autoignition - ARM | 5) Control wheel – Pull out of dive w/ smooth, steady back press |
| ### Weights | 6) Condition Lever - LOW IDLE |  |
| Max ramp weight 9710 lbs | 7) Power - As Required | Low Altitude Windshear |
| Max takeoff weight 9650 lbs |  | 1) Power – Maximum Allowable |
| Max landing weight 9168 lbs | Prohibited Maneuvers | 2) Attitude – Set and Hold Approximately 15° Nose Up |
|  | 1) Intentional Spins | 3) Gear – UP |
|  | 2) Aerobatics | 4) Flaps – Maintain Current Setting |