- C-5B PILOT BOLDFACE

- Ref TO 1C-5A-1 (current as of 17 Jul 09)

1.	ENGINE FIRE ON THE GROUND (Para 3.5)

1.  FIRE HANDLE – PULLED

2.  AGENT - DISCHARGED

2.	PYLON FIRE (Para 3.33.2)

1.  FIRE HANDLE – PULLED

2.  AGENT - DISCHARGED

3.	EMERGENCY ENGINE SHUTDOWN (Para 3.26)

1.  FIRE HANDLE – PULLED

2.  AGENT – AS REQUIRED

4.	FOUR ENGINE FLAMEOUT (Para 3.29)

1.  RAT – DEPLOY

2.  NO. 3 STARTER BUTTON – IN

3.  NO. 2 FUEL AND START IGNITION – AIR START

5.	FUSELAGE/WHEEL WELL FIRES (Para 3.32.3)

1.  OXYGEN – ON, 100%

6.	SMOKE/FUMES IN THE AIRPLANE (Para 3.34)

1.  OXYGEN – ON, 100%

7.  	LOSS OF PRESSURIZATION (Para 3.39)

1.  OXYGEN – ON, 100%