# CPT AC

# EMERGENCY IN-FLIGHT KNEELING (3-113)

- FWD MLG CAN BE FULLY KNEELED REGARDLESS OF BOGIE POSITION

- AFT MLG CAN BE FULLY KNEELED EXCEPT IF NOT IN THE ROTATIONAL POSITION OF 45 DEG TO 85 DEG, BETWEEN THESE POSITIONS A TIRE WILL CONTACT THE FARING AND STALL OUT THE KNEEL DRIVE MODULE, ALSO TIRES MAY NOT CLEAR THE GROUND ON LANDING

WHEELS UP CRASH LANDING (3-98)

- BASICALLY REVIEW CHECKLIST AND FOLLOW CAREFULLY

# LANDING WITH A FLAT TIRE

- NLG TIRE FLAT

- MOVE CG AFT IF POSSIBLE

- NORMAL APPROACH

- HOLD NOSE GEAR OFF AS LONG AS POSSIBLE

- SMOOTHLY LOWER TO RUNWAY

- USE REVERSE THRUST AND MINIMUM BRAKES

- MLG TIRE FLAT

- MAKE NORMAL APPROACH

- LAND ON SIDE OF RUNWAY WITH THE GOOD TIRES

LANDING WITH NOSE GEAR RETRACTED

- NORMAL APPROACH AND LANDING

- NOSE LOWERED TO THE RUNWAY SMOOTHLY WILL MINIMIZE DAMAGE

- ADDED NOSEUP PITCH TRIM SHOULD HELP BEFORE INITIAL TOUCHDOWN

- DEPLOYMENT OF SPOILERS AFTER TOUCHDOWN IS NOT RECOMMENDED

- USE BRAKES AFTER NOSE HAS CONTACTED THE RUNWAY, ONLY AS REQUIRED TO STOP  FROM DEPARTING THE RUNWAY

- CONSIDER FUEL JETTISON

- SHIFT CARGO AFT IF POSSIBLE

## CPT AC

PAGE 2

ANTI SKID SYSTEM FAILURES (3-116)

- CHECKLIST CHECKS THE NUMBER OF BRAKES THE ARE INCAPABLE OF BEING PROTECTED BY THE ANTI SKID SYSTEM

- ANTI SKID SYSTEM TURNED OFF

- 6 PAIRS OF BRAKES (12 WHEELS) 6 LIGHTS ON

- 4 PAIRS OF BRAKES ON THE SAME SIDE(8 WHEELS) 4 LIGHTS ON

THRUST REVERSER FAILURE (3-88)

- TR NOT LOCKED LIGHT OR EXTEND LIGHT ON BELOW 250 KTS

# EMERGENCY ENGINE SHUTDOWN CHECKLIST

- TR NOT LOCKED LIGHT OR EXTEND LIGHT ON ABOVE 250 KTS

# PLACE THROTTLE TO IDLE

# DETERMINE IF TR IS CLOSED

# IF CLOSED, CONTINUE NORMAL OPERATIONS

# PRIOR TO SLOWING BELOW 250 KTS ACCOMPLISH PRECAUTIONARY SHUTDOWN CHECKLIST

# UNCOMMANDED TR EXTENSION OR FAILURE TO RETRACT ABOVE 250 KTS ACCOMPLISH PRECAUTIONARY ENGINE SHUTDOWN CHECKLIST

# ENGINE STARTING MALFUNCTIONS

- CLEARING PROCEDURES

- MOTOR ENGINE FOR 30 SEC

- USE IF ENGINE DOES NOT REACH N2 IDLE

- SHEARED STARTER SHAFT

- BUTTON POPS OUT APPROXIMATELY 1 SEC

- POSSIBLE HOLDING COIL

- CLOSE PYLON BLEED VALVE

- IF BUTTON STAYS IN, SHAFT IS SHEARED

- CSD FAILURE

- DISCONNECT, CHECK MEL

- HAVE TO CHECK OIL QUANTITY, MX IF AVAILABLE

- MAX 50 HOURS OPERATION

- HOT START

- FIRST INDICATIONTIT SURPASSES N2

- STOP START, CLEAR ENGINE, CALL MX

- 800 – 860 FOR 25 SEC

- > 860 FOR 2 SEC

## CPT AC

# PAGE 3

- HUNG START

- INDICATIONSSTALLED AT 55%, TIT RISING

- STOP START, CLEAR ENGINE

- SECOND START WITH AUG AIR VALVE CLOSED

- START AND IGNITION SWITCH TO START20%

- HIGH FUEL FLOW

- FUEL FLOW > THAN 800 PRIOR TO IGNITION

- STOP START, CLEAR ENGINE, CHECK WITH FEMX

- NO IGNITION

- NO LIGHT OFF WITHIN 30SEC

- STOP START, CLEAR ENGINE

- CHECK CB’S AND STARTER LIMITATIONS

- TRY AIR START POSITION

- NO FUEL FLOW

- CHECK WITH FE, STOP START

- CHECK CB’S

- CALL MX ,POSSIBLE FUEL SHUTOFF VALVE PROBLEM

- NO N1 OR N2 INDICATIONS

- CHECK WITH FE

- FOR NO N1 WAIT 5 MIN AFTER N2 IDLE

- CHECK WITH SCANNER

OTHER POSSIBLE MALFUNCTIONS

- NO OIL PRESSURE BY N2 IDLE

- STARTER BUTTON NOT OUT BY N2 IDLE

- STARTER VALVE OPEN LIGHT NOT OUT BY N2 IDLE

- HOLDING COIL FAILURE

- NO LIGHT IN BUTTON