Equal Time Point (ETP)

An ETP must be computed for:

- CAT I route/route segments of 5 hours or more

Navigator will recompute ETP when:

- ATA at a reporting point is 15 minutes or more ahead or behind the planned time if the change was caused by erroneous wind information.

Flight Charts

Chart must include:

- Navigator’s name

- Date

- Chart number and edition (stripped chart)

- Flight plan course line and waypoints (waypoints labeled)

- Emergency airfields (within 75nm)

- Portions of ADIZ/FIR boundaries pertinent to the route

- Approximate location of the ETP

In-Flight Procedures

The Navigator will:

- Monitor and record ATC instructions/clearances/read-back

  - Not required when ATC instructions require immediate execution by the pilot, or when such action interferes with the timely performance of other time-sensitive navigator duties.

- Monitor the aircraft position on approach/departure using an appropriately scaled chart (ONC, TPC, JOG, etc)

- Flight follow on all missions using a suitable plotting chart (JNC, JNCA, or GNC)

- Maintain a flight log on CAT I routes/route segments of 3 hours or more.

Fixes/Position Recording

Required on CAT I route/route segments of 3 hours or more.

- Fix as soon as possible after Level-off/Coast-out (whichever occurs last)

- Fix as soon as practical prior to coast-in

- Time between fixes will not exceed 1 hour, but

  - Present position must be recorded every 30 minutes.

- Fix Procedures:

  - Before fix time:

    - Record, as a minimum, GMT, present position, true heading, spot wind, TAS, altitude and ETA to the next waypoint

  - At fix time you should:

    - Record the primary navigation solution and corresponding deltas

  - Between fixes/recorded positions, record spot readings at regular intervals to allow for calculating a DR.  Spot readings should include:

    - As a minimum, time, heading, drift angle, ground speed, wind vector, and true airspeed.

TAS Check

Required on all Category I routes of 3 hours or longer.

- Accomplish within one hour of initial cruise altitude

In-Flight Fuels

Required on CAT I routes/route segments of 5 hours or more

- As soon as practical after level-off

- Regular time intervals not to exceed 1 hour and 30 minutes.

- Navigator may terminate these procedures one hour from destination, when the CAT I route segment is completed, or at the discretion of the PIC

Dev Check

Required On CAT I routes/route segments of 3-hours or more.

- Complete as soon as practical after level-off or coast out

- Navigators should reconfirm deviation on CAT I legs every 3 hours or after planned course changes of greater than 30 degrees.

- A deviation check is not required on flights transiting Category I routes of less than three hours if:

  - The aircraft is equipped with two or more operable heading systems (the standby compass is not considered a system for this requirement).

  - The difference between systems does not exceed 2-degrees.

ARA Procedures

Prior to entering the terminal area, the navigator will brief the following items to the crew:

- Desired pattern altitudes and headings.

- Distance on final where descent will commence.

- Glide slope angle and initial rate of descent

- MDA and missed approach point/procedures. Missed approach will conform to published procedures for a useable approach, if able.

During the ARA the navigator will:

- Advise the PF when positive radar identification of the airfield complex is made.

- Direct the aircraft by headings to the final approach course.

- The turn onto base leg (if required) should be made to allow for a 10 NM final (or as required).

- State the distance from touchdown each mile from the end of the runway beginning 10-miles out

- Give heading information at least every nautical mile during the final approach.

- Advise the PF of the drift and groundspeed. If PFs can view this information on the selected SCNS/INS display, this advisory is not required.

During the ARA the navigator should:

- Give a glide path warning 10 seconds prior to the "begin descent point."