# COLD WEATHER PROCEDURES

## WARNING: DON’T FLY IN FORECAST OR KNOWN SEVERE ICING

- MODERATE ICING – CAN BE TOLERATED FOR SHORT PERIODS OF TIME IE., CLIMB OR DESCENT (EXIT AS SOON AS POSSIBLE)

- FLYING IN FREEZING PRECIPITATION IS NOT RECOMMENDED

- REMOVE ALL SNOW, ICE, & FROST FROM WINGS, FLIGHT CONTROLS & FUSELAGE BEFORE FLIGHT (DO NOT CHIP)

- ALLOW EXTRA TIME FOR INTERIOR WARM-UP & PREFLIGHT CHECKS, START APU’S TO WARM UPPER DECK (30 MINUTES)

- INSTRUMENT / AVIONICS PREFLIGHT MAY NOT FUNCTION UNTIL WARMED

- DO NOT CHECK O2 SYSTEM PRIOR TO CARGO COMPARTMENT WARM-UP

- BE ALERT FOR HYDRAULIC LEAKS, CAVITATIONS, AND FUEL SYSTEM LEAKS

- BELOW – 40 C, USE COLD START SWITCHES, ( SWITCHES TO NORM, CYCLE SWITCH 5 SEC – ON, 10 SEC – OFF  GRADUAL WARM-UP TO        – 40 C

- PREHEAT ENGINES, WATCH FOR HIGH / LOSS OF OIL PRESSURE

- OIL PRESSURE MAY EXCEED 100 PSI

- LEAVE THROTTLES IN IDLE UNTIL OIL PRESSURE IS IN NORMAL IDLE RANGE, (10- 22 PSI) IF OIL PRESSURE DROPS TO “0”, SHUT DOWN ENGINE

- MAY EXPERIENCE HUNG STARTS

- CLOSE PRESS AUG VALVE SEC II, (20% PRIOR TO RUN)

- APPLY EXTERNAL FUEL HEAT TO FUEL CONTROL

- ENGINE ANTI-ICE ON BELOW  8 C WITH VISIBLE MOISTURE

- TURN ON IMMEDIATELY AFTER ENGINE START IF ICING PRESENT OR IS EXPECTED TO BE ENCOUNTERED ANY TIME PRIOR TO SETTING CLIMB POWER

- ICE CLEARING PROCEDURES

- GROUND – ADVANCE POWER TO 90% N1 EVERY 5 MINS

- SYMMETRICAL ENGINES TO PREVENT SKIDDING

- BLOWING SNOW COULD BE PROBLEM, CLEAR AFT

- FLIGHT – ADVANCE POWER TO 90% N1 EVERY 6 MINS BELOW 15,000 FT AND EVERY 10 MINS ABOVE 15,000 FT

- ONE AT A TIME, CONTINUOUS IGNITION ON

# COLD WEATHER PROCEDURES

## PAGE 2

- TAXI

- 5 KTS ON ICE OR SLIPPERY TAXIWAYS (INTERMITTENT BRAKING)

- AT SPEED FASTER THAN 5 KTS SKIDDING CAN BE EXPECTED

- IF NLG STEERING ANGLES EXCEED 20 DEG WATCH POWER BACK OF MLG WHEN CASTERING

- IMMEDIATELY DECREASING NLG STEERING ANGLE MAY ALLEVIATE POWER BACK PROBLEM

- EXPECT SIGNIFICANT INCREASE IN TURNING RADIUS

- BRAKES MAY BE INEFFECTIVE ON ICY SURFACES

- USE ASYMMETRICAL THRUST IF NECESSARY, HOWEVER AVOID USING OUTBOARDS IF OVER BANKED SNOW

- CASTER PRIOR TO TURNING

- CENTER APPROXIMATELY 45 DEG PRIOR TO COMPLETING TURNS

- USE 45 DEG NLG STEERING FOR MINIMUM TURNS

- BE ALERT FOR NLG SKIDDING

- AT LOW TAXI SPEED

- WITH MODERATE NLG STEERING INPUTS

- DECREASE NLG STEERING ANGLE IF SKIDDING OCCURS

- AVOID TAXIING IN DEEP SLUSH OR SNOW

- MINIMIZE TAXIING IF ICE FOG FORMATION CONDITIONS ARE PRESENT

- TAKEOFF

- IF ICING IS ANTICIPATED PRIOR TO ESTABLISHING CLIMB POWER, USE ANTI-ICE FOR TAKEOFF

- IF STANDING TAKEOFF IS NECESSARY AND SKIDDING OCCURS PRIOR TO BRAKE RELEASE ABORT THE TAKEOFF

- RECOMPUTE / DOWNLOAD TO ROLLING TAKEOFF

- TRT TAKEOFF REQUIRED IF RCR < 12

- TAKEOFFS SHALL NOT BE ATTEMPTED WITH MORE THAN ½ INCH OF WATER, WET SNOW, SLUSH ON THE RUNWAY OR MORE THAN 3 INCHES OF DRY SNOW

- DELAY GEAR RETRACTION 2 MIN AFTER TAKEOFF FROM WET OR SLUSHY RUNWAYS TO CLEAR GEAR WELLS AND PREVENT DOOR LOCKS FROM FREEZING

- IF AIRCRAFT IS COLD SOAKED, IE BELOW –40 DEGREES FOR MORE THAN 6 HOURS, GEAR RETRACTION WILL BE SLOWERMLG 40 SEC, AND NLG 2 MIN

- IF GEAR IS DELAYED CORRECT T/O DATA

# COLD WEATHER PROCEDURES

## PAGE 3

- CHECK FOR ICE 5 MIN PRIOR TO TAKEOFFESCAPE HATCH #1

- IN-FLIGHT

- MADAR VIBRATION LIMITS APPLY ONLY AFTER ICE CLEARING PROCEDURES HAVE BEEN USED

- AVOID CONTINUOUS POWER SETTINGS DURING EXTENDED EXPOSURE TO ICE 67% TO 87% N1VIBRATIONS SUSCEPTIBLE

- AVOID 75% TO 85% THE REMAINDER OF THE FLIGHT

- DO NOT EXTEND THRUST REVERSERS DURING DESCENT IN ICING CONDITIONS

- TURN ON ANTI-ICE 10,000 FEET PRIOR TO ANTICIPATED ICING CONDITIONS IN A DESCENT (CONTINUOUS IGNITION ON PRIOR TO ANTI-ICE)

- WRITE UP  ENGINES FOR A FAN BLADE INSPECTION IF ICING CONDITIONS ARE ENCOUNTERED

- IF ICE IS KNOWN OR SUSPECTED ON HORIZONTAL TAIL SURFACES ADD 5 KTS TO APPROACH SPEEDS

- IF CG IS FORWARD OF 23% USE 62.5% FLAPS

- WINDSHIELD WIPERS GOOD TO 150 KTS AND ADEQUATE TO 175 KTS BUT MAY FLOAT ABOVE 150 KTS

- COLD WX AFTER LANDING

- ON ENGINE SHUTDOWN, LEAVE PYLON BLEED VALVES “ON”

- RETRACT FLAPS AND SLATS

- TRIM STAB FULL NOSE DOWN

- INSTALL ALL PROTECTIVE COVERS ( ENGINE, WING INLET, PITOT, APU)

- REMOVE AIRCRAFT BATTERIES AND INS BATTERIES

- DRAIN WATER SYSTEMS AND LATRINES