FAM-1: 5/16/05

- Outside Scan Techniques

10 degrees, looking at blocks, look at sky in pieces.  Also, clear into any turn, anywhere you’re going.  Pilot + OBS Clears Left.  IP clears rt.

- See and Avoid

Defensive Posture, Listen for all traffic advisories, 99 calls on VHF in contested transitions.  Default to rt if head to head, as per AIM.  Clear, head outside, even above FL180.  Radios: 'Traffic in sight, negative contact.'

- Crosswind T/O and Landings

Maintain corrections through landing roll/t and g.  Aileron kills the drift, rudder keeps you on the runway (e.g. Left cw, left ail, rt. Rudder)  Put in a little more power for cross controlling.

- Aborted T/O

Practice: Departure Option.  Can T/O or abort.  Cannot T/O following abort.  Execute the memory items.

1.  Announce 'Abort'

2. Power-Idle

3. Rev-A/R

4. Brakes-A/R

Leave RW:

5. Condition Lev-Fuel C/O

As soon as practibile:

6. Firewall Valves-Close

7. Boost Pumps- Off

8. Fire Ext-A/R

9. Gang bar-off

10. Evac.

Watch for hot brakes.  NOTE associated w/ brakes.

- Landing Pattern

Pre-briefed.

- Full Stop

Any config, nosewheel on rwy, then reverse.  Slow taxi before turns, rev more eff at higher a/s, brakes at lower.  After land, clear of rwy.  If at 1900, advance forward props before reverse.

- T&G Procedures

In accordance with brief.  If 2k ft left, and pwr past 12 o'clock, fly with it.  7-10 deg NH.

- Training Time-Out

Anytime safety is compromised, student does not feel situation is safe.

- NATOPS Brief

IAW card.

* Still need one for IFG.

- Dual Engine Waveoff

* Add pwr, positive ROC, offset, report.

Level at 500', do not exceed 130 kts.

* Ip calls above 100', does below 100'.

COWGUT: Clearance, Overshoot, Waveoff, Gear, Unsafe, First Third

Conditions warranting a waveoff in the landing pattern: [1] Excessive overshoot/  >30AOB during appr turn [2] Landing clearance not received by short final [3] Any verbal or visual waveoff signal issued by IP, wheels watch, Tower, or RDO [4] 3 green lights for gear not visible after rolling onto final [5] pilot feels unsafe condition exists [6] Consider waveoff if can't touchdown in first 1/3 of rwy

- Ops Limits

ITT

1. Normal (green arc) op range: 400-790C

2. Max ITT for start is 1090.  Time limited to 2 seconds.

3. The right engine is limited to 790C degrees during the start procedure for the left engine.

4. Always report the amount and duration of start exceeding 925C.

5. Always report the amount of operating ITT exceeding >790C.

6. Max ITT low idle 685C.

7. Max ITT in reverse 790C.

8. Max ITT for acceleration 850C (for 2 sec).

9. Max ITT for cruise climb 765C.

10. Max ITT for cruise 740C.

TORQUE

1. Normal (green arc) operating range: 400-1315ft-lbs.

2. Max at 2200RPM is 1520 ft-lbs.

3. Max at 1900RPM is 1315 ft-lbs.

4. Acceleration limit is 2100 ft-lbs.

PROP RPM (N2)

1. Normal (green arc) op range 1800-2200RPM.

2. Max RPM for takeoff 2,200 RPM. (NATOPS 10-12)

3. Min RPM for takeoff 2,200 RPM. (NATOPS 10-12)

4. Max RPM for acceleration 2420RPM (for 2 seconds).

5. Max RPM in reverse 2100RPM (limited by pneumatic section of primary governor)

6. Overspeed governor limited to 2,288(+/- 40)RPM.

7. Fuel topping governor limited to 2,332RPM.

N1 LIMITS (%)

1. Normal (green arc) operating range 50 - 101.5%

2. Low Idle range 50-53%

3. High idle range 64-67%

4. Max N1 acceleration 102.6%

5. Max N1 reverse 86%

6. Min N1 in reverse 80%

7. Max N1 can decrease during manual feather check NONE.

OIL PRESSURE AND TEMPERATURE

1. Normal (green arc) op range for oil pressure 85 - 100psi

2. Normal (green arc) op range for oil temp 10 -99 C

3. Min oil temp for engine start -40C

4. Min oil pressure on the ground 40psig.  Min oil pressure in the air 40psig.

5. Normal oil pressure must be obtained any time the engines are at pwr settings above 72%N1.

- Engine Start Procedures

Pre-briefed.

- Abnormal Starts

Hot Start-toward 925 (Midpoint towards 1090) kill it.  Racing up.  Caused by weak batt, not enough starter air.

Hung/Stalled Start-Lightoff, but power stalls.  Possibly failure of secondary injectors.

No start-No lightoff in 10 sec.

Execute memory items

1. Cond Lev- Fuel C/O

2. Starter-Off

Observe duty cycle: 40 sec on/60 off/40/60/40/30 min off

Caution: Starter use limited to 40 sec on, 60 sec off, 40 sec on, 60 sec off, 40 sec on, then 30 min off.

Note: If starting attempt discontinued, allow 60 sec delay to drain fuel and cool starter, then motor starter for 15 sec min (toggle starter switch DOWN).  Allow engine to come to complete stop before another start attempted.

- Brake System

NATOPS+

1. Procedures for BRAKE FIRE (CALL, STOP, CHECK)

*1.  Firefighting assistance - REQUEST

*2.  Stop aircraft

*3.  Emergency Engine Shutdown on Deck

[1] Condition Levers - FUEL CUTOFF

[2] Firewall Valves - CLOSED

[3] Boost pumps - OFF

[4] Fire extinguishers - AS REQ

[5] Gang Bar - OFF

[6] Evacuate aircraft

Warning: Step well away from aircraft (preferably well behind incase wheel explodes sideways).  Don't fire the fire!

2. Procedures for LOSS OF BRAKES

[1] Maintain directional control with rudder, nosewheel steering, or differential power. [2]Use prop reversing or beta to decelerate. [3] If possible, maneuver to open area and let aircraft stop [3] Don't taxi! [4] Try to reseat shuttle valve by pulling aft on the top of the brake pedals.

3. Procedures for HOT BRAKES

[1] Stop aircraft by reverse thrust and minimal braking

[2] Request assistance, if needed

[3] Cool brakes with propwash

[4] Inspect brakes before using them again.

Caution: Brakes can lock during max braking, which may blow tires.

- HIGH WORK

LEVEL SPEED CHANGE

Norm     150     (650-700)

Fast        170     (900-1000)

Slow        130      (450-500)

Norm      150      (650-700)

TURN PATTERN

Cardinal hdg, 150K (Lead turns 1/3 AOB)

15 AOB for 90

30 AOB for 180

45 AOB for 360

SLOW FLIGHT

P-400ft-lbs (int)

F-APPR (174 max, balloon, trim)

G-DOWN (155 max, land checklist)

A-100K (600-700 ft-lbs)

T-90 (30 AOB) reverse

F-FULL (balloon, trim)

A-90K (800-900 ft-lbs), TRIM FULL UP

T-90 (30 AOB) reverse

Recover (level):l

P-Max allow

F-APPR

G-UP

F-UP

A-150K

APPROACH TO STALLS

Pre-stall check (LACY PC)

L-Loose items stowed

A-Alt  5000AGL (VMC to deck)

8000AGL (5000 above cloud deck)

C-Cabin sign-BOTH

Yaw damp-OFF

P-Props-1900

C-Clear turn (90 at 30AOB)

Clean, Pwr Off

1. Immed after clear turn, set 400 ft-lbs

2. Roll out after 90 deg

3. P-IDLE, Pitch 10 UP, Trim to 100K

(Maintain wings level 10deg UP)

4. Immed recover 1st stall indication

Clean, Pwr On

Same as Clean Pwr Off except don't pull pwr to IDLE!

Dirty, Pwr Off

1. Immed after clear turn, set 400 ft-lbs

2. F- APPR

3. G-DOWN (Landing Checklist)

4. Roll out after 90deg

5. F-DOWN, P-IDLE, Pitch up 10deg, Trim to 100K

6. Immed recover 1st stall indication

Dirty, Pwr On

Same as Dirty Pwr Off except don't pull pwr to IDLE!

Appr Turn

1. Immed after clear turn, set 400 ft-lbs

2. F- APPR

3. G-DOWN (Landing Checklist)

4. After 90deg, maintain 30AOB

5. P-reduce to 300ft-lbs, Pitch 10deg UP, Trim to 100K, then stop trim.  Maintain turn and pitch.

6. Immed recover 1st stall indication

Stall Recovery

1. MAX, RELAX, ROLL

MAX ALLOW PWR (2nd redline, 790 ITT)

RELAX yoke pressure

ROLL wings level

2. Once recovered, incr pitch and accel

3. Once level or climbing (>85K) immed F-APPR

4. G-UP

5. F-UP

6. Complete when climb on hdg < 110K