FAM-10: 6/15/05

- Aircraft Systems

Brakes

NATOPS+

1. Procedures for BRAKE FIRE (CALL, STOP, CHECK)

*1.  Firefighting assistance - REQUEST

*2.  Stop aircraft

*3.  Emergency Engine Shutdown on Deck

[1] Condition Levers - FUEL CUTOFF

[2] Firewall Valves - CLOSED

[3] Boost pumps - OFF

[4] Fire extinguishers - AS REQ

[5] Gang Bar - OFF

[6] Evacuate aircraft

Warning: Step well away from aircraft (preferably well behind incase wheel explodes sideways).  Don't fire the fire!

2. Procedures for LOSS OF BRAKES

[1] Maintain directional control with rudder, nosewheel steering, or differential power. [2]Use prop reversing or beta to decelerate. [3] If possible, maneuver to open area and let aircraft stop [3] Don't taxi! [4] Try to reseat shuttle valve by pulling aft on the top of the brake pedals.

3. Procedures for HOT BRAKES

[1] Stop aircraft by reverse thrust and minimal braking

[2] Request assistance, if needed

[3] Cool brakes with propwash

[4] Inspect brakes before using them again.

Caution: Brakes can lock during max braking, which may blow tires.

Fuel

387.6 gal, 384 useable

61 gal in nacelles.

XFER on at 42-51 gal, 59 backup.

XFER fails, lose 28 gal trapped fuel.

3/8 full nacelle tank opens gravity feed.

500 pph for XFER pumps.

1. Total fuel system capacity 387.6 G

2. Total usable fuel 384 G

3. Crossfeed allows use of all tanks to either engine.

4. Fuel is transferred from the wing tanks to the center section tank by gravity.

5. Fuel is transferred to the nacelle tank by the transfer pump.

6. Nacelle float switches: 	High level (59G)- turns transfer pump off if 51 Gal switch fails.

Mid level  (51G)-turns transfer pump off

Low level (49G)-turns transfer pump on

7. In override, the transfer pump runs continuously, the float switches are bypassed, & the 3psi press switch reset

8. With a failed transfer pump, the fuel gets to the nacelle tank by gravity feed.

9. There are 10 fuel drains on the aircraft.

10. A fuel capacitance gauging system is used in the T-44.

11. The only diff between the left and right fuel system: (1) the left has crossfeed (2) the right has thermal relief valve & line from crossfeed system..

12. A minimum of 42 gallons of fuel is req in the Nacelle tank to obtain a good test of the transfer pump.

13. The boost pump provides 30 psi of pressure to the engine driven fuel pump.

14. It takes 5psi of pressure to activate the FUEL PRESS light.

15. It takes a 30 second pressure delay to activate the NO FUEL TRANSFER light.

16. The engine driven fuel pump supplies 800 psi pressure to the engine.

17. The indications of a boost pump failure: with crossfeed in 'auto': [1] red 'FUEL PRESS' light illums and goes out very quickly [2] yellow 'FUEL CROSSFEED' light illums [3] master caution light flashes [4] gradual loss of power on affected engine beginning at 13,000'.

18. The TBO for an engine driven fuel pump operating on suction lift is 10 hours.

19. The TBO for an eng running on AVGAS is 150 hours.

20. The crossfeed is not left open with the boost pumps operating because fuel may be inadvertently crossfed from either fuel system to both engines because of variances in pump pressure

21. Since the crossfeed valve is electrically opened, if it losses total electric power (i/e battery drained and generators out), it will be closed.

22. The firewall valve is not used to secure an engine because severe damage could occur to the engine-driven high pressure fuel pump (while cavitating) because it obtains essential lubrication from fuel flow.

24. Fuel quantity error can in total can by as high as 100 lbs, while nacelle can only by 50 lbs.

25. The syphon break is located on the outer wing and opens to prevent negative pressure build up in the fuel system (normally caused by thermal expansion).

26. The purpose of the fuel vents are to provide ram air pressure to prevent tanks from collapsing as they lose fuel to the engine.

27. If either fuel bus fails, the boost pump will still work because it's wired to the hot battery bus.

28. If the transfer pump CB popped, I would get the following indications: red 'NO FUEL TRANSFER' light WON'T ILLUM!

29. The min distance the fuel truck can come to the airplane while refueling it is 50 ft.  For the refueling point ______ft.

30. Fuel flow from outboard tanks to the engine: gravity feeds to the center inboard tank, through transfer pump to the nacelle tank, through the boost pump, by the firewall shutoff valve, through the fuel filter, through the fuel flow transmitter, through the fuel heater, through the engine driven fuel pump, controlled by FCU, controlled by start control unit, to the engine fuel nozzles, into the combustion chamber.

31. Refueling distances: (1) 10' from any port of an aircraft (2) 20' from fuel vehicle or fuel filler point.

FUEL SYSTEM EMERGENCIES (NATOPS 15-4, 15-6 to 15-7)

1. NO FUEL TRANSFER LIGHT

[1] Check total and nacelle fuel quantity

If no fuel in wing tanks (if nacelle quantity = total quantity):

[2] Transfer pump - OFF

If fuel in wing tanks:

[3] Transfer pump - OVERRIDE

If light remains on:

[4] Transfer pump - OFF (lose 28 Gallons)

[5] Land as soon as practicable

2. CROSSFEED light illumination: illuminates when boost pump fails or engine fails.  If boost pump failure: [1] close crossfeed switch.  If boost pump failed: [1] Failed boost pump - OFF [2] crossfeed - OPEN [3] Land as soon as practicable.  If not a boost pump failure, reset the crossfeed valve to 'AUTO.'  If engine failure, accomplish EMERGENCY ENGINE SHUTDOWN CHECKLIST.

3. Failure of the engine driven fuel pump - Engine WILL FLAMEOUT.  Accomplish EMERGENCY ENG SHUTDOWN CHECKLIST.

4.  If fuel leak - consider securing electric systems which may contribute to wing fire.  Secure nav lights, strobe lights, fuel vent heaters, and AOA.  Secure inboard wing items with gang bar.  If destination can be reached without dual engines, consider shutting down engine on side of fuel leak.  Secure engine as follows:

[1] Condition Lever - FUEL CUTOFF

[2] Emergency Shutdown Checklist - EXECUTE

5. If fuel syphoning - [1] Airspeed - 140K [2] Land as soon as practicable.  Avoid extreme nose low attitudes.

6. Single engine crossfeed:

[1] Boost pumps - ON

[2] Transfer pumps - AUTO

[3] Crossfeed - OPEN (check crossfeed light on)

[4] Boost pump (nonfeeding tank, tank w/o FUEL PRESS light ON) - OFF

Environmental

* W/O Power, no pressurization: Bleed air valves electrically opened, springloaded closed.

* 1 minute to go from max manual temp incr to max manual decr.

* Receiver/Dryer in wheel well.

* Firewall Shutoff Valve for environmental system held open by bleed air.

* Sidewalls: heating...  Cooling system=shunt after A/C Evaporator.  Rest of air thru heater, mixed w/bleed air.

* 16,000 BTUs for A/C

* Pneumatic: Boots, Door seal, Flight Hour Meter

* Unpressurized fresh air: from flapper ram air in A/C, or from bleed air heating system.

* NATOPS position 500-700 fpm cabin climb/descent.

* 2 o2 pressure gages, one in cockpit, other rt. Side of fuselage.

* 10k ft press starts climbing.  25 k ft, cabin alt > 10k ft.

* Below 20k ft on mask, can use normal.  3.7 L/min for passengers.

* 45 - 1 hour O2 for 3 man crew.

1)  Pressurization - Bleed air tapped after 4th stage compressor, delivered to cockpit at constant rate, controls pressurization through 2 flow control valves (outflow valve and safety valve.)  Components:  Cabin Pressure controller, Dual Cabin Altimeter, Cabin Rate of Climb Indicator, Outflow Valve, Safety Valve, Flow control units (bleed air valves).  Cabin pressure controller controls rate of outflow from cabin.   DUMP:  opens safety valve, relieves pressure differential.  TEST: Safety valve closes, PRESS:  Normally Open Preset Solenoid, Normally closed safety solenoid power removed with squat switch up.  Pressurizes via vacuum forces through preset solenoid.  Rate knob:  50 fpm-2000 fpm, Altitude knob - 1,000 ft to +10,000 ft.  Dual cabin altimeter, long needle = actual cabin altitude, short needle = pressure differential.  Cabin rate of climb = 0-6000 fpm.  Altitude warning annunciator: 9500-10000 ft.  Pressure switch on forward bulkhead.  Outflow valve:  Meters the outflow of cabin air in response to vacuum control forces from controller, Contains a preadjusted relief valve so that cabin diff does not exceed 4.7 psi.  Incorporates negative pressure relief diaphragm which keeps cabin altitude from becoming higher than ac altitude.  Safety valve: 4.9 psi.  Closes when:  landing gear strut extends, cabin pressure is TEST, vacuum source is lost, electrical power lost.  Flow control unit:  controls bleed air, Draws ambient air in with venturi to mix with bleed air.  Before start w/o eng: safety and outflow valves closed.  After start, vacuum opens safety valve.  On T/O, dump solenoid closes, releases vacuum closes safety valve.  Preset solenoid opens and time delay circuit lets left engine begin pressurization 6 sec before righ engine (no excessive pressure bump.)  Flow control unit mixes bleed and ambient air.  Goes through heat exchanger in wing center section  (or bypasses it).  Outflow valve keeps 4.7 psid or less (selected rate and altitude.)

2)  Heating and cooling:  Compressor bleed air, Ambient air, flow control, Heat exchanger, Electric heater, ducts, outlets  Cooling: same minus heater plus condenser, receiver dryer, and evaporator, and vent blower, and cabin ceiling outlets.    Electric heater:  8 elements in GND MAX, 4 in NORM.    Best heating:   Heat GND MAX, Vent blower high, open bleed air valves, close overhead outlets, Cabin Temp to Incr Heat, Mode Auto, Cooling: Close one or both bleed air valves, set vent blower high, open all overhead outlets.  A/C evap has 16,000 BTU freon.   Heater locked out with windshield heat, prop de-ice, engine lipboot heat.

3)  Oxygen system:  emergency, + 10,000 ft cabin alt.  Oxygen cylinder, Pressure reg control valve, Pressure gauges, Outlets, masks.  Cylinder: 49 cu ft, 1850 psi, 70 degF.  Reg valve controlled by push-pull knob up front.  Diluter demand masks: proper dilution of 02 with cabin air to conserve 02 at lower alts.  Green detector = flow, red = no flow, control knob on front lets you control.  100 percent vs normal on front, EMER = 100% at positive pressure.  Mic Normal/Oxygen to switch.

1. Primary purpose of the environment system is to provide for heating, cooling, and pressurization of the aircraft.

2. Safety valve: completely relieves all pressure differential when pressure control switch in DUMP or switch is in PRESS and aircraft is on the ground.  The LH landing gear safety squat switch opens the valve on the ground.

3. Outflow valve: closed on ground.  Moving the pressure control switch to TEST opens the solenoids, closing the safety valve and giving control of the outflow valve to the controller.  The LH landing gear safety squat switch opens the switch in the air.

4. Excessive pressure bump is controlled by the LH landing gear safety squat switch which: [1] removes power from the preset and safety valve solenoids [2] actuates time delay relay that allows left engine to commence pressurization 6 seconds prior to right engine.

5. Max psid for outflow valve is 4.7psid.

6. Max psid for safety valve is 4.9psid.

7. Actual control of psid in safety valve operation is by the pressure control switch when in DUMP or the LH landing gear safety squat switch when on the ground (the safety valve solenoid opens to supply vacuum to the safety valve holding it in OPEN or DUMP position).

8. Electric heater lockout system: prevents use of electric heater if windshield anti-ice, prop deice, and/or lip boot heaters are operating.

9. Max cooling: [1] Bleed Air Valves - CLOSE one or both [2] Vent Blower - HIGH [3] All overhead outlets - OPEN

Max heat: [1] Electric heat - GND MAX [2] Vent Blower - HIGH [3] All overhead outlets - CLOSE  [4] Bleed Air Valves - OPEN (after engine start)

10. The electric heater will work only when CABIN TEMP MODE is in AUTO.

11. With the vent blower in auto, the fan is operating in low speed.

12. With a total loss of electrical power, the T-44 will depressurize!

13.  None of the pressurization systems work in conjunction with the RIGHT squat switch.

14. Procedures for an altitude warning light:[1] Descend [2] Restore cabin pressurization [3] Check pressurization and/or don O2 mask.

15. Loss of cabin pressurization (NATOPS 15-11):

If gradual pressurization loss:

[1] Cabin altitude - Checked (P)

[2] Pressurization controller - Checked (CP)

[3] Bleed Air - Checked (CP)

[4] Press dump test switch - TEST (CP)  (hold for 15 sec)

If test switch restores press, may be necessary to hold switch in TEST until cabin profile at 10,000' or less.  If press  is regained through test switch, pull the PRESS CONTROL CB to maintain pressurization.

If unable to restore pressurization:

[5] Oxygen mask/microphone switch (100%) - AS REQ (P,CP,OBS)

[6] Descend - AS REQ (P)

On descent when cabin alt matches press alt, ensure PRESS CONTROL CB is reset to preclude landing

pressurized.

16. Explosive decompression: occurs in less than 1 second, causes fog, if high alt - rush of air from lungs, dazed sensation, gas pains, hypoxia if oxygen not used immediately.

[1] Oxygen mask/ MIC switches (100%) - AS REQ

[2] Descend - AS REQ

17. Supplemental oxygen is required when at cabin altitudes above 10,000'.

18. The oxygen system capacity is 1850 (+/-50) psig for one 49 cubic foot cylinder.

19. Between 17,000' to 20,000' the diluter demand regulator should supply 100% oxygen while in the normal position.

20.  The oxygen mask should be stored in the 100% position.

21. The min O2 requirement for local flights is 1000psig.  For cross-country flights: 1500psig.

Cracked windshield

External Panel: Heating elements inop, don’t use wipers.

Internal panel: descend/depressurize to 2.5 psi in 10 mins.  Lose some vis.

Cracked cabin window

Depressurize/Descend

Engine

* Engine cooled by forward->aft air flow, in front of stacks in, and out after stacks and after in aft section.

* Purge line is for FCU, not for EPA pump.

* Less than 3,500 ft PA, Idle is regulated constant.  Above that, can be as high as 83% due to min fuel flow.

* Starters go from 700 amps to 300 amps during start.

* 1) Engine:  PT6A-34B, 550 shp, reverse flow, free turbine, three stage axial single stage centrifugal compressor.  Airflow around plenum and into intake.  Diffuser vanes btw compressor and combustion chamber.  Compression ratio: 7:1.  Max N1 = 37,500 rpm.  (100%, 101.5%=38000 rpm.)  Combustion:  25% of intake air is mixed with fuel.  14 fuel spray nozzles (10 prim, 2 secondary), and 2 igniter plugs.  RGB drives prop governors through spline gears. RGB: two stages, 15:1 reduction ratio.

2) Fuel control system:  Engine driven fuel pump, Fuel control unit (inlet temps + power lever position, adjusts fuel flow, hence N1, hence power produced.)  Start control unit: mechanical fuel control valve, tied to FUEL CUTOFF position.

3)  Oil to fuel heater: preheats engine fuel with engine heat.  Oil heats it in a radiator form.  When fuel temp >70degF, valves restrict the amount of oil flow.  90degF, valves closed completely, oil bypasses core.

4)  Fuel drain collector system (after engine shutdown):  Collector tank:  residual fuel from  manifold flows in, Fuel Drain Collector Pump: Automatically  transfers residual fuel back to nacelle tank (Batt powered by #1 and #2 bus feeders.  Has CB.)

4) Starter/Gens:  Initial starter draw 700 Amps, drops rapidly to 300 amps as engine reaches 20% N1.  IGN & ENG START:  completes starter circuit, energizes igniters, open purge valve in fuel control unit, activates IGN IND light.  Purge solenoid electrically connected to ignition system and relieves any trapped fuel vapor.

5)  Compressor bleed valvu:  Centrifugal works best @ high RPM, Axial at low, so to prevent compressor stall, bleed valve between 3rd and 4th compressor stage.  Fully open less than 62% rpm, fully closed >75%.

6)   Engine limitations:  ITT 790 max continuous, 1090 max starts... Self powered.  Torquemeters: 1315/1520 foot lbs redlines, will not work with loss of 26VAC power...  Prop Tachs self powered from tach generator in RGB.  Turbine Tachs: slaved to a tach gen in the RGB.  Fuel flow: Calibrated for JP-5.  Oil PX:  Redline: 40 psi.  Oil temp redline: 99degC.

Ops limits:  Max allowable power:  Max power available for takeoff (5 min max.)  Max continuous power:  Max power (emergency use, pilots discretion) not limited by time.  Cruise climb:  Max power for normal climb.  Cruise:  Required to maintain level flight.  ITT: 400-790 degC, max allow: 790, max continuous: 790, Max cruise 740, Max criuse climb 765, max low idle: 685, max accel 850 (2sec)  Max reverse: 790, max start: 1090 (2sec), if likely to exceed 925, fuel cutoff.  Torque: 400-1315 ft.lb.   Max allow + avail: 1315 ftlb @ 2200 rpm for 5 mins... Max conti4ous: 1520 ftlb@ 1900 rpm, 1315 ftlb @ 2200 rpm.  Prop tach: 1800-2200 rpm, Max: 2200, Max  accel: 2420 (2sec), Max reverse: 2100.  Turbine tach:  50-101.5, max allow: 101.5, max continuous, cruise, cruise climb: 101.5, max accel: 102.6, max reverse: 86.  Oil temp: 10-99 degC, max:99, Oil PX: 85-100 psi (above 72%n1), max: 100, min 40, below 40 unsafe, shutdown/minimum power, btw 40-85 undesirable, completion of flight at reduced power.

6) Power control quadrant:  PCL: regulates developed power from idle to max, increase power levers, increase N1.  Reverse:  15 to -5 deg.  Movement of props in beta controls prop pitch only.  Reverse range: -5 to -11 ged, controls pitch and n1.  Condition levers:  start or stop fuel, control idle from 50 - 67% n1.

7) Oil system:  Oil tank between engine air inlet and accessory case, oil -cools and lubes engine, -operates prop pitch change mech, - controls torque meter system + torque px switches.  Capacity: 14 quarts (9 quarts in thank, dipstick can read from 5 quarts.)  Max allow oil consumption = 1/4 pint / hr.  Components:  Integral oil tank, oil filler + dipstick, Oil cooler, oil-to-fuel heater.  Oil pressure pump, Oil PXZ transmitter, chip detector, Prop gov ernor.

8)  Fire detection system:  3 fire detectors (3 photocell detectors per engine), fire detector amplifiers: one each side of airplane, cockpit indicators.  Test:  position 3 checks fuel manifold, forward nacelle and exhaust outlets detectors, 2: upper accessory compartment, 1: lower accessory compartment.  Fire extinguisher: Supply  cylinders (fire bottles), one on each sided of main wheel well,  charged with 2.5 lbs of extinguishing agent (bromotrifluromethane), pressurized with 0.8 lb dry Nitrogen.  Push to ext switches: in cockpit, Explosive squibs: at base of each fire bottle, Extinguisher system network: hoses and turbine spray nozzles distributed strategically in engine.

ENGINE SYSTEMS

1. Each engine is rated at 550 shp.

2. Total compession for each engine is equivalent to 7:1.

3. There are 2 igniter plugs per engine located at the -3 and -9 o'clock position.

4. Approximately 75% of intake air is actually used for cooling.

5. Reduction gear box (RGB) provides reduction ratio of 15:1.

6. The systems operating off the N1 compressor turbine are the compressor and accessories case: [1] fuel control system - engine driven fuel pump, fuel control unit [FCU], starter control unit [SCU], fuel spray nozzles, oil-to-fuel heater [2] fuel drain collector system - [a] fuel collector tank [b] fuel drain collector pump (EPA pump) [3] starter/generators [4] turbine tachometers

7. The systems operating off the N2 (power) section are [1] reduction gear box (RGB) [2] propeller system [3] engine torquemeters [4]propeller tachometers

8. Inputs are sent to the Fuel Control Unit (FCU) are: [1]Power lever [2]Compressor turbine speed (Ng) - fuel control's governor flyweights driven by accessory section [3]Compressor air (P3 air): air sample in which air density determined [4]Propellor governor speed (Nf) - pneumatic system  determining prop speed (the fuel topping governor).

9. Inputs to the starter control unit are through the condition lever.

10. The compressor progressive bleed valve opens at low N1 to reduce compressor pressure between the axial and centrifugal compressors.  As N1 increases, starts to close at 62%N1 and is fully closed by 75%N1.  When power is reduced, the valve opens to bleed excess pressure in the compressor.

11. The fuel drain collector system consists of the fuel collector tank, fuel drain collector pump, and associated fuel lines.  The fuel collector pump provides automatic transfer of accumulated residual fuel back into the fuel supply by after shutdown when the fuel collector tank is full and its internal float switch actuated.

12. When the starter switch is accuated to 'STARTER ONLY,' the engine is motored without providing ignition.  In 'IGN ENG START' position: completes the starter circuit, energizes the igniter plugs, opens the purge valve in the FCU, provides fuel to the fuel nozzles, and actuates the 'IGN IND' light on the annunc panel.

13.Torque is measured by a movable ring gear located in the first reduction gear stage.  Metered oil pressure across the orifice is measured against a calibrated pressure sensor in proportion to torque sensed.  The measurement is sent to the torquemeters through a 26V AC signal.

14. With a failed torquemeter, auto ignition and autofeather are still available.

15. The oil system has a 14 Quart capacity.

16. The top 5 Quarts of oil is measurable.

17. The oil tank can hold 9 Quarts.

18. There are 4 scavenger pumps per engine.  They return oil to from the oil pickups to the oil cooler.

19. Autofeather is accomplished by: [1] Propellor Autofeather switch in 'ARM' [2] both power levers > 90 (+/- 2)% N1 [3] Torque on one engine <260(+/- 50)ft-lbs.  The autofeather relay applies current to the dump valve, causing the release of oil pressure holding pitch angle on the prop.  As the pressure releases, the feathering springs (assisted by centrifugal force applied to the blade shank counterweights) feather the prop.  After one prop feathers automatically, the other prop can only be feathered manually.  The right prop may not fully feather with the prop sync on.  Autofeather only used during takeoff and landing.

20. Oil temp is maintained by running oil through a heat exchanger unit.  They are attached to the nacelles below the engine air intake.  As oil is returned from the integral oil pickups, it is returned via scavenger pumps to the oil cooler.  Air passing through the oil cooler cools the engine oil.  Normal oil temp (10 - 99C) is maintain by a thermal sensor that controls the bypass valve allowing some oil to bypass the oil cooler.

21. The oil-to-fuel heat exchanger utilizes heat from engine oil to preheat engine fuel.  A fuel temp sensing oil bypass valve regulates fuel temp by either allowing oil to flow through the heater core or bypassing it.  A temp sensing element controls the oil inlet valve.  At a fuel temp >70F, the bypass valve begins to open; at 90F, the oil completely bypasses the heater core.

22. The chip detector is located at the bottom of the reduction gearbox housing.  Its purpose is to warn the pilot of ferrous metal in the oil and a possible engine failure.  If a large particle (or several small particles) complete the circuit, the 'CHIP DETECT' annunc light and the 'FAULT WARNING' light will illum.

23. Approved oil for the T-44 is MIL-L-23699, NATO O-156, or EXXON 2380.  (List found in NATOPS 3-2)

24. The fire bottles are normally pressurized to 450psi.

25. The minimum N1 required on the deck for generator loads exceeding 0.5 are [1] 0.5 - 0.75 = 57% [2] 0.75 -0.9 = 60% [3] 0.9

-1.0 = 63%

26. The oil system provides: [1] cools and lubricates the engine [2] operates prop pitch change and engine torquemeter system.

27. The procedures for a cold oil check: [1] Open the upper right cowling to gain access to the oil filler cap [2] Remove the oil filler cap. [3] Replenish oil to within 2 quart below MAX mark on dipstick. [4] Check oil filler cap for damage and general condition [5] Check for oil leaks.

Electrical

* 5 Amp fuses/CB for everything on fuel bus but fuel bus boost pump (10)

* AC ground: 1000A for 0.1 sec, 300A continuous

* Inverter circuits: each 26V:5A fuse, each 115V:6A fuse, 1A fuse for everything on bus.

* Lose inverter select realy fuse, lose that side's 115VAC bus.

Malfunction: Gen Fail...

Evaluate Current Limiters, 3 possibility:

1) Good Current Limiters: no drop in batt, everything works.  Keep load under 1.0.

2) Bad CL (same side): lose avionics bus, fuel bus, inverter for that side.

3) Bad CL (opp side): batter discharging, everything works.  Conserve batt.

1) Gen - OFF, Reset, back on

2) Gen - OfF (if still broke)

If works in reset, Gen Control Box broke.

3) CLs check

4) Op gen - don’t exceed 1.0

5) Land as soon as practibile

NOTE: Fail w/ smoke+fumes=internal failure.  Check by closing bleed air.  Consider engine shutdown.

Dual-gen Fail:

Ensure maximum battery duration

1) Cabin Temp, heater, antiice/deice, autoignition, lights, radar - OFF

2) Emergency voice report - complete

3) VMC - Gang Bar - OFF, IMC - Gens -OFF

4) Boost pumps - OFF

5) VMC - Inverters - OFF, IMC  - #2 Inv:OFF

6) Pull a lot of CBs.

7) VMC: Avionics Master - OFF, Batt - ON

8) IMC w/ GCA: Pull a lof of Nav CBs

9) IMC w/VORTAC: Pull more CBs for other stuff.

Note: No-flap, pump down the gear.

Excessive Load:

Due to ground fault or excessive batt charge.

1) Battery/Ammeter - Check

>30 A: 2) Battery - OFF

3) Battery/ammeter - Check

>30 still: battery relay failed.  Land as soon as possible.  If it drops:

4) Recheck loadmeters

If normal, excessive battery charge.  Land as soon as practibile.  If excessive, ground fault land as soon as possible.  Watch for fire.  Note: load splits >0.1=bad paralleling.  W/ AC on, CL broke.

Inverter Fail

1) Failed Inv - OFF

2) Check for AC Switchover.

3) Land as soon as practibile.

Inst Inv Fail

Torquemeters Lie.  Can’t fix.

Tripped CB:

1. Nonessential - Don’t reset.

2. Essential- Push to reset.  If pops again, do not reset.

DON’T Reset Subpanel Feeder CBs.

Avionics Master

Lose all avionics, poweris good.  Pull MASTER POWER CB.

1)  Battery:  24v 42a  lead acid, in right wing root.   Emergency power, starts engines, acts as damper.

2)  Starter/Generator:  250 amps (Accessory gearbox) Generator Control Box located under main wing spar in center aisle.  (Regulates voltage, automatic paralleling, overvoltage protection 31VDC, undervoltage protection 18VDC, reverse current protection)  Load is measured in percent of 250 amps.  (.3-.6)

3) APU in rt engine nacelle...  No more than 28VDC, capable of 300A with 0.1 sec bursts of 1000A.

4) DC Busses:  10:  Hot Batt Bus, Batt Bus, #1/2 Subpanel Bus, #1/2 Avionics Bus, L/R Main Bus, #1/2 fuel bus.  Hot batt bus:  Singularly powered:  L/R Fire Extinguishers, Threshold lights (wing spar, baggage door, cabin door observer lt, aft compartment lt, xfeed lt) Dual powered (w/fuel bus): L/R Boost Pumps, L/R Firewall Valves, Xfeed valve.

5) Current limiters: Bus ties.  Allow either bus to get power from other generator..   Also isolates shorts from rest of system.  325A Slow fuse.

6)  Inverters:  750 Volt-Ampere single phase inverters.  Supplies 26Volt AC,for Torquemeters, 115 vac for the avionics.  Inverter relays driven off inverters, fail-safe?  System adjusts for failed inverters.  If you lose an inverter relay select fuls, you lose the AC avionics on that side.

7)  Lights:  Master Cockpit is everything except for Threshold lights, Door locking light, Seatbelt light, reading light, utility light, indirect instrument lights, aft compartment ligght.  Pull GYRO INST CB if Master Cockpit lights fails, for all lights other than gyro lights.  Exterior lights (Landing/Taxi) shutoff with gear retraction.

8) Emergencies:  Generator fail boldface:  good voltage in RESET indicates failed gen control, not bad generator.  Lose both gens, on battery power (42Ah remains.) Run off math from batt amp. Current limiter fail inflight = 0.2 difference btw busses.  Ususally L. Main higher.  Both curr limiter fail, same thing as one failing.  Variable split in loadmeters.  Gen fail w/opp Curr Lim fail:  everything works, but batt being discharged.   Same side gen fail curr lim fail, lose avionics bus, lose fuel bus on that side.  Inverter fails, turn it off and land.  Total AC failure: can only uses fuel flow, oil px, itt, n1, pitot static, prop rpm, clocks, ncs31, aoa, radalt, compass, turn and bank.  (Heading, attitude, nav all fail, and torque.)  Only have DME, ADF audio, XPDR, marker beacons, UHF VHF audio.

9) CBs:  Can reset once, but NOT Subpanel feeder, non-essential.  If Avionics Master switch fails, pull Master Power CB to bypass.

1. Sources of DC power: 1 24V/42A battery, 2 28V/250A starter-generators

2. Sources of AC power: DC power routed through 2 750V-A single-phase inverters.

3. The generator control box: overvoltage protection (31V), undervoltage protection (18V), reverse current protection, & automatic paralleling.

4. The generator control boxes are under the cabin center aisle aft of the main spar.

5. The inverters are located outboard of the engines on the wings.

6. The current limiters: tie the main buses & provide fault protection.

7. To get around a faulty master avionics switch, pull the master power CB.

8. Limitations of the generators: 28.25 (+/- 0.8)V.

9. Limitations of the inverters: 114 (+/-7)V, 400 (+/-6)Hz.

10. For APU start, the unit must be able to provide 1000 Amps for 0.1sec and 300Amp continuous load thereafter.

11. A failed Inverter Select Relay Fuse will cause failure of respective AC bus without illum of inverter light (switch over impossible).

12. Illum of INST INV OUT light indicates loss of 26VAC inverter select relay & torquemeters.

13. Procedures for INVERTER failure: [1] Fail Inverter - OFF [2] AC Bus switchover [3] Land ASA practicable

Total ac failure, still have: fuel flow, oil press, oil temp, turn & bank, pitot static, N1, ITT, prop RPM, clocks, NCS-31A, AOA indexers, RADALT, wet compass14. Procedures for GENERATOR failure: OFF, reset momentarily, ON.

15. If generator fails to reset, yet registers normal volts in reset position, indicates failure of generator control.

16. Total loss of power, still have: copilot turn and slip, VSI, altimeters, airspeed indicators, wet compass, N1 and N2, ITT, & clocks.

17.  Dual inverter failure, still have DME, radar, VOR (audio), ADF-LF (audio), xponder, MKR BCNs.  For comm still have, UHF & VHF.

18. Excessive load indications (>1.0): [1] Battery/ammeter - Note.  If charge rate in excess of 30A: [2] Battery - OFF. If loadmeter still over 1.0: [3] Non-essential loads - OFF.  Splits of >0.1 indicate abnormal generator paralleling.

19. If get a GEN OUT light, check Current Limiters: [1] Battery- no discharge, no other equip failures = CL intact. [2] Battery - discharge, no other equip failures = opposite CL failed. [3] Battery - no discharge, other equip failures = same side CL failed.

Gear

* Dynamic braking and limit switches limit gear extension and retraction.  Overload clutch prevents damage with a mechanical malf.

* Gear extension should not exceed 8 sec, retraction 10 sec.

* Only PIC can direct horn silencing.

* Cannot override horn during final approach, or in pattern.

* J-hook may not stop gear on ground if rt. Squat switch doesn’t have enough weight on it.

* Shock spring in NWS absorbs vibrations before they get to rudders.  Shimmy damper as well.

* On emergencies, get visual confirmation.  Is possible to get 3 green with unsafe indications.  Wheel falls off, strut still works, etc.

* Don’t taxi with unsafe gear indications.

Emergencies: All Generally Deferred, to your advantage to burn off fuel, decrease weight.

Unsafe Up:

1. Gear Handle - Down

2. Gear position - Check

3. If safe down, land as soon as practibile

4. If not, go to unsafe down procedures

Unsafe Down:

1-3. Check CBs

4. Check lights (press-to-test, check if wheels up warning system works under 79% N1, check AoA indexer lights.

No AoA: motor cutoff, manually extend.  Check prop sync lt (rt.), L/T lights (nose), GND MAX ht (hold in pos, won’t lock w/ squat switch) (left) (wont work w/ windshield heat, prop deice, lip boot heat.)

5. Visual gear pos check.

6. If safe, land ASA practibile.

7. If not, determine if electrical.  If electrical, emergency extend.  If not:

8. Gear handle - up

9. Gear pos visual check.

10.  Visually up - gear up landing.  If not, unsafe gear landing.

Airborne Gear Inspection

1. Join up with comms

Don’t erratically maneuver

2. Check tire inflation, J-hook, mechanical downlock, condition, gear doors and hydraulic leak.

One Main up Landing

Favor the good wheel, drag in the center, no cables, turn on lights for night emergency egress.  Execute checklist, brief, lock harnesses, landing check, kill engines, use rudder and ail to keep bad wing off as long as possible, brakes, emergency shutdown on deck.

Main Down, Nose Up.

Difficult to get out of AC because of nose low, DO NOT USE BRAKES!  Hold nose off as long as possible.  Same as one bad main, but no brakes, no rudder/ail.

Gear Up

Tends to overshoot, touchdown at 85 kts.  Brief, secure seatbelts, disengage clutch, stow manual extension handle, pull LDG GR CB, Flaps - APP, Gang bar off if possible, immediately prior to TD, Cond lev - Fuel C/O, After Touchdown - Power - Idle, Cond Lev - Fuel C/O, Brakes - A/R (can use brakes and rudders for dir control.  Finish emergency shutdown, shoot the fire extinguishers.

Main Flat

Put the drag in the center, use opposite brake for directional control.

Nose Flat

Less stable NWS, aerobrake, light wheelbraking only to maintain dir control.

Brake Failure

Dir control w/ rudder + asym power, stop from reversing to aid in decel, do not taxi.

1)  Gear:  operated by motor (single, split field, 28vdc, under co seat.)  Motor limit switches prevent overtravel (under pilot seat)  Torque shafts drive main actuators, duplex-chains drive nose gear.  Spring loaded locks fix gear in down position, jackscrew in nose fixes it in overcenter down, jackscrews holds all three up.  GEAR MOTOR CB on pedestal.  Gear retaction time, 5-7 sec., grae extension time 4-6 sec.  2 red bulbs in warning lights, means:  unlocked, in transit, HD LT TEST, below 79%+-2 power, below approach flaps, gear not down.

2)  Wheels up warning system :  warning horn, lights, when gear not down and locked + power below 79%+-2 N1, Flaps beyond approach, can silence horn.  DON’T override on traffic pattern or final approach.  Silence only works w/ flaps up.

3)  Squat switches:  Mechanically activated.  Rt.:  (act.) Gear inop, downlock hook,  (de.) Flight hour meter, (de) Indexer for AoA, Stall warning (de), Heat rt. Eng. Lip boot (de.).    Left: Heat AoA vane reduce to 14 from 28 (de), Heat electric heater GRD MAX (de from NORMAL), Heat lip boot left engine (de), Ambient air solenoid (de.), Pressurization: preset (de.), dump (act.), door (closes)

4)  Downlock and uplock switches:  each strut has one (3) of each downlock and uplock.  You can check which one is not fully retracted by PROP SYNC; if rt. Uplock closed Propsync won’t turn on.  Landing lighs... If nosewheel uplock closed, landing lighs won’t come on.    Electric heater: checks left.   Motor limit switches: up limit deactivates motor, deactivates g-meter when gear is up.  Down limit switch deactivates motor, activates AOA indexer lts.

5)  Alt gear extension:  Can be manually exteended, but not retracted through handcrank.  Do not do it if a malfunction is suspected.  Right side of pilot seat.  Must disengage landing gear clutch to pump handle, lift up clutch, turn clockwise, reengage by doing opposite.  Airspeed - 120 KIAS recommended (155 max), A/P-A/R, landing gear relay CB - pull, Gear handle - down, Clutch disengage - list, turn clockwise, Manual handle - pump until 3 green (50 ish pumps)

6)  Brakes:  Multidisc hydraulic brakes, Shuttle valve allows changeover from one set of brakes to the other.  Dual parking brake valves lock pressure.  Fluid from hyd brake system resevoir, NBO emergency brakes,  out setting maintains check valves, only Px in not out.  Only Pilot, not during flight.  With wheelbrake fail:  directional control w/ diff power, rudder, NWS.  Use reverse or beta to help.  Maneuver into open area, NO taxi.  Shuttle valve sometimes fail.  Reset by pulling aft on top of brakes.

7)  NWS:   Direct:  12L/14R, w/ brakes: 48 deg.  Retraction autodisenages.

1. Nosewheel steering actuates nose gear 12 degrees left and 14 degrees right.  (48 degrees either side with braking)

2. The function that up and down limit switches share is deactivates landing gear motor and prevents overtravel of gear.

3. The up limit switch also deactivates the g-meter when the landing gear is extended.

4. The down limit switch also activates the AOA indexer lights.  On the ground, the right main squat switch deactivates the AOA indexer lights.

5. The three up locks all will turn on the red lights in the gear selector handle, illum the wheels up light, and activate the gear warning horn if the gear don't fully retract.

6. The right up lock also deactivates the prop sync light when the prop sync switch is ON.  If the right up lock has gear fully retracted, the prop sync light will not come on when the prop sync switch in ON.

7. The left up lock also keeps the loading for the electric heat in NORM & GND MAX within slight fluctuation or no significant generator load difference.   If the left up lock has gear fully retracted, switching the electric heat from NORM to GND MAX will yield only a slight or no significant change in generator loading.

8. The nose up lock also deactivates the landing lights.  If up lock has gear fully retracted, no illumination of landing lights and no increase in generator loading when landing lights are turned ON.

9. The right squat switch: [GFISH] on ground

Gear Circuit inop

Flight hour meter inop

Inlet lip heat inop

Stall warning & AOA indexer lights inop

Handle (gear) & hook (downlock) activated

10. The left squat switch: [HHHAP] on ground

Heat for left engine lip boot inop

Heat for AOA sensor - LOW (14V)

Heat - GND MAX

Ambient air solenoid - CLOSED

Pressure control safety valve - OPEN

11. Down lock switches: close when strut fully extends.  When all 3 down lock switches activated (all 3 gear down & locked): [1] WHEELS UP light deactivates [2] Gear horn deactivates [3] 3 green down lights illum

12. The red light in the gear handle will illuminate when: [1] the gear is unlocked, in transit, or gear warning horn activated [2] both power levers retarded below 79%N1 when gear not down and locked [3] pressing the Landing Gear Handle Warning Light Test Button [4] Flaps below APPR & gear is not down and locked.

13. The warning horn silence button and the WHEELS UP light can be turned off in flight (with power retarded, gear up, and flaps up by pressing the WARN HORN SILENCE button.  After silencing the horn, it will remain silent until the flaps are extended or at least one power lever is advanced beyond 79%N1 and then pulled back below 79%N1.  Don't silence the horn in the traffic pattern or during the final segment of an IAP when potential to land exists.  Only the PIC will cancel the horn.

14. During manual gear extension, the first green down lock indicator to illum will be the nosegear light.

15. The extension time of the gear is 4-6 seconds.  The retraction time of the gear is 5-7 seconds.

16. The prop reverse not ready light will not illum with the landing gear relay CB pulled (during manual emergency extension of gear).

-- The warning horn, wheels up light, and gear handle will all come on when all 3 landing gear struts are not down and locked and: [1] both power levers retarded < 79(+/-2)%N1  OR  [2]Flaps extended beyond approach position (Flaps - FULL)

-- The wheels up light & gear handle light illum when: [1] both power levers retarded < 79(+/-2)%N1 with Gear UP  OR  [2]Flaps extended beyond approach position with Gear UP.

17. It is possible to have a safe gear indication and not have safe gear (such as not having 3 complete tire or wheel assemblies).

18.  The main gear are held down and locked by spring loaded locks. The nose gear is held down by a jackscrew actuator.

19.  The gear are held up by the jackscrew in the actuator.

20. I can tell the difference between an electrical and mechanical malfunction on an unsafe gear by comparing visual indications with cockpit gear indications and gear circuit breaker positions.

Pull the following circuit breakers: [1] Landing gear relay CB [2] Landing gear indicator CB [3] Landing gear motor CB [4] Cockpit gear position indicators.  If during pulling any of the following , the gear indicates safely down and is visually confirmed safely down, land as soon as practicable.  ***Absence of illuminated AOA indexers could indicate premature landing gear motor cutoff which is indicative of an electrical problem.

Prop

* 3-bladed aluminum prop.

* Autofeather oil pressure dump is on the overspeed governor.  Intentionally redundant: alt feather procedure.

* Rt. Autofeather doesn’t totally work w/ prop sync on.

* Recenter prop sync if not working.  Prop sync - off, Manually sync, Prop Sync - On.  Takes 6 seconds.  Tolerance of synchrophaser is +-30 rpm.

* Normal ops: pneumatic section of primary gov.=fuel topping.  Limits to 2332, also limits to 2100 in reverse.

* Gov pump: 375-400 psi.

1) Governors:  engine oil provides hydraulic control in prop sys, thru primary governor by governor pump (375 to 400 psi)...  Senses flyweight rotation around speeder spring, metering oil through pilot valve.  If primary governor malfunctions and exceeds 2200 prm by 4%, overspeed governor will cut in and reduce oil pressure and maintain prop speed at 2288 RPM.  Prop lever sets speeder spring tension setting default oil level.     In prop dome, similar to T-6, feathering spring, oil in pushes back, through pitch change linkage changes blade angle.  Higher oil Px decreases blade angle, increases prop speed.  Overspeed governor 2288 +-40.  If prop speeds exceed 2332 rpm, overspeed governor failed and pneumatic section of primary acts as fuel topping governor.

2) Blades:  Twisted to maintain even thrust, with bladeshank counterweighs to balance.  Feaqther: 87 deg, Low Pitch 15 Deg, Zero Thrust -5 deg, Reverse -11 deg, Cruise 25-35 deg.,

3)  Prop levers:  HIGH RPM: 2200

FEATHER: self-explan.  Reverse:  High RPM.  Caution ligh if gear down and not High RPM, tells you to not reverse.

4)  Test switch:  Resets overspeed governor to 1900 to 2100 rpm.  Autofeather system dumps oil in the event of engine malf.  If engine loses power at a position 90 percent +-2 N1, 2 torque sensing switches actuate.   Test position of AF switch bypasses 90% N1 Microswitch.

5) Synchrophaser system:  in-flight use, minimizes stress, reduces noise.  Synchs props at same RPM, phase the blades so 2 blades do not pass the fuselage at the same time.  Left propeller is master, righ is slave.  Mag speed pickups in overspeed governor + 3 mag phase pickups on props de-ice slip ring, transmits to control unit.  Actuator motor on rt. engine trims prop governor to exactly match left rpm.  Synchrophaser limited to +-30 rpm, so doesn’t autofeather.  Righ prop may not fully feather with propsync, and warning will go off if landing gear extended (rt.)

6) Emergencies:  past 2200 n2: prop governor failure.  Overspeed to 2288.  Sticks or moves too slowly during transient condition, prop speed exceeds 2332, then overspeed gov failed, and pneumatic section of primary governor acts as fuel topping governor.  Limits fuel flow to the engine.

1. Prop characteristics: hydraulic controlled, constant-speed, full-feathering, and reversible.

2. Feathering: feathering springs assisted by centrifugal force applied to the blade shank counterweights.  Moving Prop Lever to FEATHER lifts pilot valve to allow complete dump of high press oil.

Manual feathering: pull prop lever to feather.

Automatic feathering (ARMED): sense loss of torque oil press and feather an unpowered prop - only can occur when: [1]

autofeather - ARMED [2] both power levers > 90 (+/-2)%N1 [3] torque on one engine < 260 (+/-50)lbs.  Before auto prop

feather, interlock disarms autofeather circuit of opposite propeller at 410 +/- 50 ft-lbs.

Feathers for: [1] loss of boosted prop governor oil pressure (engine shutdowns on ground and anytime prop not turning).

Once one prop feathered, other prop must be manually feathered.

Right prop sync may not feather with prop sync on.

3. Low Pitch Blade angles: 	Feathered = 87	Low Pitch = 15	0 Thrust = -5	Reverse = -11	Cruise = 25 to 35

Idle = 15 to -5	Beta = -5 to -11

4. Max Reverse = -5 to -11 at 86% max N1 at 2100 prop RPM

5. Max N1 in BETA _______ (condition levers - LOW IDLE)

6. Centrifugal counterweights, assisted by feathering spring, move blades to low RPM (high pitch) and feather.  Governor boosted oil press moves prop to high RPM (low pitch) hydraulic stop and reverse.  Low pitch by mechanically activated hydraulic stop, causes beta valve on primary gov to stop oil flow to prop dome.  Low pitch blade pos by primary low pitch stop.  Allows blades to rotate beyond low pitch into reverse.  Beta and reverse provided by adjusting low pitch stop, controlled by power lever when aft of idle.  Prop is at low pitch when prop RPM is below selected governor PRM (when prop lever full forward position 2200, but N2 reads 2000).  No high pitch stop so prop can feather.

7. Norm op range of primary gov is 1800-2200RPM.

8. If prop RPM exceeds 2200 by more than 4%, the overspeed gov 2288 +/-40.  Fuel topping gov 2332.

9. In reverse, pneumatic section of primary gov reset to allow max 2100RPM.

10. Prop gov test switch - actuates solenoid so overspeed gov reset to 1900 to 2100

--  Condition levers - HIGH IDLE - Max RPM 2200.

--  Power levers - IDLE & Condition levers - LOW IDLE - Prop RPM 900 to 1100.

--  If prop RPM not btwn 900 to 1100 with power levers - IDLE & condition levers - LOW IDLE, perform low pitch torque check (NATOPS 10-5).

11. Reduction gearbox failure and/or N2 turbine damage occurs at 2420RPM.

12. Prop reverse not ready light: gear down and prop levers not HIGH IDLE.

13. Autofeathering: used only on t/o & landing.  ARM.  Engine loses power, ARMED, power levers > 90 (+/-2)% N1, 410 (+/-50)ft-lbs & 260 (+/-50)ft-lbs torque switches actuated by loss of torque pressure.  Current through autofeather relay to dump valve, releases oil press holding pitch angle on affected prop.  Feathering springs assisted with centrifugal force applied to blade shank counter weights.  TEST pos on switch enables check < 90% N1 on ground only.

14. Syncrophaser master engine = left.  Inflight use only.  Maintains blade positions of both props via magn speed pickup on prop overspeed gov and mag phase pickups on prop deice slip ring adjacent to brush block.  Creates pulse rate, transmitted to actuator motor on right eng.  Motor trims right eng to left.  Trimmer screws in  & out.  Limited to +/- 30 RPM gov setting.   Right prop may not fully (manual) feather with prop sync on. If props won't sync, actuator at limits - turn prop sync OFF, synch props manually, turn sync ON.  Prop Sync On light when in system in use and landing gear extended.

15. The prop lever controls the primary gov - varies tension on Speeder Spring to reposition Pilot Valve, allows more gov boosted oil into prop dome for (Higher RPM, low pitch) or oil dumped from dome (Lower RPM, higher pitch).

16. Power lever IDLE or above - sets low pitch at 15 blade angle.  Beta = -5.  Max reverse = -11.

17. Fuel topping gov - pneumatic section of primary gov, limits fuel flow to eng, reduces N1 RPM, on at 2332RPM.  Pneumatic line routes bleed air from eng compressor to FCU to primary gov.  If >2332RPM, airbleed orifice of prim gov opened by flyweight to bleed off P3 press through primary gov, closes fuel control bellows, reducing fuel flow.

18. In reverse, the pneumatic sect of primary gov reset to allow max of 2100RPM.

19. A prop would feather in flight if autofeathered armed, power levers > 90%N1, drop in torque below 260ft-lbs on same engine OR prop lever manually pulled to feather.

20. Max N1 acceleration 102.6%.

PROPELLER MALFUNCTIONS (NATOPS 15-10)

Primary Governor Failure (memory item): feather or overspeed (outside normal limits). * Certain failures can cause beta valve to be inop, causing overspeed. Control w/ overspeeding prop very difficult at lower power settings and airspeeds, highly discouraged.  If necessary, landing with overspeeding prop carry extra airspeed.  Gives more rudder authority, reduces prob of in-flight beta.

If prop rpm out of normal governor range (1800 - 2200RPM):

[1] Propeller RPM  -- Adjust to Normal RPM.

By prop lever. If norm restored, continue ops.  If norm can't be obtained:

[2] Power Lever - IDLE (concur)

[3] Propeller Lever - FEATHER (concur)

Notes: Prop RPM > 2420 may result in RGB failure and/or N2 turbine damage.  Engine with disabled prop can provide electric power.  Right prop may not full feather with prop sync ON.  Fully feathered prop will still be running at about 200 rpm.

[4] Alt Prop Feather Checklist  - AS REQ

Prop Linkage Failure: prop gov ctl linkage fails, prop goes to 2200 or maintains last rpm setting.

[1] Manipulate prop lever to determine if cockpit control lost

[2] If cockpit control lost and RPM in safe limits, match opposite prop speed with uncontrollable prop and land as soon as practicable.

CAUTION: Reverse w/o high RPM can damage prop.

Alternate Prop Feathering: Linkage or gov fails and prop doesn't feather by itself or normal feathering procedures don't work, use autofeather.

[1] Power Lever - IDLE (failed propeller) (concur)

[2] Condition Lever - FUEL CUTOFF (concur)

[3] Autofeather - ARM

[4] Power Levers -- > 90% N1 *Don't pull power lever to IDLE during autofeather, may disarm system.

[5] Emergency Shutdown Checklist - EXECUTE

Anti-Ice

* Wipers, AoA Heat, Pitot Heat, W.S. Heat, Prop Heat, Lip Boot Heat, Fuel Control Heat, Oil-to-fuel heater, Fuel Vent Heat, Ice Vanes, Boots (5.)

* Wipers: Park/Slow/Fast (#2 subpanel bus)

* 95degF lockout for the windshield anti-ice heater.  Mag compass won’t work w/ it, objects may appear blurred.

* Engine anti-ice concerned w/ intake screen.  Ice vanes eject high-mass air out an intake in the back of the engine.

* Fuel Control Heat is for FCU pneumatic heat.  SHOULD NOT be turned on w/o cooling air over the engine.  (After engine start.)

* Prop Deice:  Don’t use stationary: will cause arcing on the slip ring!  RH prop out, RH prop in, LH out, LH n.  34+-6 sec.  Only older mechanical types show fluctuation when switchover.  5-6 A current draw each element x3.

* Boots: default: vacuum supplied for hold down.  Pressure comes in, drains thru check valve to 1 in Hg, the vacuum sucks rest out.  Either engine can supply enough bleed air, passes thru 18 psi regulator.  Vacuum thru venturi, proportional.  Never cycle fast, can cause ice to form that can’t be booted off.  Inflate for 7-8 sec.

* AoA heat, 14-28 Vdc.  Don’t use pitot heat on ground, no cooling airflow.

Anti-ice/deice:  Windshield wipers, Windshield antiice 95 degF, Powerplant Ice protection system (lipboot heat, Ice vanes [moves plenum, decreases torque, increases ITT, reduces range 10-12%]), fuel system antiice (fuel-to-oil heater, electric fuel vent heating element protection, fuel control unit pneumatic line heated jackets [subpanel #1,#2 busses]), Prop electrothermal deice (outboard/inboard element, power from brush block, slip ring, de-icer timer [sequence RH outboard, inboard, LH out, in, 34+-6 sec] should draw 14-18 amps), Pitot/AoA AntiIce (14 on ground, 28VDC in air), Surface deicer (7-8 second single cycle.  Standby compass will not work w/Windshield antiice, wipers, AC, elec heat.

Windshield Wipers: Don't operate on dry glass.

Windshield anti-ice: Temp sensing unit maintains 95F at windshield surface.  Electric heater lockout.  Mag compass will be unreliable when windshield anti-ice in use.  May blur objects.

Powerplant Ice Protection: Mechanically operated ice vanes or electrical lip boot heat.  With ice vanes extended slight decr in engine torque (40-60 ft-lbs) (Range decreases 10-12%)

Fuel System Anti-icing: Oil-to-fuel heat exchanger; operates continuously; no controls; one external fuel vent in each wing for both nacelle and wing tanks (protected from icing by electric heat elements).

Fuel Control Heat: Each FCU pneumatic line protected from ice by electrically heated jackets.  To prevent overheat, FUEL CONTROL HEAT should not be turned on unless cooling air will soon pass over the jackets.

Propeller Electrothermal Deicer System: Deicing boots on each prop blade; outboard and inboard element that receive electrical power through the deicer timer, slip ring, and brush block.; current measured by the prop ammeter.  Sequence: RH OUT, RH IN, LH OUT, LH IN (RORI LOLI); each segment heats for 34+/-6 sec; req 2 minutes for entire cycle; old mechanical timers sequence to next segment (show amperage fluctuation at timer switches to next segment); electrical timers always reset to RH OUT when power applied (no flux in amperage); each blade draws 5 to 6 amps (3 blades working correctly read between 14 to 18 amps).

Surface Deicer System: Ice removed from outboard wing leading edge, horizontal stab, and vertical stab by flexing pneumatic deicer boots.  Engine bleed air from the engine compressor supplies press to inflate boots and vacuum to hold down boots; press regulator protects system from overinflation;  when not operating, a distributor valve applies vacuum to hold boots down; when the DEICE CYCLE switch is toggled to SINGLE or MANUAL, a servo valve changes the inlet boots from vacuum to pressure that allows the boots to inflate; when the solenoid is deenergized, airflow through the valve is cutoff and it discharges out the boots through the integral check valve until press reaches approx 1-inch Hg, at which time the boots port to vacuum and the remaining air is evacuated; the boots are again held down by vacuum; either engine can provide the bleed air for the system; bleed air passes through an 18-psi regulator and then enters the deicer and vacuum systems; vacuum press is created by the distribution valve and is proportional to pneumatic pressure by the deicer press regulator valve.  Stall speeds significantly increase with the wing deice boots inflated; never cycle the system rapidly, may cause ice to accumulate outside the contour of the inflated boots and prevent ice removal.  In SINGLE, boot inflate for 7 to 8 seconds, then deflate.  In MANUAL, all boots inflate and stay inflated until switch is released.  The boots can't be overinflated.

Pitot & AOA Heat Systems: On the ground, AOA heating elements pass through a current limiter lowering voltage to 14 volts DC (low heat).  At lift off , a squat switch on the left main gear causes the current limiter to be bypassed and heating elements receive 28 Volts DC (high heat); except for takeoff roll, PITOT heat should not be used on the ground (might overheat because of lack of cooling airflow over heating elements).

* Electrothermal Prop De-ice

Abnormal readings (amps)

0:  1. Check Switch

2. If off, turn on.

3. If on, inop.  Turn off.

0-14:	1.  Continue operation.

2.  If imbalance occurs, increase prop rpm briefly to aid in ice removal.

18-23: See above (0-14)

>23: Do not operate system.

Surface Deice Emergency Procedure: If boots fail to inflate, operate in MANUAL.  If boots fail to deflate, pull the SUR DE-ICE CB and reset as necessary to operate boots.

* Stall Speed significantly increases w/ boots inflated.

Flight Controls

* Go-around button in CP stick+left power lever.  10 NH, disengage AP.

* Trim: Cable-Drum System, Elevator anti-servo, Ail: Left side, 15+-1.5 deg travel up/down.

* Map lights in yoke.  ENG INST light.

* Elevator electric trim takes precedence over manual trim.

* DON’T TOW with rudder lock in (NWS problems)

* Flap Motor: Dynamic Braking (2 sets of motor windings), limit switches (Right flap dominates.)  APPR:35%/15', DOWN:100%/43'

1)  Primary flight controls: self explanatory.  Trim through manually actuated cable-drum system.  On control wheel:  Mic switch, AP/YD/ Trim Disconnect Switch.  Elevator trim Split-Thumb Switch, map ligh switch, pitch sync & CWS Switch, Go-around switch, 8 day clock.  Rudder pedals adjusted by switches on the bottom of them.

2)  Secondary flight controls: trim, flaps.  Trim tabs, jackscrew actuator and cable drum system.  Elevator also uses electric.  Anti-servo action: makes control surface force required.  Co's switch takes priority.   15 deg aileron trim capable.    Wing flaps:  each has separate jacktooth actuator actuators driven through flexible shafts by single reversible electric motor on forward side of rear spar, dynamic braking system through the used of 2 sets of motor windings.  Approach: 15 deg, 35%, Down: 100%, 43%.  CBs:  Flap and Stab: indicator, control circuits.  Morot:  Wing Flap Motor)

3)  Flight controls lock: 2 pins and U-strap.  Ensures positive locking of rudder, aileron, engine controls.  DON’T Tow with Rudder lock on.

Primary flight control system: rudder, elevator, and aileron.  Manually operated by mechanical linkage using control wheel for ailerons and elevators and adjustable rudder/brake pedals for rudder.  Trim control through manually actuated cable-drum system.

Control Wheels: Pitch sync on pilot control wheel only.  Go-around switch on CP control wheel only.  Control wheel map lights power through ENG INST CB.

Rudder Pedals: Two positions - fore and aft.

Trim Tabs: Manually actuated and mechanically controlled by cable-drum and jackscrew actuator system.  Elevator trim tab is anti-servo (as elevators displace, trim tab moves in same direction, increases effective control surface area and increases force required to apply it).

Manual Elevator Trim Tab Control: Elevator tab deflection indicated by position arrow.

Electric Elevator Trim: Controlled by dual element thumb switches and trim disconnect switch on control wheel.  CP takes priority.  ELEC TRIM CB.  Operates only when both elements of thumb switch moved.  Electric trim can't be overridden by manual trim wheel.  Disconnect is a bi-level, momentary push-type switch button.  First depress disconnects autopilot and yaw damp.  Second level disconnects electric trim.

Aileron Trim Control: Adjusts left aileron trim tab.  Relative only, not in degrees.  Full travel 15 +/-2 degrees up or down.

Flight Controls Lock: Don't tow with rudder lock installed; could result in serious damage to steering linkage.

Wing Flaps: All metal, slot-type, electrically operated, two sections for each wing.  Both sections operate as a single unit, actuated by a separate jackscrew actuator.  Driven through flexible shafts by a single, reversible electric motor mounted on fwd side of the rear spar.  Motor powered by right main bus.  WING FLAP MOTOR CB.  Motor uses dynamic braking, prevents over travel.  Flaps indicated in percent of travel.

Wing Flap Selector Handle: 0% = UP.  35% = APPR.  100% = DOWN.  Limit switches, actuated by right inboard flap control flap travel.  FLAP & STAB CB.  Intermediate flap positions can't be selected.

Wing Flap Position Indicator: Show percent of travel from 0 to 100.  APPR = 15 degrees.  FULL DOWN = 43 degrees.  FLAP & STAB CB.

MALFUNCTION

On Ground Flight Control Failure: Check yaw damp & autopilot off; leave controls in jammed position; call for immediate inspection from maintenance.

Flap System Failure: If flaps inop, land in existing flap configuration.  If split flaps, return flaps to previously selected position.  Pull the wing flap CB to prevent inadvertent flap movement.

In pattern: Select Flaps - APPR.  [2] During "Flaps" call in Gear Down Landing Checklist, check position of flaps on flap indicator.  If indicator doesn't correspond to position of flap lever: [1] Visually check position of flaps on right and left.  [2a] If flaps selected APPR, indicator shows UP, but visually checked APPR - leave flap lever at APPR and pull WING FLAP CB.  [2b] If flaps selected APPR, indicator shows UP, and flaps visually checked UP - position flap lever UP, pull WING FLAP CB, and brief new speeds (110 on final, 105 over threshold).

- Forced Landing [Check NATOPS]

* ELP:  Can slip into High Key, flaps may not be available.  High Key: 130 KIAS, feathered, 3000-2500 AGL, approach checklist complete.  Low Key: 130 KIAS 1500-1800 AGL, Gear Down, Base Key (Field Made): 110 KIAS, 800-900 AGL, Flaps Approach, Landing Checklist, Final (Landing Assured): 110 KIAS, Flaps Full, Prior to landing: Battery Off.  Aimpoint: 1/3 Down Runway.

If suitable altitude, perform a circular pattern to observe surface terrain and wind conditions.  After terrain and wind noted, set up normal pattern and extend gear when landing assured.  Plan to overshoot (less drag).  Use flaps as needed.  If positive gear down and locked indication can't be determined, do a gear up landing.  Night egress: preposition threshold, spar and aft compartment lighting ON.

1. Configuration:  [A] Don't extend full flaps until landing assured [B] Gear up landing based on typ of touchdown surface

2. Landing Area: Adequate size to land, free of obstacles, smooth.  Cultivated fields are best.  Swamps, bogs, shallow lakes, and forests should be avoided. Land into wind.

3. Landing: Make at slowest speed that retains complete control.  If landing into trees, fly into them; don't attempt to land on the tops.  (Refer to NATOPS Figure 16-5 on page 16-12 for Emergency Landing Pattern illustration)