FAM-3: 5/21/05

?? Bromotrifluromethane: Halon 1301

- Environmental/Pressurization  Systems + Malfunctions + Oxygen System

* W/O Power, no pressurization: Bleed air valves electrically opened, springloaded closed.

* 1 minute to go from max manual temp incr to max manual decr.

* Receiver/Dryer in wheel well.

* Firewall Shutoff Valve for environmental system held open by bleed air.

* Sidewalls: heating...  Cooling system=shunt after A/C Evaporator.  Rest of air thru heater, mixed w/bleed air.

* 16,000 BTUs for A/C

* Pneumatic: Boots, Door seal, Flight Hour Meter

* Unpressurized fresh air: from flapper ram air in A/C, or from bleed air heating system.

* NATOPS position 500-700 fpm cabin climb/descent.

* 2 o2 pressure gages, one in cockpit, other rt. Side of fuselage.

* 10k ft press starts climbing.  25 k ft, cabin alt > 10k ft.

* Below 20k ft on mask, can use normal.  3.7 L/min for passengers.

* 45 - 1 hour O2 for 3 man crew.

1)  Pressurization - Bleed air tapped after 4th stage compressor, delivered to cockpit at constant rate, controls pressurization through 2 flow control valves (outflow valve and safety valve.)  Components:  Cabin Pressure controller, Dual Cabin Altimeter, Cabin Rate of Climb Indicator, Outflow Valve, Safety Valve, Flow control units (bleed air valves).  Cabin pressure controller controls rate of outflow from cabin.   DUMP:  opens safety valve, relieves pressure differential.  TEST: Safety valve closes, PRESS:  Normally Open Preset Solenoid, Normally closed safety solenoid power removed with squat switch up.  Pressurizes via vacuum forces through preset solenoid.  Rate knob:  50 fpm-2000 fpm, Altitude knob - 1,000 ft to +10,000 ft.  Dual cabin altimeter, long needle = actual cabin altitude, short needle = pressure differential.  Cabin rate of climb = 0-6000 fpm.  Altitude warning annunciator: 9500-10000 ft.  Pressure switch on forward bulkhead.  Outflow valve:  Meters the outflow of cabin air in response to vacuum control forces from controller, Contains a preadjusted relief valve so that cabin diff does not exceed 4.7 psi.  Incorporates negative pressure relief diaphragm which keeps cabin altitude from becoming higher than ac altitude.  Safety valve: 4.9 psi.  Closes when:  landing gear strut extends, cabin pressure is TEST, vacuum source is lost, electrical power lost.  Flow control unit:  controls bleed air, Draws ambient air in with venturi to mix with bleed air.  Before start w/o eng: safety and outflow valves closed.  After start, vacuum opens safety valve.  On T/O, dump solenoid closes, releases vacuum closes safety valve.  Preset solenoid opens and time delay circuit lets left engine begin pressurization 6 sec before righ engine (no excessive pressure bump.)  Flow control unit mixes bleed and ambient air.  Goes through heat exchanger in wing center section  (or bypasses it).  Outflow valve keeps 4.7 psid or less (selected rate and altitude.)

2)  Heating and cooling:  Compressor bleed air, Ambient air, flow control, Heat exchanger, Electric heater, ducts, outlets  Cooling: same minus heater plus condenser, receiver dryer, and evaporator, and vent blower, and cabin ceiling outlets.    Electric heater:  8 elements in GND MAX, 4 in NORM.    Best heating:   Heat GND MAX, Vent blower high, open bleed air valves, close overhead outlets, Cabin Temp to Incr Heat, Mode Auto, Cooling: Close one or both bleed air valves, set vent blower high, open all overhead outlets.  A/C evap has 16,000 BTU freon.   Heater locked out with windshield heat, prop de-ice, engine lipboot heat.

3)  Oxygen system:  emergency, + 10,000 ft cabin alt.  Oxygen cylinder, Pressure reg control valve, Pressure gagues, Outlets, masks.  Cylinder: 49 cu ft, 1850 psi, 70 degF.  Reg valve controlled by push-pull knob up front.  Diluter demand masks: proper dilution of 02 with cabin air to conserve 02 at lower alts.  Green detector = flow, red = no flow, control knob on front lets you control.  100 percent vs normal on front, EMER = 100% at positive pressure.  Mic Normal/Oxygen to switch.

1. Primary purpose of the environment system is to provide for heating, cooling, and pressurization of the aircraft.

2. Safety valve: completely relieves all pressure differential when pressure control switch in DUMP or switch is in PRESS and aircraft is on the ground.  The LH landing gear safety squat switch opens the valve on the ground.

3. Outflow valve: closed on ground.  Moving the pressure control switch to TEST opens the solenoids, closing the safety valve and giving control of the outflow valve to the controller.  The LH landing gear safety squat switch opens the switch in the air.

4. Excessive pressure bump is controlled by the LH landing gear safety squat switch which: [1] removes power from the preset and safety valve solenoids [2] actuates time delay relay that allows left engine to commence pressurization 6 seconds prior to right engine.

5. Max psid for outflow valve is 4.7psid.

6. Max psid for safety valve is 4.9psid.

7. Actual control of psid in safety valve operation is by the pressure control switch when in DUMP or the LH landing gear safety squat switch when on the ground (the safety valve solenoid opens to supply vacuum to the safety valve holding it in OPEN or DUMP position).

8. Electric heater lockout system: prevents use of electric heater if windshield anti-ice, prop deice, and/or lip boot heaters are operating.

9. Max cooling: [1] Bleed Air Valves - CLOSE one or both [2] Vent Blower - HIGH [3] All overhead outlets - OPEN

Max heat: [1] Electric heat - GND MAX [2] Vent Blower - HIGH [3] All overhead outlets - CLOSE  [4] Bleed Air Valves - OPEN (after engine start)

10. The electric heater will work only when CABIN TEMP MODE is in AUTO.

11. With the vent blower in auto, the fan is operating in low speed.

12. With a total loss of electrical power, the T-44 will depressurize!

13.  None of the pressurization systems work in conjunction with the RIGHT squat switch.

14. Procedures for an altitude warning light:[1] Descend [2] Restore cabin pressurization [3] Check pressurization and/or don O2 mask.

15. Loss of cabin pressurization (NATOPS 15-11):

If gradual pressurization loss:

[1] Cabin altitude - Checked (P)

[2] Pressurization controller - Checked (CP)

[3] Bleed Air - Checked (CP)

[4] Press dump test switch - TEST (CP)  (hold for 15 sec)

If test switch restores press, may be necessary to hold switch in TEST until cabin profile at 10,000' or less.  If press

is regained through test switch, pull the PRESS CONTROL CB to maintain pressurization.

If unable to restore pressurization:

[5] Oxygen mask/microphone switch (100%) - AS REQ (P,CP,OBS)

[6] Descend - AS REQ (P)

On descent when cabin alt matches press alt, ensure PRESS CONTROL CB is reset to preclude landing

pressurized.

16. Explosive decompression: occurs in less than 1 second, causes fog, if high alt - rush of air from lungs, dazed sensation, gas pains, hypoxia if oxygen not used immediately.

[1] Oxygen mask/ MIC switches (100%) - AS REQ

[2] Descend - AS REQ

17. Supplemental oxygen is required when at cabin altitudes above 10,000'.

18. The oxygen system capacity is 1850 (+/-50) psig for one 49 cubic foot cylinder.

19. Between 17,000' to 20,000' the diluter demand regulator should supply 100% oxygen while in the normal position.

20.  The oxygen mask should be stored in the 100% position.

21. The min O2 requirement for local flights is 1000psig.  For cross-country flights: 1500psig.

Cracked windshield

External Panel: Heating elements inop, don’t use wipers.

Internal panel: descend/depressurize to 2.5 psi in 10 mins.  Lose some vis.

Cracked cabin window

Depressurize/Descend

- Lost Comm VFR [RICES?]

(Course Rules III-10, FIH Section A):

[1] Squawk lost comm (7600)

[2] Obtain current ATIS (CH 1 or 114.0 Truax VORTAC)  ATC may attempt to contact you on ATIS.

If unable to obtain ATIS and wx permits, overfly Navy Corpus > 3500'MSL to determine duty rwy.

[3] Intercept course rules via appropriate VFR arrival route.  Make all radio calls "in the blind"  Rock wings inbound for

break and check for interval prior to breaking.

[4] On short final, if no ALDIS lamp signal observed, perform a waveoff.  On 2nd pass, if no lamp signal observed and

rwy is clear, land.

- Smoke + Fire of Unknown Origin

The last five items of the checklist: (1) Isolate problem's source (2) restore power for safety of flight.  Pausing between the last five steps may help isolate the fire if its electrical.

1. Alert Crew

2. Cabin Temp Mode - OFF

3. Vent Blower - AUTO

WARNING: Bromotrifluoromethane is bad.  Don’t breathe.  May cause frostbite if contacts skin.

Note: If fire source is known immediately turn off all affected electrical circuits and fight fire with handheld fire extinguishers.  If fire source cannot be isolated, continue checklist.

4. Oxygen masks/microphone switch (100%) - AS REQ

Note: While using oxygen, if interphones selected, the speaker function will deactivate interphones.  A headset is required to monitor external comm while simultaneously monitoring interphones.

5. Bleed Air - CLOSED

6. Emergency descent - AS REQ (when oxygen provided for entire crew, staying at high altitude and depressurizing may help control fuselage fires.)

7. Pressurization - DUMP

8. Emergency transmission - AS REQ

9. Gang Bar - OFF (after securing electric power, the following instruments still available: copilot turn and slip, pitot-static, N1, ITT, prop RPM, clocks, and standby compass)

10. All electrical switches - OFF (If fire can't be controlled, land or ditch immediately)

11. Battery - ON

12. Generators (one at a time) - ON

13. Inverters (one at a time) - ON

14. Essential equipment - ON (individually until fire source located)

- Emergency Descent

FTI: Recover NLT 2000', complete recovery by 1000'.  2000' or less alt to lose, or battery only, stay clean.  Full flaps: 140 kts.  Clean: 227 kts.  Windshield heat from high alt to warm/moist low alt.  N/R if descending from low alt.

[1] Power levers - IDLE

[2] Props - FULL FWD

[3] Flaps - AS REQ

[4] Landing Gear - AS REQ

[5] Airspeed - AS REQ (Don't exceed VNE (227K))

[6] Windshield heat - AS REQ

Consider meteorological + damage conditions.

Recover: 500 feet above tgt, add pwr, raise nose.  Flaps,gear,flaps-A/R.  Props-1900, recover.

- Smoke+Fume Elimination

(NATOPS 15-5): Locate, isolate, and extinguish fire or source of smoke and fumes prior to initiating smoke removal procedures.  Isolate the engine as a possible source by closing the bleed air valves.  Prior to depressurizing, consider MEA and crew O2 requirements.

If immediate smoke removal necessary:

[1] Oxygen masks/MIC switches (100%) - AS REQ

[2] Pressurization - DUMP

After depressurizing, storm windows may be opened (may draw smoke into flight station).  Consider emergency descent and immediate landing.

WARNING: Mag compass fluid very flammable and toxic.  If it leaks, do smoke + fume elimination and consider securing electrical equipment.

NOTE: Nauseating fumes from desalination solution can enter cockpit via bleed air for air-conditioning/pressurization and is often detected immediately after takeoff.  Providing no secondary indications exist, closing the bleed air valves will significantly reduce the fumes.

- No-Flap Landings

FTI: Take into account higher speeds for turn/groundtrack.  Reduce power because of less drag.  Close power levers while flaring.  No abrupt movement.  A/C will balloon, sink rapidly.

Same airspeeds and altitudes through the 90, then roll onto final at 110K.  Cross threshold at 105K.

- Dynamic Engine Cut

'Numbered hdg, 5000', prop sync off.'

'300 ft, lbs, 2 deg NH (trim), full forward'

'Flaps up, Gear down, Landing Checklist'

'Approaching 95 kts, T/O power, 7-10 deg NH'

Above 91 kts, IP pulls one power lever to idle.

'Power up, rudder up (5' into failed engine), clean up, speed up (102-110 kts)'

Emergency Shutdown- Exec (failed engine)

After first 3: '1900 RPM, max power, fire or fuel?'

YES: Rest of checklist

NO: 'You have the comms, declare an emergency, dead engine checklist.'

- Ditching

DITCHING (POWER ON)

Complete 'DITCHING' checklist on back of cockpit card.  Note: Don't unstrap until all motion stops.  When evacuating through emergency exit or cabin door, take life raft & first-aid kit.  Don't remove raft from carrying case inside aircraft.  Don't inflate raft before launching.  Tie first-aid kit to raft.  Remain in vicinity of aircraft at long as it remains afloat.  Best way to hit the water is: [1] if parallel to major swells - on crest [2] if perpendicular to swells - on back side.

If power available:

[1] Gear - UP

[2] Flaps - APPR (at last 300' AGL)

[3] Rate of descent, 100fpm during final stages of approach (last 300' using RADALT); (Dual Eng) approx 21 units

AOA.

[4] Dual Engine: 90K (No flap = 100K)

DITCHING (SSE)

Complete 'DITCHING' checklist on back of cockpit card.  Note: Don't unstrap until all motion stops.  When evacuating through emergency exit or cabin door, take life raft & first-aid kit.  Don't remove raft from carrying case inside aircraft.  Don't inflate raft before launching.  Tie first-aid kit to raft.  Remain in vicinity of aircraft at long as it remains afloat.  Best way to hit the water is: [1] if parallel to major swells - on crest [2] if perpendicular to swells - on back side.

If power available:

[1] Gear - UP

[2] Flaps - APPR (at last 300'AGL)

[3] Rate of descent, 100fpm during final stages of approach (last 300' using RADALT) approx 19 - 21 Units AOA.

[4] 91K (No flap = 100K)

DITCHING (POWER OFF)

1. Gear - UP

2. Flaps - UP

3. Rate of descent - 130K (max glide)

4. At 200'AGL, Flaps - APPR

5. Bleed off airspeed with slight nose up.

6. Water entry at 90K, 21units AOA, max 500' rate of descent on VSI.

Notes: Flaps and/or RADALT may be inop.  If no flap, enter water at 100K with max 500' rate of descent on VSI.   Most essential to be wings level at time of water impact.