FAM-5: 5/25/05

- Engine Failure After Takeoff

* Execute the checklist.  Right at rotate, and outside accel-stop distance, take into air unless greater emergency.  Dynamic engine cut simulates this.

* If autofeather is working, allow it to work before IDLE: IDLE will disengage it.

* Positive ROC impossible w/ windmilling inop engine's prop.

If sufficient runway to stop on ground, ABORT!

If insufficient runway to abort takeoff:

(POWER UP, RUDDER UP, CLEAN UP, SPEED UP, CHECKLIST)

1. Power - AS REQUIRED

2. Gear - UP

3. Airspeed - (102 (obstacles) or 110)

4. EMERGENCY SHUTDOWN CHECKLIST

[1] Power Lever - IDLE

[2] Propeller - FEATHER

[3] Condition Lever - FUEL CUTOFF

If fire or fuel leak, continue checklist.  If not, proceed to dead engine checklist.

[4] Firewall valve - CLOSED

[5] Fire Extinguisher - AS REQUIRED

[6] Bleed Air - CLOSED

[7] Dead Engine Checklist - AS REQUIRED

Note: Before addressing the dead engine checklist, transfer comm to CP and have him declare an emergency.

- Engine Secure + Restart

* FTI: Actual in flight engine shutdown only by cond lever: fuel c/o.  Only daylight VMC, VMC descent possible to 4900x100 field w/firefighting equip.  Not lower than 4k.

* Feather: Complete emer eng shutdown, then preload starter assisted a/s to Gen-off.  Actual: Preload A/R.

* Windmilling: do not do above.

* Do not touch FSV.  See NATOPS note on cavitation of fuel pump and +10 sec restart after FSV use.

AIRSTARTS

Accomplished with battery or generator.  Normally starter-assisted.  Determine cause for engine failure before airstart.  Airstarts above 20,000 ft are hotter - periodically cycle condition lever to FUEL CUTOFF to avoid overtemping.  Reduce electric loads.

STARTER-ASSISTED AIRSTARTS

Don't airstart if engine died from MOVEOFF - Mechanical malfunction, Overheat condition, Vibration, Explosion, 0 N1, Fire, or Fuel fumes.

1. Power lever (failed engine) - IDLE

2. Prop lever (failed engine) - FEATHER

3. Condition lever (failed engine) - FUEL CUTOFF

4. Cabin Temp Mode - OFF

5. Vent Blower - AUTO

6. Radar - OFF/STANDBY

7. Anti-ice/deice - AS REQUIRED

8. Firewall valve - OPEN

9. Transfer Pump - AUTO

10. Boost Pump - ON

11. Crossfeed - AUTO

12. Generator (failed engine) - OFF

13. Starter (failed engine) - IGN & ENG START (check annunc light illums)

Note: If engine shutdown by closing firewall valve, time to light-off may exceed 10 sec.  Op engine ITT may increase approx 50C because of gen loading.  Retard if possible.

14. Condition lever - LOW IDLE

15. Starter (N1 > 50%) - OFF

16. Prop - UNFEATHER

17. Power - AS REQUIRED

18. Generator - RESET/ON

19. Fuel Control Heat - ON

20. Bleed Air - OPEN

21. Electrical Equipment - AS REQUIRED

22. Condition lever - HIGH IDLE

23. Instruments/Nacelles - CHECKED

WINDMILLING AIRSTART

Use for emergency relight if engine inadvertently shutdown.  Will cost alt+as therefore time crit.

1. Power Lever (failed engine) - IDLE

2. Prop lever (failed engine) - FULL FORWARD

3. Condition lever (failed engine) - FUEL CUTOFF

4. Firewall Valve - OPEN

5. Autoignition - ARM

Note: Windmilling airstarts above 20,000 ft or below 2,200 prop RPM may exceed ITT limit.

6. Condition Lever - LOW IDLE

Note: If engine shutdown by closing firewall valve, time to light-off may exceed 10 seconds

7. Power - AS REQUIRED

8. Generator - RESET/ON

9. Condition lever - HIGH IDLE

10.  Prop levers - Match A/R

- Electrical System/Malfunctions

[2-18 - 2-22 NATOPS]

* 5 Amp fuses/CB for everything on fuel bus but fuel bus boost pump (10)

* AC ground: 1000A for 0.1 sec, 300A continuous

* Inverter circuits: each 26V:5A fuse, each 115V:6A fuse, 1A fuse for everything on bus.

* Lose inverter select realy fuse, lose that side's 115VAC bus.

Malfunction: Gen Fail...

Evaluate Current Limiters, 3 possibility:

1) Good Current Limiters: no drop in batt, everything works.  Keep load under 1.0.

2) Bad CL (same side): lose avionics bus, fuel bus, inverter for that side.

3) Bad CL (opp side): batter discharging, everything works.  Conserve batt.

1) Gen - OFF, Reset, back on

2) Gen - OfF (if still broke)

If works in reset, Gen Control Box broke.

3) CLs check

4) Op gen - don’t exceed 1.0

5) Land as soon as practibile

NOTE: Fail w/ smoke+fumes=internal failure.  Check by closing bleed air.  Consider engine shutdown.

Dual-gen Fail:

Ensure maximum battery duration

1) Cabin Temp, heater, antiice/deice, autoignition, lights, radar - OFF

2) Emergency voice report - complete

3) VMC - Gang Bar - OFF, IMC - Gens -OFF

4) Boost pumps - OFF

5) VMC - Inverters - OFF, IMC  - #2 Inv:OFF

6) Pull a lot of CBs.

7) VMC: Avionics Master - OFF, Batt - ON

8) IMC w/ GCA: Pull a lof of Nav CBs

9) IMC w/VORTAC: Pull more CBs for other stuff.

Note: No-flap, pump down the gear.

Excessive Load:

Due to ground fault or excessive batt charge.

1) Battery/Ammeter - Check

>30 A: 2) Battery - OFF

3) Battery/ammeter - Check

>30 still: battery relay failed.  Land as soon as possible.  If it drops:

4) Recheck loadmeters

If normal, excessive battery charge.  Land as soon as practibile.  If excessive, ground fault land as soon as possible.  Watch for fire.  Note: load splits >0.1=bad paralleling.  W/ AC on, CL broke.

Inverter Fail

1) Failed Inv - OFF

2) Check for AC Switchover.

3) Land as soon as practibile.

Inst Inv Fail

Torquemeters Lie.  Can’t fix.

Tripped CB:

1. Nonessential - Don’t reset.

2. Essential- Push to reset.  If pops again, do not reset.

DON’T Reset Subpanel Feeder CBs.

Avionics Master

Lose all avionics, power is good.  Pull MASTER POWER CB.

1)  Battery:  24v 42a  lead acid, in right wing root.   Emergency power, starts engines, acts as damper.

2)  Starter/Generator:  250 amps (Accessory gearbox) Generator Control Box located under main wing spar in center aisle.  (Regulates voltage, automatic paralleling, overvoltage protection 31VDC, undervoltage protection 18VDC, reverse current protection)  Load is measured in percent of 250 amps.  (.3-.6)

3) APU in rt engine nacelle...  No more than 28VDC, capable of 300A with 0.1 sec bursts of 1000A.

4) DC Busses:  10:  Hot Batt Bus, Batt Bus, #1/2 Subpanel Bus, #1/2 Avionics Bus, L/R Main Bus, #1/2 fuel bus.  Hot batt bus:  Singularly powered:  L/R Fire Extinguishers, Threshold lights (wing spar, baggage door, cabin door observer lt, aft compartment lt, xfeed lt) Dual powered (w/fuel bus): L/R Boost Pumps, L/R Firewall Valves, Xfeed valve.

5) Current limiters: Bus ties.  Allow either bus to get power from other generator..   Also isolates shorts from rest of system.  325A Slow fuse.

6)  Inverters:  750 Volt-Ampere single phase inverters.  Supplies 26Volt AC,for Torquemeters, 115 vac for the avionics.  Inverter relays driven off inverters, fail-safe?  System adjusts for failed inverters.  If you lose an inverter relay select fuls, you lose the AC avionics on that side.

7)  Lights:  Master Cockpit is everything except for Threshold lights, Door locking light, Seatbelt light, reading light, utility light, indirect instrument lights, aft compartment ligght.  Pull GYRO INST CB if Master Cockpit lights fails, for all lights other than gyro lights.  Exterior lights (Landing/Taxi) shutoff with gear retraction.

8) Emergencies:  Generator fail boldface:  good voltage in RESET indicates failed gen control, not bad generator.  Lose both gens, on battery power (42Ah remains.) Run off math from batt amp. Current limiter fail inflight = 0.2 difference btw busses.  Ususally L. Main higher.  Both curr limiter fail, same thing as one failing.  Variable split in loadmeters.  Gen fail w/opp Curr Lim fail:  everything works, but batt being discharged.   Same side gen fail curr lim fail, lose avionics bus, lose fuel bus on that side.  Inverter fails, turn it off and land.  Total AC failure: can only uses fuel flow, oil px, itt, n1, pitot static, prop rpm, clocks, ncs31, aoa, radalt, compass, turn and bank.  (Heading, attitude, nav all fail, and torque.)  Only have DME, ADF audio, XPDR, marker beacons, UHF VHF audio.

9) CBs:  Can reset once, but NOT Subpanel feeder, non-essential.  If Avionics Master switch fails, pull Master Power CB to bypass.

1. Sources of DC power: 1 24V/42A battery, 2 28V/250A starter-generators

2. Sources of AC power: DC power routed through 2 750V-A single-phase inverters.

3. The generator control box: overvoltage protection (31V), undervoltage protection (18V), reverse current protection, & automatic paralleling.

4. The generator control boxes are under the cabin center aisle aft of the main spar.

5. The inverters are located outboard of the engines on the wings.

6. The current limiters: tie the main buses & provide fault protection.

7. To get around a faulty master avionics switch, pull the master power CB.

8. Limitations of the generators: 28.25 (+/- 0.8)V.

9. Limitations of the inverters: 114 (+/-7)V, 400 (+/-6)Hz.

10. For APU start, the unit must be able to provide 1000 Amps for 0.1sec and 300Amp continuous load thereafter.

11. A failed Inverter Select Relay Fuse will cause failure of respective AC bus without illum of inverter light (switch over impossible).

12. Illum of INST INV OUT light indicates loss of 26VAC inverter select relay & torquemeters.

13. Procedures for INVERTER failure: [1] Fail Inverter - OFF [2] AC Bus switchover [3] Land ASA practicable

Total ac failure, still have: fuel flow, oil press, oil temp, turn & bank, pitot static, N1, ITT, prop RPM, clocks, NCS-31A, AOA indexers, RADALT, wet compass14. Procedures for GENERATOR failure: OFF, reset momentarily, ON.

15. If generator fails to reset, yet registers normal volts in reset position, indicates failure of generator control.

16. Total loss of power, still have: copilot turn and slip, VSI, altimeters, airspeed indicators, wet compass, N1 and N2, ITT, & clocks.

17.  Dual inverter failure, still have DME, radar, VOR (audio), ADF-LF (audio), xponder, MKR BCNs.  For comm still have, UHF & VHF.

18. Excessive load indications (>1.0): [1] Battery/ammeter - Note.  If charge rate in excess of 30A: [2] Battery - OFF. If loadmeter still over 1.0: [3] Non-essential loads - OFF.  Splits of >0.1 indicate abnormal generator paralleling.

19. If get a GEN OUT light, check Current Limiters: [1] Battery- no discharge, no other equip failures = CL intact. [2] Battery - discharge, no other equip failures = opposite CL failed. [3] Battery - no discharge, other equip failures = same side CL failed.

- SSE Waveoff @ Altitude

*   Not a player, but simulates SSE waveoffs in the pattern.  Level, 800' + sim ground, sim downwind, Case 3, IP calls the 180, come off the 180, SSE final, waveoff procedures, give CP the comms.

SSE WAVEOFF AT ALTITUDE: allows for safe transition from SSE descending flight to max power, SSE climbing flight; prepares for SSE waveoffs in traffic pattern.

[1] Level off at 1000' altitude + 800' (i/e 4800, 5800, 6800), 120K, on a numbered hdg.

[2] Align CDI with hdg (tail at top)

[3] IP will reduce one power lever to IDLE

[4] Power - UP, Rudder - UP, Clean - UP

[5] Complete Emergency Shutdown Checklist

[6] When IP calls 'Approaching the 180,' complete the Gear Down Landing Checklist

[7] When checklist complete, immediately start a descending left turn to arrive at the 90 position at 500'& 120K (min 110K).

[8] Continue turn to roll out on final at the head of the CDI, 250', min 110K (max 120K)

[9] Smoothly place Props - FULL FWD.

[10] When IP calls waveoff:

[a] Power - MAX ALLOW.  Transition to climb, add right rudder, keep ball nearly centered (1/4  to ½ out twds op

eng) while banking 5 deg twds op engine.  Min 102K.  Max 110K (preferred).

[b] Flaps - APPR

[c] Gear - UP (When descent stopped or no possibility of touchdown)

[d] Flaps - UP

[e] Props - 1900RPM (after CP calls 'Gear Up.')

- Ditching- SSE

*  FTI discusses likely scenarios for SSE would include wing fire, and other time critical ditches.  Need to get down faster.  With wing fire: 90 seconds, and slip away from it.  Emergency descent A/R.

* Keep in mind heading will want to slip SSE, and level will be more difficult. Level/Heading priority one.

* NATOPS: Avoid full flaps due to high power required and hence rudder controllability issues.  Approach strongly recommended.

DITCHING (SSE)

Complete 'DITCHING' checklist on back of cockpit card.  Note: Don't unstrap until all motion stops.  When evacuating through emergency exit or cabin door, take life raft & first-aid kit.  Don't remove raft from carrying case inside aircraft.  Don't inflate raft before launching.  Tie first-aid kit to raft.  Remain in vicinity of aircraft at long as it remains afloat.  Best way to hit the water is: [1] if parallel to major swells - on crest [2] if perpendicular to swells - on back side.

If power available:

[1] Gear - UP

[2] Flaps - APPR (at 300' AGL)

[3] Rate of descent, 100fpm during final stages of approach (last 300' using RADALT) approx 19 - 21 Units AOA.

[4] 91K (No flap = 100K)

- CRM during SSE

[Check with someone for more details]

*  Use CRM for comms, concurrence, preloading on chip lights, dead engine checklist, and right hand patterns, as well as normal duties.

- SSE Pattern Work

[Check FTI 1-22 for SSE Pattern Cases]

*  Emergencies: Flameout

Check 0% N1, Not fire or fuel related, no restart.

* E: Fire

Check for secondaries, Fire or fuel (FSV,FE,BA), no restart.  Might delay shutdown in Case 2 until downwind.

* E: Chiplight

Check for secondaries.  No secondaries, preload starter assisted airstart.  Not fire or fuel related.

* E: Oil Pressure

<40 psi, unsafe, secure eng.  Not fire or fuel.  Might consider securing once established downwind.

* Cases: 1 (Upwind)

Execute Eng fail T/O Boldface (Max pwr, gear up, (flaps up), speed up (102-110), Props 1900), Execute Emergency Eng Sht BF (Power idle, prop feather, cond f c/o, fire/fuel?, FSV, FE, BA, comms, emer, dead eng checklist)

Don’t interrupt autofeather, turn CW.

* C:2 (Crosswind)

Roll wings level, Maintain 110 kts (102 min), roll out in power up rudder up clean up speed up, resume CW turn.  Standard checklist, accel to 120 on downwind, 800 ft.  At cabiness, turn back in quick to avoid extending pattern.

* C:3 (Downwind)

Standard, PU, RU, CU, SU, checklist.  Configure just prior to 180, NLT 90.  (App, gear)  120 kts, 110 min.  Can not clean up if able to maintain level.  Maj Riddle technique: always clean up.  Can tell tower if can't configure on profile.  Can’t hold landing checklist, must restart.

* C:4 (Past 180 to 90)

Power up to maintain 110-120 kts (dont cleanup unless committed to waveoff), continue turn, checklist (only first 3 required past 180)  Once on final, no checklist.  Not required, or first two for dead engine checklist.

* C:5 (Past 90)

Slight additional power, to keep 110 min.  Maintain dir control, accomplish first three (recommended), props full forward, three d&l, review me complete, you have the comms declare an emergency once safely on deck.  Note that both props full forward, not feathered.  Waveoff will autofeather prop.  (90% switch.)  A/C will float.  SSE full stop.

* Special: Pattern after SSE T&G

Combine all the cases.  Lots of rudder.  Don’t do flaps until the 180.  Time for dead engine checklist.

- SSE Landings, Waveoff, T&G

* SSE T&G exactly the same as any normal T&G.  Power levers are already at idle.  Actual single engine T&Gs would not happen.

SSE Waveoff

* 1900 RPM Enhances climb performance

* Full Flap Not Recommended

[1] Power - AS REQ (Max allowable on operating engine, pos ROC, Vxse min)

[2] Flaps - APPR (unless already up)

[3] Gear - UP (+ rate of climb & no possibility of touchdown)

[4] Flaps - UP

[5] Props - 1900RPM (when CP calls "Gear Up")

Min altitude for IP to intiate SSE waveoff is 200'AGL.

SSE Landing

* NATOPS: Full flaps only w/ gear down + locked, no possibility of waveoff, still probably not a good idea.

* Reduce power in the flare.  Reverse A/R.  Use extreme caution if surface not hard + dry.

* Execute SSE full stop brief.

* FTI: Use most favorable RWY.  Good engine into wind airborne, bad engine into wind on gnd.

Upon touchdown:

[1] Power - IDLE

[2] Reverse - AS REQ

[3] Rudder & Braking - AS REQ (counter act yaw)

[4] Scan to end of runway

[5] If yaw excessive, reduce reversing and stop with brakes (don't lock brakes)

[6] Complete when at slow taxi on rwy

[7] Clear rwy if practicable, then shutdown

[8] Don't taxi on one engine!

[9] On rollout, dead engine into wind.

[10] In air, good engine into wind.

[11] Must complete a SSE Full Stop brief to perform SSE Full Stop Landing.  Initiate from a Case One or Two scenario and only when required in syllabus.