FAM-6: 5/31/05

- Actual vs. Simulated Engine Shutdown

*  Actual only by condition lever -> Fuel cutoff.  4000 feet min.  VMC Daylight, can transition to a 4900x100 runway.  Shutdown w/ feather: preload S-A Airstart.  Actual malf: preload A/R.  Never touch FSV.

- PIC/CRM during SSE

* Already Briefed.

*  Use CRM for comms, concurrence, preloading on chip lights, dead engine checklist, and right hand patterns, as well as normal duties.

- P-Factor

*  Function of differential AoA on the blades of the prop.  Rt side has higher relative AoA hence more thrust than left side on a CW prop.  More pronounced on Rt engine where moment of P-factor farthest from centerline, hence left engine critical.  More pronounced at low A/S, high torque (high AoA conditions).

pertinent force after takeoff rotation; causes aircraft to yaw left; caused by imbalance of thrust in props plane of rotation; must add right rudder to compensate for.

- VMC Air/Ground

*Vsse above Vmca+Stall.

* 7 assumptions on Vmca: 1) Takeoff power, 2) Critical engine windmilling, 3) Takeoff flaps (up), 4) Gear up, 5) 5' AOB into operating engine, 6) Max allowable CG (min pitch performance), 7) Full rudder

* No brakes/NWS for VMCG.

VMCA: (86K) airspeed below which it is not possible to maintain hdg despite max rudder deflection and 5 degrees AOB into operating engine.  Set under following conditions: takeoff power on operating engine, critical engine (left) windmilling, flaps at takeoff setting, gear up, 5 AOB into operating engine, max allowable aft CG, full rudder deflection.  Demonstration only flown by instructor.  Speed decreases with altitude.

VSSE: (91K) Single engine safety speed.  Intentionally failing one engine below this speed is prohibited.

VMCG: (63K) The min control speed on the ground.  The min speed aircraft can maintain runway centerline (using max rudder) using rudder/ail inputs alone with critical engine windmilling, and opposite engine at full power.  Below 63K, insufficient airflow exists to directionally control the aircraft.

- Landing Gear Sys/Malf.

* Dynamic braking and limit switches limit gear extension and retraction.  Overload clutch prevents damage with a mechanical malf.

* Gear extension should not exceed 8 sec, retraction 10 sec.

* Only PIC can direct horn silencing.

* Cannot override horn during final approach, or in pattern.

* J-hook may not stop gear on ground if rt. Squat switch doesn’t have enough weight on it.

* Shock spring in NWS absorbs vibrations before they get to rudders.  Shimmy damper as well.

* On emergencies, get visual confirmation.  Is possible to get 3 green with unsafe indications.  Wheel falls off, strut still works, etc.

* Don’t taxi with unsafe gear indications.

Emergencies: All Generally Deferred, to your advantage to burn off fuel, decrease weight.

Unsafe Up:

1. Gear Handle - Down

2. Gear position - Check

3. If safe down, land as soon as practibile

4. If not, go to unsafe down procedures

Unsafe Down:

1-3. Check CBs

4. Check lights (press-to-test, check if wheels up warning system works under 79% N1, check AoA indexer lights.

No AoA: motor cutoff, manually extend.  Check prop sync lt (rt.), L/T lights (nose), GND MAX ht (hold in pos, won’t lock w/ squat switch) (left) (wont work w/ windshield heat, prop deice, lip boot heat.)

5. Visual gear pos check.

6. If safe, land ASA practibile.

7. If not, determine if electrical.  If electrical, emergency extend.  If not:

8. Gear handle - up

9. Gear pos visual check.

10.  Visually up - gear up landing.  If not, unsafe gear landing.

Airborne Gear Inspection

1. Join up with comms

Don’t erratically maneuver

2. Check tire inflation, J-hook, mechanical downlock, condition, gear doors and hydraulic leak.

One Main up Landing

Favor the good wheel, drag in the center, no cables, turn on lights for night emergency egress.  Execute checklist, brief, lock harnesses, landing check, kill engines, use rudder and ail to keep bad wing off as long as possible, brakes, emergency shutdown on deck.

Main Down, Nose Up.

Difficult to get out of AC because of nose low, DO NOT USE BRAKES!  Hold nose off as long as possible.  Same as one bad main, but no brakes, no rudder/ail.

Gear Up

Tends to overshoot, touchdown at 85 kts.  Brief, secure seatbelts, disengage clutch, stow manual extension handle, pull LDG GR CB, Flaps - APP, Gang bar off if possible, immediately prior to TD, Cond lev - Fuel C/O, After Touchdown - Power - Idle, Cond Lev - Fuel C/O, Brakes - A/R (can use brakes and rudders for dir control.  Finish emergency shutdown, shoot the fire extinguishers.

Main Flat

Put the drag in the center, use opposite brake for directional control.

Nose Flat

Less stable NWS, aerobrake, light wheelbraking only to maintain dir control.

Brake Failure

Dir control w/ rudder + asym power, stop from reversing to aid in decel, do not taxi.

1)  Gear:  operated by motor (single, split field, 28vdc, under co seat.)  Motor limit switches prevent overtravel (under pilot seat)  Torque shafts drive main actuators, duplex-chains drive nose gear.  Spring loaded locks fix gear in down position, jackscrew in nose fixes it in overcenter down, jackscrews holds all three up.  GEAR MOTOR CB on pedestal.  Gear retraction time, 5-7 sec., gear extension time 4-6 sec.  2 red bulbs in warning lights, means:  unlocked, in transit, HD LT TEST, below 79%+-2 power, below approach flaps, gear not down.

2)  Wheels up warning system :  warning horn, lights, when gear not down and locked + power below 79%+-2 N1, Flaps beyond approach, can silence horn.  DON’T override on traffic pattern or final approach.  Silence only works w/ flaps up.

3)  Squat switches:  Mechanically activated.  Rt.:  (act.) Gear inop, downlock hook,  (de.) Flight hour meter, (de) Indexer for AoA, Stall warning (de), Heat rt. Eng. Lip boot (de.).    Left: Heat AoA vane reduce to 14 from 28 (de), Heat electric heater GRD MAX (de from NORMAL), Heat lip boot left engine (de), Ambient air solenoid (de.), Pressurization: preset (de.), dump (act.), door (closes)

4)  Downlock and uplock switches:  each strut has one (3) of each downlock and uplock.  You can check which one is not fully retracted by PROP SYNC; if rt. Uplock closed Propsync won’t turn on.  Landing lighs... If nosewheel uplock closed, landing lights won’t come on.    Electric heater: checks left.   Motor limit switches: up limit deactivates motor, deactivates g-meter when gear is up.  Down limit switch deactivates motor, activates AOA indexer lts.

5)  Alt gear extension:  Can be manually extended, but not retracted through handcrank.  Do not do it if a malfunction is suspected.  Right side of pilot seat.  Must disengage landing gear clutch to pump handle, lift up clutch, turn clockwise, reengage by doing opposite.  Airspeed - 120 KIAS recommended (155 max), A/P-A/R, landing gear relay CB - pull, Gear handle - down, Clutch disengage - list, turn clockwise, Manual hnandle - pump until 3 green (50 ish pumps)

6)  Brakes:  Multidisc hydraulic brakes, Shuttle valve allows changeover from one set of brakes to the other.  Dual parking brake valves lock pressure.  Fluid from hyd brake system reservoir, NBO emergency brakes,  out setting maintains check valves, only Px in not out.  Only Pilot, not during flight.  With wheelbrake fail:  directional control w/ diff power, rudder, NWS.  Use reverse or beta to help.  Manuver into open area, NO taxi.  Shuttle valve sometimes fail.  Reset by pulling aft on top of brakes.

7)  NWS:   Direct:  12L/14R, w/ brakes: 48 deg.  Retraction autodisenages.

1. Nosewheel steering actuates nose gear 12 degrees left and 14 degrees right.  (48 degrees either side with braking)

2. The function that up and down limit switches share is deactivates landing gear motor and prevents overtravel of gear.

3. The up limit switch also deactivates the g-meter when the landing gear is extended.

4. The down limit switch also activates the AOA indexer lights.  On the ground, the right main squat switch deactivates the AOA indexer lights.

5. The three up locks all will turn on the red lights in the gear selector handle, illum the wheels up light, and activate the gear warning horn if the gear don't fully retract.

6. The right up lock also deactivates the prop sync light when the prop sync switch is ON.  If the right up lock has gear fully retracted, the prop sync light will not come on when the prop sync switch in ON.

7. The left up lock also keeps the loading for the electric heat in NORM & GND MAX within slight fluctuation or no significant generator load difference.   If the left up lock has gear fully retracted, switching the electric heat from NORM to GND MAX will yield only a slight or no significant change in generator loading.

8. The nose up lock also deactivates the landing lights.  If up lock has gear fully retracted, no illumination of landing lights and no increase in generator loading when landing lights are turned ON.

9. The right squat switch: [GFISH] on ground

Gear Circuit inop

Flight hour meter inop

Inlet lip heat inop

Stall warning & AOA indexer lights inop

Handle (gear) & hook (downlock) activated

10. The left squat switch: [HHHAP] on ground

Heat for left engine lip boot inop

Heat for AOA sensor - LOW (14V)

Heat - GND MAX

Ambient air solenoid - CLOSED

Pressure control safety valve - OPEN

11. Down lock switches: close when strut fully extends.  When all 3 down lock switches activated (all 3 gear down & locked): [1] WHEELS UP light deactivates [2] Gear horn deactivates [3] 3 green down lights illum

12. The red light in the gear handle will illuminate when: [1] the gear is unlocked, in transit, or gear warning horn activated [2] both power levers retarded below 79%N1 when gear not down and locked [3] pressing the Landing Gear Handle Warning Light Test Button [4] Flaps below APPR & gear is not down and locked.

13. The warning horn silence button and the WHEELS UP light can be turned off in flight (with power retarded, gear up, and flaps up by pressing the WARN HORN SILENCE button.  After silencing the horn, it will remain silent until the flaps are extended or at least one power lever is advanced beyond 79%N1 and then pulled back below 79%N1.  Don't silence the horn in the traffic pattern or during the final segment of an IAP when potential to land exists.  Only the PIC will cancel the horn.

14. During manual gear extension, the first green down lock indicator to illum will be the nosegear light.

15. The extension time of the gear is 4-6 seconds.  The retraction time of the gear is 5-7 seconds.

16. The prop reverse not ready light will not illum with the landing gear relay CB pulled (during manual emergency extension of gear).

-- The warning horn, wheels up light, and gear handle will all come on when all 3 landing gear struts are not down and locked and: [1] both power levers retarded < 79(+/-2)%N1  OR  [2]Flaps extended beyond approach position (Flaps - FULL)

-- The wheels up light & gear handle light illum when: [1] both power levers retarded < 79(+/-2)%N1 with Gear UP  OR  [2]Flaps extended beyond approach position with Gear UP.

17. It is possible to have a safe gear indication and not have safe gear (such as not having 3 complete tire or wheel assemblies).

18.  The main gear are held down and locked by spring loaded locks. The nose gear is held down by a jackscrew actuator.

19.  The gear are held up by the jackscrew in the actuator.

20. I can tell the difference between an electrical and mechanical malfunction on an unsafe gear by comparing visual indications with cockpit gear indications and gear circuit breaker positions.

Pull the following circuit breakers: [1] Landing gear relay CB [2] Landing gear indicator CB [3] Landing gear motor CB [4] Cockpit gear position indicators.  If during pulling any of the following , the gear indicates safely down and is visually confirmed safely down, land as soon as practicable.  ***Absence of illuminated AOA indexers could indicate premature landing gear motor cutoff which is indicative of an electrical problem.

- Vmca Maneuver

* Flown only by instructor

* Smooth power application: rapid addition during recovery can lead to spin.

* Some AoB can be taken out to simulate  Vmca at higher altitudes.  (Less asymmetric thrust production, Vmca lower)

Flying procedure: Numbered hdg, 150K, prop sync - OFF

[1] Altitude - 8000'AGL

[2] Power - 400ft-lbs

[3] Flaps - APPR

[4] Gear - UP

[5] Props - FULL FWD

[6] Airspeed - 110K

[7] Power - IDLE on left & Max Allowable on right

[8] Attitude - Raise nose so a/s bleeds off at 1K per second, ball centered, trim rudder, bank 5 deg into op eng.  Observe rudder full throw with full rudder applied and significant aileron to maintain 5 degrees bank.  Nose yaws into dead engine (vis, turn needle, RMI) at approx 86K.  Note position of rudder and ailerons.

[9] Recover - Slowly reduce power on op eng and decr AOA.  As airspeed rises above 86K, reapply power to op engine.  At 102K, execute single engine waveoff.  Complete when in a climb , clean, with > 102K.

- Landing Gear Manual Extension

* Easier at lower airspeeds.  Prop Rev Not ready light will not come on w/ LDG Gr. Relay CB pulled, AoA indicator might not come on if you don’t hit limit switch.

* Only manually extend for electrical problems, NOT mechanical problems.

* for practice: reduce stroke length when nose gear indicates safe, don’t pump past 3 green.  For real: same, but pump until significant resistance is encountered.  Do not stow handle or move controls, switches or CBs until on ground.

1.  A/S - 120 kts (135 max)

2. A/P-A/R

3. LDG GR CB - Pull

4. L.G. Handle - Down

5. Clutch Disengage Lever - Lift, turn CW

6. Manual Extension Handle - Pump until 3 green.  50 strokes as required.

7.  Gear Position - Check (Visual)

To retract after practice:

1. Emergency Engage Handle - rotate CCW, push down.

2. Extension Lever - Stow

3. LDG GR CB - In

4. Gear Handle - Up.

- Starter-Assisted Airstart

* Already Briefed

Don't airstart if engine died from MOVEOFF - Mechanical malfunction, Overheat condition, Vibration, Explosion, 0 N1, Fire, or Fuel fumes.

1. Power lever (failed engine) - IDLE

2. Prop lever (failed engine) - FEATHER

3. Condition lever (failed engine) - FUEL CUTOFF

4. Cabin Temp Mode - OFF

5. Vent Blower - AUTO

6. Radar - OFF/STANDBY

7. Anti-ice/deice - AS REQUIRED

8. Firewall valve - OPEN

9. Transfer Pump - AUTO

10. Boost Pump - ON

11. Crossfeed - AUTO

12. Generator (failed engine) - OFF

13. Starter (failed engine) - IGN & ENG START (check annunc light illums)

Note: If engine shutdown by closing firewall valve, time to light-off may exceed 10 sec.  Op engine ITT may increase approx 50C because of gen loading.  Retard if possible.

14. Condition lever - LOW IDLE

15. Starter (N1 > 50%) - OFF

16. Prop - UNFEATHER

17. Power - AS REQUIRED

18. Generator - RESET/ON

19. Fuel Control Heat - ON

20. Bleed Air - OPEN

21. Electrical Equipment - AS REQUIRED

22. Condition lever - HIGH IDLE

23. Instruments/Nacelles - CHECKED

- SSE Pattern Work

* Already Briefed

*  Emergencies: Flameout

Check 0% N1, Not fire or fuel related, no restart.

* E: Fire

Check for secondaries, Fire or fuel (FSV,FE,BA), no restart.  Might delay shutdown in Case 2 until downwind.

* E: Chiplight

Check for secondaries.  No secondaries, preload starter assisted airstart.  Not fire or fuel related.

* E: Oil Pressure

<40 psi, unsafe, secure eng.  Not fire or fuel.  Might consider securing once established downwind.

* Cases: 1 (Upwind)

Execute Eng fail T/O Boldface (Max pwr, gear up, (flaps up), speed up (102-110), Execute Emergency Eng Sht BF (Power idle, prop feather, cond f c/o, fire/fuel?, [Props 1900], FSV, FE, BA, comms, emer, dead eng checklist)

Don’t interrupt autofeather, turn CW.

* C:2 (Crosswind)

Roll wings level, Maintain 110 kts (102 min), roll out in power up rudder up clean up speed up, resume CW turn.  Standard checklist, accel to 120 on downwind, 800 ft.  At cabiness, turn back in quick to avoid extending pattern.

* C:3 (Downwind)

Standard, PU, RU, CU, SU, checklist.  Configure just prior to 180, NLT 90.  (App, gear)  120 kts, 110 min.  Can not clean up if able to maintain level.  Maj Riddle technique: always clean up.  Can tell tower if can't configure on profile.  Can’t hold landing checklist, must restart.

* C:4 (Past 180 to 90)

Power up to maintain 110-120 kts (dont cleanup unless committed to waveoff), continue turn, checklist (only first 3 required past 180)  Once on final, no checklist.  Not required, or first two for dead engine checklist.

* C:5 (Past 90)

Slight additional power, to keep 110 min.  Maintain dir control, accomplish first three (recommended), props full forward, three d&l, review me complete, you have the comms declare an emergency once safely on deck.  Note that both props full forward, not feathered.  Waveoff will autofeather prop.  (90% switch.)  A/C will float.  SSE full stop.

* Special: Pattern after SSE T&G

Combine all the cases.  Lots of rudder.  Don’t do flaps until the 180.  Time for dead engine checklist.

- SSE Landings/Waveoffs/T&G

* Already Briefed

* SSE T&G exactly the same as any normal T&G.  Power levers are already at idle.  Actual single engine T&Gs would not happen.

SSE Waveoff

* 1900 RPM Enhances climb performance

* Full Flap Not Recommended

[1] Power - AS REQ (Max allowable on operating engine, pos ROC, Vxse min)

[2] Flaps - APPR (unless already up)

[3] Gear - UP (+ rate of climb & no possibility of touchdown)

[4] Flaps - UP

[5] Props - 1900RPM (when CP calls "Gear Up")

Min altitude for IP to intiate SSE waveoff is 200'AGL.

- SSE Full Stop

* "This will be a SSE Full Stop Landing.  Once safely on the deck, I will pull both power levers over the detent, smoothly apply reverse to the operating engine, countering any swerve with rudder and aileron as necessary to maintain directional control.  As rudder effectiveness is lost, I will return both power levers toward flight idle until control effectiveness can be regained."

* NATOPS: Full flaps only w/ gear down + locked, no possibility of waveoff, still probably not a good idea.

* Reduce power in the flare.  Reverse A/R.  Use extreme caution if surface not hard + dry.

* FTI: Use most favorable RWY.  Good engine into wind airborne, bad engine into wind on gnd.

Upon touchdown:

[1] Power - IDLE

[2] Reverse - AS REQ

[3] Rudder & Braking - AS REQ (counter act yaw)

[4] Scan to end of runway

[5] If yaw excessive, reduce reversing and stop with brakes (don't lock brakes)

[6] Complete when at slow taxi on rwy

[7] Clear rwy if practicable, then shutdown

[8] Don't taxi on one engine!

[9] On rollout, dead engine into wind.

[10] In air, good engine into wind.

[11] Must complete a SSE Full Stop brief to perform SSE Full Stop Landing.  Initiate from a Case One or Two scenario and only when required in syllabus.