FAM-8: 6/5/05

- Engine Fire in Flight [NATOPS]

* Fire in flight is a critical emergency requiring pilot to assess/diagnose, take prompt corrective action.

* Sunlight can set off warning light.

Illum of FIRE warning light.  Confirm by checking eng instruments and nacelles.  Even if no secondary indications, consider shutdown by EMERGENCY ENGINE SHUTDOWN checklist.

(POWER UP, RUDDER UP, CLEAN UP, SPEED UP, Checklist)

[1] Power Lever - IDLE

[2] Propeller - FEATHER

[3] Condition Lever - FUEL CUTOFF

If fire or fuel leak, continue checklist.  If not, proceed to dead engine checklist.

[4] Firewall valve - CLOSED

[5] Fire Extinguisher - AS REQUIRED

[6] Bleed Air - CLOSED

[7] Dead Engine Checklist - AS REQUIRED

"Sir, you have the comms, declare an emergency, this will be a full stop landing."

[1] Handle emergency [2] Declare emergency [3] Squawk 7700 [4] If fire extinguishes, turn towards nearest suitable landing field and land.  [5] If fire will not extinguish, perform immediate forced landing or power-on ditch.

- Starter-Assisted Airstart

* Already Briefed

* FTI: Actual in flight engine shutdown only by cond lever: fuel c/o.  Only daylight VMC, VMC descent possivle to 4900x100 field w/firefighting equip.  Not lower than 4k.

* Feather: Complete emer eng shutdown, then preload starter assisted a/s to Gen-off.  Actual: Preload A/R.

* Windmilling: do not do above.

* Do not touch FSV.  See NATOPS note on cavitation of fuel pump and +10 sec restart after FSV use.

AIRSTARTS

Accomplished with battery or generator.  Normally starter-assisted.  Determine cause for engine failure before airstart.  Airstarts above 20,000 ft are hotter - periodically cycle condition lever to FUEL CUTOFF to avoid overtemping.  Reduce electric loads.

STARTER-ASSISTED AIRSTARTS

Don't airstart if engine died from MOVEOFF - Mechanical malfunction, Overheat condition, Vibration, Explosion, 0 N1, Fire, or Fuel fumes.

1. Power lever (failed engine) - IDLE

2. Prop lever (failed engine) - FEATHER

3. Condition lever (failed engine) - FUEL CUTOFF

4. Cabin Temp Mode - OFF

5. Vent Blower - AUTO

6. Radar - OFF/STANDBY

7. Anti-ice/deice - AS REQUIRED

8. Firewall valve - OPEN

9. Transfer Pump - AUTO

10. Boost Pump - ON

11. Crossfeed - AUTO

12. Generator (failed engine) - OFF

13. Starter (failed engine) - IGN & ENG START (check annunc light illums)

Note: If engine shutdown by closing firewall valve, time to light-off may exceed 10 sec.  Op engine ITT may increase approx 50C because of gen loading.  Retard if possible.

14. Condition lever - LOW IDLE

15. Starter (N1 > 50%) - OFF

16. Prop - UNFEATHER

17. Power - AS REQUIRED

18. Generator - RESET/ON

19. Fuel Control Heat - ON

20. Bleed Air - OPEN

21. Electrical Equipment - AS REQUIRED

22. Condition lever - HIGH IDLE

23. Instruments/Nacelles - CHECKED

- Anti-Icing System [NATOPS]

* Wipers, AoA Heat, Pitot Heat, W.S. Heat, Prop Heat, Lip Boot Heat, Fuel Control Heat, Oil-to-fuel heater, Fuel Vent Heat, Ice Vanes, Boots (5.)

* Wipers: Park/Slow/Fast (#2 subpanel bus)

* 95degF lockout for the windshield anti-ice heater.  Mag compass won’t work w/ it, objects may appear blurred.

* Engine anti-ice concerned w/ intake screen.  Ice vanes eject high-mass air out an intake in the back of the engine.

* Fuel Control Heat is for FCU pneumatic heat.  SHOULD NOT be turned on w/o cooling air over the engine.  (After engine start.)

* Prop Deice:  Don’t use stationary: will cause arcing on the slip ring!  RH prop out, RH prop in, LH out, LH n.  34+-6 sec.  Only older mechanical types show fluctuation when switchover.  5-6 A current draw each element x3.

* Boots: default: vacuum supplied for hold down.  Pressure comes in, drains thru check valve to 1 in Hg, the vacuum sucks rest out.  Either engine can supply enough bleed air, passes thru 18 psi regulator.  Vacuum thru venturi, proportional.  Never cycle fast, can cause ice to form that can’t be booted off.  Inflate for 7-8 sec.

* AoA heat, 14-28 Vdc.  Don’t use pitot heat on ground, no cooling airflow.

Anti-ice/deice:  Windshield wipers, Windshield antiice 95 degF, Powerplant Ice protection system (lipboot heat, Ice vanes [moves plenum, decreases torque, increases ITT, reduces range 10-12%]), fuel system antiice (fuel-to-oil heater, electric fuel vent heating element protection, fuel control unit pneumatic line heated jackets [subpanel #1,#2 busses]), Prop electrothermal deice (outboard/inboard element, power from brush block, slip ring, de-icer timer [sequence RH outboard, inboard, LH out, in, 34+-6 sec] should draw 14-18 amps), Pitot/AoA AntiIce (14 on ground, 28VDC in air), Surface deicer (7-8 second single cycle.  Standby compass will not work w/Windshield antiice, wipers, AC, elec heat.

Windshield Wipers: Don't operate on dry glass.

Windshield anti-ice: Temp sensing unit maintains 95F at windshield surface.  Electric heater lockout.  Mag compass will be unreliable when windshield anti-ice in use.  May blur objects.

Powerplant Ice Protection: Mechanically operated ice vanes or electrical lip boot heat.  With ice vanes extended slight decr in engine torque (40-60 ft-lbs) (Range decreases 10-12%)

Fuel System Anti-icing: Oil-to-fuel heat exchanger; operates continuously; no controls; one external fuel vent in each wing for both nacelle and wing tanks (protected from icing by electric heat elements).

Fuel Control Heat: Each FCU pneumatic line protected from ice by electrically heated jackets.  To prevent overheat, FUEL CONTROL HEAT should not be turned on unless cooling air will soon pass over the jackets.

Propeller Electrothermal Deicer System: Deicing boots on each prop blade; outboard and inboard element that receive electrical power through the deicer timer, slip ring, and brush block.; current measured by the prop ammeter.  Sequence: RH OUT, RH IN, LH OUT, LH IN (RORI LOLI); each segment heats for 34+/-6 sec; req 2 minutes for entire cycle; old mechanical timers sequence to next segment (show amperage fluctuation at timer switches to next segment); electrical timers always reset to RH OUT when power applied (no flux in amperage); each blade draws 5 to 6 amps (3 blades working correctly read between 14 to 18 amps).

Surface Deicer System: Ice removed from outboard wing leading edge, horizontal stab, and vertical stab by flexing pneumatic deicer boots.  Engine bleed air from the engine compressor supplies press to inflate boots and vacuum to hold down boots; press regulator protects system from overinflation;  when not operating, a distributor valve applies vacuum to hold boots down; when the DEICE CYCLE switch is toggled to SINGLE or MANUAL, a servo valve changes the inlet boots from vacuum to pressure that allows the boots to inflate; when the solenoid is deenergized, airflow through the valve is cutoff and it discharges out the boots through the intregal check valve until press reaches approx 1-inch Hg, at which time the boots port to vacuum and the remaining air is evacuated; the boots are again held down by vacuum; either engine can provide the bleed air for the system; bleed air passes through an 18-psi regulator and then enters the deicer and vacuum systems; vacuum press is created by the distribution valve and is proportional to pneumatic pressure by the deicer press regulator valve.  Stall speeds significantly increase with the wing deice boots inflated; never cycle the system rapidly, may cause ice to accumulate outside the contour of the inflated boots and prevent ice removal.  In SINGLE, boot inflate for 7 to 8 seconds, then deflate.  In MANUAL, all boots inflate and stay inflated until switch is released.  The boots can't be overinflated.

Pitot & AOA Heat Systems: On the ground, AOA heating elements pass through a current limiter lowering voltage to 14 volts DC (low heat).  At lift off , a squat switch on the left main gear causes the current limiter to be bypassed and heating elements receive 28 Volts DC (high heat); except for takeoff roll, PITOT heat should not be used on the ground (might overheat because of lack of cooling airflow over heating elements).

* Electrothermal Prop De-ice

Abnormal readings (amps)

0:  1. Check Switch

2. If off, turn on.

3. If on, inop.  Turn off.

0-14:	1.  Continue operation.

2.  If imbalance occurs, increase prop rpm briefly to aid in ice removal.

18-23: See above (0-14)

>23: Do not operate system.

Surface Deice Emergency Procedure: If boots fail to inflate, operate in MANUAL.  If boots fail to deflate, pull the SUR DE-ICE CB and reset as necessary to operate boots.

* Stall Speed significantly increases w/ boots inflated.