FAM-9: 6/12/05

- Ops Limits

* Refer to ops limit sheet.

Vmo (Max)=227 (-4 kts per 1k feet above 15.5k)

Mmo=0.48

Vsse=91

Vmca=86

Va (Manuv.)=153

Vle=155

Vlr=145

Vfe(35%/15'/Appr)=174

Vfe(100%/43'/Full)=140

Accel=+3.0to-1.0gs (Full flaps +2.0-0.0)

31,000 ft alt.

CG=11.6(7850)-19.4%(9650)MAC to 31.8%MAC

9710 lbs ramp, 9650 TO, 9168 land

Flared Landings

600 fpm sink rate

20 kts crosswind.

AVOID: landing on arresting gear, nosewheel touching cable risers above taxi spd., brakes on top of cable.

4.7 psi max pressure

20,000 hours for cabin.

Structure service life: 15,000 flight hours, 37,500 landings.

ITT

1. Normal (green arc) op range: 400-790C

2. Max ITT for start is 1090.  Time limited to 2 seconds.

3. The right engine is limited to 790C degrees during the start procedure for the left engine.

4. Always report the amount and duration of start exceeding 925C.

5. Always report the amount of operating ITT exceeding >790C.

6. Max ITT low idle 685C.

7. Max ITT in reverse 790C.

8. Max ITT for acceleration 850C (for 2 sec).

9. Max ITT for cruise climb 765C.

10. Max ITT for cruise 740C.

TORQUE

1. Normal (green arc) operating range: 400-1315ft-lbs.

2. Max at 2200RPM is 1520 ft-lbs.

3. Max at 1900RPM is 1315 ft-lbs.

4. Acceleration limit is 2100 ft-lbs.

PROP RPM (N2)

1. Normal (green arc) op range 1800-2200RPM.

2. Max RPM for takeoff 2,200 RPM. (NATOPS 10-12)

3. Min RPM for takeoff 2,200 RPM. (NATOPS 10-12)

4. Max RPM for acceleration 2420RPM (for 2 seconds).

5. Max RPM in reverse 2100RPM (limited by pneumatic section of primary governor)

6. Overspeed governor limited to 2,288(+/- 40)RPM.

7. Fuel topping governor limited to 2,332RPM.

N1 LIMITS (%)

1. Normal (green arc) operating range 50 - 101.5%

2. Low Idle range 50-53%

3. High idle range 64-67%

4. Max N1 acceleration 102.6%

5. Max N1 reverse 86%

6. Min N1 in reverse 80%

7. Max N1 can decrease during manual feather check NONE.

OIL PRESSURE AND TEMPERATURE

1. Normal (green arc) op range for oil pressure 85 - 100psi

2. Normal (green arc) op range for oil temp 10 -99 C

3. Min oil temp for engine start -40C

4. Min oil pressure on the ground 40psig.  Min oil pressure in the air 40psig.

5. Normal oil pressure must be obtained any time the engines are at pwr settings above 72%N1.

- Windmilling Airstart

* Already Briefed

* immediate action: results in loss of A/S and alt due to windmilling prop.

Use for emergency relight if engine inadvertently shutdown.

1. Power Lever (failed engine) - IDLE

2. Prop lever (failed engine) - FULL FORWARD

3. Condition lever (failed engine) - FUEL CUTOFF

4. Firewall Valve - OPEN

5. Autoignition - ARM

Note: Windmilling airstarts above 20,000 ft or below 2,200 prop RPM may exceed ITT limit.

6. Condition Lever - LOW IDLE

Note: If engine shutdown by closing firewall valve, time to light-off may exceed 10 seconds

7. Power - AS REQUIRED ("Continue the checklist")

8. Generator - RESET/ON

9. Autoignition - OFF

10. Condition lever - HIGH IDLE

- Flight Control System/Malf. [NATOPS]

* Go-around button in CP stick+left power lever.  10 NH, disengage AP.

* Trim: Cable-Drum System, Elevator anti-servo, Ail: Left side, 15+-1.5 deg travel up/down.

* Map lights in yoke.  ENG INST light.

* Elevator electric trim takes precedence over manual trim.

* DON’T TOW with rudder lock in (NWS problems)

* Flap Motor: Dynamic Braking (2 sets of motor windings), limit switches (Right flap dominates.)  APPR:35%/15', DOWN:100%/43'

1)  Primary flight controls: self explanatory.  Trim through manually actuated cable-drum system.  On control wheel:  Mic switch, AP/YD/ Trim Disconnect Switch.  Elevator trim Split-Thumb Switch, map light switch, pitch sync & CWS Switch, Go-around switch, 8 day clock.  Rudder pedals adjusted by switches on the bottom of them.

2)  Secondary flight controls: trim, flaps.  Trim tabs, jackscrew actuator and cable drum system.  Elevator also uses electric.  Anti-servo action: makes control surface force required.  Co's switch takes priority.   15 deg aileron trim capable.    Wing flaps:  each has separate jacktooth actuator actuators driven through flexible shafts by single reversible electric motor on forward side of rear spar, dynamic braking system through the used of 2 sets of motor windings.  Approach: 15 deg, 35%, Down: 100%, 43%.  CBs:  Flap and Stab: indicator, control circuits.  Motor:  Wing Flap Motor)

3)  Flight controls lock: 2 pins and U-strap.  Ensures positive locking of rudder, aileron, engine controls.  DON’T Tow with Rudder lock on.

Primary flight control system: rudder, elevator, and aileron.  Manually operated by mechanical linkage using control wheel for ailerons and elevators and adjustable rudder/brake pedals for rudder.  Trim control through manually actuated cable-drum system.

Control Wheels: Pitch sync on pilot control wheel only.  Go-around switch on CP control wheel only.  Control wheel map lights power through ENG INST CB.

Rudder Pedals: Two positions - fore and aft.

Trim Tabs: Manually actuated and mechanically controlled by cable-drum and jackscrew actuator system.  Elevator trim tab is anti-servo (as elevators displace, trim tab moves in same direction, increases effective control surface area and increases force required to apply it).

Manual Elevator Trim Tab Control: Elevator tab deflection indicated by position arrow.

Electric Elevator Trim: Controlled by dual element thumb switches and trim disconnect switch on control wheel.  CP takes priority.  ELEC TRIM CB.  Operates only when both elements of thumb switch moved.  Electric trim can't be overridden by manual trim wheel.  Disconnect is a bi-level, momentary push-type switch button.  First depress disconnects autopilot and yaw damp.  Second level disconnects electric trim.

Aileron Trim Control: Adjusts left aileron trim tab.  Relative only, not in degrees.  Full travel 15 +/-2 degrees up or down.

Flight Controls Lock: Don't tow with rudder lock installed; could result in serious damage to steering linkage.

Wing Flaps: All metal, slot-type, electrically operated, two sections for each wing.  Both sections operate as a single unit, actuated by a separate jackscrew actuator.  Driven through flexible shafts by a single, reversible electric motor mounted on fwd side of the rear spar.  Motor powered by right main bus.  WING FLAP MOTOR CB.  Motor uses dynamic braking, prevents over travel.  Flaps indicated in percent of travel.

Wing Flap Selector Handle: 0% = UP.  35% = APPR.  100% = DOWN.  Limit switches, actuated by right inboard flap control flap travel.  FLAP & STAB CB.  Intermediate flap positions can't be selected.

Wing Flap Position Indicator: Show percent of travel from 0 to 100.  APPR = 15 degrees.  FULL DOWN = 43 degrees.  FLAP & STAB CB.

MALFUNCTION

On Ground Flight Control Failure: Check yaw damp & autopilot off; leave controls in jammed position; call for immediate inspection from maintenance.

Flap System Failure: If flaps inop, land in existing flap configuration.  If split flaps, return flaps to previously selected position.  Pull the wing flap CB to prevent inadvertent flap movement.

In pattern: Select Flaps - APPR.  [2] During "Flaps" call in Gear Down Landing Checklist, check position of flaps on flap indicator.  If indicator doesn't correspond to position of flap lever: [1] Visually check position of flaps on right and left.  [2a] If flaps selected APPR, indicator shows UP, but visually checked APPR - leave flap lever at APPR and pull WING FLAP CB.  [2b] If flaps selected APPR, indicator shows UP, and flaps visually checked UP - position flap lever UP, pull WING FLAP CB, and brief new speeds (110 on final, 105 over threshold).

- Forced Landing [Check NATOPS]

* ELP:  Can slip into High Key, flaps may not be available.  High Key: 130 KIAS, feathered, 3000-2500 AGL, approach checklist complete.  Low Key: 130 KIAS 1500-1800 AGL, Gear Down, Base Key (Field Made): 110 KIAS, 800-900 AGL, Flaps Approach, Landing Checklist, Final (Landing Assured): 110 KIAS, Flaps Full, Prior to landing: Battery Off.  Aimpoint: 1/3 Down Runway.

If suitable altitude, perform a circular pattern to observe surface terrain and wind conditions.  After terrain and wind noted, set up normal pattern and extend gear when landing assured.  Plan to overshoot (less drag).  Use flaps as needed.  If positive gear down and locked indication can't be determined, do a gear up landing.  Night egress: preposition threshold, spar and aft compartment lighting ON.

1. Configuration:  [A] Don't extend full flaps until landing assured [B] Gear up landing based on typ of touchdown surface

2. Landing Area: Adequate size to land, free of obstacles, smooth.  Cultivated fields are best.  Swamps, bogs, shallow lakes, and forests should be avoided. Land into wind.

3. Landing: Make at slowest speed that retains complete control.  If landing into trees, fly into them; don't attempt to land on the tops.  (Refer to NATOPS Figure 16-5 on page 16-12 for Emergency Landing Pattern illustration)