FAM-EP: 6/16/05

- A/C Control in emergencies

* A/C Control is paramount: Fly the A/C first!  Maintain Aircraft Control, Analyze the Situation, Take the Proper Action, Land as Soon As Conditions Permit.  In order to enhance PIC/PF's SA, they can direct CP/PNF to do actions outside of their scan (such as generators, esp. At night/IMC.)

- Pattern Considerations

* EP's in the pattern become more time critical, as SA is split between flying pattern and dealing with EPs.  Gives more options, though...  Can land much sooner and resolve EP on ground.  Eg Case 4,5.  If deferred EP, or landing not desired, request Delta Pattern (2000 MSL here) for dealing with EP, maintain pattern ground track.

- Copilot/Right Seat Responsibilities

* Do checklists when called (call-response standard, call-response-response for eps.), anything on your side of the cockpit you own.  Gear especially.  Clear the right side, review complete on the landing.  Add to PIC's SA, assist as directed.  PNF is a Check on safety-of-flight.