- Operating wt. = approx. 375m

- Each pax = 241 lbs (73 pax = 17.6m)

- Check winds valid for T/O time. If not valid, update and spin new times

- T/O wt = ramp wt – 3000 lbs

- Use overhead temp dev to find highest acceptable FL

BLOCK

- Time from T/O to BDP

- 10% of Cat 1 route, 1 hour max

- Total of 1 & 2

- Use overhead dest wt. Use time to most distant alternate if 2 are required. Add 8,000 lbs fuel if dest. Wx is vis only.

- Use overhead alternate wt. Use 45 min at 10,000 ft unless alternate is unavail., located in Alaska, lat > 59 deg, remote or island dest. Then use 1+15 min at 20,000 ft.

- 7,000 lbs, 2,000 for descent, 5,000 for app & landing

7a. Off course maneuvering = 600# /min for

climbout, 400# /min for cruise, 5,000#

for unreliable nav aids. Known holding

delays

7b. Tankered fuel for subsequent mission

segments. Hyd cooling if needed

# 8.   Total 3 + 4 + 5 + 6 + 7a & 7b

- STTO = 3,000 lbs or 120 lbs /min for a

## Max of 5,000 lbs

- Block 10 is required ramp, block 8 is required for flight.

- Actual fuel on aircraft

- 10 – 11 = 5,000 lbs normally. Not required to defuel for standard load.

- Block 4 + 5 + 6. As technique write actual sum so FE can use on fuel log. If less than 25,000 lbs add difference to block 7b stored fuel.

Missions will normally be planned for a step-climb profile, choose the most fuel conservative profile to accomplish mission requirements.

Cruise Ceiling: Used to determine initial level off/highest acceptable FL, known as cruise-climb altitude (300 FPM rate of climb at NRT).

Cruise Speed: Normal cruise speed is 300 KCAS / 0.77 M, whichever is less.

Fuel Requirements:  If decompression would cause descent to an altitude resulting in fuel consumption exceeding planned fuel, add fuel to recover at a suitable airfield from ETP at the appropriate altitude.  This is not required if the aircraft can recover at a suitable airfield at FL 250 at LRC speed from ETP.