| # Oil Temp |  | ## Starting TIT |  | # Fuel System |  |
| --- | --- | --- | --- | --- | --- |
| Max taxi & time | 85-100C / 30 min | ## Max with enrich on (gnd) | 100C | Main tank pump press | 15 – 24 psi |
| Max in-flight & time | 85-100C / 5 min | ## Max with out enrichment | 200C | Aux & ext tank pump | 28 – 40 psi |
| Normal range | 60-85C | ## Max for air start | 200C | Low fuel press light on | 8.5 psi |
| Oil Cooler flaps maintain | 78-82C | ## Normal start limits | 750 – 830 C | Aux & ext tank empty lt. | 23 psi |
| Min for take off | 40C and increasing | ## TIT < 720 C: Do what? | Write up, see MX | Rated output - all pumps | 6000 pph |
| Min (start & warm-up) | -40C | ## TIT 720 – 750 C: | Write up, temp ctl ck | Dump outbrd/inbrd/ext | 1600/1500/65 |
| ## Engine Oil Pressure |  | ## TIT 830 – 850 C: | Write up | Min dump altitude | 5000’ no circle |
| Max (start & warm-up) | 100 psi | ## TIT 850 – 965 C: | stop, write up, 1 retry | Primary fuel mgt | 6500outbrd/6900inbd |
| Normal range / flux | 50-60 psi 10 psi | ## TIT >965 C: | stop, write, see MX | # P/N inbrd to outbrd | 0 to 500 lbs |
| Min (LSGI) | <50psi if nml@100% | ## Abnormal torch | stop, write, see MX | Sym Tank Diff (excp Aux) | 1000 lbs |
| ## Gearbox Oil Pressure |  | ## Engine TIT |  | Wing to Wing (excp Aux) | 1500 lbs |
| Max (start & warm-up) | >250 psi | ## Takeoff range / time | 1067 - 1083C / 5 min | External Tanks | 1000 lbs |
| Normal range / flux | 150-250 psi  20 psi | ## Military limit / time | 1049 C / 30 min | Aux Diff. | 1 Empty 1 Full |
| Mission continuation | 130 psi | ## T/O Reduced Min | 900 |  |  |
| Min (LSGI) range | 50 | ## Max continuous | 1010 C | ## APU/GTC |  |
| # Propellers |  | ## Accel peak limits | >1083(5s)/1175 max | Normal EGT range | 260-650 C |
| Prop brake disengage | 23% or starter app | ## Crossover | 800-840 | Max EGT | 650 C |
| Feather blade angle | 92.5 deg | ## Normal Limiting | 1083 C | Normal RPM range | 95 - 105% |
| # Blade angle in reverse | -6 to -8 degrees | ## Max downshift TIT | 850 C | Max RPM | 106% |
| Pitch lock range | 25-55 deg | ## Vol. 3 for climb/cruise | 970 C | Stop switch P/N | 110% |
| Pitch lock RPM | 103% | ## # of thermocouples | 18 (2 elements each) | Starter cycle P/N | 1 min on, 4 min off |
| In pitch lock, mtn. RPM | 96-98% | # Propulsion Check |  | APU Warm up for bleed air | 1min / 4 min <0c |
| To unlock pitch lock | Increase 2 deg | Wind note at > 7000” lbs | W/in 30 of 10k wind | Start and operate below | 20,000’ |
| Lo pitch stop blade angle | 23 deg | Icy – plane will move w/ | > 8000’lb on 4 eng | Door open ground / flight | 35 deg / 15 deg |
| Low pitch stop test | >200”lb increase | Min RPM for check | 8000’lb | GTC Warm up | 1 min / 4 min <0c |
| Prop oil quantity (total) | 26 quarts (6.5 gal) | Rev ck – max diff | > 1000” lbs | ## Start Light goes Out | 35% |
| Prop oil sump capacity | 6.5 qts in press side | # Oil Quantity |  | ## On Speed Light ON | 95% |
| Prop low oil light on at | 2 qts low in press side | Capacity range | 4 to 12 gal | Windshield Anti-Ice |  |
| Torque to decouple | -6000”lb | Expansion space | 7.5 gal | Cold Start Procedure | 5s on, 10s off, till -43C |
| Prop aux pump limit | 1 min on, 1 min off, | Low light at | 4 gal | Max OAT operations | 81F (27C) |
|  | not > 2 min in 30 min | Allowable burn rate | 1 qt/hr (.35 gal/hr) | ## Nesa temp / # of windows | 80-100 F / 9 |
| Stat. fethr ck time (R - F) | 25 sec | # After –G, mont. eng for | 2 minutes | ## Prop De & Anti Ice |  |
| Pull out if not pop out in | 6 sec | -G: Press shd run within | 10 sec (if not do ESP) | Timing cycle (De-Ice) | 15 sec on / 45 sec off |
| NTS operates at | -1260  600”lb | -G: Loss of oil when? | 30 to 90 sec | Max Gnd ops time (De) | 2 cycles (2 minutes) |
| # Throttle Positions |  | # Hydraulic System |  | Deicing amp load range | 65-90 Amps |
| LSGI range | 9 - 30 deg | Engine Pump Norm/Max | 2900-3200 / 3500psi | Anti-Ice amps | 20 Amps |
| Reverse | 0 - 18 deg | Aux & Emer Norm/ Max | 2900-3300 / 3500psi | ## Leading Edge Anti-Ice |  |
| Ground idle / beta range | 18 deg / 0 - 34 deg | RPM for indication | On speed | Max ground ops time | 30 sec |
| Flight idle / alpha range | 34 deg / 34 – 90 deg | RPM for norm press | 30 sec after on speed | Normal operating range | 75-200C |
| Crossover for TD | 65 deg | LSGI minimum press | 2550 psi | Overheat range | >200C |
| Temp Limit Range | 0-65 deg | Aileron boost pressure | 2050 psi | Control valve open/close | 158-180C |
| # Temp Control Range | 65-90 deg | Low press light on / off | 1000 psi / 1350 psi | Norm temp of anti-ice air | 350C |
| ##### Engine RPM |  | Suction boost light on | 20 psi | LE CHK, alt / temp | >5000’ or < 20C |
| Oil pres By | 35% | Norm - 2 brake app left | 2900 - 3200 psi | # AC Electrical |  |
| Ignition By | 35% | Norm – 1 brake app left | 2250 psi | Frequency range | 380-420 cps |
| Starter Disengage | 60% | Emer – 1 brake app left | 2900 psi | Voltage range | 110-125 v |
| Normal flt & Takeoff | 98 – 102% | Shutdown if press over | 3900 psi | Max AC load | 1.05 |
| Norm flight idle (on gnd) | 92.5 – 100.5% | Do not shut off pump if > | 3450 | Generator capacity | 40 KVA |
| Normal gnd idle | 94 - 102% | Emer. accum preload | 1000 100 psi | Gen cau (out) light illum | any ph =< 95 for 4 sec |
| Max reverse | 96 – 106% | Utility preload | 1500 100 psi | Bus cau (off) light illum | av ph<96v/any<100v |
| Low speed ground idle | 69 – 75.5% | Booster preload | 1500 +100 psi | ## # Gen to Power all Buses | 2 |
| Allowable Flu | +/- .5 total 1.0 | AUX P model | 300 psi | APU generator capacity | 40 KVA |
| Ignition Relay on / off | 16% / 65% | Loss of sys press cklist | <2500 psi | APU generator load | 1.05 |
| ## Speed sens. valve actuate | 94% | Brake pressure P | 2050 | ATM Gen capacity | 30 KVA/ 20 no fan |
| Fthr on airst if no nts by | 10% | ## Brake pressure N | 3000 | ATM Gen Rating | 115-200 |
| Air start light off nts/mx | 30% / 40% max | Thermal CB on suc boost | 11 amps | ATM Load | .66/1.0 below 104F |
| ## Speed of Eng / Prop | 13,820 / 1021 | Utility reservoir capacity | (N) 6.5 gal (P) 7.0 | ## DC Electrical |  |
| # Gearbox reduction ratio | 13.54 to 1 | Booster reservoir cap. | (N) 3.1 gal (P) 2.0 | Voltage Range | 25-30v/28v |
| ###### Engine Torque |  | Aux HYD reservoir cap. | (N) 5.8 gal (P) 6.22 | Max DC load | 1.03 |
| Oil temp < 0 C | Minimum | Flaps 0-15: normal | 1100-1400psi | DC power sources | Batt. 4TRs, Ext DC |
| Oil temp 0 to 40 C | 4500”lb max | Caution | 1600 psi | Batt. Voltage norm / min | 24v / 21v |
| Max continuous | 19600”lb | Flaps 15-100: norm/cau. | 2900- 3200 psi | Min INS Batt Volts | 21v |
| Allowable Rev. Diff. | 1000/ Just a write up | Caution | 3500 psi | V to close/open batt relay | 18v –close/ 12v-open |
| Propulsion Check | 30 deg @ 10 Kts | # Amt of fluid in isol circ | 1 gal | # Amp-hr rating of battery | 31 (1hr) / 36 (5hr dis) |
|  |  | # Fluid quantity on board | 21 quarts (5 gal) | Battery – min temp / time | -29F for >=4 hrs |

| # Bleed Air System |  | ## Cargo Compartment |  | ## Air Start with no NTS | 130kias below 5000’ |
| --- | --- | --- | --- | --- | --- |
| Diffuser air temp/press | 600 F / 125 psi | Milk stool req’d for ramp | >2000 lbs across rmp | ## Airspeed for prop malf | 150 ktas |
| # Regulated output (Norm) | 40 - 50 psi | # Ground Troops | 92 | ## Miscellaneous |  |
| # Regulators must be w/in | 3 psi of each other | # Paratroops with equip | 64 | Prop fail to fthr, hold for | 30 sec |
| Ground check for engine | 65–35 psi >= 16 sec | Litters (2 attend/6 attend) | 74 / 70 | Accel bleed valve open | Below 94% RPM |
| APU min output press | 35 psi | 2 loadmasters required | > 40 pass/simo AR | Time to extend full flaps | 8 to 15 sec |
| Ground check for APU | 30–15 psi >= 16 sec | # Lightest cargo jettison able | 2500 lbs | Time to retract flaps | 10 to 15 sec |
| GTC min output press | 35 psi | CG limits | 15-30% | Gear extension/retraction | 19 sec or less |
| Gnd Check GTC | 30-15 psi>=8.5 sec | # of Tie downs Rings – 10k | 175 (7 colms of 25) | Preheat eng & apu below | -37 C |
| Ground check for engine | 65-35 psi >=10 sec | # of Tie downs Rings– 25k | 12 (2 avail w/ dul rls) | Cold WX ops temp | <= 0C (32F) |
|  |  | # of Tie downs Rings – 5k | 55-11 ea wal 33 rmp | Hot WX ops temp | > 35C (95F) |
| # Pressurization System |  | ## Cargo area height | 9’ | Hot brake peak temp time | 1 to 5 minutes |
| Gauge range | -1.2 to 15.8” Hg | ## Cargo area length | 50’ | Peak hot tmp n wheel/tire | 20 to 30 min |
| Safety valve open | < -0.76 & > 15.9” Hg | ## Cargo area space | 41’ x 10’3” | Hot brakes: no tow for | 15 min |
| Differential Press Max | 15.16” Hg | ## Speeds, Angles, & Feet |  | Full anti-skid; no retr. for | 15 min |
| Aux Vent Valve Opens | Hg / 0.28 psi | ## Max airspeed (mach) | 0.64 mach | # of Nacelle Detectors | P/ 7 N/8 nacelle |
| W.shield cracked (1 or 2) | 10 inches max | ## T-storm penetration | 65+ p off stall <=180 | Temp for Nacelle ovheat | 300 Degrees |
| Cargo cracked (1 pane) | 10 inches max | ## Windmill Taxi Start | 100k till 4000’ / 40% | # of Turbine ovheat Det. | 4 turbine |
| Cargo cracked (2 panes) | 0 inches max | ## Flaps 10 % | 220 | Temp for Turbine ovheat | 700 Degrees |
| Rate ctl Min Rate | 30-200 | ## Flaps 50% | 180 | Under floor heat temp | 80 F |
| Rate ctl Max Rate | 1600-2900 | ## Flaps 100% | 145 | Start: st. vlv light on in | 5s max (2-3 psi to st) |
| Crew door jettison press | < 3.1”Hg | ## Door Only | 185k | Start: st vlv opn lite out in | 15s after start release |
| # Oxygen & Life Support |  | ## Ramp and Door | 150k | Seat inertial reel lock | 2 to 3 G’s |
| First aid kits | 23 stations | ## Air deflector / troop door | 150k | ## Max Symm / Unsym G’s | 2.0 Gs / 1.5 Gs |
| Oxy ports flt deck / total | 2 / 09 | ## Lights | 250k | ## Most reliable fuel reading | 0 deg roll,  3 d pitch |
| ## Min lox per VOL 3 | 5 liters or ETP | ## Gear | 165 k | Max land wt / max vvi | 155000lbs / 300 fpm |
| ## Fully serviced quantity | 25 liters | ## Wipers ineffective | 180k | Norm land wt / max vvi | 130000lbs / 540 fpm |
| ## Low light on | 2.5 liters on gage | ## Troop door open / locked | 250k | TO weight/ max EWP | 155000# / 175000# |
| ## Press range No Flow | 270-455 psi | ## Crew door bailout | 150k | ## Max wt for windmill txi | 135,000 lbs |
| ## Press. range Cont.Flow | 270-340 psi | ## Paintd surface w/o stencil | 250k | Overtorque tiers | 19600, 21500, 23000 |
| ## 1 Full sys–min supply-hr | 96 man hours | ## Tire rotation (nose/main) | 139 / 174 | Redo told card w/ chng of | 5000’, 5*, 1000’ PA |
| # of life rafts / capacity | 2 / 20 men | ## Crosswind taxi norm/max | 30k / 60k | RCR for no brakes | 2 |
| Fire extinguishers | 4 | ## Ditching | 10+ stall w/ full flaps | ## Elev trim tab – elec stop | 6 deg dn / 25 deg up |
| Fire ext. press-halon1211 | 125  50 psi | ## Max bank angle | 60 deg | ## Elev trim tab – mech stop | 8 deg dn / 27 deg up |
| Emer lights illum | 2.5g’s or Ess DC off | ## Max bank angle w/ flaps | 45 deg | ## Eng strt cycle (1 on, then) | 1 off, 5 off, 30 off |
| # of Emer exit lights | N- 7 P- 8 | ## Min space for max turn | 169’ 7” | # Tire Fuse able Plugs | 390* F |
| POK time of oxygen | 10 to 14 min | ## Lowest vert clearance | 5’ 9” (inbrd prop) | # Battery Bus (8) |  |
| EEBD time of oxygen | 14 to 16 min | ## Max spd/angle >= 155M | 10 kts / 20 deg | Fire agent | Voltmeter |
| ELT / ULB range | 1000 mi / 2k-4k yds | ## Angle of flaps at full | 35 deg | Alarm bells | Emer exit lights |
| LPUs | 1 per crewmember | Going into Ground Idle | Below 115 kts | AC external power | Emer depress |
| ## Portable oxygen bottles | 10 | Wing span / length / high | 132’7”/ 97’9” / 38’3” | Jump lights | ELT |
| Hand Axe | 2 | ## Max spd w/ 60d nosewhl | 5 kts |  |  |
| ## Smoke MASK/Quick Don | 7 | Max turn angle >=20 kts | 20 deg max |  |  |
|  |  | #### Boldface |  |  |  |
| ### FUSELAGE FIRE/SMOKE AND FUMES ELIMINATION. |  |  | GTC/APU EMERGENCY SHUTDOWN (GROUND/INFLIGHT). |  |  |
| 1. OXYGEN “ON, 100 PERCENT” (ALL) |  |  | 1. FIRE HANDLE “PULLED” (E) |  |  |
|  |  |  | 2. AGENT “DISCHARGED” (FOR FIRE) (E) |  |  |
| ## LOSS OF DROUGE OR BREAKAGE OF REFUELING HOSE |  |  |  |  |  |
| 1. Reel Fuel valves switch CLOSE (E) |  |  | ENGINE SHUTDOWN PROCEDURE. |  |  |
| 2. Reel control power switch OFF (E) |  |  | 1. CONDITION LEVER “FEATHER” (CP) |  |  |
| 3. Pumps for Transfer OFF (E) |  |  | 2. FIRE HANDLE “PULLED” (CP) |  |  |
| 4. Proceed to Post Air Refueling Checklist |  |  | 3. AGENT “DISCHARGED” (For Fire or Nacelle Overheat) (CP) |  |  |

| ## General Memory Aids |  | Gearbox Accessory Drive | Generator | ## P/ Shutdown Conditions |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ## Prop safety features | Pitch lock |  | Hydraulic pump | Generator Failure |  |  |
|  | Low Pitch Stop |  | Oil pump | Engine Fire |  |  |
|  | Feather |  | Starter | Turbine Overheat |  |  |
|  |  |  | Tach generator | Nacelle Overheat |  |  |
| ## Gearbox safety features | NTS |  |  | ## Uncontrollable Power |  |  |
|  | Decouple | ## Engine Accessory Drive | Oil pump | Cert. Prop Malfunctions |  |  |
|  | Prop brake |  | Fuel pump | Uncont. Rise in TIT |  |  |
|  |  |  | Fuel filter | Uncont Drop Oil Pressure |  |  |
| ## Fire Handle (ess dc) (9) | Engine fuel control |  | Speed sensitive ctl | Uncont. Rise in Oil Temp |  |  |
|  | Engine oil control |  | Speed sensitive vlv | Unusual Vibration or Rough |  |  |
|  | Firewall fuel |  |  | Throttle Control Failure |  |  |
|  | Firewall hydraulics | ## CB Panel – copilot side | Main | ## Excessive Visible Fluid Leak |  |  |
|  | Engine bleed air |  | Radio |  |  |  |
|  | Eng start ctl circuits |  | Start (ESS DC) | ## N/ Shutdown Conditions |  |  |
|  | Prop feathered |  | Engine (ESS DC) | Engine Fire |  |  |
|  | Positions fire ext vlvs |  | Airframe (ESS DC) | Turbine Overheat |  |  |
|  | Arms agent discharge |  | Lights (Main DC) | Nacelle Overheat |  |  |
|  |  |  |  | ## Uncontrollable Power |  |  |
| ## CB Panel – Pilot side | LH AC | ## Sec AC – 26 v ac, 1 PHz | #1 step-down xformer | Cert. Prop Malfunctions |  |  |
|  | RH AC |  | ## Fuel pressure ind. |  | Uncont. Rise in TIT |  |
|  | ## AC inst & eng fuel ctl |  | Emer hydraulic press |  | Uncont Drop Oil Pressure |  |
|  | Fuses |  | Booster HYD press |  | Uncont. Rise in Oil Temp |  |
|  | Battery bus |  | #3 eng&grbx oil pres |  | Unusual Vibration or Rough |  |
|  | Isolated DC |  | #4 eng&grbx oil pres |  | Throttle Control Failure |  |
|  | Essential AC |  |  |  | ## ST VLV caution light |  |
|  |  | # Air Refueling System |  |  | ## Excessive Visible Fluid Leak |  |
| ## Sec AC – 26 v ac, 1 PHz | #2 step-down xformer | Length of Hose | 81-93 ft. |  |  |  |
|  | Aux HYD press | Min length for AR | 90 ft. |  |  |  |
|  | Utility HYD press | Cut Hose grater than | 7 ft. |  |  |  |
|  | Normal brake press | Refuel Range | 56-76 ft. |  |  |  |
|  | Antiskid test | Unlatch Pressure | 1000 psi  50 psi |  |  |  |
|  | #1 eng &grbx oil pres | Latch Pressure | 400 psi  50 psi |  |  |  |
|  | #2 eng &grbx oil pres | Air Refuel Reel Acc. psi | 1350 psi  50 psi |  |  |  |
|  |  | Refuel Surge Suppressor | 40 psi  50 psi |  |  |  |
|  |  | LD Speed with Hose Ext | 130 not < approach |  |  |  |
|  |  | Fuse Tank Capacity gal. | 1800 gal |  |  |  |
|  |  | Dump down to lbs left | 570 lbs |  |  |  |
|  |  | Fuse Tank pump press. | 28-40 psi |  |  |  |
|  |  | Tank Empty light | 23 psi |  |  |  |
|  |  | Hyd press Off light | 1700 psi |  |  |  |
|  |  | Tanker Ready light w/in | 5 sec. |  |  |  |
|  |  | Power source for AR sys | ESS DC |  |  |  |
|  |  | Pwr source for Guillotine | ISOL DC |  |  |  |
|  |  | Trial Speed | 5-8 fps |  |  |  |
|  |  | Rewind Rate | 5-15 fps |  |  |  |
|  |  | Emergence Trail | 3 fps |  |  |  |