# INSTRUMENT TAKE-OFF

- Select, tune, and identify navigational aids to be used for departure

- Align horizon bar on each ADI

- Place MODE/NAV SEL switch to primary nav aid to be used

- Place Flt Dir switch to manual

- Align airplane on runway, set heading marker to most logical heading, and set departure course in the course window on the HSI

- Apply takeoff power

- At Minimum control speed or 5 knots below charted takeoff speed, whichever is greater, rotate and establish 7 deg nose-high on ADI

- Confirm two positive climbs, retract gear

- With a minimum climb rate of  300 VVI and the airplane accelerating, retract the flaps at a minimum. of  takeoff speed + 20 KIAS

- Trim

- Establish climb power, and turn on anti-icing as required.  Be alert for power loss

## BACK COURSE LOCALIZER OPERATION

- T.I.M. station and place mode selector to VOR/ILS

- Set the front course in the course selector window on the HSI

- Set the HDG marker on the tail of the course arrow, and place the FLT DIR switch to manual

- Use HSI CDI to fly the ILS (Localizer Back Course)

### 3 ENGINE BRIEFING

Check2 – EPS/Decent/Landing			Go:  Throttles toward max as necessary

S - Simultaneous Transmissions			        50 bank into good engines

T – Trim: Add/Remove on Final			        Flaps 50%/Gear up when able

A – Review Landing Speeds			        Flaps no < 20% prior to 2 engine VMCA

R – Reversing				        Clean-up/Climb at 3 engine climb speed

APPROACH AIRSPEEDS

LOW ALTITUDE APPROACH:  (4 OR 3 ENGINES)

PRIOR TO ENTRY – 170 KIAS

OUTBOUND -- 150 KIAS OR APPROACH SPEED (WHICHEVER IS HIGHER)

FINAL – APPROACH SPEED

LOW ALTITUDE APPROACH:  (2 ENGINE)

PRIOR TO ENTRY – 170 KIAS

OUTBOUND – FLAPS UP, GEAR AS REQUIRED; 160 KIAS MIN

INBOUND – 150 KIAS OR APPROACH SPEED (WHICHEVER IS HIGHER)

FINAL – 150 KIAS OR APPROACH SPEED (WHICHEVER IS HIGHER)

RADAR:  (4 OR 3 ENGINES)

DOWNWIND -- 170 KIAS

BASE LEG – 150 KIAS

FINAL – APPROACH SPEED

RADAR:  (2 ENGINES)

DOWNWIND – FLAPS UP, GEAR AS REQUIRED, 160 KIAS MIN

BASE LEG – FLAPS UP, GEAR AS REQUIRED, 160 KIAS MIN

TURN TO FINAL – 150 KIAS OR APPROACH SPEED (WHICHEVER IS HIGHER)

FINAL – 150 KIAS OR APPROACH SPEED (WHICHEVER IS HIGHER) UNTIL LANDING IS ASSURED

WARNING!!!

#### DO NOT EXTEND FULL FLAPS OR SLOW BELOW 2 ENGINE MIN CONTROL SPEED UNTIL LANDING IS ASSURED