LL-3 (N4403): 9/4/05

- Modified Contour/Nav Procedures

* Using visual references and the RADALT, flying 500 AGL with reference to the base contour for the area, allowing momentary deviations above and below for the base altitude for terrain depressions and obstacles to permit a smooth flight profile.

- Natural features preferable to cultural features.

- Do not overfly built-up areas.

- Use terrain masking if available.

- Avoid being skylighted, go around hills rather than over.

- Fly to shadows, place airplane's shadow on terrain shadows.

- Turns should not be made into significantly higher terrain unless climb-out performance is analyzed.