LL-4 (N4501): 9/4/05

- Low-Level Brief

* As required

- Night Illusions and Spatial D

* [11-217 Ch.22]

- General Spatial D: Equilibrium: Semicircular Canals (angular accel), Otolith Organs (Linear Accel.)   Vision: ambient (peripheral) system=where, little processing.  Focal (central) vision=what, high processing.  NVGs help see, but reduce FOV.  SD Type I: Unrecognized, Type II: Recognized, Type III: Incapacitating.  Night Join-Ups very disorienting.  Watch for somatosensory/vestibular illusions, counteract w/ xcheck, trust your instruments.

- Night Visual Illusions:  Blending of earth + sky: Denies a horizon.  NVG Flight over water: difficult to discern surface; use inst. X-check.  False vertical + Horiz cues: sloping cloud deck/aurora; cross-check.  No cues: very dangerous for formation flight.  Undetected IMC: can occur on NVGs w/ fog; watch for image degradation.  Vection: self movement illusion by movement of other options.  Autokinesis: stare at a point of light and it will seem to move.  Black hole: water/no horizon; use peripheral vision.  Induced motion: perceived motion of objects w/ no discernable bkgnd.

- Night Low-Level Nav Techniques

* Primarily through DR.  Fly planned hdg, planned time, planned A/S.  BARALT primary altitude reference.  Unlit landmark may be impossible to see.  Fixing on points very difficult.  Use red lights.  Moonlight may show up some landmarks, river/lakes can stand out.  Large cities show up but not small.  Distinctive light patterns on airfields.  Busy highways have headlights.

- Descending Slowdown

*  [FIH] Slowdown:  Drop altitude = highest LZ altitude + 1000 ft.  Slowdown should be planned to stabilize on airspeed and altitude 1 min prior to TOT.  During day, climb required (500 AGL->1000 AGL.)  Call "thirty sec to slowdown, five seconds to slowdown, slowdown, slowdown, now."  PAC power idle, 1000 fpm climb.  On speed, flaps approach.  Slow to 120 KIAS.  During Night, same, but stay level.  When DZ in sight and will remain in sight, or a positive solution established and terrain clearance ensured, can descend to DZ alt.  No timing adjustments from slowdown to DZ.