The Ultimate P-3 Limits Gouge…

Airspeeds:

Vne	sea level	405 kts

8000 feet	410 kts

Flaps	up (0%/0°)	Vne

manuever (39 or 40%/10°)	275 kts

takeoff/approach (77%/18°)	190 kts

land (100%/40°)	170 kts

minimum retraction speed	140 kts

Heavy or turbulent retraction:

takeoff to manuver	160 kts

manuver to up	180 kts

Landing gear	extended	300 kts

extending (at 1 g)	300 kts

extending (> 1 g)	190 kts

retraction 	190 kts

Landing gear warning system 	can overide	153±3 kts (48° coord.)

can't overide	142±3 kts (60° coord.)

Bomb bay doors	open or close 	Vne

Internal jettison 	(all stores)	180-330 kts (at 1 g)

External jettison 	(all stores)	140-200 kts (at 1 g)

IRDS turret	extended	Vne

extending/retracting	375 kts

Overwing exit	in flight	170 kts

Main cabin door	max tested	300 kts

APU Doors	in flight	225 kts

Fuel dump	in flight	140-300 kts

Shifting RPM	on ground	50 kts maximum

Rudder effectiveness	approximately	50 -60 kts

Rotate	below 108,000 1bs.	115 kts.

Climb schedule airspeed	4 engine	220 kts to 10,000 then - 2 kts/1000 ft.

3 engine	190 kts to 20,000 then - 2 kts/1000 ft.

2 engine	180 kts

Airstart	normal	170-210 kts

Range critical	210 or above

maximum	345 - 6/1000 ft. alt kts.

NTS check	reccomended	170-190 kts (minimum 1.52 Vs) 			(climb airspeed ok)

Airspeeds (cont.):

Loiter Airspeed		Gross weight plus 90 kts

Linger Airspeed		Fuel Totalizer plus 140 kts

Max range 	configurations A,B,C	205+1/2 gross wt. - alt. (in thousands)

configurations D,E	180+1/2 gross wt. - 2 x alt. (in thous.)

Max range wind adjustment	headwind 	Increase TAS by 30% of headwind

tailwind 	Decrease TAS by 25% of tailwind

Landing lights 	extended/extending	260 kts

Max crosswind component		35 kts

Crosswind of 30 kts or more		minimum 135 kts until touchdown

12 units AOA equals		1.52 Vs flaps up

1.42 Vs flaps at manuever

1.35 Vs flaps at approach

- Vs flaps at land

Approach speeds	straight in	1.35+5 kts, min. 130 kts

circling	1.35+5 kts, min. 140 kts

No flap approach/landing	gear up	1.52 Vs, min. 160 kts

final, gear down	1.2 Vs, min. 135 kts

Two engine approach/landing	clean	1.52 Vs, min. 160 kts

final, landing config.	1.35 Vs, min. 145 kts

Land flap ditch		Power off Vs + 10 kts

(approx. 1/2 gross wt. + 63 kts)

Approach flap ditch		Land flap ditch + 5 kts

Manuever/Up flap ditch		Land flap ditch + 20 kts

Three Engine ditch (approach flaps reccomended)	1.35 Vs down to approach flap 			ditch speed with control check

Windshield heat inoperative		240 kts or less below 10,000 ft

Max speed w/ HF ant. separation		170 kts

Altitudes:

Max APU altitude	generator use	20,000 ft.

engine starting	6,000 ft.

Partial Load monitoring	single engine-driven generator	All altitudes

APU only	< 8,000 ft.

Full load monitoring	APU only	> 8,000 ft.

Minimum on-station	day/VMC	200 ft. AGL

night or IMC	300 ft. AGL

Two engine loiter	1,000 ft. AGL

RAWS warnings	three second	380 ft. ±20

continuous	170 ft. ± 10

Unreliable Radalt signals inhibited		1,000 ft. above reference alt.

NTS check		Below 8,000 ft

Cabin Press light on		10,000 ±500 ft.

Cabin Press light out		by 8,000 ft.

APU full load monitoring	activates	8,000-8,200 ft.

deactivates	7,500 ft. minimum

Maximum Airstart altitude	with JP-5/JP-8	25,000 ft.

with JP-4	35,000 ft.

Pressurization setting	below 14,000 ft	Local altitude + 800 ft.

above 14,000 ft	(Cruise alt./1000 - 14)  x 5

(Cruise alt/1000 - 11) x 6

(w/o AFC 341)

Minimum altitude for fuel dump 	(OPNAV 3710)	6000 ft. AGL

Recovery altitude	Ditching drill	4,000 ft

Vmc air demo	4,000 ft

Steep turns	5,000 ft

Stalls	10,000 ft

Power loss with boost pump failure	(fuel aeration)	13,000 ft. (approx.)

Oxygen requirements	flight crew above 10,000 ft. CA	full time

(CA = cabin altitude)	guys in back 10,000 - 13,000 ft. CA	three hours

guys in back above 13,000 ft. CA	full time

flight crew above 27,000 ft.	“immediately available”

Life vests required		< 1000 feet overwater

Bailout 	minimum reccomended	3,000 ft.

“should not be attempted below”	1,000 ft.

System Capacities:

Fire bottles 	Bromotrifloromethane	10.5 lbs. each

Fire bottle 	nitrogen charge	600 psi

Oil system 	system capacity	15.75 gal.

usable oil	8.65 gal.

foaming space	5.7 gal.

unusable oil	1.4 gal.

Fuel Tanks	1&4	1,606 gal.

2&3	1,671 gal.

tank 5 	2,646 gal.

Total	9,200 gal.

Minimum fuel for hydraulics	tanks 2 and 3	1,000 lbs.

Windshield washer tank	nose wheel well	1 gal.

Hydraulic system #1	resevoir	5.6 gal.

system	16.2 gal.

w/brake accumulator charged	5.0 gal.

w/brake accumulator charged, gear up	4.0 gal.

minimum for flight(resevoir)	4.2 gal. (charged)

4.8 gal. (not charged)

cavitation occurs at (guage mark)	2.0 gal.

Hydraulic system #2 	resevoir	1.0 gal.

system	4.5 gal.

minimum for flight	3/4 gal.

cavitation occurs at (guage mark)	3/4 gal.

Brake accumulator air charge		800 psi

Emergency Brake Pressure	maximum	3,000 psi

minimum for flight	1,900 psi

Bomb bay emergency resevoir	(empties to #1 system)	3/4 gal.

Urinal/toilet		6.5/6.0 gals

Water breakers	Head	4.5 gals

Galley	2.0 gals

Emergency	2.5 gals

EDC oil quantity	engines 2 and 3	1.0 gal.

APU oil quantity		1.0 gal.

(fill at 1/2 or below on lower window)

Prop fluid		6.2 gal.

Refer turbine		0.7 pint

Add if below 3/4 full

Air multiplier		8.45 oz.

Add if at or below 1/2 full

Oxygen System (at 70 F)	maximum (minimum for flight)	1,800 psi (1,500 psi)

Rate Measurements:

Hydraulic pump output		8 GPM (2 GPM for cooling)

Prop pump #1 output		20 quarts per minute

Prop pump #2 output		40 quarts per minute

Transfer pump rate	pump	5,000 1bs./hour

scavenge	2,500 Ibs./hour

Boost pump rate	pump	5,000 1bs./hour

scavenge	2,500 Ibs./hour

Dump rate	all pumps operating	1,000 lbs./minute

Fueling capacity	2 hoses	600 GPM (55 psi max pressure)

1 hose 	300 GPM

APU fuel usage		300 lbs./hour

Shaft Horsepower:

Max/Military/Normal rated		4600 SHP

Torquemeter “Flopover”	(inspect as soon as practical)	5300 SHP

Cruise power gouge		double GW in thousands

Minimum shaft for touch and go		2500 SHP

Decoupler actuation		-500 to -1700 SHP

Minimum SHP for NTS check		800 SHP

NTS action	engines 1 and 4	-150 to -500 SHP

engines 2 and 3	-100 to -500 SHP

Max shaft for oil temp	oil temp below 0°C	Minimum SHP

oil temp 0 to 40°C	1000 SHP Max

oil temp 40°C and rising	Max allowable

RGB absorbs		31 SHP + 0.5% of power

Accessory loading	four engines operating	75 SHP

(average per engine)	three engines operating	100 SHP

engines 2 and 3 operating	150 SHP

engines 3 and 4 operating	110 SHP

Pressures:

Oil pressure	normal RGB	130-250 psi

normal power section	50-60 psi

caution light RGB	130 psi (±5)

caution light power section	40 psi (±5)

fluctuations RGB	±10 psi

fluctuations power section	±5 psi

Hydraulic pressure	normal	2,960-3,200 psi

low press light	1,800 psi

Autopilot disconnect	1,100 psi (#1 system)

Brake accumulator air charge		800 psi

1B pump pressure		2200-2900/2950 psi

In-flight brake pressure	(when landing gear are retracted)	90-105 psi (115±5 psi max.)

Emergency Brake Pressure	maximum	3,000 psi

minimum for flight	1,900 psi

Transfer pump press low light		below 4 psi

Boost pump press low light	electrical boost pump in fuel tank	below 2 psi

Filter light	low pressure fuel filter	greater than 7.5 psi differential

Press low light	engine driven boost pump	less than 19 psi differential across pump

Normal fuel crossfeed pressure		15-30 psi

Max crossfeed pressure		50 psi

Prime cutout	fuel manifold pressure	50 psi

Fuel pressure for parrelling light		150 psi

Fuel manifold drain valves	(close at 16% RPM)	8-10 psi fuel pressure

Combustion liner drain valves	(open on shutdown)	2-4 psi air pressure

Feather pump cutout		600-800 psi

Tank 5 overpressure guage 	green	0-7 psi

(on refuleing panel)	red	7-12 psi (needle latches)

Rudder power light servo switches 	#1 system	2,000 psi open/l,500 psi close

#2 system	200 psi open/50 psi close

Oxygen System (at 70 F)	maximum	1,800 psi

minimum for flight 	1,500 psi

pressure reducer	to 65 psi

pressure relief	at 145 psi

Minimum air pressure	at 16 % on start	25 psi

Wing plenum press. relief doors	(spring loaded closed)	open at approx. 5 psi

Pressures (cont.):

Pressurization System	Isobaric range	0-13.3 in. Hg differential

Differential range	13.3-13.9 in. Hg differential

Pressure relief range	13.9-14.4 in. Hg differential

Open pyropistol port		9 in. Hg differential or less

Air-cycle cooling system 	Green	0-2.4 in. Hg differential

Ice-limiting system:	Yellow	2.4-4.0 in. Hg differential

Red	> 4.0 in. Hg differential

EDC pressure ratio limiting:	Green 	> 72 psi EDC oil pressure

Yellow	50-72 psi EDCoil pressure

Red	0-50 psi EDC oil pressure

EDC Press Low light	EDC oil pressure	< 50 psi

Normal tire pressure		150 psi

Temperatures:

Max rated power		1077° C (5 minutes)

Military rated power		1049° C (30 minutes)

Normal rated power		1010° C (continous)

TIT guage red light		1080° C (±2)

TIT in Low RPM		410-730° C

TIT in Normal RPM		445-645° C

Maximum Reverse		580-790° C

Flight Idle		430-705° C

Min. TIT w/TD null, max power		1007° C

Starts	hot start above	830° C

continue and record	830-850° C

discontinue and reattempt	850-965° C

discontinue and inspect	> 965° C (or torch)

Restart residual TIT	Ground	200° C

In-flight	150° C

Using prime	100° C

TD temperature ranges	Start limiting (put 0%/take 50%)	830° C

Temp limiting (put 0%/take 20%)	1077° C (+6/-10)

Temp contolling (put 15%/take 20%)	819° C at 66° coord.

1077° C(+6/-10) at 90° coord.

minimum scheduled temperture 	566°C (0-43° coord.)

Engine oil hot light		100° C

Oil temperature	Normal	60-90° C

Max in ground range	100° C (30 minutes)

Max in flight range	100° C (5 minutes)

Min. for start	-40° C w/mil 23699 oil

-54° C w/mil 7808 oil

Min. for takeoff	40° C and rising w/normal oil press

Min. inflight	50° C in cold temps.

Minimum SHP	0° C and below

1,000 shp max	0-40° C

Max power	40° C and rising

Fuel pumps auto shutdown	pump case temperature	400° F (transfer and boost pumps)

Fuel temp 	(heater strainer)	40-70° F

Hydraulic oil hot light		175° F (177° F?)

Hydraulic pump auto shutoff	pump case temperature	380° F

EDC temp high light 	EDC oil temperature	121° C

Tire thermal fuses activate at		145° C

Bomb bay cold light on		-3 to +2° C

Bomb bay hot light on		54 to 60° C

Bomb bay normal range 	(cycling thermostat)	+3 to +7° C	..

Leading Edge Hot Light		110° C

TIT rise when wing de-ice on		10° C min. (in 20-30 sec.)

Wing Hot light	Outboard sensors	104° C

lights extinguish at	96° C

Inboard/Center sensors	88° C

lights extinguish at	79° C

Windshield heat		90-95° F

Rack overheat light	On	135° F (±3)

Off	128° F

T/R overheat light		150° C

Air conditioning temp range in auto		18-29° C (65-85° F)

Min. air conditioning duct temp.		5° C

Min. electronic equip. usage above		27° C cabin temp.

Radiant floor and wall heat		69-80° F

No loiter below		10° C and visible moisture

Loiter not reccomended below		0° C for prop life

Engine anti-ice for takeoff		< 8° C and heavy moisture

Engine anti-ice in flight		Temps near icing range and

visible moisture

APU EGT limits	-2 APU	688°C continous

710° C max (10 seconds)

715° C max peak

-3 APU	649° C continous

704° C max peak

APU fire detection		400° F

Engine fire detection	Zone 1 Turbine section	600° F

Zone 2 Compressor/Intrconnect	400° F

Zone 3 Power section accessories	400° F

RPM measurements:

Normal mech. prop governing		99-101% RPM

Ground Start normal RPM		96.3-99.1% RPM

Ground start low RPM		71.0-73.8% RPM

RPM in max reverse		96.7-104.9% RPM

RPM at flight idle		94.5-98.4% RPM

Press prime button by		16% RPM

Lightoff RPM	normal range	16-33% RPM or max motoring

nominal	24% RPM

Fuel Pump Parallel light	normally on	16-65% RPM

nominal	35%

EDC press low lights out	normal	16-65% RPM

nominal	35% RPM

Oil pressure	must be indicated by	35% RPM

Stalled or Stagnated start	RPM hangs between	35-50% RPM

Prop pump lights out	ground start	By low RPM

inflight restart	By 55% RPM

Start button pops out at		57-64% RPM

Speed sensitive control switches		16,65,94% RPM

Speed sense valve switch	(closes 5th and 10th stage bleed air)	94% RPM

Mechanical feather		0-10% RPM

Speed bias motor range		-4% to +6% (96-106%) RPM

Speed bias lock out (new system)	disables speed bias motor	2.5% or greater change signal

(requires ground maintenance)

Resync (old syncs)		±2% RPM per application

Master Trim knob (2 or 3)		±1% RPM

Overspeed	(must be audible)	1% greater than mech. governing

Underspeed	(must be audible)	1% less than mech. governing

Pitchlock actuation		103.5% RPM or loss of control pressure

Pitchlock reset		109% RPM

Fuel topping governor		104.2 to 106.7% RPM

Fuel Control minimum RPM		97% RPM

Fuel goveror test switch 	resets onspeed to	106% RPM

Prop brake	Applied	< 21% RPM

Locked	Backward rotation

Released	0-21% RPM (starter torque)

Released	21 - 100% RPM (oil pressure)

Temperature Datum System	Start limiting	< 94% RPM

Start limiting to Temp limiting	94% RPM

Temp limiting	>94% RPM (< 66° coord.)

Temp controlling 	>94% RPM (> 66° coord.)

APU RPM limits	fuel and ignition on	10% RPM

starter disengages	35% RPM

ignitors off	95% RPM

ground limit	98-102% RPM

inflight limit	98-103% RPM

auto shutdown (overspeed)	106% RPM

Coordinator Positions: (BA=blade Angle)

Ground range		0-34° coord.

Flight range		34-90° coord.

Ground start		15° coord  (0° BA)

Flight Idle		32°/34° coord.

Flight start		48° coord

Max reverse		0° coord. (-13° BA)

Ground idle		9° coord. (-6° BA)

NTS cammed out below		24° coord

Pictlock Reset		< 28° coord. (> 10° BA)

Backup valve open		< 28° coord. (> 24° BA)

Oil cooler inducing available		< 32° coord.

Flight idle stops retract at		32° coord.

Prop sync available above		32° coord.

Low RPM available below		36° coord.

Landing gear warning system 	can overide	48° coord. (153±3 kts)

can't overide	60° coord. (142±3 kts)

Autofeather armed above		60°±2 coord.

Feather (E-handle pulled)		100°

Beta Followup Schedule	< 68° coord.	minimum 11° BA

78° coord.	minimum 17° BA

88° coord.	minimum 22.5° BA

Blade Angle Positions: (BA=blade angle)

Blade angle range		-13° to 86.65° BA

Feathered		86.65° BA

Ground start		0° BA (15° coord.)

Beta light		< 10° BA

Max reverse		-13° BA (0° coord.)

Ground idle		-6° BA (9° coord.)

Pitchlock available		17° to 57° BA

Pictlock Reset		> 10° BA (< 28° coord.)

Low Pitch Stop		13° BA

Backup valve closed		< 24° BA (> 28° coord.)

Airstart switch		45° BA (NTS inop light)

Beta Followup Schedule	< 68° coord.	minimum 11° BA

78° coord.	minimum 17° BA

88° coord.	minimum 22.5° BA

Weights:

Max takeoff weight		135,000 lbs.

Max overweight takeoff		139,760 w/CO's permission

Max landing		114,000 lbs.

Max normal landing		103,880 lbs.

Landings over 103,880 lbs. 	(before inspection required)	10

Max ZFW	normal cofig.	77,200 lbs.

foam tanks	78,015 lbs.

Max fuel tank differential	takeoff 1&4	1400 lbs.

inflight/land 1&4	4389 lbs.

takeoff 2&3	5025 lbs.

inflight/land 2&3	n/a

Electical System:

Generator frequency range	normal	375-435 Hz (±5 Hz)

at 109% RPM	432 Hz

Generator overvoltage trip	can use “reset”	129 volts

Generator undervoltage trip	engages lockout relay (must pull CB)	105 volts

Feeder fault lockout	difference between gen. and sup. panel	35 amps or more

New sup. panel specs.	undervoltage delay	4 sec. ±1

underfrequency dropout	367 Hz ±3

generator pickup	383 Hz ±3

APU full load monitoring	activates	8,000-8,200 ft.

deactivates	7,500 ft. minimum

TR voltage range	no load	29.5 volts

full load	26.0 volts

TR power rating	TR’s 1 and 2	200 amps

TR 3	150 amps (due to CB)

Fuel system:

Dipstick after refueling		3 minutes or more

Min. wing fuel for dip 	Tank 1&4	8775 lbs./1350 gals

Tank 2&3	4225 lbs./650 gals

Primary fuel pump output exceeds secondary by	10%

Fuel control provides TD with		120% of required fuel

TD in null bypasses		20% of fuel

Hydrostatic tester calibrated at		0° roll, 1° nose down

Crossover pipe tank 5 fuel level		 4000 lbs.

Min. fuel for flight	w/dip and fuel log	6,000 lbs. (NATOPS)

VP-30	8,000 lbs., symmetric

Engine system (including oil):

100% RPM equals	engine RPM	13,820 RPM

prop RPM	1020 RPM

Gearbox reduction ratio		13.54 to 1

Compressor air for cooling		75%

Compressor air for combustion		25%

Starting time to low RPM		60 secs. max, 30-45 normal

Horsepower decrease with engine anti-ice	9%

Fuel Flow increase to make up lost SHP	5%

Fuel use increase w/engine and wing deice	500 pounds/hour

Check oil quantity w/in		30 minutes of shutdown

Max starter rotation		60 seconds

RPM stabilized after initial overshoot	w/in 10 seconds

Airframe/Hydraulic System:

Flap positions	up	0% extension, 0° deflection

maneuver	39 or 40% extension, 10° deflection

takeofflapp.	77 or 78% extension, 18° defection

land	100% extension, 40° deflection

Flaps from land to up		20 seconds max

Emergency brake applications		3 full, 6 partial

Accumulator brake applications		8

Time to fill brake accumulator	#1,#1A pump	3 seconds

#1B pump	3 minutes

Accumulator stays charged 	(parking brakes)	30 hours

Inflight brakes stop tire rotation w/in	4 seconds after gear sel. up

Nosewheel steering range		67° left or right

Bank angle	coordinated turn	70° max (3 G limit)

maneuvering 	65° max

Max angle of bank w/sim. 2 eng. failure	45°

Acceleration Limits	flaps up	+3.0/-1.0

(symetric flight)	maneuver flaps	+3.0/-0.0

approach flaps	+2.0/-0.0

land flaps	+2.0/-0.0

over 135,000 lbs./flaps up	+2.5/-0.5

over max ZFW	+2.1

(non-symetric flight/roll maneuver)	flaps up	+2.4/-0.0

maneuver flaps	+2.4/-0.0

approach flaps	+1.0/+1.0

land flaps	+1.0/+1.0

over 135,000 lbs./flaps up	+2.0/-0.0

Pivot distances 	Tail	75'5"

(locked inboard main)	Wingtip	65'

Nose	48'

Outboard prop tip clearance		at least 30 inches

Max bank angle at touchdown	(or outboard prop will strike runway)	9°

Pressurization/Environmental/Foul Weather Systems:

Output of EDC	#2	33% flight station/66% cabin

#3	100% cabin

EDC disconnect		Normal RPM only

A/C programmer drives full hot in auto w/in	90 seconds

Max time at full hot /manual mode		1 minute

Max time to stabilize temp. in auto mode	30 minutes

02 system provides	(for flight crew)	3.5 hours at 25,000 ft.

02 in normal	equals 100% O2 at	30,000 ft.

Portable bottles	No exertion	22 minutes

(100% setting)	Moderate to heavy effort	5-10 minutes

Warmup front windshield in low 		for 10 minutes before high

Time for side windshield heat effectiveness	45 minutes

Low windshield heat		40% of high

Prop deice	normal cycle length	160 seconds

test cycle length	160 seconds

cuffs/aft spinners and islands	20 seconds each

front spinner amps (test)	76-100 (74-100)

cuff amps (test)	51-72 (48-72)

aft spinners/islands amps (test)	63-82 (58-82)

Emp deice	normal cycle length	176 seconds

test cycle length	44 seconds

# of parting strips	11

# of cycling strips	20

cycling strips(normal)	8 seconds each

cycling strips(test)	2 seconds each

parting strips(normal)	continous

parting strips(test on ground)	2 seconds

parting strips(test inflight)	continous (readout in 1st  2 sec.)

deadband (normal)	8 seconds (beginning and end)

deadband (test)	2 seconds

overheat warning	104° C ±6

overheat warning extinguishes	82° C ±6

Flight Instruments:

Airspeed split pilot vs. copilot		5 kts max

Heading split between HSI's	(INS 1 vs. INS 2)	2° degrees max

Heading split HSI vs. wet compass		5° degrees max

Min. battery volts to start the APU		22 volts (NATOPS)/18 actual

Inertial time on battery pwr.		15 minutes (approx.)

Standby attitude indicator w/loss of power	9 minutes (approx.)

Emergency Equipment:

Emergency exit lights		1.5 lateral G's auto on

Life vests required	Flight Crew	all times overwater

Guys in back	< 1,000 feet overwater

Takeoff and landing overwater

When deemed necessary b y PPC

Mk 124 day/night flare		20 second duration

Strobe light		40-60 flashes per minute

8 hour life

NB-8 parachute		28 foot canopy

18 ft/sec descent rate

PRT-5	frequencies	8364 kHz and 243.0 MHz

(beacon only)	lifespan	72 hours at 25° C

Antenna	HF-9 feet, UHF-12 inches

Range	160 nm at 30,000 ft.

PRC-90-2	freqs.	282.8 and 243.0 mhz

(transceiver/beacon)	voice capable	both freqs.

beacon	243.0

lifespan	18 hours (8.5 hours w/high power)

EEBD	moderate to heavy work	15 minutes

Limits that defy all other catagories:

APU intake exhaust doors close w/in	1 minute

Landing lights w/out cooling air		30 seconds max

Autofeather actuates with less than		500 lbs. of thrust

# of electrical outlets		46

Safe radar distance: people	APS 115	75 feet

APS 137	250feet

HERO ordance/fueling	APS115	140feet

APS 137	250feet

Howgozit required		1000 nm from nearest suitable

Fuel log required 		flight over 6 hours

>500 nm to nearest suitable field

Circling mins give 300 feet 	Cat C	1.7 miles of runway

of terrain separation:	Cat D	2.3 miles of runway

Headwind/ tailwind affects landing rollout	± 1 % per knot

Extra 10 knots at touch down increases roll	1000 ft

Step Climb 2000 ft. 	when TIT decreases to:	900° C TIT w/tailwind

890° C TIT w/ no wind

880° C TIT w/ headwind

Immediate ditch		1 long ring

Prepare to bailout		4 short rings

Bailout		1 long ring (>10 seconds)

Reverse with blown tire		power lever at ground idle

Windshear indications	airspeed change	± 10 kts or more

vertical speed change	± 500 fpm or more

Windshear recovery attitude		10° nose up, disregard airspeed

Power Sources (alphabetical):

| Aux. vent |  | Mon ac/Mon dc |
| --- | --- | --- |
| Bleed air valves |  | Start dc |
| Bomb bay contol |  | Ext main dc |
| Bomb bay heat |  | Ext main dc |
| Bus A&B control cb's |  | mon dc |
| Chip detectors |  | Mon dc |
| Command bell |  | flight dc |
| Crossfeed/crosship valve |  | Mon ac |
| Dump pump |  | Bus A/main dc |
| Dump valve |  | Main dc |
| Emergency brake pressure guage |  | Inst bus #2 |
| FDI (pilot & copilot) |  | Mon ac/Flt ac backup |
| Feather pumps | 1&4 | Bus B/Mon dc (Bus A alt.) |
|  | 2&3 | Bus A/ Ext. main dc |
| Fire detection (light, horn, control) |  | Mon dc |
| Fire extinguishing | normal | Mon dc |
|  | altemate | Ext main dc |
| Flap position indicator |  | Inst. bus #1 |
| Fuel boost pumps | 1&3 | Bus A/Ext. main dc |
|  | 2&4 | Bus B/Ext. main dc |
| Fuel pressure guage |  | Inst. bus #2 |
| Fuel quantify guages | Flight station | Mon ac |
|  | Fueling panel | Bus A (1 CB for all) |
| Fuel quantity test |  | Ext main dc |
| Fuel transfer/fueling valves |  | Main dc |
| Fueling panel control relay |  | Main dc |
| Fuselage (crooship) bleed air valve |  | Start dc |
| Gen 4 aux ctrl cb | Transfer relay #7 | flight dc |
| Gen control CB’s | gen 2,3,4 | mon dc |
|  | APU gen | GOB |
| GOB relay CB |  | flight dc |
| Ground/air sensing CB |  | main dc |
| Hydraulic pressure #2 system |  | Inst bus #2 |
| Hydraulic pressure #1 system |  | Inst bus #1 |
| Hydraulic pumps | #1 | Bus A/ Ext. main dc |
|  | #1A | Bus B/ Mon. dc |
|  | #2 | Bus B/Ext. main dc |
|  | #1B | GOB |
| Hydraulic quantiy guage |  | Ext main dc |
| IFF |  | flight ac/mon dc/flight dc |
| Inverter power cb |  | start dc |
| Inverter relay cb |  | flight dc |
| Landing gear control valve |  | Ext main dc |
| Landing gear handle light |  | Ext main dc |
| Landing gear posit. indicator |  | Ext main dc |
| Leading edge temp guage |  | Mon ac |
| Main tank valves |  | Mon ac |
| Normal brake pressure guage |  | Inst bus #1 |
| Oil pressure gauges | 2&3 | Inst. bus #2 |
|  | 1&4 | Inst. bus #1 |
| Oil quantity gauges | ground test | GOB |
|  | normal | Ext main dc |
| Oil temperature guages |  | Ext main dc |
| Outflow valve |  | Flight ac |
| Pilot/ copilot turn needles |  | Mon dc (pilots backed up by Flt dc w/ ess. bus monitor switch off) |
| Pitchlock reset solenoid |  | flight dc |
| Power sensing cb |  | mon dc |
| Rack overheat detection/waming |  | Ext main dc |
| Radiant floor/ wall heat |  | Bus B |
| Red instrument lights (pilot/copilot) |  | Mon ac/Flt ac backup |
| Standby att. Indicator (peanut gyro) |  | Flt ac |
| Transfer pump | forward | Bus A/Ext. main dc |
|  | aft | Bus B/Ext. main dc |
| Vertical gyro (standby gyro) |  | Mon ac/Flt ac backup |
| WHEELS warning light/horn |  | Mon dc |
| Windshield washer pump |  | Mon ac |
| Windshield wipers |  | Mon ac |

The data contained herein carries no guarantee either expressed or implied. Offer not valid in all areas. License, tax, title and destination fees extra. Enter as often as you like. See store for details. All numbers and figures come from either the good book of NATOPS or the February 1998 edition of the Job Aid published by VP-30. If you find this data useful, please pass a copy along to a fellow aviator. The editor would like to thank the authors of the Constitution and the designers at Lockheed, Allison, and Hamilton Standard.

°±

08/31/98