NVG STUDY SHEET

NVG’s do not turn night into day, they allow you to operate more safely in the night low-level environment.  Use of NVGs allows us to see objects at night that could not be seen during unaided operations.  They do not replace proper planning and good judgment.

# Visual process-

Light energy enters the cornea and passes through the pupil to be focused onto the retina.

Cone Cells:  Used for day vision and are found mostly in the central part of the retina, called the fovea.    Used for resolution and color vision, therefore requiring high levels of energy.

Rod Cells:  Used for low level vision and located mostly outside the fovea.  Create a gray image.

Blind Spot: Created by the lack of rod cells in the fovea combined with low level environments.  This is why light sources in the dark must be viewed using off centered vision.

Components of Vision:

Focal Vision:  Limited to the central two degrees of vision and is a conscious function.  It is used to see clearly in order to recognize objects and read displays. (Decipher task oriented information)  It is a slow process and although it is not used for orientation, it can be used to acquire visual information about orientation.

AmbientVision:  Also called peripheral vision.  It is a subconscious function used primarily to orient one self in the environment.  (Maintain spatial orientation)

Dark adaptation:  Rod cells require rhodopsin(visual purple) in order for this process to occur.  For most people, the sensitivity of the eye increases 10000 fold during the first 30 mins, with little increase after that.

Length of exposure to light is one variable that determines the time it takes for adaptation.  Usually adaptation is fully achieved in 30-45 mins.

Impairment due to exposure to light depends on intensity and duration of exposure.  Usually, exposure to bright light for one second can seriously impair night vision.

NVG image is perceived in shades of green and not too bright, therefore you are using both cone cells and rode cells to see the image.  You are in an intermediate state of dark adaptation, and it usually take 5-8 mins to regain full dark adaptation.

## Types of vision

Photopic Vision:  Defines the visual capability during periods of high illumination.   Visual perception is achieved by cone cells mainly, due to the bleaching effect on rod cells.  The result is a sharp color image.  Objects are primarily viewed with focal vision but can be detected with ambient vision.

Scotoic Vision:  Defines visual capability during periods of low illumination, when the cones in the fovea are not being stimulated.  The result is a poor resolution on the order of 20/200 or less and no available color.  Due to the blind spot, scanning is required to locate objects and offset vision to keep them in sight.

Mesopic:  Intermediate stage between photopic and scotopic.  Usually viewed at dawn, dusk, and NVG operations.  Both cones and rods are used.

Spatial orientation.

More time and effort is required to maintain spatial orientation during NVG operations  due to the required input from focal  and ambient vision.

Focal vision is required to interpret the NVG image.  Also, due to the goggles reduced field of view and the lack of visual cues in the periphery, more reliance is place on focal vision during NVG ops.  All this adds up to more time and effort required to maintain spatial orientation during NVG ops.

Spatial disorientation:

The degraded visual environment combined with the high task loading required during NVG ops adversely affect focal vision and contribute to the onset of spatial disorientation.

Properties of NRG:

The human eye is sensitive to the visible spectrum (400-700nm). NVGs are designed to be sensitive to both visible and near IR wavelengths.  Thermal imaging (FLIR) are sensitive to energy in the mid and far IR regions.

Illuminance:  Amount of light which strikes and object form a source.

Luminance: The amount of light being reflected from a surface.

Albedo:  Ratio between illuminance and luminance.

Note:  The light source provides illuminace but what our eyes see and NVGs intensify is luminance.

Contrast:  Measure of luminance difference between surfaces.  At night, contrast depends on the albedo values of  each surface.

Shadows:  Direction depends on position of the light source, while length depends on the angle of the source and the height of the object.  They can completely hide obstructions such as ridgelines, towers and may make it hard to identify LZs.

Foreshadowing:  Near objects are masked by the shadow of further and higher objects.

Moon:

Illumination from the moon is caused by absorption of energy as it travels through the atmosphere.  At low moon angles there is more atmosphere for the energy to penetrate and more energy absorption occurs, therefore illumination is lowest. Particulates like water, rain, dust will also have an effect.

Phases of the moon:

New Moon:  0 – ¼  (8 days)  Relative low light level

First quarter (waxing):  ¼ -3/4  (7 days)  Good illumination is provided

Full Moon:  > ¾ (8 days)

Third Quarter(Waning) ¾ - ¼ (7 Days)   Good illumination but less than First quarter

# Stars

Provide 20% of the sky Illuminance on a moonless night yet their contribution is mainly in the form of IR NRG.

# Solar Light

Sky glow is ambient light from the sun present up to 1 ½ hours after sunset and ½ prior to sunrise.  Adversely affects NVG operations by affecting the gain of the goggle and therefore reducing the image.

# Other background illumination

The greater portion of energy in the night originates in the upper atmosphere and is produced by a ionization processes.  (40%)

Night Vision Goggle Characteristics

Air force NVG are binocular-style, helmet mounted, image intensification devices that amplify visible and near IR NRG.  It is a passive process, hence no emissions are created by the goggles.

Image Intensifier Tubes:

Objective Lens: Optical elements that focus incoming photons of light onto a photocathode.  Image is inverted in this process.

Minus Blue filter:  Coated onto the inside of the objective lens, preventing certain wavelengths from entering the intensification process.  This allows the use of properly filtered cockpit lighting to aid the pilot in viewing the cockpit instruments with unaided

vision underneath the goggles.

Photocathode:  A layered coating of gallium arsenide which releases electrons when hit by photons,

Microchannel Plate:  Thin wafer containing a million glass tubules that channel the electrons.  The tube walls are coated with a lead compound which release more electrons when impacted by incoming electrons.  Tubules are tilted to ensure impact, ensuring thousands of exiting electrons per electron that enters.

Phosphor Screen Applied to the output fiber optic system.  It is a layer that emits energy in the visible spectrum when struck by electrons.  Due to the type of lens, the resulting image is green.

Fiberoptic inverter:  Reorients the image that was inverted by the objective lens

Diopter Lens:  Final optical element adjusted for focusing the image onto the retina.

Types of tubes:

Type I (direct view):   Present an image directly from independent image intensifier tubes to each eye.  Optical performance is better than the indirect system and therefore used by the Air Force.

Type II (indirect view) Image form each tube is focused and reflected onto combiner positioned in front of each eye.  The combiner filters some of the NVG image to allow the pilot to look through and see other items of interest(IE HUD) Used by navy/MC.

NVG Characteristics:

Gain:  Ratio of output to input.  More clearly put, the amount of energy being intensified when compared to the energy that entered the process.   NVGs have circuitry that determine the amount of energy entering the process, then control the level of intensification needed to produce images of consistent brightness over different levels of illumination. At some point the maximum gain value is reached  and image quality  begins to degrade.

Image:  Monochromatic (One color…GREEN)

Visual performance:   Vastly superior to the human eyes performance under night conditions but limited when compared to daylight performance of the human eye.

Visual limitations
Field of View-  40 degree field of view.  Less than the eyes normal 120 by 80 degree FOV.  This loss of peripheral vision can contribute to the onset of illusions.

Resolution:  20/25 to 20/40 Snellen. (this is the best under optimum conditions)   Much better than the eyes 20/200- 20/400 with the naked eye at night.

Note:  NVGs will not correct for sigh deficiencies such as myopia and astigmatism.  If you wear glasses during the day, you will have to wear them at with NVGs.

Binocular Cues:  Binocular cues are needed for close tasks.  (within arm distances to 200 meters)  They require use of both eyes functioning together and include stereopsis, vergence and accommodation.  When using NVGs to asses information at these distances capability is seriously degraded due to the goggle.

Monocular cues: Do not require the coordination of both eyes, and are available at further distances than binocular cues.  They are therefore most important for deriving distance information while flying.  NVG adversely affect monocular cues by loss of sharp contrast and definition, both of these needed for determining depth and distance.   Examples of monocular cues include size constancy, motion parallax and linear perspective.

The norm is to try to fly lower or closer in order to see more detail.  Also, you must know that learned techniques may not transfer to a new area where terrain and objects vary completely in size and perspective.  In general, there is a tendency to overestimate how well you can see with NVGs

Contrast: Also degraded during NVG ops.  Any bright light soured within or near the NVGs FOV will further reduce contrast by reducing gain, creating a veiling glare.   There exists differences in sensitivity to contrast among crews, leading to differences in image interpretation.

Dynamic Visual Cues:  Provide information that helps to determine direction, altitude and speed.

Static Cue Motion:  summed effect of change in one or more static cues caused by aircraft motion. It includes elevation, known size and perspective.  Central vision tracking is a method for seeing these cues and are therefore degraded under NVGs.

Optical Flow:  Angular rate and direction of movement of objects as a result of aircraft velocity measured relative to the aviator’s eye point, necessary to determine speed and direction of motion.   Central vision is again used for these cues, and therefore degraded with NVGs.

Peripheral Vision Motion: Subconscious method of detecting optical flow.  This cue is severely degraded and central vision tracking becomes the primary attitude detection means.   This leads to flying at lower than expected attitudes, due to the lack of “speed rush” from the ground.

NVG  Scan:  Due to the reduction in FOV, an active and aggressive scan is needed along with an flight instrument analysis, in order to create a “real” picture of the environment.

Preflight adjustment and Assessment:

A small error in adjustment can significantly degrade NVG visual acuity. This problem is compounded by the fact that is nearly impossible to detect loss of visual acuity without a controlled test environment, meaning you can lose visual performance without realizing it.

Tube Alignment: Binocular fusion is when the mind subconsciously fuses the images of both eyes.  To ensure proper tube alignment you should  be able to see a complete circle independently and an image close to a circle with both eyes.  Eye strain, fatigue, disorientation or nausea can occur if NVG tube alignment errors are significant.

Assessment of Visual Acuity:  Should always be at least as good as the vision of the best image from one of the tubes alone.

Image Defects- For each defect, if the problem is not fixed or impairs the function of NVGs, do not hesitate to turn them in for maintenance.

Shading:  Dark are appears along the edge of the image and a full image can not be obtained.  Attempt to eliminate shading by readjusting either the tilt or interpupillary distance, or by shifting the helmet’s position.  This may also be caused dropping of the goggles

Edge glow: A bright area along the outer edge of the image.  It can result form an incompatible light source outside the goggle FOV, a shift in the microchannel plate, or a power supply problem within the tube assembly.

Honeycomb:  This is a result of manufacturing process during which the fiber optic inverter is assembled within the tube.  If faint, it will not affect NVG performance.

Bright spots:  The result of irregular emission points on the photocathode, usually occurring during the manufacturing process

Dark spots:  Simply where the bright spots used to be and corrected by the manufacturing facility, where they burned out part of the photocathode.  May also be caused by material allowed to enter the system during maintenance

Distortion:   Bending and shear distortion may occur.  Discontinue use of NVGs if distortion interferes with normal operations.  Flying with tube distortion can cause problems in distance and altitude estimations

Scintilliation:  Sparkling effects occurring during low illumination conditions as a result of increased goggle gain and system noise.  It can be an indication of decreasing illumination caused by deteriorating weather conditions or shadows.

## Factors affecting NVG Ops

Cockpit lighting:   NVG compatible lighting allows the crew member to see cockpit instrument underneath the NVG while not measurably affecting NVG performance.  Just because a light is green or blue does not mean it is compatible due to the significant amount of near-IR NRG being released by the filament.  Bulbs must be modified in order to make them NVG compatible.  Also, turning down the brightness of incompatible lightning will also not make them compatible.  More important, an incompatible light does not have to be within the NVG FOV for it to have an effect on the gain.

Transparency Transmissivity:  Degradation caused by windscreens, canopies and items as such, that trap near IR wavelengths.

Weather and visibility factors: Any atmospheric condition which absorbs, scatters, or refracts illumination will effectively reduce the usable NRG available to the NVG.

Clouds:  In general, NVGs easily see clouds that are dense but may not see less dense clouds.  It is possible to be invisible to thin and wispy clouds that are normally seen during the day.  This is because they are less dense, may be low level and set in against the terrain.  Very low or very high illumination may also help disguise these clouds.  The invisibility of clouds is dangerous because the cloud reduces luminance, therefore reducing scene contrast and texture, and producing a false perception of distance.  Also, the cloud may start to get thicker, luring the pilot into IMC conditions

Fog: Similar to clouds but being that fog tends to be less dense, it can be more of a problem.

Rain: Droplet size and density are key ingredients to its visibility or invisibility. Light rain or mist may not be seen with NVGs, but will affect contrast, distance estimation and depth perception.

Snow: Generally large in comparison to the wavelength  of visible light and IR, easily blocking or scattering those wavelengths.    On the ground and in mountainous terrain, snow may add contrast. In flatter terrain snow may reduce contrast.  On low illumination nights snow may reflect energy better than the terrain it covers, increasing the level of illumination.  Fresh, wet snow reflects approx. 85% of the energy reaching it, thus providing the best natural reflectivity of any terrain.

Note:  All atmospheric conditions can reduce illumination levels and recognition of this reduction is very difficult to detect.  The change is usually very subtle.   Common cues include loss of  celestial lights, loss of ground lights, reduced contrast, reduced depth perception or distance cues, reduced acuity or resolution, increased graininess or scintilliation, and a more pronounced halo effect around incompatible light sources outside the aircraft.  Also, the visual acuity achieved in the aircraft will always be less than the one achieved in the eye lane.  It is therefore imperative that you maximize NVG performance before flight and avoid doing anything to the goggle during flight to disrupt it.

Circadian Rhythm:  has a duration of approximately 25 hours.  You will never be as good at night as you are in the day because physiological functions are at their ebb.  A person’s lowest performance usually occurs between 0300-0600, called the circadian trough.

Prevention includes proper rest, proficiency and motivation.

## Fatigue

Acute: Tiredness or exhaustion experienced after a demanding mental or physical activity.  It is short-term and will usually be relieved by rest.

Cumulative: Less intense than acute fatigue and occurs over time as a result of inadequate rest or heavy workload.  It usually takes more than one night’s rest to recover to normal energy levels.

Circadian:  Tiredness produced by shifting the cycle.

Effects of fatigue:

Fatigue is similar to hypoxia.  Performance is subtly eroded, recognition is difficult and there is an unwillingness to do anything about it.

Loss of SA:  Computational and abstraction skills become degraded.  The ability to restructure the parts of the night scene are slowed, impacting the time required to interpret instruments and displays.

Channelized Attention:   Attention span and vigilance are reduced, important elements are overlooked and scanning patterns break down, due to fixation.  Fatigue usually results in errors caused by omission of a task as opposed to performing the task correctly

Complacency:  Allows for acceptance of situation that would not normally be permitted   Critical but routine tasks are often skipped because fatigue reduces overall willingness to respond.

Unrecognized Spatial disorientation: Due to short-term memory seriously being affected by fatigue, a crewmember may not recognize a bad situation or attempt corrective action in times needing complex and swift decisions

Coping with Fatigue:  Night missions can combine all three types of fatigue.  The effects are synergistic and not simply additive.  The easiest way to overcome or delay the effects of fatigue is through task familiarity and motivation.

Night Operations:

The NVG environment is always changing, so you must always be aware of what cues are presented and work to interpret them.  Even then, beware of the potential misperceptions or illusions in an NVG scene

Terrain interpretation:  Due to the variability of weather, the illumination level, and the moon angle, any given scene may look radically different on consecutive nights.

Terrain Albedo:  Difference in terrain albedo will greatly influence luminance.  Albedo will also vary with specific conditions of terrain even though the type is the same. (IE, wet vs dry)

Terrain Contrast:  Measure of the difference in reflectivity between surfaces.  The greater the contrast, the more normal the scene appears in NVG.

Terrain Shadows: The amount of terrain obscuration within a shaded are is dependent on the amount of ambient illumination and relative position of the moon. Never plan on seeing any terrain features within shadows, regardless of the moon size disc.

NVG Misperceptions and illusions:

Reduced resolution, limited field of view, and susceptibility to obscurants can intensify misperceptions and illusions.  These include:

Depth perception and distance estimation errors:  Depth perception and distance estimation DO exist when using NVG, but they are degraded by environment conditions and goggle limitations.  Most helpful cues are those with which the aircrew is most familiar. When flying over different terrain with unfamiliar features, serious errors in DP and DE can develop.  Overall, the best way to train for the lack of DP and DE cueing is through proper planning, training, and a good discussion of difference between the deployed location and the area of operation.  Also, looking at the source with unaided vision might help in determining color and eliminates the halo effect produced by the NVGs.

Terrain Contour misperceptions:  Exaggerated by anything that degrades NVG image.

Discriminating between near and distant terrain:  Pay attention to motion parallax between two objects

Gradual elevation change:  Can be difficult to detect when terrain is in low contrast.  An aggressive scan with NVGs, along with an aggressive instrument scan can help fight this misperception

Maintain scene detail:  Do not fly lower in a an attempt to regain lost detail.

Undetected or  illusory motion:  Motion illusion experienced by aircrews are usually due to flights over areas of reduced contrast, or a sudden loss of contrast and flow cues.  For example, helicopter crews hovering over low contrast terrain, (large field, or open water) can actually be moving at high speeds without knowing it.  Once again, a vigorous instrument scan can help alleviate this situation.

Note:  The best way to accomplish visual acuity is through proper preflight adjustment and assessment of the goggles, ensuring the best NVG image.  In-flight attentiveness, using all available resources and not relying solely on NVGs will help tremendously.

Emergency Situations:  If  the NVGs will not be useful during EPs, consider removing them.  It is also probable that you will forget you are wearing them under a highly stressful situation.

Inadvertent IMC:  One of the most dangerous situation experienced with NVGs, due to the inability of NVGs to see various areas of moisture and luring the pilot into an environment with no visual information.  The following cues are key to impending IMC.

Halos surrounding incompatible light sources outside the cockpit may change in appearance. Normally sharp edges to the halo can become less distinct and the halo may appear larger due to energy dispersion form the moisture.

A gradual loss of scene detail, visual acuity, or terrain contrast.

Partial or complete obscuration of the moon and stars.

An increase in Scintilliation

The glow or flash from your aircraft external lights may become visible or intensify.

Spatial Disorientation:  Although NVGs usually improve situation awareness and reduce the possibility of spatial disorientation, they can also enhance momentary disorientation, due to the limited FOV and resolution.  Constant vigilance and a good scan pattern, both inside and outside, will help prevents Spatial D.  If you feel disoriented, act the same way as if you were in a non-NVG flight.