# NOSE LANDING GEAR INDICATORS

## NOSE GEAR POSITION INDICATORS

- UP INDICATES DOOR CLOSE RELAYS AND DOOR LOCK RELAYS ARE IN THE CLOSED POSITION

- BARBER POLE INDICATES NOSE GEAR DOORS ARE UNLOCKED AND NOT FULLY OPEN

- RED WHEELS INDICATES THE DOORS ARE FULLY OPEN AND THE NOSE GEAR IS NOT DOWN AND LOCKED

- GREEN WHEELS INDICATES THE NOSE GEAR IS DOWN AND THE DOWNLOCK IS ENGAGED

MAIN GEAR POSITION INDICATORS

- UP INDICATES THE GEAR IS UP, THE DOOR CLOSE RELAYS AND THE DOOR LOCK RELAYS ARE IN THE CLOSED AND LOCKED POSITION

- BARBER POLE INDICATES THE GEAR IS IN THE IN TRANSIT FROM THE UP OR DOWN POSITION

- RED WHEELS (SHOWN AT THE 90 DEG POSITION) INDICATES THE GEAR IS DOWN , THE SIDE BRACE IS IN THE LOCKED POSITION AND THE CROSSWIND / CASTER POWER BACK POSITIONING COLLAR IS DISENGAGED

- GREEN WHEELS (ALIGNED WITH LINE OF FLIGHT) INDICATES THE GEAR DOWN AND LOCKED CONDITIONS ARE MET AND THE CROSSWIND / CASTER POWER BACK POSITIONING COLLAR IS ENGAGED