PLASTIC BRAIN JOHN WRAZIN

# PLASTIC BRAIN

# GTC / EXT PWR GUIDE

| # EQUIPMENT OPS |  |  |
| --- | --- | --- |
| # THE COMPLETE IDIOT’S GUIDE TO: |  |  |
| # THE GTC  |  |  |
| 1 2 3 4 5 6 7 8 9 10 11 |  | Battery – ON Bus Tie Switch – ON / GUARD UP Eng #2 Cross Feed – OPEN GTC Door – OPEN (Wait 5 Sec) Bleed Air / ATM – CLOSED / STOP Hold START till START (Amber) Light illuminates. Wait for ON SPEED (Green) Light to be on for 1 minute. Bleed Air / ATM – OPEN / ON while Check A-C Load Meter for Flicker Eng Gen switch to ATM GEN Check Freqs and Volts in all Phases ATM GEN – ON (Rt Side of Panel) Check all Bleed Air Gauges in Green Check All Fire Handles |
| EXTERNAL POWER  |  |  |
|  | ~~The Cart~~ Panel lights – ON (should be labeled) Choke (not your chicken) pushed all the way in. Push and hold the PUSH TO START button (black rubber) Put “Build Voltage” switch in UP pos and hold until proper freqs & volts. Attach cord to Airplane. Put CLOSE CONTACTOR switch UP. Look for Green light. ~~The Plane~~ Eng Gen Switch to EXT PWR Check Freqs and Volts in all Phases Ext AC Pwr Light comes on (L Side of Panel) - means Cart is Running Ext AC Pwr Switch – ON |  |
| FREQS AND VOLTS  FREQS 380 - 470; VOLTS 110 - 125. FOR REFERENCE ONLY |

# BRIEFING GUIDE

Crew Coordination

- ESA

- Terminology:  Ready,Ready, Roll/Turn/Hack

- Co has leg timing, Nav has update timing.

- 5 NM to go – MC, Alt, where is the Threat, time status.

- Threat plotting

- Time control – Speed vs. Geometry

Departure

- Formation position (pilot give hack on lead roll)

- Abort: formation position on interplane x3

- Combat entry checklist

- Acceleration location

ROUTE STUDY

- Orbit point and Alt (Day 1000 AGL, Nt 500 + leg Nt alt)

- Combat Entry Point

- Positive transfer of navigation (whenever out of the window)

- 20 and 10 min. advisory locations

- Bullseye location (in immediate pg.)

SKE Specific

- CP call in-track at 30 sec turn prep

And I will give turn delay time

- Where to expect lead’s GS, DA, and ALT info

- Pilots get first call on preps and all preps after SD

Run-in

- Winds

- Where is the TP (SD point)

- SD and 1 min advisory locations

- DZ size, PI location, and drop load wind limits

- a CREW SAVE is GOOD SAVE

SKE Specific

- Will notify when inside DZ entry point

- Maintain in-track distance at 1 min advis.

Recovery

- Flimsy page number

- I will monitor navaids

- Backup timing from FAF

- Outbound HDG from procedure turn (double or triple drift)

IF YOU CAN READ THIS, YOU STILL DON’T NEED GLASSES

# CHEATS

| C H E A T S ! ! ! |  |
| --- | --- |
| & Other Good things to know… |  |
| Equipment Preflight Ops “1553” Radar Altimeter 16-32 second calibration period (“1 minute”) Compasses 5 minute warm-up time ADF 5 minute warm-up time Radar 3 minute warm-up time # Defensive Systems 5 second warm-up time | # Level Off: When you reach 2/3 of your altitude you are at ½ your time to climb. # Temp DEV Passing 10K, whatever is in the Temperature Gage is the actual temp dev. # IAS: 4 x Alt Ex. FL 200, 280 TAS 4 x 20 = 80 80-280 = -200 # IAS = 200 ## IAS to CAS correction 270+ TAS –3 correction # 269– TAS –2 correction # ETA lines: 60/GS x Distance = Time (min.) DR lines: GS/60 x Time = Distance – or - Time/60 x GS = Distance ## TFF Decreases as Alt increases 100 lbs per 1000 feet of altitude change. Ex. TFF @ FL200=4.3 TFF @ FL220=4.1 Course Control: DCA = Degrees to turn L or R to intercept course a given distance down track. XT x 57.3 ± TKE =DCA DTI (Formula for Navigation) Distance to lead turns by for 90 º turns: 1% of ½ of GS [(GS/2)x.01] Ex. GS=180 180/2 = 90, 90x0.01 = 0.9 nm 1 NM = 2025 yds or 6076 ft. Wag is 2000 yds per NM Climb to TA, Descend to TL |
| Fuel Wags: for when time is short. Also good a X. Hi Level: 5000 #/hr + 1000 # Lo Level: 6000 #/hr + 1000 # (Extra 1000 lbs for mom and the kids, wind and temperature variations, weather deviations, etc.) |  |
| A Place for My Stuff: C-130E - Max Densities. 92 Ground Troops 64 Paratroopers 72 Litters 16 CDS Bundles 06 Cargo Pallets; 54000 lbs*-Hypothetical Max Cargo Wt 42000 lbs Max Airdrop Load Wt. |  |
| Radiation Hazards  300 Feet (RADAR) for Fuel and Explosives 40 Feet (RADAR) for Personnel 5 Feet (HF Radio Ant.) for Anything |  |
| --------------RUNWAY-------------- Peacetime Restrictions: 3000 x 80 (60 Assault) Wartime Operations: As Required x 80 (60 Assault) |  |
| Is it Hot in Here? Cold WX Ops - 0ºC (32ºF) and Lower Hot WX Ops – +35ºC (95ºF) and Higher |  |
| On Time? AMC & USAFE -20 Min./+14 Min. PACAF & CENTAF -30 Min./+14 Min. |  |

# SKE LEAD FOR DUMMIES

| # SKE LEAD FOR DUMMIES |  |
| --- | --- |
| # SKE Lead Script 1. “Combat Entry Checklist” 2. “20 Minute Advisory – Pre-slowdown Checklist” Relay: Barometer BAR Drift RD/LD Groundspeed # 3. “10 Minute Advisory” 4. “30 Seconds to SD” “SD Prep, Relayed” “5 Seconds to SD” “-Prep” “Slowdown Now” “Execute” 5. "" Prep “Clear to Descend” “E” “Descend” 6. “1 Minute Advisory” “” Prep 7. “5 Seconds” “” Prep 8. “Green Light” “Execute” 9. “Red Light” “Hack” 10. “30 Second + Prep” 11. “5 Second + Prep” 12. “Execute” Turn to Escape Heading | # SKE Lead Turn Script The Turn Computer is your friend. It keeps the wingman Navs from killing you post-mission. Whenever possible, set up the turn computer with the necessary data. The Turn Computer needs three bits of Info, PH, TAS, and NH. For Every Turn: “30 Second Turn Prep (Next Mag Heading) Passed” Tell Pilot, “5 Second Right/ Left Turn Prep” He says, “Passed” “Turn, Execute” Pilot says, “Passed” -- On Rollout: Mag Course Kill Drift, pass Drift Kill Heading as “PH” Read TAS from SCNS and relay as TAS. Barring threat avoidance, you’re set for the next turn. Example: 071 NH XMT. . E. 073 PH XMT. 230 TAS XMT. |
| # SKE Recovery – You aren’t Done Yet! After the Escape Maneuver (+,+,E), relay all turns as normal. On base leg of ST-IN or ARA, relay “# 999, - E” (Procedure Turn approach separation achieved on inbound turn.) SEE NEXT PAGE FOR DETAILS |  |

# SKE RECOVERY GUIDE

What?  You need a Diagram?!

DZ

#999
-
E

ARA or ILS ST-IN

ALT 020
↓
E

FAF

140 IAS
-
E

150 IAS
-
E

NOTE:  Approach Separation is achieved in the turn, so NO #999

DZ

NDB Procedure Turn

067
→

E

247
→

E

1 min

18 s.

36 s.

FAF

2nd time around

140 IAS
-
E

150 IAS
-
E

# PLANNING GOODIES

| ### PLANNING GOODIES |  |
| --- | --- |
| Things that aren’t done often enough to commit to memory… |  |
| ### FUEL PLANNING | ### KEY TO WX CODES |
| # TFF is enroute fuel for (enroute time + 1 hour) – (enroute time) (PFF+TFF)/2 = AVG DIFF/AVG = Fuel ETE Fuel ETE-ETE Dest = Extra Time ## FUEL Adjustments for Drag Cruise ceiling (ALL airspeeds) Decrease by 800 ft Cruise Fuel (280+ TAS) Add to Enroute Fuel Enroute TIME; ADD FUEL. 0 – 4 Hrs; + 800 #. 4 – 8 Hrs; + 1600 #. 8+ Hrs; + 2000 #. ## Endurance (280+ TAS) Subtract from Onboard Fuel before computing endurance Onboard Fuel; Subtract. 0 – 18000 #; - 800 #. 18000 # – 36000 #; - 1600 #. 36000 # +; - 2000 #. USE 55-19 for Endurance Calc. DIFFERENCE BETWEEN NAV & ENG CRUISE CEILING: Nav’s ceiling is theoretical max top floor, Eng’s ceiling is highest altitude reachable with 300-ft/min climb. | CLOUD COVER: OVC – 7/8 – 8/8 coverage BKN – 5/8 – 6/8 coverage SCT – 3/8 – 4/8 coverage FEW – 1/8 – 2/8 coverage SKC – Sky Clear I DON’T EVEN SEE THE CODE ANYMORE… -; Light; PL; Ice Pellets. Moderate; GR; Hail. +; Heavy; GS; Small Hail. VC; In Vicinity; UP; Unknown Precip. MI; Shallow; FG; Fog. PR; Partial; BR; Mist. BC; Patches; FU; Smoke. DR; Low Drifting; VA; Volcanic Ash. BL; Blowing; DU; Dust. SH; Showers; SA; Sand. TS; T-Storms; HZ; Haze. FZ; Freezing; PY; Spray. DZ; Drizzle; PO; Dust/Sand Whirls. RA; Rain; SQ; Squalls. SN; Snow; FC; Funnel Cloud. SG; Snow Grains; SS; Sand Storms. IC; Ice Crystals; DS; Dust Storms. (FIH C-55) |
|  | 9 to 5 My @$$ FDP is 16 hrs (waive. To 18 per A/C) TAC Duty Day is 12 Hours. Period. |
|  | A; . -; N; - .. B; - . . .; O; - - -. C; - . - .; P; . - - .. D; - . .; Q; - - . -. E; .; R; . - .. F; . . - .; S; . . .. G; - - .; T; -. H; . . . .; U; . . -. I; . .; V; . . . -. J; . - - -; W; . - -. K; - . -; X; - . . -. L; . - . .; Y; - . - -. M; - -; Z; - - . .. MORSE CODE Penguins on the Iceberg... |
| AMCP 55-19: Quick Index - Highest Acceptable Flight Level computed on pg. 22 - Time / Distance to climb computed # on pg. 18 |  |
| DD Form 175 and 1801 # Item 3 of a DD Form 175 should say “C-130/I” for CONUS based C-130’s. On DD Form 1801 file “C-130/R” for routes requiring RNAV and to indicate RNAV equipment on board the aircraft. |  |

# Equipment Ops BICU / GPS

| # Equipment Ops: |  |
| --- | --- |
| ### BICU DEV CHECK “Contra Code” |  |
| # Change IDCU hdg to Mag or change heading on cel update page to Mag. TEST--MAINT TEST--BICU TEST-BICU INPUT--. Record Compass Headings on Log Go to NAV-ENROUTE 1-5 or UPDATE-CEL & Record INS Mag Hdg. Compute Compass Deviations. # On NAV-INIT-FLT PMTR 2-3, enter the Deviation Correction (reverse the sign) and # select the most accurate compass. |  |
| INS Power On/Off If INS power switch is placed to OFF position, wait at least 10 seconds before placing it back ON. | Air Alignment If you’re reading this, you’re screwed. Requires that airplane fly straight and level for at least 1 minute (5 minutes is recommended) after commanding alignment. After 5 minutes of alignment, “S” turns are recommended to allow the Kalman filter to measure and operate on heading changes. After 30 minutes with valid DVS inputs or 15-45 minutes with valid GPS inputs (must have crypto data loaded), INS MODE becomes NAV. Note: During air alignment, FDMS switch of pilot in control of the airplane should not be placed in INU position. |
| GPS Backup - 1553B data bus failure The ARINC 429 channel will provide GPS outputs to the navigator’s IDCU only. Although always available at the navigator’s IDCU, use of this display page should be limited to times when GPS 1553B data is not available |  |
| GPS Initialization At turn-on, auto-initialization starts & requires no operator input. If critical memory info isn’t available due to battery failure or receiver replacement, GPS will perform a cold initialization to collect the almanac info. Cold init may take up to 2 hours. |  |
| User Initialization User initialization may be accomplished via GPS INIT 1-2 display page. This requires three distinct operations to be performed: 1. Setting GPS receiver to INIT mode 2. Entry of desired data on GPS INIT 1-2 page 3. Setting GPS receiver back to NAV mode |  |
|  | GPS Global Positioning System States of the satellite 1 - Normal Acquisition. Is acquiring a C/A code 3 - Code Lock. Able to maintain coarse pseudo-range measurements only 5 - Carrier Track/Data Demodulation. Maximum accuracy. 7 - Signal Reacquisition GPS Status 3-3 Carrier-to-noise ratio (C/NO); higher is better. Jamming-to-signal (J/S); lower is better. Figure Of Merit (FOM). 1 - 82’, 2 - 164’, 3 - 246’, 4 - 328’, 5 - 656’ |
| (((( SENSITIVE STEERING )))) Engages with appropriate LZ or DZ settings once TKE ≤ 15° & XTK ≤ 1.5 NM |  |

# Equipment Ops O2 / RADAR

| # Equipment Ops: |  |  |  |
| --- | --- | --- | --- |
| ### OXYGEN SYSTEM |  |  | ### AN/APN-59 RADAR |
| # Suck my hose, if you want to live… |  |  | # Feel the Tingle? |
| MINIMUM REQUIRED IS 5 LITERS! *25 L Liquid O2 system *O2 Consumption Charts in –1, sec I, pg 1-231 *96 Man-hours when fully serviced Low Warning Light: on when down to 2.5 L O2 PSI. Static; 270-340 psi. Dynamic; 270-455 psi. PRICE Check (Aisle 9…) P; Pressure (See above). R; Regulator (Airflow). I; Indicators (Flasher, Needle). C; Connections (Hose). E; EMRG. 10 Total O2 Regulators on Aircraft 6 on Flight Deck 4 in Cargo Compartment | 28v Main DC / 115v Main AC / CP Upper Test Meter does Milliamperes Line; 0.60 ± 0.08 and steady. Mag*; 0.65 ± 0.35 and steady. Mixer 1; 0.60 ± 0.30. Mixer 2; 0.60 ± 0.30. AFC 1; 0.60 ± 0.30. AFC 2; 0.60 ± 0.30. *Magnetron WARN = WX Mode L – CCW Scan @ 12 RPM R – CW Scan @ 12 RPM STC – Reduces gain on short range targets Inner knob – amount of reduction Outer knob – range of reduction (<=40) When Pac-Men Attack… Remember: 300 ft Fuel/Explosives 40 ft Personnel/Popcorn Radiation Hazard Pattern. 300 40 |
|  | MY ONLY BOLDFACE |  |  |
| Fuselage Fire/Smoke and Fume Elimination. [W] --- --- --- --- --- --- --- --- --- --- --- --- --- --- --- --- --- 1. OXYGEN [W] – “ON, 100 PERCENT” (ALL) --- --- --- --- --- --- --- --- --- --- --- --- --- --- --- --- --- |  |  |  |
| STRUCTURAL SPEED LIMITS Ramp & Door, Air Deflector Doors, Crew Entrance Door (for Bail Out), and to Open Paratroop Doors – 150 KIAS Cargo Door – 185 KIAS With Paratroop Doors Open & Locked – 250 KIAS |  |  | # RADAR Tied to N-1 #1 or C-12 #2 |
|  |  |  | # WAIT at least 30 sec. before switching the RADAR set from STBY to OFF. |

| # EQUIPMENT OPS: |  |  |
| --- | --- | --- |
| ### 1553 DATA BUS FAILURE RECOVERY |  |  |
| # DO THIS ▼ | # NAME | # LOCATION |
| 1. Pull Circuit Breakers for: | ###### Plt IDCU CPlt IDCU NAV IDCU BICU 28V DC DVS IRU GPS INU | P Upper /E DC P Upper /E DC P Upper /E DC P Upper /E DC P Upper /E AC P Upper /E AC P Upper /E AC P Upper /E AC-DC |
| 2. Starting with the NAV’s IDCU, return power to these functions until something causes the IDCUs to display “BC I” instead of the respective BC F or RT messages. | Definitions: |  |
|  | IDCU – Individual Display Computer Unit BICU – Bus Integration Computer Unit IRU – Interface Receptacle Unit (DTM Loader) DVS – Doppler Velocity Sensor GPS – Global Positioning System INU – Inertial Navigation Unit |  |
| 3. When Test Screen displays “BCI” again, you have just reactivated the problem component. |  |  |
| 4. Again pull all the assoc. circuit breakers. Wait 10 sec, and return power to all except the known problem component. | “…I need warp speed in 2 minutes or we’re all dead…” |  |
| 5. If able and needed, perform Air Alignment to the INS. |  |  |
| SOURCE – C-130 ATS – NAVIGATOR REFRESHER TRAINING – INSTRUCTOR DEFINITIONS & LOCATIONS – TO-1-C-130E(H)-1 AND TO-1-C-130-1-4 |  |  |

# Equipment Ops 1553

# Equipment Ops IDCU / Compass

IDCU and Compass System Tie-Ins

IDCU Block Diagram

– Made Easy for those from the South.

(Ref TO 1C-130-1-4,p. 1-1)

| IDCU 1 (P) | IDCU 2 (CP) | IDCU 3 (N) |
| --- | --- | --- |
| *UHF 1 HF 1 TACAN 1 | VHF 2 HF 2 TACAN 2 | UHF 2 VHF 1 |
| SKE, MLS 1, MLS 2 |  |  |
| VOR/ILS 1, VOR/ILS 2 |  |  |

UHF 1

HF 1

TAC 1

VHF 2

HF 2

TAC 2

C

2

P

1

MLS 1 & MLS 2 & SKE

VOR/ILS 2

VOR/ILS 1

N

3

VHF  1

UHF  2

GPS BK

Compass System Tie-Ins

(ref TO 1C-130E(H)-1,p. 4-230, and 4-243)

| N-1 #1 | N-1 #2 |  | C-12 #1 | C-12 #2 |
| --- | --- | --- | --- | --- |
| P HSI CP RMI CP BDHI N #2 RMI N #2 BDHI TACAN 1 VOR 1 RADAR | CP HSI P RMI P BDHI N #1 RMI N #1 BDHI TACAN 2 VOR 2 |  | P HSI *P BDHI CP RMI N #1 BDHI TACAN 1 VOR 1 | CP HSI *CP BDHI P RMI N #2 BDHI TACAN 2 VOR 2 RADAR |

# Equipment Ops AN/APQ-175

AN/APQ-175 (AWADS) Cheat Card

What Little I Actually Know About this F**king Thing.

<<<<  REFERENCE: TO 1C-130E-1-2  >>>>

AN/APN-59 is superior for finding and avoiding WX, and for high level navigation.

AWADS is superior for ground mapping at low altitudes.

AWADS also makes the flight deck up to 20º F warmer than a APN-59

5 min, 4 sec warm-up, then run BIT Checks.  RADAR is self-testing and self-tuning.

Make sure the RANGE MARKS are switched ON.

2 Bands, X and KA:

| X Band (Normal) | KA Band (AWADS) |
| --- | --- |
| Press Check 39-44 not to exceed 45 (On the regular gage). X Band is used for navigation and WX avoidance. Normal band for all ranges 50 nm and greater. | Press Check 25 +/- 1 (Make sure only One bottle is open) Uses Freon. KA Band is used for precision ground map-ping below 5000 ft AGL. Normal band for 30 nm range and lower. |

If there is no freon, switch the Band to ALTN to use X Band in the 30 nm ranges.

For WX:  Set to WX mode. Tilt 0.0 to +1.0.  In WX mode, gain is inoperative.  Use TGT SENS knob to adjust picture.  Set ISO ECHO to MOD or HVY.

- = = R A D I A T I O N   R A N G E S = = -

Explosives – 149 ft

Personnel – 96

Fuel – 64 ft

KA Band requires freon from the bottles.  To open bottles (one behind 245, one under flight deck) remember “lefty loosey.”

149

96

64

SHOW EM YOURS!

To sync the displays, set your NAV VID switch to ALTN.  This should slave the pilots scope to your own.  This is great for WX avoidance, assuming you can find the storm with this thing.  You have an equally good chance of avoiding a mountain by 20 nm.

-----------------------------------------------------------------------------------------------------------------

Technical Poo:  Radar Circuit Breakers are Ess-DC and Ess-AC on Pilot’s Upper Panel.  AWADS Computer on Ess-AC on Pilot’s Side Panel.

FREQ – Per TO 1C-130E-1-2, the system is limited to frequencies 6, 7, and 8.

-----------------------------------------------------------------------------------------------------------------

Use Radome Anti-Ice whenever icing is encountered.  “Heat On” and “Mode Auto” will set the system to automatic operation.  Leave it in “Low” rate.

<<<< REFERENCE: TO 1C-130E-1-2 >>>>

| # Miscellany |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| # Overwater Goodies | # Simple Radio Calls |  |  |  |  |
| ##### TIME Requirements ##### FIXES Initial Cruise Fix as soon as practical after LVL, time between fixes not to exceed 1+00 DRs Plotted before fix time # DEV Check as soon as practical. # Must be Straight and Level Flight. Then every 3 hours or planned alter of 30° or more. TAS Check within 1 hr of LVL. One time deal. # FUELS as soon as practical after LVL and at regular intervals 1+30. May terminate In-flight Fuel Management 1hr from destination. --- --- --- --- --- --- --- --- --- --- --- ETP Computations Required on Cat I routes or Cat I portions of routes when the total time between the LSAF and FSAF is 5 hours or more. Recompute ETP when actual arrival over any reporting point prior to ETP exceeds 15 minutes ahead or behind time when the change was caused by erroneous wind information. Category I Enroute Procedures Deviation checks should use the INU TH. DR line required for each fix (except coast-out/in) Wind required on fix and air plot DR lines. Must match drift. General Fixing Requirements Category II routes just need ETA/ATA. Category I, when the SCNS is reliable; only time, position, average heading, average true airspeed, altitude, and spot wind is needed. Time between full line entries will not exceed 1+20. | Off Call: “Command Post, ‘Raven 420’ Departed at 1530 zulu” Inbound Call: “Command Post, ‘Raven 420’ is approx. 15 minutes out, mission 90% complete, Alpha (2) for “wings off light”. Request 10K lbs of fuel, parking, and Trans for 6 crew and baggage.” Position Reports: “PTA PTP” Point you just passed. When you passed it, and your MSL Alt when you passed it. Then your next point, when you’re getting there, and then the point after that. Ex. “Bla Radio, Reach 900, Position.” “Charlie Foxtrot at 2000 Z FL210.” “Estimate Mike Foxtrot 2100 Z, then Sierra Hotel.” |  |  |  |  |
|  |  | ###### EMRG Radio Procedures |  |  |  |
|  | ###### Emergency |  |  | ###### Squawk Mode 3/A,. ###### code 7700. ###### Transmit on UHF 243.0, VHF 121.5, or HF 8364. |  |
|  | ###### Distress, ###### MAYDAY (3x) ###### Urgency, ###### PAN PAN (3x) Key for 30 seconds for SARSAT |  |  |  |  |
|  | ###### Radio Failure |  | Squawk Mode 3/A, code 7600. |  |  |
|  | Hijacked |  | ###### Squawk Mode 3/A, code 7500. Flaps set 100% on ground. (Rqst Immobilize Acft.) |  |  |
| Overwater Missions - Don’t carry more passengers and crew than life rafts will accommodate. (80 total) - LPUs will be within easy reach of each passenger and aircrew member prior to takeoff. - Crew will fit and adjust LPUs and will wear them below 2000 feet (except for takeoffs and landings). |  |  |  |  |  |
| ###### The Bells, The Bells… (or: The Lord of the Rings) Rings ↓; Bail Out (AIR); Ditch (OCEANS); Crash (GROUND). 6 Short; X; Prepare for Ditching; Prepare for Crash Landing. 3 Short; Prepare to Bail Out; X; X. 1 Long; Abandon Airplane; Brace for Impact; Brace for Impact. On the Ground, 1 Long Ring means Ground Egress, i.e. “GET THE HELL OUT!” |

# Miscellany

# TAS/DEV Check

| ### TAS & DEV CHECK FORM |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TAS CALIBRATION CHECKS WITHIN AN HR OF LVLOFF, ONCE PER FLIGHT |  |  |  |  |  |  |  |  |
| CALL SIGN |  |  |  |  |  |  |  |  |
| TIME |  |  |  |  |  |  |  |  |
| ALT (PA) |  |  |  |  |  |  |  |  |
| IOAT |  |  |  |  |  |  |  |  |
| TOAT |  |  |  |  |  |  |  |  |
| IAS |  |  |  |  |  |  |  |  |
| CAS (-3) |  |  |  |  |  |  |  |  |
| EAS |  |  |  |  |  |  |  |  |
| TAS |  |  |  |  |  |  |  |  |
| -GUAGE TAS |  |  |  |  |  |  |  |  |
| CORR |  |  |  |  |  |  |  |  |
| -SCNS TAS |  |  |  |  |  |  |  |  |
| CORR |  |  |  |  |  |  |  |  |
| HEAT OF COMPRESSION (TEMP °C) |  |  |  |  |  |  |  |  |
| TAS; TEMP. 190-210; -4. 210-230; -5. 230-246; -6. 246-263; -7. | TAS; TEMP. 263-277; -8. 277-288; -9. 288-300; -10. 300-310; -11. |
| COMPASS DEVIATION CHECKS EVERY 3 HRS OR PLANNED ALTER OF 30° + |  |  |  |  |  |  |  |  |
| CALLSIGN |  |  |  |  |  |  |  |  |
| TIME |  |  |  |  |  |  |  |  |
| CEL TH |  |  |  |  |  |  |  |  |
| MAG VAR +W / -E |  |  |  |  |  |  |  |  |
| CEL MAG HDG |  |  |  |  |  |  |  |  |
| DEV |  |  |  |  |  |  |  |  |
| #1 BICU |  |  |  |  |  |  |  |  |
| DEV |  |  |  |  |  |  |  |  |
| #2 BICU |  |  |  |  |  |  |  |  |
| DEV |  |  |  |  |  |  |  |  |
| WHISKEY |  |  |  |  |  |  |  |  |
| JMW FORM 4116.1, EXTRACTED FROM AF FORM 4116. |  |  |  |  |  |  |  |  |

# Flare/Chaff Counter

| # FOR REFERENCE ONLY # PROBABLY SECRET (WHEN FILLED IN) |  |  |  |  |
| --- | --- | --- | --- | --- |
| FLARE/CHAFF COUNTER |  |  |  |  |
| # TYPE | # CMDS | # FORMS | # PLAC | # Manual COUNT |
| M-211 |  |  |  |  |
| M-206 |  |  |  |  |
| MJU-50 |  |  |  |  |
| CH RR-170 |  |  |  |  |
| 2D 1A 2C 1B 2B 1C 2A 1D 1E |  |  |  |  |

# Fuel Planning Log

For Fuel calculations use AMCP 55-19 or TO 1C-130E(H)-1-1.

| Fuel Planning Reg. |  | PAGE NO. |  |
| --- | --- | --- | --- |
| OPERATING WT |  | CALL SIGN |  |
| CARGO/PAX WT |  | ACFT NO. |  |
| RAMP FUEL |  | MSN NO. |  |
| RAMP WT |  | TIME | FUEL |
| TAKEOFF WT |  |  |  |
| 1. ENROUTE |  | + |  |
| 2. RESERVE | TFF: |  |  |
|  | TD: / |  |  |
| 3.ENROUTE + RESERVE |  | + |  |
| ALTERNATE + 4. MISSED APPROACH |  |  |  |
| 5. HOLDING | 2.0 |  |  |
|  | 3.5 |  |  |
| 6. APPROACH/ | 1.0/4.0 |  | 5.0 |
|  | LANDING |  |  |
| 7. IDENTIFIED EXTRA |  | + |  |
| TOTAL (3 + 4 + 5 + 6 + 7) 8. TAKEOFF |  |  |  |
| 9. TAXI | 1.3 |  | 1.3 |
| 10. REQUIRED RAMP |  |  |  |
| (Ramp-Drag-Taxi)-------------------------> 11. ACTUAL RAMP |  | ENDURANCE | Planned Ramp Fuel |
|  |  | + |  |
| UNIDENTIFIED 12. EXTRA |  |  |  |
| MIN DIV OR (4 + 5 + 6) 13. REQ OVHD DEST |  |  |  |
| “Give me fuel, give me fire, give me that which I desire.” JMW FORM 4116.2, EXTRACTED FROM AF FORM 4116 |  |  |  |

# Inflight Fuel Card

| INFLIGHT FUEL MANAGEMENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| ETA DEST |  |  |  |  |
| TIME |  |  |  |  |
| PFF |  |  |  |  |
| TFF* |  |  |  |  |
| FUEL REM |  |  |  |  |
| O/H FUEL* |  |  |  |  |
| DIFF |  |  |  |  |
| FUEL ETE |  |  |  |  |
| ETE DEST |  |  |  |  |
| EXT TIME |  |  |  |  |
| AVG FF |  |  |  |  |
| MORE INFLIGHT FUEL MANAGEMENT |  |  |  |  |
| ETA DEST |  |  |  |  |
| TIME |  |  |  |  |
| PFF |  |  |  |  |
| TFF* |  |  |  |  |
| FUEL REM |  |  |  |  |
| O/H FUEL* |  |  |  |  |
| DIFF |  |  |  |  |
| FUEL ETE |  |  |  |  |
| ETE DEST |  |  |  |  |
| EXT TIME |  |  |  |  |
| AVG FF |  |  |  |  |
| STILL MORE INFLIGHT FUEL MANAGEMENT |  |  |  |  |
| ETA DEST |  |  |  |  |
| TIME |  |  |  |  |
| PFF |  |  |  |  |
| TFF* |  |  |  |  |
| FUEL REM |  |  |  |  |
| O/H FUEL* |  |  |  |  |
| DIFF |  |  |  |  |
| FUEL ETE |  |  |  |  |
| ETE DEST |  |  |  |  |
| EXT TIME |  |  |  |  |
| AVG FF |  |  |  |  |
| “E” Stands for “Enough.” Doesn’t it?” JMW FORM 4116.3-A, EXTRACTED FROM AF FORM 4116. MODIFIED FOR AVERAGE FUEL FLOW METHOD. |  |  |  |  |

# HF Radio

| # H F R A D I O ## FREQUENCIES |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MARS RADIO  Phone Patches / Message Relays |  |  |  |  |  | BBC NEWS  English Speaking News |  |
| 01 | 20.9925 |  |  | 1 | 1413 |  |  |
| 02 | 14.6060 |  |  | 2 | 1325 |  |  |
| 03 | 14.4000 |  |  | 3 | 1764 |  |  |
| 04 | 13.9270 |  |  | 4 | 1557.5 |  |  |
| 05 | 07.6335 |  |  | 5 | 1531 |  |  |
| 06 | 04.5570 |  |  | 6 | 1356.5 |  |  |
| 07 | 13.9930 |  |  | 7 | 1209.5 |  |  |
| 08 | 13.9960 |  |  | 8 | 1179 |  |  |
| 09 | 07.5400 |  |  | 9 | 970 |  |  |
| 10 | 07.0350 |  |  | 10 | 941 |  |  |
| 11 | 04.0035 |  |  |  |  |  |  |
| 12 | 04.0110 |  |  |  |  |  |  |
| 13 | 02.6560 |  |  |  |  |  |  |
| Phone Numbers: ____________________ ____________________ ____________________ ____________________ ____________________ ____________________ ____________________ ____________________ ____________________ ____________________ |  |  |  | ____________________ ____________________ ____________________ ____________________ ____________________ ____________________ ____________________ ____________________ ____________________ ____________________ |  |  |  |

| # MILL AROUND CHECKLIST T.O. 1C-130-420CL- ∏ |  |
| --- | --- |
| - “ACKNOWLEDGED” (AS REQUIRED) | ALL |
| - LOCATE AIRCRAFT COMMANDER FOR TASK REQUIREMENTS – “I DON’T KNOW.” | (P) |
| - LOCATE AND SECURE REQUIRED ITEMS – FOOD, DRINKS, TOBACCO, SUNGLASSES, CHEWING GUM, ETC. | ALL |
| - DISCUSS OPTIONS AND EVALUATE ALL REQUIREMENTS – I.E. ASK YOURSELF, “IS THIS REALLY NECESSARY?” | ALL |
| - PLACE REQUIRED ITEMS IN MOUTH (EXCEPT SUNGLASSES) | ALL |
| - FIND RELAXING CHAIR, DECK OF CARDS AND WAIT. | ALL |
| - “WHERE WE GONNA EAT?” “I DON’T CARE, BUT…” | (FE) (N) (LM) (CP) |
| - REPEAT ITEMS 2-7 PERIODICALLY UNTIL AIRCRAFT COMMANDER KNOWS SOMETHING. |  |

For Reference Only

# Mill Around Checklist

For Reference Only