UH-1H BOLDFACE

Emer Gov Opns

- GOV - switch – EMER

- Throttle—adjust as necessary to control RPM

- Land as soon as Possible

# Engine Malfunction Low Altitude/Low Airspeed or Cruise

- AUTOROTATE

- EMER GOV OPNS

# Engine Restart During Flight

- Throttle - Off

- STARTER GEN switch - START

- FUEL switches - ON

- GOV switch - EMER

- Attempt start

- Land as soon as possible

Engine Overspeed

- Collective – Increase

- Throttle—Reduce

- EMER GOV OPNS

# Main Driveshaft Failure

- AUTOROTATE

- EMER SHUTDOWN

# Fire - Engine Start

- Start switch - Press

- Throttle - Off

- FUEL switches - OFF

# Flight Control Servo Hardover

- HYD CONT switch - Select opposite position

2.   Land as soon as possible