| Maneuver | KIAS | N2 | ITT | Torque | Preflight Checklist: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maneuvering | 170 | 1700 | 600 | 1000 | -A-sheets |  |  |  |  |  |  |
| Fast Cruise | 200 | 1700 | 650 | 1200 | -TOLD |  |  |  |  |  |  |
| Slow Cruise | 140 | 1700 | 575 | 750 | -NOTAMS |  |  |  |  |  |  |
| Cruise Climb: |  |  |  |  | -TAFs/METARs |  |  |  |  |  |  |
| SL-10K | 155 | 1900 | 700 | 2230 | -Canned Dash-1 |  |  |  |  |  |  |
| 10K-20K | 135 | 1900 | 700 | 2230 | Nfwb.ntmof.navy.mil/nfwb Password ngppilot |  |  |  |  |  |  |
| 20K-25K | 125 | 1900 | 700 | 2230 | -Dash-1 Wx Brief (Wx x2124) |  |  |  |  |  |  |
| above 25K | 115 | 1900 | 700 | 2230 | -Flight Plan (DD175) x2505 |  |  |  |  |  |  |
| 500 FPM↑ | 150 | 1700 |  | 1400 | Fax x3301 |  |  |  |  |  |  |
| 667 FPM↑ | 150 | 1700 |  | 1500 | -Pubs Bag |  |  |  |  |  |  |
| 500 FPM↓ | 150 | 1700 |  | 800 |  |  |  |  |  |  |  |
| 800 FPM↓ | 150 | 1700 |  | 600 |  |  |  |  |  |  |  |

60-1 Rule Shortcuts:

| Speed | NM/MIN=TAS/60 | DME | 1nm | .8nm |
| --- | --- | --- | --- | --- |
|  | TAS=IAS + (2% per 1000’)*IAS | 8 | 7.5 | 6 |
|  | TAS=IAS+(FL/2) | 10 | 6 | 4.8 |
| Turn Radius | TR= NM/MIN – 2 | 15 | 4 | 3.2 |
| Descent Gradients | Gradient = (100s of feet) / Distance in NM | 20 | 3 | 2.4 |
|  | 1°= 100 feet at 1NM |  |  |  |
| When to begin descent | [(Altitude to lose)*2] +10 = NM from destination [or (alt to lose)*3] | 60 | 1 | .8 |
| Visual Descent Pt. | VDP = HAT / G.S. (nm from E.O.R.) |  |  |  |
| VSI | VSI= Pitch Change * (NM/MIN*100) |  |  |  |
|  | VSI for 3° G.S. = (GndSpd*10)/2 |  |  |  |
|  | VSI for 2.5° G.S. = [GndSpd*10)/2]-100 |  |  |  |

IFR Takeoff Brief:  Normal contact brief, then:

Back me up during all phases of flight, providing assistance as required.  Ensure all radios and navaids are set, including CP HSI, RMI, and MKR BCN.  Handle all comms as directed.  Record all clearances, headings, altitudes, frequencies, and ATIS.  Call out any abnormal pitch, roll, or airspeed, and when we are 1000’ from an assigned altitude or 100’ from DH/MDA.

-Emergency immediately after takeoff

-Departure we are flying, altitude, frequency