# MISC APPROACH INFO

# CROSSWIND LANDING

- COMBINATION OF WING LOW AND CRAB

- CRAB TAKEN OUT PRIOR TO TOUCHDOWN

- FLY TO RUNWAY, DO NOT ALLOW TO FLOAT

- AIRSPEED MAY BE INCREASED UP TO 15 KTS ON A DRY RUNWAY TO ALLOW OPERATION IN THE NORMAL ZONE

- 40% FLAP APPROACH A CONSIDERATION IF RUNWAY LENGTH PERMITS

- FOR 40% APPROACH PLACE SPOILER RATIO SHIFTER SWITCHES TO THE FLAPS DOWN POSITION AFTER TURNING FINAL

TWO ENGINE APPROACHES

- PRIOR TO EXTENDING GEAR AND FLAPS WITH OUTBOARD ENGINE HYDRAULIC SYSTEMS INTACT:

- TURN ON ELECTRIC HYDRAULIC BOOST PUMPS

- TURN ON ATM

- PRIOR TO TURNING ON THE ATM’S THE FLOOR HEAT SHOULD BE TURNED OFF TO REDUCE BLEED AIR LOADS AND POSSIBLE CYCLING ON AND OFF OF THE ATM

- MAINTAIN AIRSPEED WELL ABOVE MINIMUM CONTROL SPEED

- WITH FLAPS LESS THAN 80% AND THE NLG NOT DOWN OR MLG POSITIONING COLLARS NOT LOCKED THE LANDING GEAR WARNING HORN PROTECTION IS NOT AVAILABLE

- TWO ENGINE APP SPEED = 40% FLAP APP SPEED

- BANK ANGLE SHALL NOT EXCEED 30 DEGREES IN A TURN

- DOWNWIND

- TWO ENGINE APP SPEED + 40 KTS (MINIMUM)

- BASE LEG

- GEAR DOWN

- TWO ENGINE APP SPEED + 40 KTS

- FINAL APPROACH (JUST PRIOR TO GLIDE SLOPE)

- FLAPS 40%

- TWO ENGINE APP SPEED, BUT NOT < 2 ENGINE VMCA

- THRESHOLD

- TWO ENGINE APP SPEED

- FLAPS LANDING (IF DESIRED)

- TOUCHDOWN (NLG ON GROUND)

- REV IDLE (ONLY)

- SPOILERS AND BRAKES

LOSS OF MULTIPLE HYDRAULIC SYSTEMS

2                      2

1                                    4

133    23                             23    133

1

3

2

3

TR LIMITATIONS

- DO NOT EXCEED REV IDLE ABOVE 150 KTS OR BELOW 60 KTS EXCEPT IN AN EMERGENCY

- IF REV IDLE IS EXCEEDED BELOW 60 KTS A FAN BLADE INSPECTION IS REQUIRED

- DO NOT OPERATE OUTBOARD TR’S INFLIGHT

- ENTER REV IDLE WITH INBOARDS INFLIGHT ONLY AFTER ENGINE’S HAVE REACHED IDLE RPM

- TR’S LIMITED TO REV IDLE INFLIGHT, OUTBOARD ENGINES MUST REMAIN IN IDLE WHILE INBOARD TR’S ARE EXTENDED INFLIGHT

- DO NOT EXTEND THE TR’S WITH THE FLAPS DEPLOYED

WEIGHT LIMITATIONS EMERGENCY WAR PLANNING

- 840,000 LBS

- NORMAL LANDING WEIGHT 769,000 LBS

PRECISION RADAR APPROACH (PAR)

- NORMALLY STARTS 8 MILES FROM TOUCHDOWN

- CONTROLLER WILL ADVISE APPROACHING GLIDE PATH 10 – 30 SEC PRIOR

- CONTROLLER WILL ADVISE WHEN AIRCRAFT REACHES DH

- DH IS DETERMINED IN THE COCKPIT BY THE ALTIMETER AND WILL BE GIVEN BY THE CONTROLLER USE WHICHEVER OCCURS FIRST

- SELECT A BACKUP APPROACH IN CASE OF RADAR FAILURE OR LOST COMMUNICATIONS, WX PERMITTING

- IF COMMUNICATIONS ARE LOSS YOU ARE AUTOMATICALLY CLEARED PUBLISHED APPROACH UNLESS CONTROLLER HAS ISSUED A SPECIFIC LOST COMM APPROACH

- LOST COMMUNICATION CONDITIONS

- 1 MIN WHILE BEING VECTORED TO FINAL

- 15 SEC WHILE ON FINAL APPROACH FOR AN ASR

- 5 SEC ON FINAL APPROACH FOR A PAR

- CONTROLLER IS REQUIRED TO ISSUE (WX) CEILING AND VIS

- CEILING IS < 1500 FT OR BELOW THE HIGHEST CIRCLING MINIMUMS WHICHEVER IS GREATER

- VISIBILITY BELOW 3 MILES

NON PRECISION APPROACH (ASR)

- UPON REQUEST CONTROLLER WILL PROVIDE RECOMMENDED ALTITUDES

- CONTROLLER WILL PROVIDE STRAIGHT-IN MDA’S AND MAP LOCATION

- CONTROLLER WILL PROVIDE ADVANCE NOTICE OF WHERE TO BEGIN DESCENT

RADAR PATTERN

- EARLY CONFIGURATION

- 100% FLAP APP SPEED + 30 KTS

- DOWNWIND

- CLEAN, 100% FLAP APP + 60 KTS

- BASE LEG

- FLAPS 40%, GEAR DOWN

- 100 FLAP APP + 20

- FOR NO FLAP, GEAR DOWN, NO FLAP APP + 20

- FINAL APPROACH (JUST PRIOR TO GLIDE SLOPE)

- FLAPS LANDING CONFIGURATION

SLOW TO APPROACH SPEED