AIRSPEED

Vne Sea level	405

Vflap maneuver	275

Vflap t/o and approach	190

Vflaps land	170

Min flap 3 or 4 eng	140

Min flap 2 eng	145

Min flap heavy	160/180

No Flap downwind (clean)	1.52 or 160

2 Eng downwind (clean) 	1.52 or 160

Min speed with pitchlocked prop	130

Avoid hard brking above	120

Max xwind airspeed	35

Vgear extension @1g	300

Vgear extension @1g+	190

Vlanding lights extend	260

Vbomb bay doors open	Limit Dive

Voverwing exit	170

Vmain cabin door open	300

VAPU door open	225

VIRDS turret extended/extend or retract	405 / 375

VAIMS turret extended/extend or retract	405 / 350

Vfuel dump	140 – 300

Eng start	170-210 345max(-6/1000)

Max for shifting	50

Vmc ground	102

Power levers into beta w/out power	125

Power levers into beta w/ power	135

Turb air moderate to extreme	220

Turb air light to moderate	190

Ditch speed using appr flap	LFDS+5kts

Ditch speed using man flap	LFDS+20kts

NTS check	180 +-10, reference 1.52

Below Class B airspace	200

4 eng climb to 10K (-2/1000)	220

3 eng climb to 20K (-2/1000)	190

2 eng climb	180

Max w/ cracked windshield	240

Loiter	Gross weight + 90

Stall buffet gouge	Gross weight + 63

APU

Gnd RPM range	98-102

Flt RPM range	98-103

Max RPM	106

EGT  -2	688/710/715

EGT  -3	649/704

EGT -10	649/710/732

APU max altitude	20,000’

APU start temp range	-54 to 54

APU eng start max alt	6000’

APU fuel flow rate	300 pph

APU assumes a load	95%

Oil Pressure

Start (rising by XX%)	35%

Low RPM RGB	50-250

Norm RGB	130-250

Norm P/S	(100 for start and W/U) 50-60

OIL PRESS low lt P/S	40

OIL PRESS low lt RGB	130

Allow press flux RGB	+/- 10

Allow press flux P/S	+/- 5

Max time 0 or neg gs	10 sec

SHP

Max cont SHP	4600

SHP for NTS 1 and 4	-150-500

SHP for NTS 2 and 3	-100-500

SHP w/ oil tmp below 0oC	minimum

SHP w/ oil tmp 0-40oC	1000

SHP w/ oil +40, no flux RGB	4600

Oil Capacities

Total	15.75gal

Usable	8.65gal

Unusable	1.4gal

Foaming	5.7gal

Hydraulic System

Norm Pressure	2960-3200

#1 sys	5.6 gl res/ 16.2 gl total/ srv @ .8 low

#2 sys	1 gl res/ 4.5 gl total/ srv @ .25 low

Air Brake prs (min EBB for flt)	1900

Hyd pump press low lt	1800

Auto pilot disengage	1100

Hyd OIL HOT lt	175oF

Brake applications with Air	3-6

Brake applications with accumulator	8

TIT

Low RPM	410-730 (71.0-73.8%)

Flight Idle	430-705 (94.5-98.4%)

Norm RPM 	445-645 (96.3-99.1%)

Max Reverse 0o	580-790 (96.7-104.9%)

Max T/O 90o	1077 (5 min)

Military Power	1049 (30 min)

Normal Flight	1010

Hot Start (rec & cont)	830-850

Over temp Insp required	965

Cold Start (rec & cont)	720-750

TIT Overtemp lt on at	1080 +- 2

Max TIT Overtemp	1175 (5 sec)

Residual TIT (Ground)	200

Residual TIT (Air)	150

Residual TIT (Prime)	100

N1 RPM %

T/O Max 	99-101%

Fuel topping Gov	104.2-106.7

Pitchlock	103.5

Pitchlock reset	109

T/O RPM overshoot/under	106 / 96

In flt over/undershoot	sync +/- 1, no sync 2

Overshoot that must be inspected	+106

Start Button out btwn	57-64

Prop brake held released by oil 	21%

Inflight restart, PP1 light out by 	55%

Speed Sensitive Control

16% - Energizes Ignition Relay through the FSOV, energizes igniters, paralleling valve, fuel manifold drain, and enrichment

65% - de-energizes ignition relay

94% - TD to temp limiting

Laterl Imbalance

Tank 1&4 t/o, in flt or ldg	1400 / 4389

Tank 2&3 t/o, in flt or ldg	5025 / N/A

Tank 1/4 =1606, 2/3 =1671, 5=2646 ttl=9200

AOB

Max AOB 	60

ZFW / Load Limits

W/out foam / with foam	80,400/82,400

Min Alt on Station	    dayVmc/night Imc

A/C w/ RADALT and RAWS	200 / 300

A/C w/o RADALT or RAWS

w/ current alt setting	300 / 500

A/C w/o any above	500 / 1000

A/C and Press Sys

EDC TEMP HI lt on	121

EDC PRESS LOW lt	50

LE HOT lt on	110

Isobaric Range	0-13.3

Differential Range	13.3-13.9

Safety Range	13.9-14.4

CABIN PRESS lt on	10000 +- 500

CABIN PRESS lt out	8000

O2 sys time @25k’	3.5

O2 sys press at 70oF	1500

Walk around bottle work/rest	 5-10/ 22 min

EDC Max press differential	35

Oil Temp

Norm Range	60-90

Max Temp	100

Oil Hot Light	100

Max time OIL HOT gnd	30 min

Max time OIL HOT air	5 min

Min to start using 23699	-40

Min to start using 7808	-54

Other

Amount of ice to use deice	½”

Max alt to start JP-4	35000

Max alt to start JP-5/8	25000

Max aft CG limit	32%

Wind for opening radome	20kts

Max boost out AOB	20

Rack Overheat 	ON-135 oC  / OFF-128 oC

Fuel System

Fuel Boost Pump BOOST lt	2

Trans pump PRESS LOW lt	4

Normal Fuel Boost pump press	15-30

TD Start Limiting temp	830o

TD Temp Limiting temp	1077o

Fuel heater strainer	40-70of

Fueling per hose	300 GPM

Coord / Blade Angle

Max Reverse	0 / -13o

Ground Idle	9 / -6o

Ground Start	15 / 0o

Pitchlock reset	< 28o / >10o

Sync & Flt Idle Stop	>32o coord

Oil Cooler Inducers	<32o coord

Flight Idle	34o coord

Low RPM solenoid	36o coord

Flt Start & LGWS	(153) 48o coord

Auto FX & LGWS	(142) 60o coord

TD System	66o coord

Max Power	90o coord

Beta Follow up (@90 cord)	22.5 B/a

Pitchlock Cammed out	<17, >57

Force link Tab

Down 5 when trim up 21-25

Up 15 when trim down 5

Weights

Recommend T/O	135,000

Overload T/O	139,760

Max Landing	114,000

Recommended Landing	103,880

14th Stage Bleed Air

W – Wing De-Ice

E – Engine Starting

E – Engine Anti-ice

B – Bomb bay heat

O – Oil cooler

C – Closes 5th and 10th stage bleed air.

Items on RGB

S- Starter (Engine)

T – Tachometer

O – Oil pump and filter

G – Generator

E – EDC 2 & 3

Items on Accessory Section

F – Fuel Control

E – Engine Fuel pump

S – Speed Sensitive Control

S – Speed Sensitive Valve

M – Main Oil Pump

O – Oil Filter

S – Scavenge Oil Pump

E-Handle Functions (Elec/Mech)

F – Fuel Shutoff @ FCU – (E/M)

F – Fuel Tank Emer Shut Closes – (M)

F – Feather Valve put to FX – (M)

F – FX system activated – (E)

B – Bleed Air Shutoff Valve – (E)

O – Oil Tank Shutoff Valve Closed – (E)

G – Gen Cooling Air Shutoff Valve – (M)

E – EDC Dumped & Firewall Valve – (E)

Max Range TAS

½ GW + 205 – Alt

-10 kts for 3 eng / -20 kts for 2 eng

Inc 30% of HW / Dec 25% of TW

Step Climb 925

900 TIT w/ TW

890 TIT w/ no wind

880 TIT w/ HW

Hydroplaning

Dynamic= √tire pres * 9

Viscous / Rubber Reversion – as low as 5 kts

Angle Of Attack

Max Range	9

Loiter (flap up)	10.5

Loiter (Maneuver)	9.8

Max Endurence	12

Approach	12

Minimum Fuel Flow	14

Best Angle of Climb	14

Ditch	15

Max No Flap	15

Stall Buffet	19-22

Flap Positions

Maneuver – 40% - 10o – 275 kts

T/O & App – 77% - 18o – 190 kts

Land – 100% - 40o – 170 kts

Gross wt G-limits Symmetric Roll

Less than 125k                  0/ 2.5      1/2

More than 125k                 0/2.0       1/1.5

TIT exceeds 1175 for 5 seconds

Ground	reduce, shutdown, inspect

T/O	Reduce to 1077, record, inspect

Inflt	Reduce to 1010, record, inspect

Electrical

Generator Output	120v 400 Hz

APU 90 KVA output/ Eng 60 ground 90 air

TR output	28v 200 amp

TR overheat	150

Battery Rating	24v 31amp

Prop Protective Devices

Beta Follow-up	11 deg@FI / 22.5 deg@90

Brake	released (21%), applied, locked,

Auto FX	 Less than 500lbs/ no othr FX bttn in/ abv 60 deg cord/ armed. 4, 1, 3, 2

Pitchlock	>103.5%, loss fluid (<17,>57)

Pitchlock Reset	resets to 109% (>10,<28)

Low Pitch Stop	13 deg BA(<28)

Air Start Protection (45deg)  	FX btn out, PCO not depressed, BA less than 45 deg.

NTS	2/3 (-150,-500) 1/4(-100,-500)(<24)

Decouple	 -500 to -1700

MOSA

1000’ abv, 30nm (every 5, then  every 1 inside 10)

SS3 Shall verify all Nav MOSA calls

RADAR can be secured by MC

Can be modified by PIC when Vmc

Transfer Relays

2/3 A – 2 to A & 3 to B(#2/3 sup ACR/PMG)

2B – alt to A (Bus A cont. MEDC)

3B – atl to B (Bus B cont. MEDC)

4 – Ext Pwr/Gen 4 (EXT PWR Cont GOB)

5 &6 –2&3 to backup (gen no.4 xfer MEDC)

7 – Gen 4/APU (Gen for Aux cont FEDC

Bus Priorities

Bus A – Gen 2, E, 4, A, Gen 3

Bus B – Gen 3, E, 4, A, Gen 2

MEAC – Bus A, 2, E, 4, A, Gen 3

FEAC – Gen 2, E, 4, A, 3, Bus A

Start AC – MEAC, SEDC inverter

Loiter Requirements

NTS check

All buses energized normally

Top 3, Bottom 3

Oat > 10 deg with visible moisture

Flight AC System

P – Pitot Heat (1st indication)

O – Out flow valve

S – Standby Vertical Gyro

S – Standby Attitude Indicator

F – FDI

I – IFF

R – Radar Altimeter (off Flag)

E – Essential Indicator light

Fire System

Temp for zone 1	600of

Temp for zone 2/3	400of

HRD N charge psi	600 psi

Flight DC Systems

B – Bell (command)

U – Utility light

G – Gen 4 Aux Control Circuit

G – GOB

P – Pitchlock Reset

I – Power Inverter Relay

I – IFF

I – INS #1 & INS #2

T – Turn Needle

Bus A Systems

Gen 2 off

Hyd pump 1

Boost Pump 1+3

Bus B Systems

Gen 3 off

Fan out light

Hyd Pump 1a+2

Boost pump 2+4

MEAC Systems

Right Pitot heat

TIT off

SHP

Flight Station lighting

MEDC Systems

AOA off

No Annunciator Light

No ICS

MDC Systems

Hyd Pump 1+2

Oil quantity/temp

Landing Gear Indicators

Hyd Quantity

Fan out

All boost pumps

*Rack Overheat

EMDC Systems

Same as MDC but no RACK OVERHEAT

SEDC Systems

Ground Air conditioning

Bleed air valves

Engine anti-Ice

9% SHP drop

5% increase in Fuel Flow to compensate

500 lbs/hr if using wing and engine anti ice

When to use Hi windshield heat

Ice on windshield

Ice is anticipated and OAT is 0 to –25

Below 10000 in high bird activity

Fog on window

Ice control times

Prop de-ice	160 sec

Emp de-ice	176sec/44 sec for test

Wing deice	20-30 sec with 10 deg rise

Fuel log required

> 6 hours

> 500 NM from alternant

Fuel gauge malfunction

Minimum runway length

6000 ft for touch–n-go, short field down to 5000’-below 5K you need CO’s permission