NAVIGATOR BRIEFING

CREW COORDINATION

- ESA

- Orbit Point (if applicable)

- Combat Entry Checklist following After T/O Checklist

- THREATS – Bullseye Location

  - CEP/CExP

  - Handoff Navigation

  - Navigator Plot

IMMEDIATE – Between _ and _ Bullseye radials

Make course correction

Tell Pilot where we’re going

DELAYED

Co-Pilot Plot/Compare

Tell Pilot Threat Location and Plan

- Co-pilot IDCU in Enroute 4-5 or Airdrop 1-3

- Co-pilot Leg Timing/ Navigator Update Timing

- AWADS FCI CONTROL

  - Inside 10NM to the turn – load SKE turn only

  - Pilot has preps after 30 sec “SD” prep-Escape “E”

  - ELEMENT LD – Don’t pass/verbalize LDs drop preps

  - 5 sec and Execute – pilot verbalize “Passed”

- Takeoff Timing (“Timing”, 5 sec, Ready…Ready…Roll)

- Abort – Navigator on Interplane/Co-Pilot on ATC

  - Confirm VHF/UHF

  - Backup Co-Pilot on ATC

# ROUTE STUDY

# Leg Briefing prior to 5 NM to go (AWADS/SKE-10NM)

  - Next point (VIS-Description; AWADS/SKE-RAD/DME)

  - Outbound Course, Distance and Altitude

- The Turn

  - VISUAL

    - First person call in sight with clock position

    - Second person call in sight with distance

  - AWADS/SKE

    - 30 sec prep – IN-TRACK

    - LEADs Execute – “Timing”

    - Backup Computer Turn timing

  - Call “Ready, ready, turn” with two indicators

    - Timing, SCNS, Visually-RAD/DME

- Rollout Briefing

  - Drift and Drift Killed Heading

  - LEAD’s Time Status to Next Point

  - Next Point flying to or next update point

  - VIS – Give the pilot something to fly to

  - What LEAD will do to correct

  - Time control – Ahead/Behind at Low Approach

    - Wind Analysis

    - 1 Min with single airspeed adjustment

    - Remainder with off-course maneuvering

XTK > 5NM – New MSA

    - Airspeed adjustment points

      - At initial acceleration

      - 20 Minute Advisory

      - 10 Minute Advisory

- Call “Ready, ready, hack” at update points

  - Distance to go

  - XTK

  - Description of update target

- Advisories

  - Pre-Slowdowns at 20 min

  - Back me up on “advisory/briefing”

- BIG PICTURE Route Study

# RUN-IN/DROP ZONE

- IP Identification (i.e. Pointers, Description)

- TP at Slowdown

- Run-in Checkpoints and Terrain

  - XTK in Yards (1NM=2000 yds)

  - SKE – In-Track at LEADs 1 Min and maintain

  - AWADS

    - XTK > 200 yds/nm – Echelon

      - First to see it call Echelon Heading

      - 20 degree cut – based on winds

      - Average rollout @ 500 yd XTK

    - Updates – when to expect them

      - “Updating Ballistic Wind/Cursor”

      - CDI Effect

No “Update” call – Drift killed hdg

      - “Cleared to Chase”

Element LD – on drop altitude

    - “Drift relayed” > 2 degrees OR 3 to 4 degrees

    - Run-in verbiage

“30 sec to SD – passed”

“5 sec to SD – minus prep; SD Now – execute”

“5 sec to descent – down prep”

        - “Inside DZ Entry”

        - Pilot check “In position” all aircraft

“Execute” descend to Drop Altitude

“1 MINUTE ADVISORY – down prep”

“5 SECONDS – down prep”

“GREEN LIGHT – execute – Timing”

“RED LIGHT – Timing”

“30 sec – plus prep”; “5 sec – plus prep”

“Escape – execute”

    - Talk the Box – “Time to go (SD/GL)-Drift-XTK”

    - “On Course” within 25 yards (Course after GL)

  - Minimum of 3 update points – BIG PICTURE Review

  - Slowdown Visual Reference

  - Drop Zone Restrictions – Doors, Escape Timing, Racetracks

- DZ Identification

  - First person call in sight

  - Second person in sight with position and distance

  - One Minute Advisory Checkpoint

  - “NEW CARP” – Repeat the Updated CARP

  - Where I think you are.  Where to go.

- Drop Card Review

  - Cover Drop Card Items – Drop Altitude: + 50’/Element

  - Confirm Block letter or No-Drop signals

  - AWADS “No Drop”

    - Our aircraft – SEND drop preps

    - Element – Call “Element No Drop” – NO preps

    - Formation No Drop – SEND No Drop prep

  - Special “No Drop” – Range Clearance, WX, NVG (+,+,E)

  - DZ Size and PI placement

  - CARP

  - Wind Vector – Expansion/Compression

  - 5 Second reference point

  - Safety Box Identification

    - LEAD AWADS-200yds L/R; 2 sec

    - AWADS Wing- LEADs XTK > 400yds diverging

    - VISUAL

  - Wind Limits

  - SALVO Procedures (CHECK FIRE, Airfields?)

# RECOVERY

- Escape Timing Deviations – Expansion/Compression

- Assume VIS procedures location/signal

- Timing for Acceleration (if applicable)

- Recovery Route Study

- Combat Exit Checklist

- IFR

  - Computer ARA

    - “Update Cursor”

    - Clear to Chase”

  - NDB PT – Expansion/Compression

- VIS - DW or OH: “Hack for _ sec”- “Timing”

- Timing for Landing (Minimum/Desired)

  - VIS – 15 sec/20 Sec

  - AWADS In-Track – 5000’/6000’

THREAT POSSIBILITIES – AVIODANCE/TIME CONTROL PLAN