CREW COORDINATION / MISSION STUDY GUIDE

1.  INTRODUCTION

P / CP / N – Training requirements / Seating

N - Route study

N / CP - Form 70 / Stick Diagram with times, waypoint #’s, and bullseye

2.  CREW COORDINATION

GROUND

P – Walk around, SKE , Nav aids

CP – Radios & Nav aids, Have Quick, Secure Voice, Mode IV.

N – Mark load (date & callsign), Check chutes

P – Stations/Comm Check time | Crew brief time (Stations – 20?)
	NVGs – Taping Complete /  NVG-AL – Check Radar & Radar Alt (P & N)

NVGs – Extra set / Extra Batteries / Lip Lights / Battery Packs

PRIOR TO TAXI

P / CP -  plan (especially DZ coord.),  Predicted CDS flaps  & SKE timing

SKE – CP - Parameters - (set for invertent wx penetration)

DEPARTURE

FCI  prior to departure

N – (Wing) call “timing, 10 sec, 5 sec, ready ready hack” (phantom spacing?)

N - Combat Entry , Non-flying pilot briefs the drop card

CP - all ATC radios / P – interplane

CP / N - Abort calls (CP: ATC / N: Interplane)

SKE -CP - ADI: Radial/DME;  T.O. in SKE unless leading

ENROUTE

CP / N - Positive transfer of Navigation

CP / N -  Call out objects by clock position and distance (walk me to it)

CP – Identify items or terrain features on centerline along flightpath

P / CP ADI: VIS - both fly in SCNS

N / CP - Chart read during off-course maneuvers, AVOID Restricted Areas,

- Avoid Class C / D airspace; use CTAF / Traffice advisories as req.

- Timing

CP - Leg times

N – Time from update points /  TOT control (TOA?)

N – Airspeed changes –vs- off course maneuvering (Update MSA)

P / CP / N – Nav 4-5 page, Airdrop page?

- Turn Point Brief  (5 NM)

N - Course (predicted heading) / Distance / MSA /  Night Altitudes

- VIS Turns

Concur on TP /  TALK ME ON TO IT / Nav can pass the turn call off

Turn call example: “Ready, Ready, Right turn heading 245”

On rollout – SCNS XTRK / Winds & Drift/ Lead’s time status

- SKE Turns

SKE -P / CP / N  –VERBALIZE ALL PREPS

SKE -N - I get 5 sec and E preps as well as SD thru escape (advise on preps)			SKE -CP - give intrack distance at 30 secs

SKE -P - Will work for position until 5 sec and make time adjustment

SKE -P / CP - Pilot not flying watch PPI and call trends / directive calls

SKE -P / CP - Turn call example: “Ready, Ready, Right turn heading 245”

SKE -N - On rollout – Advise XTRK / Winds & Drift / Lead’s time status

NVGs – CDI: 1 dot (1.5nm) – Check position & Hdg | 2 dots (3nm) – CLIMB / Turn

NVGs – Segmnt Alt Calls – “CLIMB to ___” prior / “Clear DOWN to ___” after

NVGs FAIL – Through SD: Maint Posit | Inform LD | Clmb to MSA | Xfer Cntls | Spare Set

THREATS

-If notified via ABCCC or AWACS (AUTHENTICATE)

N / CP - identify location of threat while the other navigates then compare // P’s Stick Diag

N / CP - Formulate plan // Rendezvous Page? // Tell LM where to look // Update Plan!

-Pop up threats

N / CP – SCNS coordination and techniques (Mark Pt / Bulleye in Immed or WP 60?)

N / CP - Mark new threat on chart

CP - Inform Formation  & ABCCC

P – Discuss evasive maneuvers and crew coord

RUN-IN

N - Hit IP upwind on drift killed heading (TP?)

CP - Make sure we have DZ freq. set / Range clearance – if required.

P / CP - Memorize key terrain features  //  micro navigate

SLOWDOWN - location / signal / what will SCNS read at our SD?

-Recheck drift and winds (>3° during Vis = intrack with DZ)

N  -  After winds, UPDATE CARP ASAP - Notify the crew! (CP update safety box)

N / CP - Check wind limits for drop

P / CP / N -  Positive DZ identification  - (1st notify /  2nd notify w/ clock and dist.)

N / CP – Check predicted GS -vs- actual GS and update usable DZ time

SKE - N – Echelon Turns: 200yd/NM  What will SCNS Read @ IP? Decide EARLY!

SKE - N - Mark 50 foot addition to drop alt. by element & Drop alt. AGL

SKE - N - DZ entry point, what will our SCNS read

SKE - CP – Reset Secondary Control Just INSIDE the IP

NVGs FAIL – Past SD: Vis w/DZ? Can continue  Maint Posit

Otherwise  Brk Out | Inform LD | Clmb to MSA | Xfer Cntls | Spare Set

DROP

N - put me where you want me (Be Assertive, I should respond to you)

P – Tend to listen to CP when they speak due to safety and drop clearance

P / CP / N - “NO DROP  PARAMETERS / SAFETY BOX”

CP - “Two handed” for all drops

N / CP – Verbally verify Block Letter!

N – Call five seconds on time and not early. (GPS backup techniques)

CP - CDS, after “Load clear” if I don’t say “Flaps 50%”- 1 Pimp Then Track to 50%

Malfunctions - CP - red light / configuration /  (70%) / alt & airspeed / radio call

N - racetrack / salvo -- get us there and run all checklists for subsequent drops

SKE - N –Make sure we get advisories (DN PREP)   (CP help)

SKE -CP – Reset Secondardy Control after RED LIGHT / “LOAD CLEAR”

SKE - P / CP / N - “NO DROP  PARAMETERS / SAFETY BOX”

ESCAPE

Heading / altitude / airspeed

N – Acceleration

SKE - N / CP – CAT (Climb - Accelerate - Time/Turn)

RECOVERY

N - Combat Exit  , Descent 

Type / altitude / approach seperation

CP – Ensure we get clearance for the next route – if required

N – OH & DW, hack 12 seconds for turn and 15 secs for landing dist.

SKE - Navaids – ADI Mode Sel Switch after final turn within 18 DME

SKE - CP – Cleared to set multiples of 6, 12 or 18 off of lead during separation

NVG – AIR LAND

T/O – CP Call 40 50 60,  N – Call 100 200 300 400

– NVGs FAIL – BLO Vr: Lights & Abort | ABV Vr: Lights & ITO or Xfer Cntls

LND – CP – Call  “-3/OnSpd/+5 | __ Down”, N – Call Dist to LZ & Glideslope

– NVGs FAIL – P / CP inside 2NM  Call Go Around | Otherwise: Advise Crew