# SCNS AVIONICS DIAGRAM

C-130E(H) Acft. / Ref. TO 1C-130-1-4

## MLS 1 & 2

P-1

C-2

## HF 1

## HF 2

## IDCU

## IDCU

## TAC 1

## TAC 2

## E-DC

## E-DC

## UHF 1

## VHF 2

## REF PT

## VOR 1

# N-3

BICU

## VHF 1

## FAIL HIS

## IDCU

## VOR 2

## PRESET

## GPS BKP

## E-AC

## E-DC

# DATA BUS

# (1553)

## UHF 2

## INS BATT

## TAS CMP

## UPDATE

# INU

DVS

## I-DOP

## I-INS

# GPS

INS

VERT

REF

## E-AC

## E-AC

## E-AC

## AA

## INS BATT

## AA

## I-GPS

## GPS

VERT

REF

# SCNS NOTES AND LIMITATIONS

Alignment:

First usable alignment is DEGRADED NAV.  Occurs 90 sec after a medium accuracy alignment is initiated.

Full medium accuracy alignment (ground) is complete within 8 minutes.

Enhanced interrupted alignment allowed only once during each power up INU cycle.  Must be initiated within 10 min of the initial transition of INU to NAV mode and takes 4 minutes to complete.

Must make aircraft heading change of 70 degrees or more.

| Time (Min) | 1.5 | 2.0 | 2.5 | 3.0 | 3.5 | 4.0 | 4.8 | 6.3 | 8.0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INS CEP | 7.2 | 6.4 | 5.6 | 4.8 | 4..0 | 3.2 | 2.4 | 1.6 | 0.8 |
| SCNS CEP | 4.5 | 4.0 | 3.5 | 3.0 | 2.5 | 2.0 | 1.5 | 1.0 | 0.5 |

Power Interruptions:  Less than 5.0 ± 3.5 sec, comm. and nav radios return to their pre-interrupted state.  If greater than 5.0 ± 3.5 sec, the radios are powered off, but all control settings are retained.  In some cases, TESTING may be displayed.

Special Waypoints:

Drop Zones – 8 – (DROPZONE)

Time Over Target – 6 – (TARGET)

Landing Zones – 6 – (LZONES)

Air Align:

Requires valid DVS and/or GPS (P-coded) inputs to complete AA