FOR REFERENCE ONLY

Mumbles’ SCNS SETUP GOUGE for Copilots

1.  Turn on SCNS (by NAV station): pull 3 switches to up position (SCNS, INU, DVS) from right to left *ensure INU switch goes all the way up!  Halfway up gives you 90 degree error*

2.  Verify present position (from copilot/pilot SCNS IDCU): TEST (then compare SCNS coordinates with present parking spot’s in 79 RQS Tactics guide pg 3-3).  Press VERIFY if present position correct, otherwise input correct north and west coordinates and then VERIFY.

3.  Medium accuracy alignment: NAV > INIT > INS ALIGN 1-2 press COMMAND MODE once (upper right screen button).  Alignment complete after 8 minutes.  Press COMMAND MODE button again prior BEFORE TAXI checklist or prior to brake release to complete alignment.  Once the button is pressed, you have 10 minutes to initiate enhanced alignment if desired.

4.  Set GPS time:  NAV > ↑(page up) > GPS STATUS > ↑(page up) from here, press lower left (UTC) button twice to set the UTC time on SCNS.

5.  Clear flight plan: NAV > INIT > FLT PRMTR 1-3 hit CLEAR PLN twice to VERIFY (the upper right screen button) This action deletes flight plan data from the previous flight.

6.  Enter waypoints:  PLAN > WAYPOINT   here you can enter waypoints manually (DEF TYPE defaults to E for manual north and west coordinate entry)--VERIFY after entering coordinates for each waypoint.  Starts with WPT 1. Press ↑(page up) to cycle to the next WPT.

7.  Set SNCS to flight plan mode: NAV > MODE CTRL  under FLIGHT MODE, select FP on this page.  NOTE: If you are cleared direct to a waypoint in flight, this is also the page where you enter the new waypoint (directions at bottom of page)

8.  Check flight plan: PLAN > CHECK PLN  here you can verify courses and distances with your printed flight plan.↑(page up) to see more points.

-----USEFUL IN-FLIGHT TIPS-----

- IMMEDIATE page --NAV > IMMEDIATE  This single page provides an accurate Mag Course (MC) direct to any one waypoint from your present position--simply enter this waypoint as one of the following DEF TYPE choices:

1. coordinates: “E” is the default DEF TYPE choice for coordinate inputs, so 1)enter the north and west waypoint coordinates manually, 2)press VERIFY.

2. waypoint: select DEF TYPE “W”, then enter waypoint # from your flight plan you wish to steer direct to.  …For example, if you want to go direct to D-M which is WPT #5 on your flight plan: 1) press upper left button (DEF TYPE) to select “W”, 2) press upper right-hand button, 3) enter “5” on number keypad, 4) press VERIFY.

3.  reference point:  select DEF TYPE “R” Only use if reference points are loaded.  1)Enter identifier (such as DMA for D-M) by selecting upper right-hand button and entering on letter pad.  2)Press VERIFY.

The IMMEDIATE page is static, so in order to update, press RCL button on IDCU then IMMEDIATE again.  This will provide an exact MC, distance, and ETA/ETE (based on current GS) from your present position to the point entered on this page.  Great backup for a fix-to-fix.  **NOTE: IMMEDIATE page entries will not effect SCNS steering unless IMMEDIATE is the mode selected on the NAV > MODE CTRL page.

- SEQUENCING SNCS or GOING DIRECT to WPTS  NAV > MODE CTRL.  Here you can sequence SCNS or have it go direct to any waypoint in the flight plan.

-To sequence SCNS,  press the SEQUENCE button on the right as many times as you wish.  You will see it cycle 1-2,2-3,3-4,4-5, etc. as it cycles through the from/to flight plan waypoints.

-To cycle SCNS direct to a waypoint, press the button just below where you normally input the waypoint sequence.  Press VERIFY.

--Ok, an example: let’s say SCNS shows the current sequence is WPT 2-3.  We are cleared direct to D-M, which is waypoint 5.  Simply enter “5”, press the button (it will show 0-5), then VERIFY.  SCNS is now going direct to waypoint 5.  To find out the new MC, go to NAV > ENROUTE 3-5.  The direct MC is displayed at the top of the page, to the right of “0-5”

- TURNING OFF SCNS: 1) Zeroize SCNS: NAV > INIT > GPS INIT > ↑(page up) DAILY KEY: zeroize, then VERIFY  2) Turn OFF (by NAV station): pull down/turn off DVS and INU switches from left to right, wait 10 sec., & turn off SCNS switch (get clearance from engineer)