Flowchart: SSE Pattern

* Note: Case 1: Check for 1900 RPM after step 3.

1) 'Left Chip Light'

- 'Secondaries?'

- Power Up

- Rudder Up

- Clean up (Until Case 3)

- Speed up (Case 1)

- 'Left Power Lever Idle, Concur'

(Check Max on remaining engine)

- 'Left Prop Feather, Concur'

- 'Left Condition Lever Fuel C/O, Concur'

- 'You have the comms, declare an emergency, give me the first two items of  the dead engine checklist, preload the starter-assisted airstart (no secondaries)'

- Approaching 180: 'Flaps Approach, Gear Down, Landing Checklist'

2) 'Low Oil Pressure'

- 'Whats my Oil Pressure?'

- Power Up

- Rudder Up

- Clean up (Until Case 3)

- Speed up (Case 1)

- 'Left Power Lever Idle, Concur'

(Check Max on remaining engine)

- 'Left Prop Feather, Concur'

- 'Left Condition Lever Fuel C/O, Concur'

- 'You have the comms, declare an emergency, give me the first two items of  the dead engine checklist, preload the starter-assisted airstart (>40 psi)'

- Approaching 180: 'Flaps Approach, Gear Down, Landing Checklist'

3) 'Fire - Left Engine'

- 'Secondaries?'

- Power Up

- Rudder Up

- Clean up (Until Case 3)

- Speed up (Case 1)

- 'Left Power Lever Idle, Concur'

(Check Max on remaining engine)

- 'Left Prop Feather, Concur'

- 'Left Condition Lever Fuel C/O, Concur'

- 'Left Firewall Valve Closed, Concur'

- 'Left Fire Extinuisher Required, Concur'

- 'Close Left Bleed Air'

- 'You have the comms, declare an emergency, give me the first two items of  the dead engine checklist, preload the starter-assisted airstart (no secondaries)'

- Approaching 180: 'Flaps Approach, Gear Down, Landing Checklist'

4) 'Fuel Leak - Left Engine'

- 'Fuel Leak or Siphon?'

IF SIPHON: <140 kts, land.

IF FUEL LEAK: Continue.

- Power Up

- Rudder Up

- Clean up (Until Case 3)

- Speed up (Case 1)

- 'Left Condition Lever Fuel C/O, Concur'

- 'Left Power Lever Idle, Concur'

(Check Max on remaining engine)

- 'Left Prop Feather, Concur'

- 'Left Condition Lever Fuel C/O, Concur'

- 'Left Firewall Valve Closed, Concur'

- 'Left Fire Extinuisher Not Required, Concur'

- 'Close Left Bleed Air'

- 'You have the comms, declare an emergency, give me the first two items of  the dead engine checklist'

- Approaching 180: 'Flaps Approach, Gear Down, Landing Checklist'

5) 'Prop Malfunction - Left Engine'

- 'Whats my Prop RPM?'

IF 2200/NC: Manipulate Left, Match Right.

IF >2200: Continue.

- 'Can I adjust it to a normal operating range?'

IF NOT: Continue.

- Power Up

- Rudder Up

- Clean up (Until Case 3)

- Speed up (Case 1)

- 'Left Power Lever Idle, Concur'

(Check Max on remaining engine)

- 'Left Prop Feather, Concur'

- 'Does the Prop Feather?'

IF NOT: Alt Feathering Checklist.

- 'You have the comms, declare an emergency'

- Approaching 180: 'Flaps Approach, Gear Down, Landing Checklist'

6) Sudden Loss of Power - Rt. Wing high.

- Power Up

- Rudder Up

- Clean up (Until Case 3)

- Speed up (102-110) (Case 1)

- 'Left Power Lever Idle, Concur'

(Check Max on remaining engine)

- 'Left Prop Feather, Concur'

- 'Left Condition Lever Fuel C/O, Concur'

- 'Is it fire or fuel related?' (Props 1900)

IF SO: Continue.

IF NOT: Go to Comms Transfer.

- 'Left Firewall Valve Closed, Concur'

- 'Left F.E. (Not) Required, Concur'

- 'Close Left Bleed Air'

- 'You have the comms, declare an emergency, give me the first two items of  the dead engine checklist'

- Approaching 180: 'Flaps Approach, Gear Down, Landing Checklist'