The Condensed T-44 NATOPS 
Last updated Apr 02  Not sanctioned by VT-31 so use at your own risk.

PT6A-34B Engines General Info 
550 shp reverse flow, free turbine type 
3 stage axial, single stage centrifugal compressors 
7:1 compression ratio 
# of igniter plugs:                2   (3 and 9 o’clock)
Primary spray nozzles:       10 
Secondary spray nozzles:   4 (makes 14 total) 
2 stage planetary red: 15 to 1 
Location progressive bleed valve: 5 o’clock 	Opens approx: 62% N1 
	Closes approx: 75% N1 
Auto-ignition: Armed  <410+/-50 torque 
Fire ext press @ 70°F: 450 psi (see p. 3-8) 
Agent: 2.5 pounds bromotrifloromethane           Torque 
Normal		400-1315ft.4bs 
@ 2200 rpm 	1315 max 
@ 1900 rpm 	1520 max 
Torque varies linearly between 1900-2200 rpm N1 Turbine Speed 
Normal			50-101.5% 
Max N1 101.5% = 38,000 rpm compressor speed 
Acceleration		102.6% for 2 sec 
Reverse 			86% 
Gen load to .5 		50% 		     .5 to .75			57% 
.75 to .9 			60%
.9 to 1.0			63%
Every 10° be1ow -30°C:	Reduce max N1 2.2% 
Low idle 		50-53%
Hi idle 			64-67%
Gnd ops >3500’PA	Low idle as hi as 83% 
N2(prop) speed 
Normal			1800-2200 rpm 
Max 			2200 rpm
Reverse			2100 rpm
Acceleration 		2420 for 2 sec 
Idle 900-1100 (if not, do low pitch torque check) 
Test resets over speed gov 2000 ± 100 
Norm gov check 		1600-1800 
Over speed gov 		2288 +/-40 (+4%) 
Fuel topping 		2332 (+6%) 
Reduction gear box failure/turbine damage may 
result if N2 exceeds 	2420 
Synchrophaser limited gov +/-30 rpm 
Synchrophaser re-centers in approx 6 sec 
Propellers 
Governor oil pump 	375-400 psi 
Beta 			15° to -5° 
Reverse 			-5° to -11° 
Feather 			87°                        Low pitch stop 		15°  
Dc-ice amps 		14-18A

Each blade 		5-6A

Cycles every 34+/-6 sec, RT outboard lst   
Props at low pitch if:	RPM decays 
Auto-feather: Auto-feather switches - ARM 
Both PL at or above 90+/-2% arms sys. 
One eng <410+50 ft-lbs dis-arms other side, 
then <260+/-50 ft-lbs will feather prop. 
Oil System 
Measurable 		5 qts 
Tank holds 		9 qts 
System Total 		14 qts 
Allowable burn rate 	1/4 pint/hr 
Oil pump has 		2 gears 
# scavenge pumps	4 
If 3qts low: motor starter 15 sec & recheck 			If still low contact MX 
Oil to fuel heater 
Core vlv cls, bypass open 70°F fuel temp 
Core fully closed & bypass open 90°F 
Bypass valve diverts oil from exchanger 
Oil Pressure 
Normal 		85-100 psi above 72% N1 
Low idle 	40 psi min 
<85 psi undesirable; tolerated only for 
completion of flt @ reduced pw setting. 
Consider shutdown & landing ASAP; OR 
reduce pwr & land as soon as practical 
<40 psi in air unsafe; requires shutdown OR 
landing ASAP using min pwr to sustain flt 
<40 psi on gnd requires shutdown 
Oil Temp 
Normal 10-99°C 
Lo & hi idle -40to99°C 
Starting -40°C mm 
>99°C unsafe; requires shutdown or landing 
ASAPossible using min pwr to sustain flt 
Interstage Turbine Temp (ITT) 
Normal 		400-790°C 
Climb 		765°C 
Cruise 		740°C 
Lo idle 		685°C (if>, reduce gen load 
		or increase Nl) 
Max Continuous 	790°C (emergency use) 
Acceleration	850°C for 2 sec 
Max starting 	1090°C Abort start 925°C 
Report amount/duration start ITT > 925°C 
Report amount/duration operating ITT > 790°C 
Max allowable pwr: max pwr for t/o; 5 ruin 
Max continuous pwr: max pwr not limited by 
time, emergency discretion of pilot 
Cruise climb pwr: max pwr approved for normal 		climb         	            Cruise pwr: Pwr required to maintain level flight a desired alt, norm speed.

The Condensed T-44 NATOPS 
Last updated Apr 02  Not sanctioned by VT-31 so use at your own risk.

Pneumatic 
Normal			16+/-4 psi 
Max 			20 psi
Suction 
Normal 3.0-5.9 in. mercury (ref Natops 4-3) 
Normal for engine run up 4.3-5.9 in. (ref FTI) 
Powers: deice boot deflation, suct. gauge, CP turn & slip 
Anti-ice/Deice systems 
Full preflight if: 	First leg of x-country 
		Icing conditions arc forecast 
Use lip boot heat		When icing expected 
Use prop heat		Before flight into 
			icing conditions 
Prop imbalance 		Increase RPM briefly 
Prop de-ice 14-18 amps sys. 34±6 sec cycle x 4. 
Min ice to use boots 1/2 to 1” accumulation 
Single anode time delay 7-8 sec inflate/deflate 
If 5°C in visible moisture: Auto-ignition - on 
(41°F) 			Ice vanes - extend 
Ice vane torque drop 	40-60 ft-lb 
Ice vane range decrease 	10-12% 
Windshield heat inop 	>95°F 
AOA heat 14 vdc gnd, 28 vdc air 
Gear System (ref. blue brain for switches) 
Extension time		4-6 sec 
Retraction time 		5-7 sec
Manual extension takes 	approx. 50 strokes
Nose-wheel deflection: 	12° left, 14° right

48° either w/ braking 
Warning if no gear &:	Both Nl <79+/-2% 
			or flaps full 
Warning indications:	Wheels up lights 
			Horn 
			Red light in handle

Controls 
Flap settings   app: 15°=35%   full: 43°=100% 
Aileron tab moves 	15+/-1.5°, relative 
Rudder & Elev trim	marked in degrees E1ectric elev. Trim: Copilot’s priority, anti-servo 
Fuel System 
No takeoff range 	 0-265# per side (yellow arc) 
Total capacity 		387.6 gal 
Usable fuel 		384 gal 
Outboard tanks 		132 gal per side 
Nacelle tank 		61 gal per side 
Nacelle switches 		59, 51,42 gal 
Fuel no xfr light 		2.5 psi; 30 sec delay 
Transfer pump rated: 	500 lb/hr 
Min nacelle fuel for xfr pump test:  42 gal

Grav feed starts when nacelle <3/8 full 			@ 150# fuel in nacelle 
Grav flow traps 		28 gal (190#) 
Boost pump nominal 	30 psi 
Fuel press light 		<5 psi 
Suction lift (no boost pump) limited 10 hrs TBO 
Avgas usage 		150 hrs TBO 
Before start fuel bal within 100# tot, 50# nac 
If eng shutdown by firewall valve, restart may 
exceed 10 sec and lack of lubrication during 
cavitation could damage engine-driven fuel pump 
Eng driven fuel pump press: 800 psi 
Electrical 
Battery 		24Vdc, 42 amp-hr 
Generators	28 Vdc, 250 amp-hr 
APU 	28V, l000 A for .l sec, 300A continuous 
System Voltage 	28.25+/-.8 
Min V for battery start 	22V 
Min V for APU start	20V 
Min V for APU charge 	18V (replace batt if 
			less) 
Inverters: 750 volt/amps 
Ac Volts 		114+/-7 and 26 V 
Ac Freq 			400+/-6 Hz 
Current limiters: 325 amp slow blow fuse 
Current limiter functions: tie main busses, fault 			protection 
Gen control boxes: paralleling, reverse current 		protection, over/under voltage 
Dual powered items: boost pumps, firewall 			valves, xfeed valve 
Single powered hot bus: L, R fire extinguishers, 		xfeed light, threshold lights 
		Threshold lights: baggage 			door, aft dome, threshold, spar 		cover, cabin door observer 
Batt functions: Emer pwr, damper for system, 		seIf.contained starts 
Pressurization/Environmental 
Outflow valve Limits 	0-4.7 PSID 
Safety Valve Limits 	0-4.9 PSID 
Valves are opened by  	suction 
Valves are closed by 	spring 
Valves are controlled by controller & solenoids. 
Cabin alt warn light 	9,500-10,000’ msl 
Cabin alt settings 		-1,000 to 10,000’ msl 
Rate knob settings 	50-2,000 fpm 
Set cabin alt to: 		500 msl above airfield 
30.40 - curr alt setting x 100Q = cab alt setting 
Crack windshield 	inner 2.5 psid 10 min 
			outer don’t use wipers 
			windshield heat arcs

Crack side window 	0 psid
Bleed air regulated to 	18 psi
Door seal psi 		4 psi

The Condensed T-44 NATOPS 
Last updated Apr 02  Not sanctioned by VT-31 so use at your own risk.

Dump switch: 	1. Opens dump solenoid			2. Closes preset solenoid 
		3. Opens rarn air flapper door

Test switch: 	Opens circuits to preset & 			dump solenoids (they go to 
		“normal” positions) 
Ambient air solenoid: 
	Closed by left sqt. sw. on ground 
	6 sec delay after take off to avoid 
	pressure bump 
Ambient air thermostatically controlled in flight Electric heat locked out by use of: 
		1. Lip boot heat 
		2. Windshield heat 
		3. Prop deice 
Max Cooling: 	Close bleeds, Vent blower-Hi, 		Open all overhead outlets 
Max Heat: 	Open bleeds, Vent blower-Hi, 		Close all overhead outlets, 
		electric heat - GND MAX 
Manual temp control may take 1 min 
Oxygen 
Capacity			49 cubic feet 
X-country min:		1500 psi
Local min:		1000 psi
System pressure	1850 +/- 50 psi @ 70°F

(add/subtract 3.5 psi per 1°F) 
Diluter control lever when stowed: 100% 
Lever when above 20,000’: 100% 
Radar 
160 nm range 
No trans: <50’ from metal surfaces 
	<18’ to person, flammable liquids, 
	or in vicinity of refueling 
Approx. 2 min warm up 
preflight if T-stms forecast or x-country

Airframe 
Main, nose tire press 	51-57, 50-55 psi Winds >75 kts can cause structural damage 
Max aft CG all weights	160” 
Max ramp weight		9,710 lbs 
Max T/O weight		9,650 lbs
Max land weight 		9,168 lbs
Navy approved ceiling	31,000’ 
Max G clean 		+3, -1 
Max G flaps down	+2,0 
Prohibited maneuvers	Intentional spins 
			Acrobatic maneuvers 
Landing limitations	600fpm 
			20 kt x-wind 
			Flared only 
			No arresting cable When crossing cable: not faster than taxi speed 
		no braking during rollover

VMCA			86 KIAS 
VSSE			91 KIAS 
VX VXSE			102 KIAS 
VY			108 KIAS 
VYSE			110 KIAS 
VLR			145 KL4S 
VLE			155 KIAS 
VFE 100%			140 KIAS 
VFE 35%			174 KIAS 
VA			153 KIAS 
Min in icing		140 KLAS 
VNE OR VMO		227 KS(-4kts per 
			1,000’ above 15,500’)

Loading-Seat tracks 	200# per sq foot
Loading Walkways 	100# per sq foot
Max cargo aft	      200# on floor, 150# shelf