INTERIOR PRE-FLIGHT CHECKLIST

FLIGHT STATION

CONTROL LOCKS.......................................REMOVED AND STOWED

CAUTION: CONSIDER WINDS BEFORE REMOVING CONTROL LOCKS

WINDSHIELD.........................................................CHECK CONDITION

OXYGEN..........CHECK (MIN 1,000/1,500 FOR CCX) CHECK MASKS

TRIM.........................................................................CHECK/SET/ZERO

FLAPS........................FLAP HANDLE AND POSTION CORRESPOND

PARKING BRAKE.......................................................SET/PUMP FIRM

EMERGENCY GEAR CONTROLS............................CHECK STOWED

START & SUTO IGN SWITCHED....................................................OFF

AVIONIC MASTER...........................................................................OFF

ENVIROMENTAL CONTROLS........................................................OFF

PROP SYNC.....................................................................................OFF

TRANSPONDER............................................................................STBY

BATTERY...........................................................................................ON

VOLTAGE..........................................................................................CHECK

FUEL QUANTITY/CURRENT LIMITERS...........................................CHECK

GEAR LIGHTS...................................................................................CHECK

THREE GREEN........................................................................CHECK

GEAR HANDLE..............................................DEPRESS HD LT TEST

WHEELS UP LIGHT.................................................PRESS TO TEST

EXTERIOR LIGHTS...............................................CHECK (AS REQ’D)

BATTERY.........................................................................................OFF

MALFUNCTION SWITCHES............................................................OFF

EMERGENCY EQUIPMENT.......................................................CHECK

OXYGEN MASKS

FIRE EXTINGUISHER UNDER RIGHT SEAT

LIFEVEST

CHECK RADAR SECURE IN MOUNTING

PASSENGER/CARGO COMPARTMENT

WINDOWS..................................................................................CHECK

EMERGENCY EXIT..........................................LATCHED AND FLUSH

EMERGENCY EQUIPMENT................................. CHECK (AS REQ’D)

PAX OXYGEN MASKS, LIFEVEST

FIRE EXTINGUISHER

RAFT

FIRST AID KIT

AFT COMPARTMENT...............CHECK ALL LOOSE ITEMS STOWED