Tac Check Study Guide

AIRDROP:

PERSONNEL:	Altitudes: 	Tactical training	800 ft

SATB-P		500 ft

Emergency Bailout	400 ft (static line)

Emergency Bailout	2000 ft (free-fall)

NOTE:	Advise jumpers if bailout must be made in excess of 150 KIAS.

Airspeeds: 	Static Line		130 kts

High Altitude		130 kts desired

(110 min-150 max kts)

Winds:		Air Force		13 kts

Water Jumps	17 kts

Army		DZSO discretion

High Glide Ratio	18 kts(?)

SATB-P		25 kts

NOTE:	If surface winds are unknown during personnel drops, advise the jumpmaster and army airborne mission commander when drop altitude winds exceed 30 kts.

DZ Size:	600 yd. X 600 yd. Minimum (Up to 1000 ft AGL)

Add 30 yds to width and length for each 100 ft above 1000 ft AGL

Add 75 yd for each additional jumper to trailing edge

For day vis formation increase width by 100 yds.

For SKE formation increase width by 400 yds.

Official sunset to sunrise increase width by:

Single-ship: 100 yds.

Vis Form: 200 yds.

Increase length by 100 yds.

Emergency:	If jumper is being towed by anything other than a static

line, the loadmaster will attempt to free the jumper.

Procedures:        1.  Jumpmaster will stop remaining jumpers and notify the pilot.

2.  Co-pilot will turn on the red light.

3.  Pilot will maintain drop altitude and airspeed, and avoid

flying over or upwind of water or built up areas.

4.  If jumper is tangled outside a paratroop door exit, the pilot

should lower landing gear then set flaps 100%.

5.  Avoid turning into the jumper.

Priorities: 	Paratroop Door-Retrieve the jumper.

Cargo Ramp & Door-Cut the jumper free.

Required Calls:	20 & 10 minute advisory (6 minute advisory optional)

Pre-slowdown Checklist, Slowdown Checklist,

1 minute advisory and 5 second advisory

Loadmaster 	Pre-slowdown Checklist:

Duties:		   -Jumpmaster is alerted

-Helmets are placed on

-CDS arming switch-Normal

-ADS ramp support arms are connected

-Aft anchor cables lowered (paratroop door exits only)

-Static line retriever cables are safety tied and checked

Slowdown Checklist:

-Jumpmaster is alerted

-Air deflector doors checked (paratroop door exit only)

-Open and lock paratroop doors after hearing co-pilot call

air deflector doors open

-Jump platforms locked in place

-Aft anchor cable supports lowered (tailgate jumpers)

-Positioning of the door bundles is the responsibility of the user

Notes:		Max paratroopers – 64

Tailgate Drops – 1 cable used per pass with 20 jumpers max

Forward throw of a jumper – 250 yards

EQUIPMENT:  Altitudes:	G-12D			650 ft.

G-12E			550 ft.

Door Bundle (G-13/14)	300 ft.

SATB-H			See type chute simulated.

Notes:	Airdrops at or above 300 ft. AGL will use high altitude chutes

Max airdrop weight::  42,000 lbs.

Forward throw of Heavy Equipment:  500 yds.

Airspeed:	All equipment – 140 kts.

Winds:		Air Force

Chutes with release assembly	17kts.

Chutes without release assembly	13 kts.

Army				DZSO discretion

SATB-H				25 kts.

DZ Size:		600 yds. X 1000 yds. Minimum (up to 1100 ft. AGL)

Above 1100 ft AGL: Add 30 yds. To width and length for

each 100 ft. above 1100 ft. (add 15 yds. To DZ sides)

Add 400 yd for each additional platform to trailing edge.

For day vis formation increase width by 100 yds.

For SKE formation increase width by 400 yds.

Official sunset to sunrise increase width by:

Single-ship:  100 yds.

Vis Form:      200 yds.

Increase length by 100 yds.

EQUIPMENT	For all considerations, the pilot will maintain drop altitude and

Emergencies:	airspeed, and avoid flying over or upwind of water or built-up

Areas to the max extent while turning towards the pre-briefed salvo area.

Loose Platform:	1.  Loadmaster will notify the pilot..

2.  Apply emergency aft restraint on all loads/platforms.

3.  Raise the aft anchor support cables.

4.  Clear and close the ramp and door.

5.  Re-lock/Re-secure all loose equipment.

6.  Completion of Malfunction/Drop Checklist.

Extraction parachute fails to release mechanically or falls on the ramp:

1.  Loadmaster will notify the pilot.

2.  Apply emergency aft restraint on load.

3.  Raise the aft anchor support cable

4.  Clear and close ramp and door.

5.  Re-lock or secure all platforms.

6.  Completion of Malfunction/Drop Checklists.

Load fails to extract with a single extraction chute deployed outside the

Aircraft:

1.  Loadmaster will notify the pilot.

2.  Apply emergency aft restraint from front to rear.

3.  Cut the extraction chute aft of the attachment point.

4.  Raise the aft anchor support cable.

5.  Clear and close ramp and door.

6.  Re-lock and secure all platforms.

7. Completion of Malfunction/Drop Checklists.

Multiple 28-foot extraction parachutes fail to release mechanically or falls

on the ramp:

1.  Loadmaster will notify the pilot.

2.  Raise the aft anchor support cable.

3.  Clear and close the ramp and door.

4.  Re-lock or secure all platforms.

5. Completion of Malfunction/Drop Checklists.

Load fails to extract with multiple 28-foot extraction parachutes outside

the aircraft::

1.  Loadmaster will pull the right hand rail control

handle to emergency.

2.  Loadmaster will verify the position of the left hand

rail locks (If engaged, loadmaster will sustain a

10-20 sec. Pull on the simul-handle).

3.  Loadmaster will notify the pilot.

4.  Prepare to land!!!

Loadmaster	     Pre-Slowdown Checklist:

Duties:			-Jumpmaster alerted (if required)

-Helmets are placed on

-CDS arming switch-Normal

-Forward loads not being dropped are secured

-Emer. restraint chains-removed/positioned

-Right rail locks-checked for proper settings

-Vertical restraint flanges-retracted/secured

-ADS ramp support arms are connected

-Load and extraction system are checked

-Left rail locks are sequentially unlocked

Slowdown Checklist:

-Jumpmaster alerted (if required)

-Loadmaster clears ramp and door to be

opened

-Aft anchor support cables-lowered (static line

system)

Required Calls:	Pre-slowdown Checklist

Slowdown Checklist

1 minute advisory

5 second advisory

Extraction Release Systems:

Static Line Connector Strap: 1,086 to 17,000 lbs. Max-Lapes

Type II platform

PEFTC:  2,520 to 15,000 lbs. max (platform extraction only)

EFTC:  2,520 to 42,000 lbs. max (item or platform extraction)

Emergency Aft Restraint Chain Requirement:

For each		15 ft parachute		2 chains

22 ft parachute		4 chains

28 ft parachute		6 chains

Double 28 ft.		None

CDS:	Altitudes:	G-12D (1-6 bundles)	475 ft.

(7+ bundles)	575 ft.

G-12E (Single chute)	475’ @ 130 kts/375’ @ 140-150

(cluster of 2)	550 ft

G-13/14 (1 or 2)		400 ft./300 ft.

SATB-C			Type chute being simulated.

Airspeeds:	Less than 120,000 lbs.	130 kts.

120,000 lbs. or greater	140 kts.

Winds:	Air Force

G-12D/E		13 kts.

G-13/14			20 kts.

Free fall/High velocity	Unlimited

Army				DZSO discretion

SATB-C				25 kts.

DZ Size:		400 yds. X 400 yds. Minimum (up to 600 ft. AGL)

Above 600 ft. AGL: Add 40 yds. to width and length

for each 100 ft. above 600 ft..

Add 50 yds. for each single/double CDS container set

up to 4/7-8.  5-8/9 or more add 700 yds.

For day vis formation increase width by 100 yds.

For SKE formation increase width by 400 yds.

Official sunset to sunrise increase width by:

Single-ship:  100 yds.

Vis Form:      200 yds.

Increase length by 100 yds.

Emergency	1.  Loadmaster will dearm the CDS/LAPES switch.

Procedures:	2.  Notify the pilot (give a brief description)

3.  Upon being notified, the Copilot will extend

additional flaps

4.  Alert parachutists to stay forward and clear of load.

5.  Raise aft anchor support cable.

6.  Clear and close ramp and door (if possible).

7. Completion of Malfunction/Drop Checklists.

CAUTION:  If the load is jammed in the ramp area,

Notify the engineer to stop closing action when

The cargo door releases from the uplock.  If

this is necessary, the loadmaster will close

the ramp after securing the load.

NOTE:    If the malfunction was due to a static line

Retriever failure, the mission may be continued

using the opposite static line retriever and

manually activating the retriever rewind switch

at FS 245.

Loadmaster	Pre-Slowdown Checklist

Duties:			-Jumpmaster alerted (if required *)

-Helmets are placed on

-CDS arming switch-Normal

-Forward barrier is checked

-Forward and aft restraint straps are removed

-Static line retriever cables are safety tied (*)

-ADS ramp support arms are connected

-CDS retriever cable and knife are checked

-Vertical restraint straps are removed

Slowdown Checklist

-Jumpmaster alerted (*)

-Loadmaster clears ramp and door to open

-Aft anchor cable support arms are lowered

-Static line retriever cable compression

spring-seated

-CDS switch is armed

Required Calls:	Pre-Slowdown Checklist

Slowdown Checklist

1 minute advisory

5 second advisory

Notes:	During training, 6,000 ft. in-track spacing is required when below

600 ft. AGL

Max CDS weight is 37,248 lbs (2,328 per bundle X 16 bundles)

Nose up attitude attained: 6-8 degrees

Max containers: 8-single stick / 16-double stick

CDS/LAPES switch: 3 sec retraction of static line at green light

Loads above 5,000 lbs. requires a buffer stop assembly for a

Single pass (limitation: 5,000 lb. tiedown rings on sidewalls).

Loads above 26,650 lbs. require the buffer stop to have

additional chains.  (limitation:26,650x3G’s=79,950

<4locksx20,000lbs. forward restraint)

The CVR provides all necessary vertical and lateral restraint

provided the container is secured to the skid board for 2 G’s.

CDS delivery not recommended below 104,000 lbs. (slow extr.)

Departing the DZ, the flaps should be reset to 50% as soon as

the loadmaster calls “LOAD CLEAR”

In case of overrun during formation CDS run-in, the pilot

will not reduce airspeed until clear of the formation with

flaps reset at 50%.

CDS platforms can be either 48”x53.5” without or 48”x48” with

the CVR.

Using 48”x53.5” platforms, any number can be dropped up to

8 in a single stick, above that they can only be in even numbers

(10,12,14, or 16 bundles).

Forward throw of CDS:  550 yds.

TRAINING BUNDLE:	Wind Limit:	25 kts.

Forward Throw:	160 yds.

UNILATERAL WIND CRITERIA: Unilateral Air Force training airdrops will not be

made when surface winds (including gusts) exceed the

drop limits within a one-minute period prior to the drop.

MISCELLANEOUS	-Wedge airdrops allow 1-6 containers to be dropped

AIRDROP NOTES:	  along with 40 troopers.

-During combination airdrops, notify the jumpmaster

if the point of impact falls within 150 yds. of any DZ

boundary.

-Equipment/CDS airdrops: ADS light must remain

illuminated throughout the airdrop run-in, otherwise

it is a “NO DROP”

-Door bundle containers can weigh up to 500 lbs. and

are limited to one bundle per exit used

0-100 lbs.	1 Loadmaster

100-350 lbs.	2 Loadmasters (or 1 with another person)

250-500 lbs.	3 Loadmasters (or 1 with 2 other people)

-Prior to proceeding aft to operate the ramp an door

controls, the loadmaster will ensure the lifeline is

attached and locked to a tiedown ring no farther

aft than FS 677.

-The navigator’s “30 seconds to slowdown” call will

initiate the slowdown checklist.

AIRDROP EQUIPMENT AND CAPABILITY:

Dual Rail Controls:	Left-hand locks	20,000 fwd/10,000 aft

Controls:  STOWED/OPERATIONAL/AFT RESTRAINT

RELEASED/SIMULTANEOUS

Right-hand locks	20,000 fwd/variable up to 4,000 aft.

Controls:  CHECK/NORMAL/EMERGENCY/LOAD

Cargo Ramp Capacity:	5,000 lbs. without dual rails installed

4,664 lbs. with dual rails

2,000 lbs. can be loaded across an unsupported ramp

provided the plane’s aft CG limit is not exceeded.

Cargo Compartment:	Pallet positions 1-4 max loads each of 10,000 lbs.

Pallet position 5 max load of 8,500 lbs.

Tiedown Rings:  25,000 lbs. Capacity: 12 on floor (access 2 with dual rails installed)

10,000 lbs. Capacity. 175 on floor (also on dual rails)

5,000 lbs. Capacity:  11 on sidewall (also on cargo ramp)

Cargo Parachutes:	G-11A:Flat, circular 100-ft. diameter canopy: 250 lbs wt

20 35-ft. type III nylon suspension lines.

G-11B:Same as G-11A except has 95-ft. long type V

Nylon webbing.  Note: Can’t mix G-11A and G-11B

on a load.

Cargo Releases:		5,000 lb capacity release – 20-second timer

Min/max wt. 1,300/1,500 lbs.  Releases when a

no load condition exists.

M-1/M-2 release: 12-16 second mechanical timer

M-1:  15,000 lbs. limit

M-2:  42,000 lbs. limit

Personnel Airdrop:	Maximum – 64 fully equipped paratroopers

(Two loadmasters required with more than 40 pax)

In-flight Rigged: 44 max.

All personnel airdrops will carry a minimum of 3

Parachutes.  2 will be made available for Army use.

Arctic Paratroopers: 40 max.

Cabin Altitude Check:	Check accomplished for airdrops above 10,000 ft. MSL.

Check will be performed at altitudes of 10,000 and

15,000 ft. MSL.  At a cabin altitude of 20,000 ft., it will

be accomplished every 15 minutes or with each 5,000 ft.

increase in altitude.  Above 30,000 ft, it will be

accomplished every 5 minutes.

EMERGENCY EQUIPMENT:

Hand Held Fire Extinguishers:	4 (1 aft of Rt. Paratroop door /1 Aft of left

Wheel well / 1 either side of FS 245)

Portable Oxygen Bottles:		4 (2 on flight deck / 1 aft side of FS 245 /

1 aft of right paratroop door)

Hand Axes:		2 (1 either side of FS 245 / 1 aft of right paratroop door)

Emergency Exit Lights:	7 (located by each emergency exit)

Will illuminate with:     1.  2.5 G’s deceleration

2.  Essential DC bus failure

Also powered by 4 D-cell batteries

Escape Ropes:		3 (One by each overhead escape hatch)

ELT System:	Installed in the dorsal fin.

Broadcasts on 121.5 and 243.0.

ELT is completely independent of aircraft power other than to

Reset or check the system.

Senses a decelerating impact force along the longitudinal axis.

AIRLAND:

Engine Running Onload/Offload:

Restraints:	After the aircraft is slowed to taxi speed, the loadmaster may

remove all tiedowns except for 1 forward and 1 aft restraint,

open the aft cargo door and lower the ramp to 12” above

horizontal

WARNING:	Do not onload or offload through the crew entrance door and

ramp or paratroop doors at the same time.

Combat Onload/Offload:

Method “A”:	Used for offload of single, multiple, ramp or married pallets,

Airdrop platforms, and CDS containers.

A taxiway or ramp at least 500 ft. long is required; however

1000 ft. is desired to provide a margin of safety.

This method can be used up to a weight of 12,000 lbs. (or

greater if excess is offset w/ballast at least equal to the excess

Ramp will be placed in the ADS position.

With the exception of small arms ammunition, explosives and

Munitions shall not be combat offloaded without approval

Of MAJCOM/DO (?)

Power setting for Method A is roughly 5,000 in/lbs. of torque

Method “B”:	Used for offloading married pallets that do not fit the category

For method

“A” or for which no ballast is available for married pallets over

12,000 lbs.

Use 4 serviceable 55-gal drums under each pallet to be offloaded.

The load weight on the ramp must not exceed 15,000 lbs.

Do not use for airdrop rigged platforms.

AIRLAND OPERATIONS:

Day VMC:	Plan a minimum of 300’ AGL modified contour altitude above the

terrain using visual references from the IP to a point where the

approach begins

Night VMC:	ARA’s will be planned for all night VMC tactical LZ operations.

IMC:		Except for contingencies, ARA’s conducted in IMC must use

approach plates published by the Defense Mapping Agency.

During contingencies, the DIRMOBFOR may approve IMC ARA

approach plates.

ASSAULT OPERATIONS:

Assault Takeoff:	Min Runway Length – Min field length for max effort takeoff or

3,000 ft.

Min Width – Runway 60 ft. wide and taxiway 30 ft. wide min.

Speeds:		Rotation – Takeoff Speed (Vmeto vsVmca)

Climb – Obstacle clearance speed or Vmca + 10 kts

Flap Retraction – Obstacle clearance + 10 kts / Vmca + 20 kts

Assault Landing:	Limits – Max weight is 130,000 lbs. (without waiver)

External fuel tanks empty

540 fpm sink rate allowed if gross weight is less than 130,000

(otherwise max sink rate is 300 fpm)

Outboard main tank fuel: 6,200 lbs. or less (215 psi struts)

5,000 lbs. or less (285 psi struts)

Total fuel 23,500 lbs. or less

Perform no more than 3 consecutive heavy weight landings

Without a 15 minute gear cooling down period.

NOTE:  A propulsion system check should be accomplished

When the first landing of the day will be an assault landing.

Night Heavyweight Assault:		Minimum weight is 115,000 lbs.

LOW LEVEL PLANNING:

Miscellaneous	Category II Route: Any route on which the position of the aircraft

can be accurately determined by the overhead crossing of a radio aid

(NDB, VOR, TACAN) at least ONCE each hour with positive course

guidance between such radio aids.

Category I Route: Any route that is not Category II.

Lead Crew: A mission ready crew on which the aircraft

commander and navigator have received lead training

prescribed by AMCR 51-130 and an evaluation in a formation

Lead position.

Airdrop Qualification: 	Any airdrop Qualified crew: Personnel – All altitudes

Equipment – 0 – 5000 ft. AGL

Equipment 5001 ft. AGL and above require one of the

following method: AWADS, GRADS, SCNS/ZM, or

radar beacon airdrop.

Formation Lead Requirements:

Unilateral Training:	VMC – Lead crew is not required.

IMC (single element form.) – One instructor a/c located

within the formation.  If this crew aborts, the

formation must either go single ship or abort.

IMC (Multiple element form.) – Lead crew or instructor

pilot is required in the lead and deputy serial position.

Element lead positions require an instructor pilot,

a lead crew, or AWADS crew.  Deputy lead or an

element lead can abort after stations time with any

crew taking their position (w/ MC approval).  Any

crew can fly last ship.

Other than Unilateral Training:  IMC & VMC – A lead crew is required in the lead

and deputy serial lead positions.  Element lead positions require

a lead or AWADS crew.  If deputy lead or an element lead aborts

after stations time, any crew can assume their position

(with MC approval).  Any crew can fly last ship.

Crew Duty Requirements: Combination Tactical or Pilot Proficiency and Airlift

Mission – 16 hrs if the tactical or pilot proficiency portion is completed

within the first 12 hours and a fully operative autopilot is available for

use during the rest of the mission.  Otherwise, Tactical and Pilot

Proficiency is 12 hours.  (AC cannot extend the 12-hr duty day for TAC)

Loadmaster Requirements: 2 loadmasters or 1 loadmaster and another qualified

crewmember are required if more than 40 pax are scheduled to be carried

(except for unit moves/contingencies)

-Only one loadmaster is required for tactical missions if

a).  Using only 1 static line for personnel or door bundle less than 100 lbs

b).  Dropping SATB’s only

c).  No drop or extraction is planned and ground time is sufficient to

permit onload and offload by 1 loadmaster

-2 loadmasters will be used on high altitude equipment drops

-1 loadmaster may be used on personnel drops up to 13,000’ MSL

provided only one exit is used (except ramp and door static line drops)

-During the aircrew briefing, the aircraft commander must advise the

loadmaster when the mission profile requires flight below 800 ft. AGL

with the doors open.  Loadmasters must wear a restraint harness when

performing duties near an open exit below 800 ft. AGL or above

14,000 ft. MSL.

Oxygen Equipment:  Equipment must be checked within 30 days before each flight

requiring its use.  As a minimum, it will be inspected every 120 days.

Quick-don oxygen masks are not approved for prebreathing or for use

during High Altitude operations (at or above 18,000 ft.)

The aircraft liquid oxygen converter will be full for the first HALO sortie of

the day and filled as needed for subsequent sorties.

Thunderstorm Avoidance:  FL 230 and above – 20 NM

Below FL 230 – 10 NM

Tactical Low Level Operations – 5 NM (outside air temp

must be above 0 degrees C at flight altitude)

Do not fly directly above thunderstorms (w/i 2,000 ft.)

A favorable condition for lightening strikes exists when

flying within 5,000 ft. of the freezing level.

Fuel Requirements:  Minimum Overhead – 7,000 lbs.

Minimum planned landing – 4,000 lbs.

Fuel distribution for primary fuel management above

28,000 lbs.

#1 and #4 fuel tanks – 7,500 lbs. each

#2 and #3 fuel tanks – 6,900 lbs. each

Additional fuel will be put into the auxiliary tanks

NOTE: Primary fuel management is defined as “main

tanks full when the external and/or auxiliary

tanks contain usable fuel.”

Altitude Requirements:

Note:  Aeronautical charts do not depict changes in terrain less than the chart

contour interval, or man-made obstacle less than 200’ AGL.  Additionally,

the highest spot elevation on any given leg may not be the highest terrain,

as in the case of gradually rising elevations.

Day VMC – 300 ft. modified contour

Night VMC – 500 ft. above the highest man-made obstacle or terrain feature and

spot elevation, or 400 ft. plus one basic contour interval above the highest

depicted basic contour, whichever is highest, within 5 NM’s either side

of route centerline to include the aircraft turning radius.

IMC – 1,000 ft. (2,000 ft. in designated mountainous terrain) above the highest

man-made obstacle, spot elevation or contour interval within 5 NM’s of

route centerline (10 NM’s outside the CONUS).

MSA – Same as Night VMC (500 ft. or 400 ft. + 1 contour interval)

NOTE: MSA is an initial VFR altitude that provides terrain clearance while

the crew analyzes situations that require interruption of low level ops.

ESA – 1,000 ft.. (2,000 ft. in mountainous terrain) within 22 NM of route of flight.

Can be rounded up to next highest 100 ft.

NOTE:  Climb to ESA in the case of inadvertent weather penetration.

Climb to MSA when either pilot must leave the seat during low level flight,

in the case of an aircraft malfunction which will distract the crew, and in

the case of aircrew disorientation.

IFR Drop Altitude – 500 ft. above the highest man-made obstacle or terrain feature

and spot elevation, or 400 ft. + 1 contour interval above the highest

depicted basic contour, whichever is highest, within 3 NM’s either side of

run-in centerline from DZ entry point to DZ exit point.

Night VMC Drop Altitude- After slowdown when the DZ is in sight and will remain

in sight or when a positive position is identified and adequate terrain

Clearance is assured; the aircraft may descend to drop altitude specified

in MCR 55-40, AMT, SPINS, or mission planning sheet.

Weather Requirements:

Unilateral Airdrops: 300-ft. ceiling and ½ mile ground visibility.  AF DZCO will

cancel drops (except SATB) with weather lower than this.

Joint Airdrops: Discretion of the user.

NOTE: Army training airdrop mins are usually 200 ft. AGL but can be

waived.

Contingency/Combat: Weather minimums are at the discretion of the theater

or task force commander.

NOTE:  When the ceiling is less than 600 ft. AGL, all personnel including

the combat controllers, will be cleared from the DZ NLT five minutes prior

to the airdrop TOT and will remain clear until drop completion.

Planning Restrictions:	Prohibited areas – avoid by 3 NM

Restricted area – do not fly through without permission

Sensitive area – Avoid by 1 NM (3 NM below 2,000 ft.

AGL and in excess of 250 KIAS)

Nuclear Power Plants - Avoid by 3 NM

Towns – do not fly over magenta shaded areas on TPC’s

Congested Areas – do not fly over towns, settlements

Etc., if your altitude does not ensure at least 1,000

ft. above the highest obstacle within a 2,000’ radius.

Non-Congested Areas – do not fly less than 500 ft. from

any person, vehicle, vessel, or structure.

Emergency Airfields: Optimum emergency airfields are located within 50 NM of

intended route approximately every 100 NM’s

Stabilization Point: Personnel – Be at or above drop altitude and stable not later

than 1 minute (2 minutes for jumpmaster directed jumps).

Equipment: - Be on drop altitude and drop airspeed by green light.

No Drop Signal: In VMC, a no-drop, or mission cancellation is communicated by

the absence of prebriefed markings, red smoke, red flares, or any other

Pre-coordinated signal.  The temporary postponement of an airdrop may

be indicated by panels formed into 2 parallel bars placed perpendicular to

the drop zone axis.

No Drop Decision: Before the “ONE MINUTE” advisory, any crewmember

observing a condition that could jeopardize a safe drop will notify the

aircraft commander.  After the “ONE MINUTE” advisory, any crew member

observing a condition that could jeopardize a safe drop will transmit a “NO

DROP” call on interphone.  A no-drop will be called if the slowdown

checklist is not completed prior to the “FIVE SECOND” call.

The Loadmaster and Copilot (or pilot not flying) will acknowledge all

“NO DROP” calls.

Methods of Aerial Delivery:

Ground Marked Release System (GMRS): The supported unit is responsible for

computing a release point and providing ground markings (panels or

lights).  The airdrop is made over this marker or abeam a flanker marker

(inverted “L”).

Verbally Initiated Release System (VIRS): Ground personnel compute a release

point, provide verbal steering guidance to the aircraft, and give the

release call.  Qualified CCT and TALO personnel only perform VIRS.

NOTE: Use half-standard rate turns, STANDBY is given approximately

5 sec. out, and call green light on the first EXECUTE command.

Jumpmaster Directed Airdrops: Limited to single ship airdrops.  One minute prior

to the navigator’s release point, copilot will turn on the green light to

indicate clearance for the jumpmaster to determine the exact exit point.

After slowdown checks are completed, the loadmaster permits the

jumpmaster access to the door to begin “spotting procedures”.

Ground Radar Aerial Delivery System (GRADS): It is an IMC airdrop method using

fixed radar facilities to provide navigational guidance to position the

aircraft over the navigator’s CARP.  Not recommended during periods of

heavy rain.

Dyess LATN Area: Airspace within 100 NM of ABI VORTAC from 300’ to 1500’ AGL.

Marrion No-Drop Conditions:

- HE airdrop surface winds above 13 kts (including gusts) and the direction is between 220 and 280 degrees

- Any CARP, other than personnel, plots off the DZ

- A static line personnel CARP plots east of the Dyess main runway

- An aircraft is on the compass rose

- An aircraft is on either LZ

- Contact has not been made with the DZCO

##### FORMATION PROCEDURES

Taxi and Takeoff:	Formation Taxi -	4 engines: 1 aircraft length

2 engines: 2 aircraft lengths

Minimum Takeoff Interval: 15 seconds (30 sec vertical and

horizontal (IFR)

Formation Takeoff: Formation Leaders will brief takeoff torque

when different series C-130 aircraft are in the same

formation

Reduced power formation takeoff (Dyess): 970o TIT

Reduced power formation takeoff (LRAFB): 932o TIT

Minimum Weather for formation takeoff: Lowest compatible approach mins for the

airport, but no lower than 200 ft. and 1 mile visibility (RVR 50 at both

ends of the runway).  If weather is lower than the approach mins but

higher than 200-1, the formation can takeoff using a departure

alternate.  Adhere to both ceiling and visibility minimums.

Inadvertent Weather Penetration:

Without SKE:	1. Lead directs inadvertent wx penetration without SKE calling

out base heading and altitude.

2. Initiate 1000 ft/min climb to or above ESA at cruise airspeed.

3. Once climb is initiated, #2 and #3 turn 45o right or left

respectively, from the base heading for 2 min before resuming

base heading.

4. The last element will occupy the base altitude and all other

elements will stack 500 ft. higher.

5. Do not change heading while IMC.

6. The formation leader will contact ATC for individual clearances

With SKE:	1. Lead directs inadvertent wx penetration with SKE calling out

base heading and altitude.

2. Element wingmen will initiate a 1000 ft/min climb to base

altitude, select SKE, and set cross-track to 1000 left or right

as appropriate.

3. When level at base altitude, the leader commands his section

to assume IFR interval.

4. Wingmen reduce airspeed 15 kts and drift back into position

at 4000 ft and 8000 ft respectively.

5. Wingmen reset cross track and fly appropriate SKE position.

NOTE:  	If you inadvertently enter the weather without SKE, the formation must be

directed to climb to or above Emergency Safe Altitude (ESA).

Non-Standard Formation: A formation in which the wingman exceeds 100 ft.

vertical spacing or 1 NM (~6000 ft) horizontal spacing from the leader.

Visual Procedures:

Orbit:	At a prebriefed point enroute to the orbit area, aircraft will descend to

1,000 ft. above the briefed orbit altitude.  Aircraft must be VFR at or before

the orbit point.  Aircraft will slow to and maintain 180 kts at the orbit

point.  When beginning the first turn outbound in the orbit, aircraft will

descend to and maintain 500 ft. above the orbit until preceding aircraft

are in sight or until beginning the turn inbound; whichever is later.

Acceleration:  Lead sets 900o TIT at briefed time or reference point.

Airspeed Changes: At, night, lead will announce unplanned airspeed changes of

15 knots or more.

CARP:	Each element flies an independent approach to the DZ until the end of the

usable DZ time (red light).

NOTE – In a crosswind greater than 3o drift, wingmen adjust their

formation position to maintain the same relative ground track as lead.

Drift effect will approximately double after slowdown.

Slowdown:  All night slowdowns will include an aural or visual signal.  Lead sets

1,000 in-lbs. of torque and configures.

VIS Run-ins: Element leaders fly independent run-ins from slowdown to the DZ.

Wingmen maintain position but drop on individual CARP

Escape:  At the end of usable DZ, lead turns and accelerates to 140 kts

Recoveries:  Tactical:

St-in:  	3 NM is required to slow from 200 KIAS to threshold speed.  From

250 KIAS, 4.5 to 5 NM’s are required.

Teardrop:  Start slowing about 1 NM from the approach end and offset

to the other side.

Beam:	Cross field at 220 KIAS and initiate base turn at ~150 KIAS.

Random Steep:  Extend gear and flaps 100%, then commence descent

while maneuvering to intercept final.  Maintain 140 KIAS (or

approach speed, if higher) until wings level on final.

Downwind:  Entry – 200 KIAS and 1,000 ft. above field elevation

Wingmen stack ~50 ft.

Break approximately ½ NM past the approach end.

Wingmen break approximately 10-12 seconds apart.

Roll out on final no less than 300 ft. and ¼ mile from runway.

Overhead:  Entry – 200 KIAS and 1,500 ft. above field elevation.

Wingmen stack approximately 50 ft.

Downwind leg is flown at 150 KIAS.

Break approximately ½ mile past the approach end.

Turn to final 12-14 seconds apart (for wingmen).

Roll out on final no less than 300 ft. and ¼ mile from runway.

Landing Interval:	Minimum – 12 seconds

Desired – 20 seconds

Geometry’s:

Fluid Trail:  This tactic allows wingmen to optimize terrain masking, provides more

freedom for evasive maneuvers, and breaks up the symmetrical visual

pattern of the in-trail formation.  Wingmen may maneuver up to 90o on

either side of the in-trail position.  Element size is limited to no more than

3 aircraft.

Modified V:  This tactic shortens the overall length of a formation to improve fighter

escort coverage, while retaining the advantages of a dispersed flexible

formation.  Normally flown as 3 two-ship elements.  Aircraft within each

element maintains 2,000 ft and 4,000 ft intervals.  #2 element lead flies

approximately 9,000 ft.  #3 element lead flies approximately 18,000 ft.

Element wingmen maintain fixed positions on each element lead, but each

follower element is free to maneuver on an arc behind the lead element.

Line Abreast and Box:  This formation is useful over areas that provide minimal

terrain masking.  It is also useful over a large valley or LOC.  The wingmen

(or element lead) can fly abeam lead with 4,000 ft to 24,000 ft lateral

spacing or drop back into a box formation (In-trail spacing for box is

2,000 ft to 24,000 ft.).

In- Trail:  Designed primarily for mass airborne assault.  Masses large number

of aircraft over the target in minimal time.  Element wingmen are 2,000 ft.

apart while element leads are 12,000 ft. apart.

Inverted Vic:  Exploits a limited number of rear vision bubbles to enhance visual

lookout for the formation.  The first two aircraft fly line abreast while the

third aircraft flies in-trail (approximately 6,000 ft.).

SKE OPERATIONS:

-Zone marker should be placed no more than 1,500 meters away from PI.

-Master placement:  ZM drops – lead aircraft	  Non-ZM drops – middle

-SCNS/SKE/ZM system is not operational for drop when the SKE INVALID

message is not present, and dynamic deltas are displayed for at least

45 seconds with “S” selected as the drop reference.

-If a valid mix is not received by one minute prior to drop (frozen deltas

and/or a SKE INVALID meaasge is displayed), a no drop will be called.

If VFR conditions exist, a visual drop may be accomplished.  Wingmen

may drop using SKE timing procedures if not receiving a valid zone

marker mix.

SKE Procedures:

Miscellaneous:	Max of 36 aircraft, 34 w/zone marker (to 31 due to 10 mile limit).

Max range of SKE is 10 NM.

Max range of Track-While-Scan (TWS) is 4 NM.

Priority of SKE signals – Altitude, Heading, Airspeed

If multiple formations are within 80 NM, use slot enable

Zone marker uses both slots 01 & 02 for synchronization and to

extend its range from the master to 20 NM. (do not enable 02)

Turn data is lost if “E” is not given within 1 min of prep command

Unusable Slots – 10, 20, 30

5 SKE circuit breaker – 3 Main AC, 1 Main DC, 1 Battery Bus

SKE clock requires a 15-min warm-up

SKE transmitter requires a 2-minute warm-up

ADI Bank Steering Bar Sensitivity:

+1,000 ft full scale when in-track is: 1,000 – 4,000 ft.

+2,000 ft full scale when in-track is: 5,000 – 9,000 ft

+4,000 ft full scale when in-track is: 10,000 – 24,000 ft

Glideslope Indicator Sensitivity:

+1,000 ft full scale, except +5,000 ft with zero offset

when “50” is entered into the ALTITUDE function

in the SKE Secondary Control Panel.

Range Indicator: +1,000 ft/-750 ft. full scale long

SKE Parameter and FCI Checks: Must be accomplished prior to takeoff.

Departure and Assembly: Lead climbs at 180 kts and 1,500 ft/min.  Wingmen climb

at 190 kts.  Upon reaching assembly altitude, wingmen may accelerate

up to 210 KIAS.

Climb Schedule:	100 fpm below 10,000 ft	Maximum  <10,000 ft  1500 fpm/180

(technique)	750 fpm below 15,000 ft		   <15,000 ft  1200 fpm/170

500 fpm above15, 000 ft		    >15,000 ft  1000 fpm/160

Spacing:  #2 & #3 are 4,000 ft and 8,000 ft respectively behind lead.  Offset

distance is 500 ft left or right respectively.  Element leads maintain

12,000 ft. with “00” cross-track.

Overrun:  Element Lead – Set 1,300 ft cross-track and turn in the safest direction

away from lead.  If a cross-track is already selected, set an additional

1,300 ft.  Announce overrun giving element number, bas heading, and

airspeed.

Wing Position: Start a turn toward the direction of the set cross-track, set an

additional 300 ft. cross-track and monitor position.  Announce overrun

and formation position.

NOTE:  To perform a drop after an overrun, the aircraft must be back in

position prior to the 1-minute warning.

Lead Change:  The aborting leader turns 45o away from the bas heading in the

safest direction establishing a position approximately 1 NM from the

formation.  If VMC, you can rejoin to any pre-coordinated position.  If IMC,

you must rejoin at the end of the formation.  The new leader will perform

a FCI check.

IMC Rejoins:  	1. Set leader # as required to join at the end of the formation and

enable all formation slot numbers.

2. Set range rings to maximum range (16).

3. Approach formation from 1,000ft above or below the formation.	4. Establish radio contact with the formation and confirm SKE

info.

5. When the formation appears on the scope, check that the

master lost indications have extinguished

6. The rejoining aircraft will identify the appropriate element

leader.  Upon positive identification, lead will complete an FCI

check prior to the rejoin.

7. Join on position while maintaining 1,000-ft. altitude separation

When stabilized in position and the last formation aircraft is

positively identified, request rejoin clearance from lead and

climb or descend to formation altitude.

8. The rejoining aircraft must be stabilized in position at

formation altitude by the IP (IMC) or by 1 minute prior to TOT

(VMC) or abort the rejoin.

Airspeed Changes: Unprogrammed airspeed changes will be in increments of 10 kt

unless otherwise announced by ESKE or radio.

Deceleration – use 600o TIT

Acceleration – Use 900o TIT

Heading Changes: Unprogrammed heading changes of more than 10o will be

Signaled using the FCI or a radio call.

Wind drift applied to a planned turn is not an unplanned heading change.

Plan enroute turns to be less than 90o.

CARP: All aircraft must use the same drift to determine airdrop crosstrack.

Element leaders maintain position with reference to the TWS until

attaining drop altitude at which time they fly an independent approach

to the CARP and through DZ escape.

Bank: Element leaders are limited to 20o of bank until slowdown.  Then, they are

limited to 10o of bank.

Echelon Turns:  Echelon turns may be performed after the IP and prior to DZ

escape.  After slowdown, element leads are limited to 10o bank maximum.

Sensitive Steering:  Sensitive steering will automatically be engaged when track

angle error is less than 15o, crosstrack error is less than 1.5 NM, and

the aircraft has passed the SCNS-generated pseudo-waypoint called

the turn point (TP).

Slowdown:  Do not initiate descent from last enroute altitude until:

- Lead’s position is positively known as determined by on-board radar, radar beacon, zone marker, ground based radar, radial/DME fix, or visual reference

- The entire formation is within 3 NM of planned DZ run-in centerline course

- The last aircraft in the formation is at or past the DZ entry Point

NOTE:  Each element stacks 50 ft above the preceding element.

Slowdown Signal:  An FCI “SD, -, E” and a simultaneous radio call (at night or in

IMC if the tactical situation allows).

Min IFR Drop Altitude:  500 ft. above the highest man-made obstacle or terrain

feature and spot elevation, or 400’ plus one basic contour interval above

the highest depicted basic contour; whichever is highest, within 3 NM

either side of the run-in centerline from DZ entry point to DZ exit point.

IMC Drop Clearance:  Zone marker, radar beacon, radio call, or other coordinated

signal will be used to confirm drop clearance a minimum of 2 minutes

prior to release.

Uncontrolled Airspace:  If a SKE drop is to be made in uncontrolled airspace,

provide a NOTAM to the FAA flight Service station nearest the objective

area at least 48 hours in advance, regardless of weather.

DZ Entry Point:  A fixed point on the DZ run-in course where the last aircraft in the

formation can begin descent from IFR enroute altitude to IFR drop altitude

Approaches:

Procedure Turn:  Entry will be within 70o of the published inbound course on the

non-maneuvering side, within 20o on the maneuvering side or conveniently aligned for a teardrop, and a minimum of 1,000 ft. above the procedure turn altitude.  The procedure turn is limited to formations of 9 aircraft or less.

Do not enter from the quadrant requiring a turn to the non-maneuvering

side.

Reduce airspeed to 170 KIAS if holding is required.  When holding is

not required and within 3 minutes of the IAF, or when in the holding

pattern and cleared to approach, reduce airspeed to 150 KIAS and

configure for landing.

In-bound turn timing:	#2 – 18 sec (if at 4,000 ft.)

#3 – 36 sec (if at 8,000 ft.)

Element Lead – 54 sec

Adjustments:	Add 2 seconds for each 1,000-ft. long

Subtract 2 seconds for each 1,000-ft. close

When over the FAF, lead signals an airspeed reduction to 140 KIAS or

final approach speed, whichever is higher.  (Radio call only if this speed

is other than 140 KIAS and unable to transmit this speed using the FCI).

ICAO Approaches:  These approaches will be flown single-ship only.

Straight-In:  Upon receiving the call (establish approach separation now), all

follower aircraft reduce airspeed by 30 kts (to no lower than 150KIAS),

reset SKE in-track distance to 6,000 ft., and establish new separation.

Airspeed inbound to the FAF will be 150 KIAS.  Each aircraft will fly its

own approach when established on final.

Landing Interval on Final:		Desired – 6,000 Feet

Minimum – 5,000 Feet

Missed Approach:  If the airfield is reported below minimums after the approach

is started, the formation executes a missed approach, maintains 150 kts

and approach separation.

High Altitude Terminal Recovery:  This procedure is designed to allow formation

holding in IMC and use single ship penetration with a minimum of 3 min

separation between aircraft.  Lead will slow the formation to 170 KIAS

prior to arriving over the fix.  If cleared for an immediate approach (no

holding), the formation will approach the letdown fix on a track within 54o

of the reciprocal to the outbound penetration course at the assigned base

altitude.

Over the fix, the lead aircraft in the first element of the first section

immediately turns to the outbound penetration approach course, descends

1,000 ft, and executes the published letdown procedure.

DZ Entry Point Calculation:

Min DZ Entry Point = 	Stabilization Point (Usually 6 NM)

+Slowdown Distance (Fig 1-5, ch 1/Vol 2 55-130)

+Deceleration Adj. from 210 kts (.25 NM for each 10 kts)

+Formation Length (in NM Note: 4,000 ft per aircraft)

-Deceleration Factor (from a/s to 140 kts/ Fig 1-5 note)

Down Prep Distance (for lead):  Take DZ entry and subtract formation length.

Stabilization Point:  A point on the DZ run-in course at which the lead aircraft

Should plan to be stabilized on drop airspeed and drop altitude.  This

point will be at least 6 NM prior to the point of impact and is a planning

tool only.

SKE Drop:  Do not attempt AWADS/SKE airdrops in areas of thunderstorm

activity, heavy precipitation, or during icing conditions.

-During the drop, AWADS or ZM equipped elements wingmen will maintain

formation position via TWS and use AWADS/ZM timing (primary) or

SKE timing (secondary) for release.

DZ Exit Point:  A fixed point on the DZ escape flight path centerline, established

during planning, at which each aircraft will be at minimum IFR enroute

altitude.  Plan this point for a minimum of 4 NM track distance from the

trailing edge of the DZ.

DZ Exit Point Calculation:

Min DZ Exit point = DZ Escape Point (@ 140 kt and 1 min escape = 2,5

NM)

+Climb Distance to Min IFR Alt (2.5 NM per 1,000 ft alt)

NOTE:  The distance between the DZ entry and DZ exit point must not

exceed 25 NM.

DZ Escape:  Escape from the DZ commences 1 min after each element lead’s red

light time.  At escape, each element establishes 140 KIAS and 1,000

ft/min climb (if climb is required) and turns to DZ departure heading

(on SKE timing for wingmen).  Heading changes will be as small as

possible, not normally to exceed 90o.

Salvo:  AWADS or zone marker capable aircraft will depart the formation for the

salvo area.  If the salvo aircraft is formation or element lead, direct a

SKE lead change.  SKE wingmen in VMC, or IMC SCNS equipped and

SCNS position is verified, should be directed to exit the formation for the

salvo area.  Otherwise, in IMC flight conditions, the formation lead will

proceed to the salvo area with the entire formation.

Enroute Descent (technique):  Triple the altitude you need to lose and add 20 NM

to give you a comfortable descent gradient.

Holding:  If more than 3 aircraft must hold use 2-minute patterns for low altitude

and 3-minutes for high altitude patterns.

#2 and #3 will track outbound on the reciprocal to the outbound

penetration approach course +20o for 1 ½ and 3 minutes respectively.

After expiration of time, each aircraft will turn inbound to the fix and

descend 1,000 ft.

Second and third elements will enter a 3-minute holding pattern.  After

one 3-minute pattern, the second element lead will start the approach

while #2 and #3 turn outbound.  #2 will turn inbound after 1-½ minutes

and #3 will turn inbound after 3 minutes.  Upon turning inbound, each

aircraft will descend 1,000 ft.  The third element performs the same

procedure on the second holding circuit.

During the penetration, aircraft will maintain 3,500-fpm rate of descent

until reaching 230 kts.  Maintain 230 kts until 1,000 ft above level off

altitude.

Vertical IFR Formation:	Spacing:		Longitudinal spacing – 1.5 NM

Element Lead spacing – 4.5 NM

Vertical separation – 500 ft.

Takeoff Interval:	30 sec between aircraft in a section

4 min. between section leads

30 min. between serial leads

Takeoff:  Aircraft maintain runway heading for 2-min

after break release, then turn to departure

heading.  Lead is limited to 900o TIT and

1,000 fpm.

Aborting:  Aircraft aborting the formation will break out

in the safest direction 45o from base heading.

After 1 ½ minutes (wings level), return to the

base heading and comply with separate IFR

clearance.

Altitudes:  During intermediate level offs, section leads

will level off at the highest altitude in the

assigned block and each proceeding aircraft

will level off 500 ft. lower.  Last aircraft in the

section will report level at his altitude.

At cruise altitude, section leads level; off at the

lowest altitude in the assigned block.  Each

preceding aircraft will level off 500 ft. higher.

Last aircraft in the section or serial calls

entering the block and level at assigned

altitude.

During descents, all section aircraft descend at

cruise airspeed and 1,000 fpm.  Lead aircraft

levels off at base altitude and each aircraft

levels off 500 ft. higher.

Weather Mins:	(2,500 – 5)  – 18 or less aircraft

(5,000 – 5) – more than 18 aircraft