| ## TACTICAL AIRLIFT DROPZONE CRITERIA |  |  |  |  |
| --- | --- | --- | --- | --- |
| PERSONNEL |  |  |  |  |
| ALTITUDE (AGL) | WIDTH (Note 1) | LENGTH (Notes 2 & 3) 1 Jumper Additional Jumpers |  |  |
| Up To 1,000 | 600 yd | 600 yd | Add 75 yd for each additional jumper to trailing edge (100 yd when using CAPES) |  |
| Above 1,000 | Add 30 to width and length for each 100 above 1,000 (add 15 to each side of the DZ) |  |  |  |
| # CDS |  |  |  |  |
| ALTITUDE (AGL) | WIDTH (Note 1) | # OF CONTAINERS Single Double |  | LENGTH (Note 2) |
| Up To 600 | 400 yd | 1 2 3 4 5 - 8 | 1 - 2 3 - 4 5 - 6 7 - 8 9 or more | 400 yd 450 yd 500 yd 550 yd 700 yd |
| Above 600 | Add 40 yd to DZ width and length for each 100 above 600 (20 yd added to each side of the DZ) |  |  |  |
| # HEAVY EQUIPMENT |  |  |  |  |
| ALTITUDE (AGL) | WIDTH (Note 1) | LENGTH (Notes 2) 1 Platform Additional Platforms |  |  |
| Up to 1,100 | 600 yd | 1,000 yd | Add 400 yd to trailing edge for each additional platform |  |
| Above 1,100 | Add 30 to width and length for each 100 above 1,100 (add 15 to each side of the DZ) |  |  |  |
| Note 1: a. For day vis formation, increase width by 100 yd (50 yd to each side) b. For SKE formation, increase width by 400 yd (200 yd each side) c. Official sunset to sunrise, increase width by 100 yd for SS form (50 yd each side) or 200 yd (100 yd each side) for vis formations Note 2: Official sunset to sunrise, increase length 100 yd for vis drops (50 yd each side) Note 3: For CCT/Pararescue unilateral ops, see 55-60 |  |  |  |  |

| ### Forward Throw |  |
| --- | --- |
| SATB | Approx. 160 yds |
| Personnel | Approx. 250 yds |
| Door Bundle | Approx. 250 yds |
| Heavy Equipment | Approx. 500 yds |
| CDS | Approx. 550 yds |

| ## DROP ALTITUDES |  |  |  |
| --- | --- | --- | --- |
| HEAVY EQUIPMENT |  |  |  |
| TYPE OF CHUTE | CLUSTER SIZE | ABSOLUTE MIN | OPS MIN |
| G-12D | N/A | 575 | 650 |
| G-12E | N/A | 475 | 550 |
| G-11A | Single Canopy 2 - 7 Canopies 8 Canopies |  | 900 1,100 1,300 |
| G-11B | Single Canopy 2 - 4 Canopies 5 - 7 Canopies 8 Canopies |  | 700 750 1,100 1,300 |
| G-11C | 1 - 2 Canopies 3 - 4 Canopies 5 Canopies 6 - 7 Canopies 8 Canopies | 975 1,025 1,075 1,125 1,225 | 1,050 1,100 1,150 1,200 1,300 |
| SATB-H |  | See type of Chute Being Simulated |  |
| Door Bundle (G-13/14, T-10) |  | 225 | 300 |
| Note: Min drop alt for HE (5,000 lb parachute release) is Greater of 1,000, or type |  |  |  |
| CDS |  |  |  |
| G-12D | 1 Canopy, 1 - 6 Bundles 7 or More Bundles 2 Canopies | 400 500 450 | 475 575 525 |
| G-12E | 1 Canopy,1 - 40 Bundles 130 KIAS 140 - 150 KIAS | 350 300 | 425 375 |
| G-13 | 1 - 2 Canopies 3 Canopies |  | 400 500 |
| G-14 | 1 - 2 Canopies 3 Canopies |  | 300 400 |
| 12/22/26 Hi-Velocity |  | 100+vert dist for the load being dropped |  |
| SATB-C |  | See type of Chute Being Simulated |  |
| PERSONNEL |  |  |  |
| Combat Operations |  | Determined by Airborne & Airlift CC |  |
| Tactical Training |  |  | 800 |
| Basic Airborne Trainees |  |  | 1,250 |
| HALO (min opening) |  |  | 2,500 |
| SATB-P |  |  | 500 |

| TYPE DROP | Distance from Appch End (Yards / Meters) |  |
| --- | --- | --- |
| FOR SINGLE/MULTI AIRCRAFT (See NOTE 1 & 2) |  |  |
|  | DAY | NIGHT |
| CDS | 200/183 | 250/226 |
| PERSONNEL | 300/274 | 350/320 |
| EQUIPMENT | 500/457 | 550/503 |
| NOTE 1: PI location may be adjusted for special operations, or to meet specific mission requirements. Participants must be briefed NOTE 2: PI location may be adjusted for aircrew PI acquisition training. The PI may be located anywhere within the surveyed DZ boundaries as long as the minimum required DZ size for that type airdrop fits within the boundaries. All participants must be briefed when using this option. NOTE 3: For HV CDS and HAARS position the PI in the center of the DZ. |  |  |

| Surface Wind Limits for CDS / Heavy Equipment Airdrops |  |
| --- | --- |
| AF Equipment | 17 |
| AF CDS using G-12 parachutes | 13 |
| AF CDS using G-13/14 parachutes | 20 |
| HAARS or HV CDS | No Restriction |
| AF Training Bundles (SATB) | 25 |
| Non-AF Equipment | Discretion of unit DZSO |
| Surface Wind Limits for USAF Personnel Airdrops |  |
| AF Static Line (Land) | 13 |
| AF Static Line (Water) | 25 |
| AF MFF (Land) | 17 |
| AF MFF (Water) | 25 |
| AF Intentional Tree Jumps | 22 |
| NOTE: Non-AF Personnel IAW airborne unit DZSO |  |

Formation Airspeeds

|  | Below 10,000' | 10,000 - 15,000 | Above 15,000' |
| --- | --- | --- | --- |
| Assembly | 180 KIAS | 170 KIAS | 160 KIAS |
| Climb | 180, 1500 FPM | 170, 1200 FPM | 160, 1000 FPM |
| Cruise | Normally 210 | Normally 210 | Normally 190 |
| Descending SD | 140, 1,000 FPM | As Briefed | As Briefed |
| En Route Alt Change | En route Airspeed or As Briefed / 1,000 FPM or as Briefed |  |  |
| DZ Escape | 140, 1000 FPM | As Briefed | As Briefed |

Inadvertent Weather Penetration With SKE

- Immediately upon penetrating the weather, on the command of lead, the formation climbs to a base altitude at or above the emergency safe altitude (ESA) for the route.

- Lead will direct his flight to execute weather penetration with SKE, giving base heading and base altitude as a minimum.

- Element wingmen initiate climb, select SKE on the attitude directional indicator (ADI) mode select switch, set cross-track to 1,000 feet  left or right as appropriate, and climb to the base altitude on the base heading at cruise airspeed and 1,000 feet per minute while maintaining SKE separation.

- When level at the base altitude, the leader commands their section to assume IFR interval.  At this command, number 2 and 3 aircraft in each element reduce airspeed by 15 knots and drift back until they establish intervals of 4,000 and 8,000 feet, respectively, then reset the appropriate cross-track distance.

- If visual conditions cannot be reestablished, lead contacts the appropriate ATC facility for clearance.

Inadvertent Weather Penetration Without SKE

- Immediately upon penetrating the weather, on the command of the flight lead, the formation climbs to a base altitude at or above the ESA for the route

- Lead will direct their flight to execute weather penetration without SKE, giving base heading and base altitude as a minimum.

- Climb at cruise speed and 1,000 feet per minute.

- When wingmen have a rate of climb of 1,000 feet per minute established, number 2 and 3 wingmen turn 45 degrees right or left, respectively, from the base heading.  Maintain this divergent course for two minutes before resuming base heading.

- The last element will occupy the base altitude.  All other elements will stack 500 feet higher, with the first element occupying the highest altitude.

- Do not change base heading while in instrument meteorological conditions (IMC).

- The formation leader will contact ATC for individual clearances